IEEE P802.22
Wireless RANs

| Initialization Issues for September Interim 2007 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

The Geolocation Tiger Team has been working on doc. IEEE 802.22-06/248 to develop Section 6.15, titled, “Initialization and Network Association.”  Initially the work was focused on updating Section 6.15 with the necessary geolocation edits.  However, during the course of the work, the Geolocation Tiger Team has recognized that Section 6.15 is unacceptably full of errors and has therefore expanded its focus to develop the section in its entirety.

Although some of the issues presented in this document do not pertain to geolocation, all of the issues are essential to both the development of Section 6.15 and to the standard as a whole.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Important Issues Identified by the Geolocation Tiger Team Related to Initialization:

- There must be normative text in the standard describing a compliant installation.  If the compliant installation is not followed, the devices could cause interference.  Create new section.

- Need to define how the available channel list is determined.  Spectrum Manager.

- If during intialization the CPE detects an incumbent in or adjacent to the BS operating channel, should the CPE inform the BS of this presence?  In order for the CPE to inform the BS of the presence of an incumbent signal, the CPE must associate to that BS.  The CPE could disregard the BS in this case and not attempt to associate with that BS.  There are concerns that the CPE would never know if the sensed signal is either not actually a protected incumbent or is an incumbent signal from substantially far away with which the CPE would not interfere.  The spectrum manager should make it possible for the BS, especially on its own during initialization, determine if a signal needs to be protected.  The CPE should not use the channel if an incumbent is detected until the group determines how a BS is able to make the decision that the incumbent is not something that the CPE needs to protect.  Spectrum Manager/CPE automaton flowchart.

- Verify that the keep-out-regions are defined correctly to take into account if a CPE is pointed in the wrong direction (at the NLC, for example).  Recommended Practice.

- The name of the BS needs to be transmitted frequently and this requirement needs to be stated in the standard.  Create new statement.

- The initializing CPE must know on what channel and/or frequency the BS is operating.  There needs to be message space for the BS to report this information to an unassociated CPE.  MAC.

- The BS could occasionally broadcast the set of available channels.  It could broadcast MAC address, set of available channels (or list of channels to ignore when sensing), and a set of coordinates defining range of operation.  Include all the MAC tools or elements to make room for the BS operating channel number, WISP, short description, list of available channels, and list of channels to be disregarded to be broadcast by the BS.  The rate should be every (e.g.) 2 seconds.  This would minimize the number of BSs that would be unnecessarily disregarded; the number of informative bursts between CPE and BS; and the number of channels that would need to be sensed.  MAC.

- The CPE would have to sense specifically for a wireless microphone in all channels – even the list of channels to be ignored.  An analysis needs to be performed to determine how quick the CPEs notification would have to be not to affect the audio from a protected wireless microphone service.  Sensing.

- In order to get RSSI, you need to sense the channel for a period of time.  During this time, you might hit a quiet period.  If this happens, you might miss the existence of the WRAN in the channel.  Spectrum Manager/CPE automaton flowchart.

- Text in doc. IEEE 802.22-07/275r4 pertaining to the CPE sending its resulting sensing report to the BS should be inserted into the appropriate subsection in Section 6.15.  Geolocation/Initialization.

- The tolerance for the CPE/BS ranging timing offset adjustment must be determined.  Currently the document states that the timing offset adjustment is to be tuned to the center of the cyclic prefix within ¼ of the cyclic prefix duration.  802.16 uses 1/32 of a modulation symbol for a timing offset.  The smallest cyclic prefix for 802.22 is 1/32.  Should 802.22 use 1/64? PHY

- We need to define the ranging process.  The ranging process described in Section 6.15.2.5.1 is incorrect.  The CPE is expected to make timing offset adjustments to seem co-located to the BS but the CPE does not know where it is at this point.  Section 8.10.2 contains text describing the ranging process.  Ivan/ETRI should review the text in both sections in perhaps the PHY group to develop ranging.

References: