IEEE P802.22
Wireless RANs

| Indication of Go-On Response |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |
| Ke Wang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993034 | wincle@huawei.com |

Abstract

In the current draft 1.0, there is no primitive to notify the MAC Sublayer when the PHY layer of the SPD detects the Go-On response during the ANP. Some remedies are suggested to patch this hole.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

In the current draft 1.0 [1], a special acknowledgement message called Go-On is sent during the ANP to allow the SPD to transmit an additional beacon frame without need to issue another RTS burst.. If the PPD receives an SPD's beacon frame having the NST subfield set to one, the PPD can allow the SPD to transmit an additional beacon frame by sending a Go-On during the ANP.

The problem in the current draft is that there is no primitive to notify the MAC Sublayer when the PHY layer of the SPD detects the Go-On response during the ANP.

- Suggested Remedy:

When the PHY layer of the SPD detects the Go-On response during the ANP, the PLME of the SPD should notify its MLME that the Go-On response has been detected.

- (1) Modify the primitive PLME-INITIATE-RTS-BURST.confirm

6.2.2.4 PLME-INITIATE-RTS-BURST.confirm

The PLME-INITIATE-RTS-BURST.confirm primitive is generated by the PLME of a secondary protecting device and issued to its MLME in response to a PLME-INITIATE-RTS-BURST.request primitive or to indicate the Go-On response. This primitive confirms that an RTS burst packet was sent by the PHY entity and indicates whether the protecting device has permission to send a beacon frame. Table 10 specifies the parameters for the PLME-INITIATERTS-BURST.confirm primitive.

- Table 10—PLME-INITIATE-RTS-BURST.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | （ACK, NACK, Go-On） | The result of the request to send a beacon frame. |

- (2) Replace the text in 7.4.5.2, Line 49, Page 59:

If an SPD wants to send more than one beacon frame, the SPD can send its initial beacon frame with the Rank and NST subfields set to zero and one, respectively. Upon receipt of this beacon frame, the PPD can grant permission to the SPD to send a second beacon frame by transmitting a Go-On response during the ANP. This process may decrease the RTS collision probability due to the fact that the second beacon frame is sent without transmitting an additional RTS burst during the receive period. After sending its initial beacon frame successfully (on receipt of the MLME-START-BEACON.confirm with the Status parameter set to SUCCESS), an the SPD shall send the second beacon frame to the MAC sublayer through MLME-START-BEACON.request primitive and wait for up to aNSTValidCount superframes to receive a Go-On response from the PPD. If the Go-On response is received through PLME-INITIATE-RTS-BURST.confirm primitive, the MAC of that SPD shall request the PHY layer to send its beacon frame in the next superframe through PD-DATA.request primitive. If a response is not received during this time, the SPD shall terminate the process, and it shall send a new RTS burst before sending another beacon frame.

- References:

[1] Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D1, May 2007.