IEEE P802.22
Wireless RANs

| Proposed Solution for Pseudo-Randomness of Back-off Timers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U. S. A. | 916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, P. R. China | 86-21-50993036 | zhangjianwei@huawei.com |

Abstract

In the current draft, random back-off timers will be used to avoid collisions. However, these timers do not generate perfectly random back-offs to solve collision problems. In this document, three new back-off schemes are proposed by which it is guranteed that collisions can be avoided in the next step.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. INTRODUCTION

In the current draft of TG1 standard [1], random back-off timers are used to avoid collisions. Some comments including Comments # 72, 78, 189, 317, and 337 for the first round of TG1 letter ballot were suggested to point out problems related to this random back-off issues [2].

Random back-off timers that we are considering in the current draft are not perfectly random. Therefore we can not gurantee that collision problems can be solved for the next step.

In this document, three perfect back-off schemes are being proposed. With one of these schemes, it can be guranteed that collisions do not occur by assigning a different value of back-off for each SPD for the next step.

One of typical applications is the retry mechanism for the case that multiple SPDs send the same RTS codeword in the same superframe. In the next trial, this back-off scheme can be applied for selection of a superframe for the next RTS.

2.  BACK-OFF MECHANISM

Existing SPDs

Scheme 1

Each SPD has its own value of back-off, l, {l | l = a*n, n = 0, 1, 2, 3, ….., k }. Here n is the non-negative integer and a is a constant, for example, 3. When an SPD is associated with the PPD for the first time by transmitting its first beacon frame, this SPD has a value of zero for n. Whenever it receives a beacon frame from another ordinary SPD or the NPD, it increases its value of n by one. If it receives another beacon frame consecutively from the same SPD, the value of n will be kept unchanged. Whenever it transmits a beacon frame, its value of n is reset to zero. By applying this, it is guaranteed that all SPDs have different values for this value of back-off, l. It means that each SPD always has a different value of n from values that other SPDs have.

Scheme 2

Each SPD has its own value of back-off, l, {l | l = a*n, n = 0, 1, 2, 3, ….., k } where k is the maximum number of SPDs in a channel. Here n is the non-negative integer and a is a constant, for example, 3. When an SPD is associated with the PPD for the first time by transmitting its first beacon frame, this SPD has a value of zero for n. Whenever it recognizes any addition in the subchannel map of a beacon frame from the PPD, each SPD updates its value of n by using the following rule:

If there is any addition in the subchannel map, the SPD just occupying this subchannel has a value of 0 for n. Only for this case, each SPD counts all subchannels occupied from the subchannel which has a value of 0 for n to the higher end of channel and again going back to the lower edge of the channel till it meets the subchannel which has zero for n by checking the subchannel map. A value for n can be assigned for the corresponding SPD with the result of this count. By following this procedure each SPD has its own value of n.

By applying this, it is guaranteed that all SPDs have different values for this value of back-off, l. It means that each SPD always has a different value of n from values that other SPDs have.

Scheme 3

Each SPD has a back-off value, l, corresponding to its subchannel number in the subchannel map. If this SPD cover mutilple subchannels, it selects the smallest channel number. Therefore each SPD can have its own back-off value.

New SPDs

For new SPDs which are not associated with the PPD yet, a procedure described in Section 7.4.5 of the current draft [1] can be applied. Upon initialization, a protecting device shall identify the channel it is to monitor, then monitor that channel for a period of 5 + 2*l superframe periods, where l is an integer corresponding to its smallest subchannel number for its protected devices that this SPD is to protect, to determine the presence or absence of a PPD. The judgment is that a PPD is present on the channel if the protecting device receives a beacon frame (7.4.5 Device initialization procedure, page 64). It means a random back-off mechanism is already applied to new SPDs using this procedure.

3.  ADVANTEGES OVER CURRENT BACK-OFF SCHEME

With  new back-off schemes proposed in this document, a few advanateges over courrent scheme are identified as follows:

Nearly perfect randomness achieved

By letting each SPD have a different value of n, it has its own back-off value.

No waiting time for the next action

While the current scheme has almost an average waiting time of 8 superframes if a 16 back-off scheme is applied, the scheme proposed here has no waiting time for the next action because there should have an SPD which has a value of 0 for n.

4.  CONCLUSION

In this document, new back-off schemes are proposed. Using these schemes, we can guarantee that each SPD including the NPD has a different value of back-off. It means it provides a method to avoid collisions.

References

[1] Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands, P802.22.1/D2, Nov. 2007

[2] P802.22.1d1.0_cmts_007, Aug. 2007