IEEE P802.22
Wireless RANs

| Proposed Solution for Pseudo-Randomness of Back-off Timers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U. S. A. | 916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, P. R. China | 86-21-50993036 | zhangjianwei@huawei.com |

Abstract

In the current draft, a random back-off timer will be used to avoid collisions. However, this timer does not generate perfectly random back-offs to solve collision problems. In this document, a new back-off scheme is proposed by which it is guranteed that collisions can be avoided in the next step when multiple SPDs have a collision to transmit their RTS codewords due to their selection of the same RTS codeword.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. INTRODUCTION

In the current draft of TG1 standard [1], a random back-off timer is used to avoid collisions in transmitting their next RTS codewords when multiple SPDs sent the same RTS codeword in a superframe.

Some comments including Comments # 72, 78, 189, 317, and 337 for the first round of TG1 letter ballot were suggested to point out problems related to this random back-off issue [2]. One of the authors of this document submitted a comment related to this case – Comment 413 [3].

The random back-off timer that we are considering in the current draft does not provide perfectly random numbers for SPDs. Therefore we can not gurantee that collision problems can be solved for the next step after a collision.

In this document, a back-off scheme is being proposed for the case explained in the above. With this scheme, it can be guranteed that collisions do not occur by assigning a different value of back-off for each SPD for the next step.

One of typical applications for the scheme is the retry mechanism for the case that multiple SPDs send the same RTS codeword in a superframe as explained in 7.4.3 in the current draft [1]. In the next trial, this back-off scheme can be applied for selection of a superframe for the next RTS request.

2.  BACK-OFF MECHANISM

For Existing SPDs

Each SPD has its own value of back-off, K, {K | K = a*n, n = 0, 1, 2, 3, …..}. Here n is the non-negative integer and a is a constant, for example, 2. When an SPD is associated with the PPD for the first time by transmitting its first beacon frame, this SPD has a value of zero for n. Whenever it receives a beacon frame from another ordinary SPD or the NPD, it increases its value of n by one. Whenever it transmits a beacon frame, its value of n is reset to zero. By applying this, it is guaranteed that all SPDs have different values for this value of back-off, K. It means that each SPD always has a different value of n from values that other SPDs have.

Proposed Text Change

The text of lines 14 and 15, page 64 in the current draft [1] – Following the failed request, “the SPD shall choose a random number K in the range [0, …, 15].” - can be replaced with the above description of the scheme. And “select another random number and” in line 21 and “selects a random number and” in line 22 on Page 64 will be removed.

3.  CONCLUSION

In this document, a new back-off scheme is being proposed. By letting each SPD have a different value of n, it has its own back-off value. Therefore using these schemes, we can guarantee that each SPD including the NPD has a different value of back-off. It means it provides a method to avoid collisions.

References

[1] Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands, P802.22.1/D2, Nov. 2007

[2] P802.22.1d1.0_cmts_007, Aug. 2007

[2] P802.22.1d2.0_cmts_004, Nov. 2007

Appendix

One Example

Value of n

Event			SPD1	SPD2	SPD3	SPD4	SPD5	SPD6	SPD7	SPD8	SPD9	SPD10

SPD1 sends a beacon	0

SPD2 sends a beacon	1	0

SPD3 sends a beacon	2	1	0

SPD4 sends a beacon	3	2	1	0

SPD2 sends a beacon	4	0	2	1

SPD5 sends a beacon	5	1	3	2	0

SPD6 sends a beacon	6	2	4	3	1	0

SPD7 sends a beacon	7	3	5	4	2	1	0

SPD8 sends a beacon	8	4	6	5	3	2	1	0

SPD9 sends a beacon	9	5	7	6	4	3	2	1	0

SPD3 sends a beacon	10	6	0	7	5	4	3	2	1

SPD10 sends a beacon	11	7	1	8	6	5	4	3	2	0

SPD7 sends a beacon	12	8	2	9	7	6	0	4	3	1

SPD1 sends a beacon	0	9	3	10	8	7	1	5	4	2