IEEE P802.22
Wireless RANs

| Spectrum Manager Channel Sets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

A new list of channel sets to be used by the 802.22 WRAN Spectrum Manager and a definition for each of these sets were tentatively developed during the discussions that took place on Thursday evening, 20 August 2007, in Waikoloa.

This contribution tries to describe the state of the common understanding at the end of the discussions in that meeting from the perspective of the author and has NOT been agreed as of the time of its submission.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Spectrum  Manager channel sets

1- Precluded channels

This channel set contains all channels in the VHF/UHF Broadcast bands that are precluded from broadcast incumbents and WRAN operation by regulation or otherwise (e.g., ch. 37 and >52 in the USA).  No WRAN transmission is allowed in these channels and no sensing is needed.

2- Disallowed channels

This channel set contains all channels disallowed for WRAN operation.  These channels will be enforced at the base station as a result of access to the incumbent databases and/or by the operator based on local situation and/or equipment limitations.  This will represent premanent or quasi-permanent situations.  No WRAN transmission will take place in these channels but sensing of incumbent operation may be needed.

7- Unclassified channels (any other channels not sensed yet)

This channel set contains all channels not included in the other sets, typically channels that have not yet been explored by the sensing process because the other sets contain sufficient channels for their successful operation or previously sensed channels for which the valid sensing period has lapsed.  This constitutes the pool of channels that will be accessed in case a new channel is needed.

6- Occupied channel

This channel set contains the channels where incumbent operation (TV or wireless microphones) or WRAN operation was found for which the valid sensing period has still not lapsed.  In order to maintain these channels in this set, sensing needs to be refreshed on a regular basis (< 6 sec).

3- Current operating channel
This channel set contains the channel on which the WRAN operation currently takes place.  In order to maintain this channel in this set, sensing needs to be refreshed on a regular basis (< 2 sec).

4- Backup channels (ordered list)
This channel set contains the channels that have been found available for WRAN operation by the base station (i.e., no incumbent operation is found on any potentially affected channels.  This channel list is broadcast to all CPEs.  This list is maintained in order of priority where the first channel in the list will be the one where the WRAN operation will be transferred if the operating channel becomes unavailable.  The second channel in the list is next, and so on.  No WRAN transmission will take place on these backup channels.  In order to maintain a channel in this set, sensing needs to be refreshed on a regular basis (< 6 sec).  In an incumbent or a WRAN system is found on a backup channel, it is then transferred to the occupied set.

5- Candidate channels

This channel set contains the channels that the base station wants the CPEs to sense and report on.  This channel list is broadcast to all CPEs.  This list is maintained in order of priority where the first channel in the list will be the one sensed first after the operating and the backup channels have been sensed.  The second channel in the list is next, and so on.  This channel set is the feeder for the backup channel set when the base station decides to transfer the first candidate channel that was found to be available for WRAN transmission to its backup list.  No WRAN transmission will take place on these candidate channels.  In order to maintain a channel in this set, sensing needs to be refreshed on a regular basis (< 6 sec).  If an incumbent or a WRAN system is found on a candidate channel, it is then transferred to the occupied set.