IEEE P802.22
Wireless RANs

| Remove IE of FEC and modulation type from DCD and UCD |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wu Yu-chun | Huawei Hisi | Xinxi Rd., Shangdi St, Haidian Dist, Beijing 100085 China | 86-10-8282-9018 | wuyuchun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Liu Jinnan | Huawei Hisi | Xinxi Rd., Shangdi St, Haidian Dist, Beijing 100085 China | 86-10-8288-9148 | liujinnan@huawei.com |

Abstract

Since 22 group have decided to use 6 bits for DIUC and UIUC in September 2007 meeting, we can have a one to one mapping relationship between each DIUC/UIUC value and each FEC code type, FEC rate and modulation type combination, so there is no need to include these information in the DCD/UCD which will be downloaded from the BS to CPE,  thus some wireless resource can be saved.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1.  Introduction

802.22 group has tried a lot to reduce the size of MAC IEs as much as possible to reduce the overhead of WRAN system. In September 2007 meeting, we decided to use 6 bits for DIUC/UIUC, this makes it possible to remove the information of FEC code, FEC rate and modulation type in the DCD/UCD table, thus reduce the system overhead.

2.  Remove FEC and modulation type information from the DCD/UCD table

6 bits of DIUC/UIUC gives 64 possiblilities, but the number of possible combinations of FEC code type, FEC rate and modulation type is only 49 in WRAN system. So we can define a one to one mapping relationship between each DIUC/UIUC value and each FEC code type, FEC rate and modulation type combination, for example, like in following table:

| DIUC/UIUC Index | FEC code type | FEC rate | Modulation type |
| --- | --- | --- | --- |
| 0 | CC | 1/2 | BPSK |
| 1 | CC | 1/2 | QPSK |
| 2 | CC | 1/2 | 16-QAM |
| 3 | CC | 1/2 | 64-QAM |
| 4 | CC | 2/3 | QPSK |
| 5 | CC | 2/3 | 16-QAM |
| 6 | CC | 2/3 | 64-QAM |
| 7 | CC | 3/4 | QPSK |
| 8 | CC | 3/4 | 16-QAM |
| 9 | CC | 3/4 | 64-QAM |
| 10 | CC | 5/6 | QPSK |
| 11 | CC | 5/6 | 16-QAM |
| 12 | CC | 5/6 | 64-QAM |
| 13 | DBTC | 1/2 | QPSK |
| 14 | DBTC | 1/2 | 16-QAM |
| 15 | DBTC | 1/2 | 64-QAM |
| 16 | DBTC | 2/3 | QPSK |
| 17 | DBTC | 2/3 | 16-QAM |
| 18 | DBTC | 2/3 | 64-QAM |
| 19 | DBTC | 3/4 | QPSK |
| 20 | DBTC | 3/4 | 16-QAM |
| 21 | DBTC | 3/4 | 64-QAM |
| 22 | DBTC | 5/6 | QPSK |
| 23 | DBTC | 5/6 | 16-QAM |
| 24 | DBTC | 5/6 | 64-QAM |
| 25 | LDPC | 1/2 | QPSK |
| 26 | LDPC | 1/2 | 16-QAM |
| 27 | LDPC | 1/2 | 64-QAM |
| 28 | LDPC | 2/3 | QPSK |
| 29 | LDPC | 2/3 | 16-QAM |
| 30 | LDPC | 2/3 | 64-QAM |
| 31 | LDPC | 3/4 | QPSK |
| 32 | LDPC | 3/4 | 16-QAM |
| 33 | LDPC | 3/4 | 64-QAM |
| 34 | LDPC | 5/6 | QPSK |
| 35 | LDPC | 5/6 | 16-QAM |
| 36 | LDPC | 5/6 | 64-QAM |
| 37 | STBC | 1/2 | QPSK |
| 38 | STBC | 1/2 | 16-QAM |
| 39 | STBC | 1/2 | 64-QAM |
| 40 | STBC | 2/3 | QPSK |
| 41 | STBC | 2/3 | 16-QAM |
| 42 | STBC | 2/3 | 64-QAM |
| 43 | STBC | 3/4 | QPSK |
| 44 | STBC | 3/4 | 16-QAM |
| 45 | STBC | 3/4 | 64-QAM |
| 46 | STBC | 5/6 | QPSK |
| 47 | STBC | 5/6 | 16-QAM |
| 48 | STBC | 5/6 | 64-QAM |

Thus, we only need to specify such a mapping relationship in the standard. For BS and CPE implementation, they both save such a table in their local memory, then whenever BS/CPE send a DIUC/UIUC burst profile, the CPE/BS can look up local table and knows what FEC code, rate and modulation type it corresponds to. Then we can delete the Element ID 150 from table 47 and table 59 in [1], so some wireless resource can be saved and we have a more reliable FEC and modulation type information than we download it from the BS.

Furthermore, it’s desirable to define one same table for both DS and US, thus we only need one table instead of two for both DS and US and some memory can be saved for implementation.

3.  Further consideration

On the same logic as above, we can even consider further to remove “DIUC mandatory exit threshold” and “DIUC minimum entry threshold”. For a specific wireless channel model such as AWGN, or Channel mode B as specified in [2], we can determine these two values by simulation. However, for different channel model, the corresponding value maybe different,  and moreover, actual links between each CPE and BS may have different channel characteristics and may also change slowly. Thus it seems hard to define fixed values of these two parameters for each FEC and modulation type. So we still need to communicate them between BS and CPE.

- References

- 《IEEE P802.22?/D0.3.8.1 Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands》, September 2007.

- 《22-05-0055-07-0000_WRAN_channel _Modeling》, August 2005.