IEEE P802.22
Wireless RANs

| Annex on TG1 detection for 802.22 draft |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jinxia Cheng | Samsung Electronics | China | +86 10 6439 0088 3112 | jinxia.cheng@samsung.com |
| David Mazzarese | Samsung Electronics | Korea | +82 10 3279 5210 | d.mazzarese@samsung.com |
| Baowei Ji | Samsung Telecom. America | USA | +1-972-761-7167 | bji@sta.samsung.com |
| Shan Cheng | Samsung Electronics | Korea | +82 31 279 7557 | cheng.shan@samsung.com |
| Euntaek Lim | Samsung Electronics | Korea | +82 31 279 5917 | et.lim@samsung.com |

Abstract

This Annex  summarizes the essential features of the TG1 beacon signal and protocols.It also clarifies the different possible implementation options that offer trade-offs in sensing time, sensing complexity, WRAN QoS support, , etc. .The Annex should not be considered a replacement to the normative text in the IEEE802.22.1 standard.]

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Summary of the characteristics of the IEEE802.22.1 beacon signal and protocols

d.

IEEE 802.22.1 standard defines the protocol and data format for communication devices forming a beaconing network offering enhanced protection for low-power, licensed devices operating in television broadcast bands. In beaconing network, three types of protecting devices (PD) are identified:

- Primary Protecting Device (PPD): the PPD is the main device responsible for providing incumbent protection. The IEEE 802.22.1 draft specification requires that it transmit beacon data at least every other superframe. The protection information within the PPD beacon transmissions may or may not include information aggregated as a result of inter-device communication with other PDs.

- Secondary Protecting Device (SPD): an SPD is a PD which has chosen to have another PD provide protection on its behalf. The protection information is shared with the PPD via inter-device communication, and subsequently broadcasted as part of the PPD’s regular beacon transmissions.

- Next-In-Line Protecting Device (NPD): an NPD is an SPD which has been selected by a PPD to become the new PPD in the event that the current PPD ceases beacon transmission.

A beaconing device shall operate using the following parameters:

- Offset from lower TV channel edge: 309.4406kHz

- Chip rate: 76.873kchips/s

- Symbol rate: 9.6091kBaud

- Occupied bandwidth: 77KHz for -3dB and 110KHz for -20dB

## Superframe structure

The standard employs the superframe structure shown in Figure 1. The superframe structure consists of 31 slots. Each slot is comprised of 32 DQPSK symbols, where one symbol has a duration of 1/9609.1 seconds (i.e., 104.1 microseconds). The superframe is comprised of synchronization channel and beacon channel transmitted continuously in parallel. The synchronization channel consists of a succession of synchronization bursts. The beacon channel consists of the PPDU, which contains the MAC beacon frame. Following 30 synchronization bursts and the PPDU, if the PPD is in its initial transmission period, the last slot transmits the synchronization bust of index zero, in which case the PPDU data are just zeros. Otherwise, the last slot is an inter-device communication period (ICP), which is composed of a receive period (Rx) and an acknowledgement/no acknowledgement period (ANP) as well as three transit gaps separating those periods.

(a)

(b)

Figure 1	  Superframe logical format

## Beacon frame structure

The PPDU consists largely of the MAC beacon frame. The MAC beacon frame contains information relevant to the device or devices protected by the protecting device, including the physical location of the beaconing device and the estimated duration of TV channel occupancy. Figure 2 shows the structure of the beacon frame, which originates from within the MAC sublayer of either a PPD or an SPD. The beacon frame contains three MAC sub-frames (MSF). MSF1 contains source address field, location field and three MAC parameter fields. MSF2 contains the channel/subchannel map and signature fields. MSF3 contains the certificate field. The signature and certificate fields are part of the public-key cryptography security solution. Figure 2 gives a schematic view of the beacon frame and the PHY packet (PPDU).

Figure 2  Schematic view of the beacon frame and the PHY packet (PPDU)

## Synchronization burst

Each slot contains one synchronization burst, as well as a fixed number of PPDU bits. The synchronization bursts, each of which consists of a 15-bit synchronization field followed by 7-bit index field that decrements with each burst transmission, an 8-bit parity field for detecting and correcting errors on the index value, and a 2-bit reserved field, as is shown in Figure 3. Each synchronization burst occupies one 32-bit long synchronization channel slot of duration 32 bits/9609.1Hz=3.3301ms.

In the synchronization burst, the sync field is used by the receiver to detect the presence of the synchronization burst and to synchronize to the slot timing. The index field is used to obtain frame synchronization with an incoming beacon. It contains a numerical value equal to the number of slots remaining before the start of the next superframe. The index field shall be decremented by one each time the data contained within a slot is transmitted until the index reaches either zero or one, depending on whether the PPDU will be followed by an ICP period.

If the PPDU will not be followed by an ICP period, the final index shall be zero, and the next superframe shall start immediately. If the PPDU is to be followed by an ICP period, the final index shall be one, and the next superframe shall start after ICP period.

Figure 3  Schematic view of synchronization burst sequence

## Inter-device communication period (ICP)

The inter-device communication period is only included in the superframe if the PPD is not executing the device initialization procedure. The order of symbols within the period is as follows: 5 symbols of turnaround time, 8 symbols for the receive period, 6 symbols of turnaround time, 8 symbols for the ANP and another 5 symbols of turnaround time.

The RTS period is used by an SPD to reserve a superframe to transmit its beacon frame to the PPD. Each RTS burst consist of an RTS codeword field, wherein a RTS codeword shall randomly be selected from the list of available RTS codewords. NPD codeword can also be sent by the NPD in RTS period to inform that the NPD is still active. Both RTS codeword and NPD codeword are cyclically shifted sequences of the 15-bit sync field except the first zero bit in the sequences.

The ANP period is used by the PPD to respond to information received during previous receive periods and, more generally, to communicate with other PDs. The list of possible ACKs in this period is identical of possible RTS codewords. NACK is also a cyclically shifted sequence of the 15-bit sync field except the first zero bit in the sequences.

## PHY specifications

The IEEE P802.22.1/D2 PHY shall employ direct sequence spread spectrum (DSSS) with differential quadrature phase-shift keying (DQPSK). Figure 4 provides a functional block diagram for specifying the PHY modulation and spreading function.

Data bits either belong to the synchronization logical channel, the beacon logical channel, the RTS burst, or the ANP burst are parsed between the physical I channel and the physical Q channel, which are used as input for DQPSK encoding. DQPSK encoding is a phase change applied to the previous DQPSK symbol according to the two raw data bits from the I and Q channels being encoded. After DQPSK encoding, each DQPSK symbol shall be mapped into an 8-chip, complex, pseudo-random noise (PN) sequence. The chip sequence is modulated onto the carrier with square-root-cosine pulse shaping applied separately to the in-phase and quadrature components of the complex modulation chips.

FEC is applied to portions of both the synchronization burst and the beacon frame. A (15,7) block code shall be applied to be the Index field of the synchronization burst. A half-rate, binary convolutional code shall be applied to the first MAC subframe (MSF1) of the beacon frame. Please see IEEE P802.22.1/D3 and the future revisions for details.

Figure 4  Modulation and spreading functions

# Reference receiver architecture at the WRAN

The reference receiver block diagram is shown in Figure 5. Decision points along the receiver path are highlighted with numbered boxes. The information that could be available at each decision point according to specific implementations is described in Table 1.

Two data paths are shown in Figure 5. One data path is used for sensing during a short periodic quiet period, and the other data path is used for sensing during a long scheduled quiet period. Note that a different implementation could be used for the data path used for sensing during a short quiet period, where the detection of the presence of the inter-device communication period, and the analysis of the location of the different data fields in the inter-device communication period, could be done after DQPSK demodulation. The demodulator may need to recognize silent symbols, and replace them by erasures rather than make hard bit decisions.

Figure 5  Reference receiver block diagram

| Decision point | Description | Result |
| --- | --- | --- |
| (A) | Energy detection of the TG1 beacon signal within a 76.8731 kHz provides improved coverage over the direct detection of the protected wireless microphone signal due to the larger power of the beacon. The type of signal detected cannot be deduced from simple energy detection. Measurement of the signal level is an indication of the distance of the TG1 transmitter from the WRAN receiver. | A certain device is occupying the channel. |
| (B) | The detection of the 8-chip DSSS PN sequence offers a 9.0309 dB processing gain over energy detection. This is a feature detection technique, which uniquely identifies an 802.22.1 signal. Reliable detection of the PN sequence is a strong indication of the presence of a TG1 beaconing device. Measurement of the signal level can give an indication of the distance of the TG1 transmitter from the WRAN receiver. | A TG1 beacon is in the channel. Estimated distance of the TG1 transmitter from the receiver. No synchronization information. |
| (C) | Identification of the 15-bit sync sequence offers no improvement in terms of signal level (no processing gain), nor any additional information relative to the authenticity of the TG1 beaconing device, beyond what could be achieved in (B). The sole purpose of the 15-bit sync sequence is to synchronize the bit sequence with the start of the index bit field. | Same as (B) |
|  | At this point, the WRAN receiver may also read the index, but without the error detection and correction capability. Depending on the received signal strength, the receiver may decide that the index was likely received without error and try and schedule a long quiet period to capture the beacon frame. No reduction in short sensing time can be expected with this feature that still needs to capture the 15-bit m-sequence and one index field. | Unconfirmed synchronization information from the index |
| (D) | Successful decoding of the index allows determining the start time of the TG1 frame in the Q channel. The WRAN can use that information to schedule a quiet period synchronized with the start of a future TG1 frame. | Confirmed synchronization information from the index |
| (E) | Successful decoding of the bits of the MSF1 field provides all the information that the WRAN needs to protect the Part 74 device. However, the MSF1 bits alone do not allow the WRAN to authenticate the TG1 beaconing device. The WRAN could be faced with a rogue beacon. | Beacon data available Beacon device not authenticated |
| (F) | After successful authentication of the TG1 beaconing device, the WRAN can have no further doubt on the presence of a legitimate Part 74 incumbent device in its vicinity, and it must leave the channel. | Beacon data available Beacon device authenticated |

Table. 1 - Decision points in the data paths of the receiver

# Sensing and detection requirements at the WRAN

## Sensing thresholds

We need to align this section with the scope of the annexes of other sensing techniques (for DTV, Wireless Mic).

Discuss possibility of the TG1 sensing being affected by WRAN operation on adjacent channel: desensitization? Filter rejection toward adjacent channel?  Must be less toward lower TV channel since beacon is lower in the channel.

6.8.1 – the .22 receiver designer might need to know the tolerance on the TG1 center frequency to design their frequency acquisition.

6.8.4 – knowing the modulation accuracy of the transmitter gives the receiver designers an idea of how good they need to get in their receiver matched filtering and what the overall EVM impact might be (beyond just the impact due to his receiver)

6.8.6 – an idea of what sensitivities to shoot for.

## Sensing times

The basic timing parameters of the TG1 signal, and the minimum detection times (worst cases), are summarized in Table 2. Note that due to the DQPSK modulation, one DQPSK symbol duration has been added to the duration of the signal in order to determine the minimum sensing time when demodulation is required.

| Sensing type | Signal feature | Signal duration | Minimum sensing time | Detection strategy | Related section |
| --- | --- | --- | --- | --- | --- |
| (1) | Energy | Continuous except in ICP slot | Data not available | Energy detection in 77 kHz bandwidth | 3.2.1 |
| (2) | 8-chip PN spreading sequence | 0.1041 ms (0.1041 ms period) | 0.8328 ms within initialization period | Capture 8 consecutive PN sequences * | 3.2.2 |
| (3) | 8-chip PN spreading sequence | 0.1041 ms (periodicity broken every 30 slots) | 2.8107 ms outside initialization period | Capture 8 consecutive PN sequences * | 3.2.3 |
| (4) | Synchronization information: sync sequence and index | 3.3302 ms (3.3302 ms period) | 5.1009 ms within initialization period | Capture one sync sequence, and index + parity bits either before or after the sync sequence | 3.2.4 |
| (5) | Synchronization information: sync sequence and index, and slot type (sync burst or inter-device communication period) | 3.3302 ms | 5.1009 ms outside initialization period | Capture one sync sequence and index + parity bits, otherwise recognize that the signal contains an inter-device communication period | 3.2.5 |
| (6) | Synchronization information: sync sequence and index | 3.3302 ms (3.3302 ms period) | 5.1009 ms twice within 2 seconds asynchronously outside initialization period | Capture one sync sequence, and index + parity bits either before or after the sync sequence | 3.2.6 |
| (7) | Synchronization information: sync sequence and index (no analysis of the type of slot) | 3.3302 ms (periodicity broken every 30 slots) | 8.4311 ms outside initialization period | Capture one sync sequence and index + parity bits either before or after the sync sequence | 3.2.7 |
| (8) | MSF1 (FEC encoded) | 28.3070 ms | 28.4111 ms | Capture MSF1 asynchronously (already synchronized by index) | 3.2.8 |
| (9) | MSF1+MSF2 | 70.7676 ms | 70.8717 ms | Capture MSF1+MSF2 asynchronously (already synchronized by index) | 3.2.9 |
| (10) | MSF1+MSF2+MSF3 | 98.2421 ms | 98.3462 ms | Capture MSF1+MSF2+MSF3 asynchronously (already synchronized by index) | 3.2.10 |
| (11) | TG1 superframe | 103.2374 ms (beacon period) | Irrelevant | Irrelevant | 3.2.11 |

Table. 2 – Minimum sensing times

* Capturing 8 consecutive 8-chip PN sequences is for reference herein.

Note that we did not consider the detection strategy that would consist of detecting non-adjacent parts of the beacon frame, to later reconstruct the whole beacon. This strategy could work in some cases, but there is no guarantee of the integrity of the data. One typical case would be when part of the frame is captured from the transmission of a PPD, while another part of the frame would be captured from the transmission of an SPD. It could also be possible that the information in the frame be updated from one superframe to the next, due to aggregation of SPD data into the PPD data. There may be additional concerns regarding security and authentication.

Figure 6 describes various sensing time region together with sensing threshold for the 11 sensing types listed above, according to the receiver sensitivity defined in section 6.8.6 in [1].

Figure 6  Summary of sensing thresholds and sensing durations

### Sensing Type 1

Energy detection can be performed within the 77 kHz bandwidth of the beacon signal, without possibility of identifying the type of signal detected in the case of positive detection.

### Sensing Type 2

0.8328 ms is used for capturing 8 consecutive 8-chip PN sequences within initialization period. Since 0.1041ms (1/9.6091=0.1041ms) is used for correlating 8-chip PN sequences, 8×0.1041=0.8328ms is needed for sensing 8 continuous 8-chip PN sequences. For this sensing scenario, only chip sequences from the initialization period are considered.

### Sensing Type 3

The 2.8107 ms sensing time is obtained by adding the duration of 8 PN spreading sequences to the maximum time without transmission that could occur within an inter-device communication period, which corresponds to the absence of transmission of RTS sequence surrounded by two turnaround times for a total duration of 19 symbols. (8+19) ×0.1041=2.8107 ms. The capture of 8 PN spreading sequences is taken as a first guideline, pending analysis to determine the probability of detection achievable at the edge of the protected contour with a simple threshold comparison.

### Sensing Type 4

The 5.1 ms sensing time for the sync burst within the beacon device initialization period can be achieved by capturing 5.1 ms of signal, demodulating the DQPSK symbols to obtain hard bit decisions, and then looking for the 15-bit synchronization sequence. Once the position of that sequence has been determined, the receiver looks before or after that sequence, in order to identify the uncoded and parity bits of the index. The index bits and parity bits must be consecutive, whereas the synchronization can be obtained when the synchronization sequence is located before or after the index and parity bits.

Figure 7 shows both worst cases for the receiver to detect sync burst and index within 5.1ms sensing time.

(a) one worst case to catch sync field, index field and parity field in 5.1ms sensing time

(b) the other worst case to catch sync field, index field and parity field in 5.1 ms sensing time

Figure 7  Worst cases for synchronization within the initialization period

In both Figure 7(a) and Figure 7(b), 46 DQPSK symbols need to be detected after despreading for getting the sync burst, the index and parity bits. Additionally, one more symbol shall be received for demodulating differential QPSK symbols. Therefore, at least 47 DQPSK symbols shall be received within the sensing window. The filters with roll-off factor shall be taken into further consideration for getting the whole 47 DQPSK symbols. In total, a sensing window with 49 symbols should be properly designed for receiving 47 DQPSK symbols for safety, which is equal to 5.099ms (49/9.6091=5.099ms).

In both worst cases shown in Figure 7(a) and Figure 7(b), after DQPSK decmodulation, hard decision bits shall be synchronized with the 15-bit synchronization sequence to detect one sync burst successfully. For the first worst scenario in Figure 7(a), the sync field is first detected in slot k-1, then the index field and parity field in slot k will be detected after synchronization. Therefore, index k should be decoded without errors while k-1 will be presented as current index number. For the second worst scenario in Figure 7(b), sync field is detected firstly in slot k-1, then index field and parity field in the same slot will be detected after synchronization. Therefore, index k-1 should be decoded without errors and thus k-1 will be regarded as the current index number.

For cases other than the two worst cased described above, after synchronization with the 15-bit  synchronization sequence, the position of the 15-bit sync field in 5.1ms sensing window shall be decided before decoding the index field and parity field, so that the current index number will be the output.

### Sensing Type 5

Synchronization information is obtained by decoding the index, or by obtaining non-ambiguous information about the type of slots contained in the signal captured during the sensing period. A cognitive radio could easily conclude that an inter-device communication period has been sensed rather than a sync burst slot, due to the structure of the inter-device communication period, which contains periods of signal and periods of silence of precise durations. In this case, there can be no doubt that the last slot of a superframe is contained within the sensed signal, and therefore the WRAN can acquire synchronization information as if it had decoded the index in a regular sync burst slot. The assumption in the notations of Figure 5 is that the index retained by the receiver corresponds to the most recent slot present in the sensed signal. With this additional cognition, the WRAN only needs 5.1 ms of quiet sensing time to obtain the synchronization information, whether the captured signal contains a sync burst or whether an inter-device communication period falls within the captured signal. A simple comparator with threshold detection is sufficient for this additional cognition.

According to Figure 5, the slot type will be decided after one 49-symbol DQPSK sequence is acquired by PN spreading sequence detection. Because of the special character with inter-device communication period (slot 0), the 49-symbol sequence will show obvious difference from other normal slots (slot 1-slot 30). From the inter-device communication period structure, it can be seen that the whole slot 0 is separated into five parts: 5 symbols of  turnaround time 1, 8 symbols of RTS, 6 symbols of turnaround time 2, 8 symbols of ANP and 5 symbols of turnaround time 3. If there is a whole or partial inter-device communication period involved in the 49-symbol DQPSK sequence, at least one silence symbol shall be detected within the sequence. Figure 8 gives a sketch map of detected DQPSK sequence for both normal slot 1~slot 30 and special inter-device communication period (slot 0).

Figure 8  Data and silence periods in TG1 slots (DP: data period; SP: silence period).

Figure 8 sketches a map of detected DQPSK sequence for normal slots and inter-device communication period. In the DQPSK sequence obtained after detection with the PN spreading sequence, if a continuous data period with length larger than 32 symbols is observed, the slot type for the corresponding sensing window shall be a normal slot; otherwise the slot type for the sensing window shall be the special inter-device communication period. In this case, the 49-symbol DQPSK sequence might include continuous DP with length shorter than 32 symbols and at least one silence period (SP).

In the following paragraphs, we list all the possibilities with inter-device communication period (slot 0) involved in the 5.1009ms sensing window.

- Partial or whole inter-device communication period (slot 0) of superframe N at the end of sensing window

In this case, the length of continuous DP symbols is smaller than 32, thus partial or whole inter-device communication period (slot 0) will be present at the end of the 49-symbol sequence. Figure 9 shows an example for this scenario.

Figure 9  Example of data period and silence period distribution with slot 0 at the end of sensing window

Figure 9 shows an example of data period and silence period distribution with slot 0 at the end of sensing window.

The 49-symbol DQPSK sequence starts from the third symbol of Slot 1 in beacon superframe N and ends by Turnaround Time 2 in Superframe N. After detection by PN spreading sequence, if the WRAN receiver gets a 49-symbol sequence with 30-symbol continuous DP followed by a 5-symbol SP, 8-symbol DP and another 6-symbol SP, timing analysis will decide that the present sensing window ends at the 19th symbol of Slot 0 in Superframe N. Therefore, the timing analysis will know that the next superframe will begin after receiving another 13 symbols (8-symbol DP and 5-symbol Turnaround time 3).

- Partial or whole inter-device communication period (slot 0) of superframe N at the beginning of sensing window

In this case, the length of continuous DP symbols is also smaller than 32, thus partial or whole inter-device communication period (slot 0) will be present at the beginning of the 49-symbol sequence. Figure 10 shows an example for this scenario.

Figure 10  Example of data period and silence period distribution with slot 0 at the beginning of sensing window

Figure 10 shows an example of data period and silence period distribution with slot 0 at the beginning of sensing window. The 49-symbol DQPSK sequence starts from the third symbol of RTS period in beacon superframe N and ends by the 3rd symbol of Parity field in slot 30 of Superframe N+1. After detection by the PN spreading sequence, if the WRAN receiver gets a 49-symbol sequence with 5-symbol continuous DP followed by a 6-symbol SP, 8-symbol DP, another 5-symbol SP and 25-symbol continuous DP, timing analysis will decide that the present sensing window ends at the 25th symbol of Slot 30 in Superframe N+1. Therefore, the timing analysis will know that slot 29 of superframe N+1 will begin after receiving another 7 symbols (5 symbols in parity field and 2 symbols in reserved field will be included).

- Whole inter-device communication period (slot 0) in the middle of sensing window

In this case, a whole inter-device communication period (slot 0) will be present in the middle of the 49-symbol DQPSK sequence with several symbols of DP from slot 1 of Superframe N and slot 30 of Superframe N+1 at both the end and the beginning. Figure 11 shows an example for this scenario.

Figure 11  Example of data period and silence period distribution with slot 0 in the middle of sensing window

Figure 11 shows an example of data period and silence period distribution with slot 0 in the middle of sensing window. The 49-symbol DQPSK sequence starts from the 1st symbol of parity field in slot 1 of beacon superframe N and ends by the 7th symbol of Sync field in slot 30 of superframe N+1. After detection by the PN spreading sequence, if the WRAN receiver gets a 49-symbol sequence with 10-symbol continuous DP followed by a 5-symbol SP, 8-symbol DP, 6-symbol SP, 8-symbol DP, 5-symbol SP and another 7-symbol continuous DP, timing analysis will decide that the present sensing window ends at the 7th symbol of Slot 30 in Superframe N+1. Therefore, the timing analysis will know that slot 29 of superframe N+1 will begin after receiving another 25 symbols (10 symbols in sync field, 7 symbols in index field, 8 symbols in parity field and 2 symbols in reserved field will be included).

- No data is transmitted by SPD in RTS period

One special scenario should be reminded here with no data transmitted by SPD in RTS period. In this case, the sketch map of detected DQPSK sequence for inter-device communication period is redrawn in Figure 12.

Figure 12  Detected DQPSK sequence in inter-device communication period with no data transmission in RTS period

### Sensing Type 6

If the signal in the 5.1 ms sensing window which contains an inter-device communication period can not be recognized by WRAN receiver, it is not equipped with the detection method described in section 3.2.5, then 5.1  ms sensing window might need to be used twice for capturing the sync burst and index information outside the initialization period.

### Sensing Type 7

If the inter-device communication period can not be recognized by the WRAN receiver with the method in section 3.2.5, a longer sensing window of 8.4311 ms can be adopted rather than two 5.1 ms sensing windows described in section 3.2.6. In Figure 13, two worst cases are listed to show that 8.4311 ms will be the minimum sensing time for both cases. As is shown in section 3.2.4 and both Figure 7(a) and Figure 7(b), if all continuous slots in sensing windows are captured from normal slots, it will take at least 5.1009 ms for sensing a sync burst and the index number. Since there is a special ICP period between two normal slots, one more slot of duration 3.3302 ms will be needed in the calculation of the minimum sensing time, which becomes 8.4311ms=5.1009ms+3.3302ms.

(a) one worst case to catch sync field, index field and parity field in 8.4311 ms sensing time

(b) the other worst case to catch sync field, index field and parity field in 8.4311 ms sensing time

Figure 13  Worst cases for synchronization outside the initialization period

### Sensing Type 8

MSF1 has a duration of 28.3070 ms, so the minimum sensing time for MSF1 is 28.4111 ms, including an additional DQPSK symbol involved for differential demodulation.

### Sensing Type 9

MSF1+MSF2 has a duration of 70.7676ms, so the minimum sensing time for MSF1+MSF2 is 29.2437 ms, including an additional DQPSK symbol involved for differential demodulation.

### Sensing Type 10

MSF1+MSF2+MSF3 has a duration of 98.2421ms, so the minimum sensing time for MSF1+MSF2+MSF3 is 98.3462 ms, including an additional DQPSK symbol involved for differential demodulation.

## Synchronization with the TG1 frame

The WRAN and TG1 frames are not synchronized, even after the WRAN has acquired timing information for the TG1 superframe. The WRAN receiver needs to synchronize to the TG1 frame, but this will not change the timing of the WRAN transmissions. Once the WRAN receiver has determined the start of the next TG1 frame, it needs to wait for a certain amount of time, referred to as latency.

The TG1 beacon ensures that the required quiet period duration is smaller than one WRAN superframe, so the WRAN does not need to interrupt the superframe preamble in order to schedule a quiet period, but it needs to wait until the desired part of the beacon frame does not interrupt the superframe preamble.

According to the sensing timing parameters defined in Section 3.2, the WRAN would need to capture

- 3.3302ms sync burst within 5.1ms short quiet period

- 29.1396ms MSF1+FEC of MSF1

- 70.7676ms MSF1+FEC of MSF1+MSF2

- 98.2421ms MSF1+FEC of MSF1+MSF2+MSF3

After a 5.1ms short quiet period, the WRAN receiver can successfully schedule a quiet period to capture each beacon frame with the minimum sensing duration. Therefore, once the WRAN receiver has determined the start of the next TG1 frame, it needs to wait for a latency T.

Figure 14 shows the scheduling of a quiet period synchronized with the TG1 beacon frame after the WRAN receiver has captured sync burst and index number. The WRAN superframe length is 160 ms, and the WRAN frame length is 10 ms. At the beginning of each superframe, the first symbol shall be the superframe preamble, followed by a frame preamble symbol. The third symbol shall be the SCH. Therefore, SCH is taken into account in the analysis so that the beacon can be captured in its entirety without interrupting the SCH.

In the figure,

- P is the quiet period;

- S is the WRAN superframe period;

- H is defined as the length of the beginning of each superframe that shall not be interrupted by a quiet period, which at least includes the first three OFDM symbols (2 ms is suggested here for safety);

- B is the length of the beacon frame;

- D is the beacon frame offset, which is defined as the time offset between the beginning of the first beacon frame and the beginning of the current WRAN superframe after sync burst and index capture.

Figure 14  Scheduling of a quiet period synchronized with the TG1 beacon frame

If D+P < S, the quiet period could be scheduled in the current superframe (it is assumed that B < S, which is the case with the parameters of the WRAN and beacon signals). Otherwise the minimum number of superframes to wait for before scheduling a quiet period of length P is:

Table 3 gives a list of the typical values for latency with different quiet period durations P. Here, quiet periods of duration 29.1396 ms, 70.7676 ms and 98.2421 ms are considered for detecting MSF1, MSF1+MSF2 and MSF1+MSF2+MSF3 separately. In the table, H is set as 2 ms. In addition, the maximum number of quiet frames at the end of a superframe is 16 frames (160-2=158ms), which means the quiet period can be scheduled to start at any time and end at any time within a superframe, except during the H period with SCH and preamble included.

| length of required quiet period | 29.1396ms (MSF1) | 71.600160 ms (MSF1+MSF2) | 98.2421ms (MSF1+MSF2+MSF3) |
| --- | --- | --- | --- |
| mean latency before scheduling a long period | 32 ms (<1 superframe) | 88 ms (<1 superframe) | 145 ms (<1 superframe) |
| max latency before scheduling a long period | 160 ms (1 superframe,) | 320 ms (2 superframes) | 320 ms (2 superframes) |

Table 3 Latency with different quiet period value P

As long as the quiet period can start and end anywhere within a superframe, the WRAN receiver should be able to detect the MSF1, or MSF1+MSF2, or MSF1+MSF2+MSF3, within 2 superframes (including waiting time and detection time without interruption of the SCH).

# Options for detecting the IEEE802.22.1 beacon signal

## Decision Options

Figure 15  Decision flowchart during a short sensing period

Figure 16   Decision flowchart after synchronization with the TG1 beacon frame

## Tradeoffs

# Operation scenarios for the coexistence of 802.22.1 and 802.22

## TG1 and WRAN coexistence scenarios

This section will organize and summarize the various coexistence scenarios discussed through the email reflector.

Some of these cases are:

- The WRAN is operating on TV channel N and a beacon comes up on TV channel N

- The WRAN is operating on TV channel N and a beacon comes up on TV channel N+1 or N-1

- The WRAN is operating and a beacon comes up on a backup TV channel K

- The WRAN is operating and a beacon comes up on a channel adjacent to the backup TV channel K+1 or K-1

- [Not applicable since there is no taboo channel for wireless microphone and TG1.]

- The WRAN comes up and starts monitoring a TV channel that has been used by a beacon. Is it still being used? If not, the channel availability time will need to be met before WRAN can move to this channel.

- …

- The WRAN starts sensing a channel that it hasn’t sensed for a long time (e.g. that channel was just added to the list of potential available channels). Sense during channel availability time before allowing moving in.

  - There is already a beacon?

  - There are several beacons?

- A beacon comes up when the WRAN has finished its channel monitoring time. Refresh period of 2 seconds applies.

- During a CPE power up initialization sequence, how many quiet periods would CPE need to use?

- When sensing on a new channel (OOB sensing) for the first time, how many quiet periods would CPE need to use?

- Is it possible to sense beacon frames several times for different portions with short quiet period?

- …

The list of relevant scenarios will be completed later.

## Sensing and detection strategies

Analysis of the coexistence scenarios, and description of acceptable strategies for the WRAN.

Suggestions on TG1 beacon detection from TG1 point of view. This would be useful while we are developing the sensing strategies but would not necessarily be useful in the annex to the standard.  This could be a text for the interim period only

# References

[1] P802.22.1/D32, Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands, April, 2008.