IEEE P802.22
Wireless RANs

| Primitive of PLME-NPD-HEARTBEAT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |
| Ke Wang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993034 | wincle@huawei.com |

Abstract

In the current IEEE 802.22.1/D2, there is no primitive to ask the PHY layer of the NPD to send the NPD codeword. Some remedies are suggested to patch this hole.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

When an SPD is selected as the NPD, it shall respond in two beacon frames following the reception of the PPD’s beacon frame with the NPD Indication subfield equal to 01 [1]. In other words if the SPD receives the PPD’s beacon frame in superframe n+2, the SPD shall respond by sending an NPD code during the receive period in superframe n+4. The SPD shall then set the MIB attribute macNPDAddress equal to its own address.

The problem in the current draft is that there is no primitive to notify the PHY layer of the SPD to send the NPD codeword. Since no response shall be required from the PPD to confirm receipt of the NPD code, the next higher layer of the SPD doesn’t know whether an NPD code was sent or not, it doesn’t know when to set the MIB attribute macNPDAddress through MLME-SET.request primitive. We suggest that the SPD shall set the MIB attribute macNPDAddress equal to its own address when it is informed that it was selected to be the NPD (upon receipt of the PPD’s beacon frame with the NPD Indication subfield changed as 01).

- Suggested Remedy:

(1) Add a PLME-NPD-HEARTBEAT primitive

Table 7—PLME-SAP primitives

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| PLME-ANP-DECISION | 6.2.2.1 |  | 6.2.2.2 |
| PLME-ANP-RESPONSE |  | 6.2.2.3 |  |
| PLME-GET | 6.2.2.4 |  | 6.2.2.5 |
| PLME-INITIATE-RTS-BURST | 6.2.2.6 |  | 6.2.2.7 |
| PLME-NPD-ACTIVE |  | 6.2.2.8 |  |
| PLME-NPD-HEARTBEAT | 6.2.2.9 |  | 6.2.2.10 |
| PLME-SET | 6.2.2.11 |  | 6.2.2.12 |
| PLME-SET-RTX-STATE | 6.2.2.13 |  | 6.2.2.14 |

6.2.2.9 PLME-NPD-HEARTBEAT.request

The PLME-NPD-HEARTBEAT.request primitive is generated by the MLME of the SPD which is selected as the NPD and issued to its PLME to request that the PHY entity send the NPD codeword during the receive period. The PLME-NPD-HEARTBEAT.request primitive has no parameters.

Once the NPD codeword is transmitted, the PLME will issue the PLME-NPD-HEARTBEAT.confirm primitive with the appropriate status (6.2.2.10).

6.2.2.10 PLME-NPD-HEARTBEAT.confirm

The PLME-NPD-HEARTBEAT.confirm primitive is generated by the PLME of the SPD which is selected as the NPD and issued to its MLME in response to a PLME-NPD-HEARTBEAT.request primitive. This primitive confirms that the NPD codeword was sent by the PHY entity. Table X specifies the parameters for the PLME-NPD-HEARTBEAT.confirm primitive.

Table X - PLME-NPD-HEARTBEAT.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | COMPLETE | The result of the request to send the NPD codeword. |

On receipt of the PLME-NPD-HEARTBEAT.confirm primitive, the MLME is notified that its request to send the NPD codeword was accepted.

(2) Replace the text in 7.4.7.3.1, Line 9, Page 72:

Upon receipt of the PPD’s beacon frame with the NPD Indication subfield changed as 01, the SPD is informed that it was selected to be the NPD. The SPD shall then set the MIB attribute macNPDAddress equal to its own address. The SPD shall respond two beacon frames following the reception of the PPD’s beacon frame. In other words if the SPD receives the PPD’s beacon frame in superframe n+2, the SPD shall respond by sending an NPD code during the receive period in superframe n+4. The process will be complete once the PPD receives the NPD code and sets the MIB attribute macNPDAddress equal to the SPD’s address.

(3) Update the text in 7.4.7.3.2, Line 19, Page 72:

“The NPD shall send an NPD code in at least every aNPDPeriod superframes in order to indicate that it is still active. If the NPD wants to send the RTS burst in one PPD receive period in which the NPD should send the NPD code subject to the rule of the periodic NPD code transmission, it should delay the transmission of the RTS burst. …”

(4) Modify Figure 36/37

Please refer to [2].

References:

- Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D2, October 2007.

- 22-07-xxxx-00-0001_Updated examples of MSCs