IEEE P802.22
Wireless RANs

| The message flow for the SPD with NST set to one |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |
| Ke Wang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993034 | wincle@huawei.com |

Abstract

In the current IEEE 802.22.1/D2,.the procedure for one SPD to send more than one beacon frame is not exactly correct. Some remedies are suggested in this document.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

According to IEEE 802.22.1/D2 [1], if an SPD wants to send more than one beacon frame, the SPD can send its initial beacon frame with the Rank and NST subfields set to zero and one, respectively. Upon receipt of this beacon frame, the PPD can grant permission to the SPD to send a second beacon frame by transmitting a Go-On response during an ANP. This process may decrease the RTS collision probability due to the fact that the second beacon frame is sent without transmitting an additional RTS burst during the receive period. After successfully sending its initial beacon frame (i.e., the MAC sublayer received a PLME-INITIATE-RTS-BURST.confirm primitive with the Status parameter set to ACK), the SPD shall wait for up to aNSTValidCount superframes to receive a Go-On response from the PPD. If a Go-On response is received by the MAC sublayer of the SPD via the PLME-ANP-RESPONSE.indication primitive during this time, the MAC sublayer of that SPD shall request that its PHY layer send a second beacon frame during the upcoming superframe by issuing a PD-DATA.request primitive. Note that once this second beacon frame is sent, the SPD shall transmit a new RTS burst before being able to send any further beacon frames. If a Go-On response is not received during this time, the SPD shall terminate the process, and it shall send a new RTS burst before sending another beacon frame.

The problem is that the next higher layer of the SPD should know that the initial beacon frame has been transmitted successfully and send the second beacon frame to the MAC sublayer through MLME-START-BEACON.request primitive.

- Suggested Remedy:

Replace the text in 7.4.6.2, Line 29, Page 67:

If an SPD wants to send more than one beacon frame, the SPD can send its initial beacon frame with the Rank and NST subfields set to zero and one, respectively. Upon receipt of this beacon frame, the PPD can grant permission to the SPD to send a second beacon frame by transmitting a Go-On response during an ANP. This process may decrease the RTS collision probability due to the fact that the second beacon frame is sent without transmitting an additional RTS burst during the receive period. After successfully sending its initial beacon frame (i.e., the next higher layer received a MLME-START-BEACON.confirm primitive with the Status parameter set to SUCCESS), the next higher layer of the SPD shall request the MAC sublayer to send the second beacon frame through MLME-START-BEACON.request primitive, the SPD shall wait for up to aNSTValidCount superframes to receive a Go-On response from the PPD. If a Go-On response is received by the MAC sublayer of the SPD via the PLME-ANP-RESPONSE.indication primitive during this time, the MAC sublayer of that SPD shall request that its PHY layer send a second beacon frame during the upcoming superframe by issuing a PD-DATA.request primitive. Note that once this second beacon frame is sent, the SPD shall transmit a new RTS burst before being able to send any further beacon frames. If a Go-On response is not received during this time, the SPD shall send a new RTS burst in order to send the second beacon frame.

References:

- Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D2, October 2007.