IEEE P802.22
Wireless RANs

| Updating the NPD Address |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |
| Xuesheng Zhu | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhuxuesheng@huawei.com |
| Ke Wang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993034 | wincle@huawei.com |

Abstract

In the current IEEE 802.22.1/D2 [1], only the PPD and the selected SPD, i.e. the new NPD, are defined how to update its record of the NPD address. Actually the other SPDs also need to update their own records of the NPD addresses after the NPD selection to monitor the NPD’s existence in the future.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

In the section of 7.4.7.3.1 [1], only the PPD and the selected SPD, i.e. the new NPD, are defined how to update its record of the NPD address. Actually the other SPDs also need to update their own records of the NPD addresses after the NPD selection to monitor the NPD’s existence in the future.

- Suggested Remedy:

Add text in the end of 7.4.7.3.1

“All SPDs always listens the beacon frames from the PPD and other SPDs (including the NPD). Upon receipt of the PPD’s beacon frame with the NPD Indication subfield changed to 01, the other SPD (not that selected SPD) can recall which SPD transmitted the latest SPD beacon frame and retrieve the NPD address from the Source Address field in that beacon frame. The MLME-NPD.request is issued to tell the MAC the updated NPD address. In the following superframes, after receiving the NPD beacon frame, which can be identified by reading the NPD subfield, and drawing out the NPD address, each SPD can make sure the validity of its NPDAddress record. If not, one more MLME-NPD.request is issued to tell the MAC the updated NPD address.”

Update the subclauses of 7.1.1.5 and 7.1.1.6

7.1.1.5 MLME-NPD.request

The MLME-NPD.request primitive is generated by the next higher layer of a PPD and issued to its MLME with the address of the SPD which was chosen to be the Next-in-line Protecting Device (NPD). Table 34 specifies the parameter for the MLME-NPD.request primitive. This primitive shall only be issued by a PPD.

Table 34—MLME-NPD.request parameter

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| NPDAddress | IEEE Address | A valid 48-bit IEEE address | The address of the SPD which was chosen as the NPD. |

On the receipt of the MLME-NPD.request primitive, the MLME is notified of the chosen NPD and will generate a beacon frame with the NPD subfield set to one.

7.1.1.6 MLME-NPD.confirm

The MLME-NPD.confirm primitive is generated by the MLME of a PPD and issued to its next higher layer following the transmission of a beacon frame with the NPD subfield set to one. Table 35 specifies the parameter for the MLME-NPD.confirm primitive.

Table 35—MLME-NPD.confirm parameter

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | COMPLETE | The result of the attempt to transmit a beacon frame containing an NPD notification. The result of updating the NPD address. |

On the receipt of the MLME-NPD.confirm primitive, the next higher layer is notified that the MAC sublayer has updated its NPDAddress record successfully transmitted a beacon frame with the NPD subfield set to one. The Status parameter will have the value COMPLETE.

References:

- Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bonds, P802.22.1/D2, October 2007.