IEEE P802.22
Wireless RANs

| Actual Quiet Period Duration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | (858) 658-1874 | Shellhammer@ieee.org |

Abstract

The worst case duration of the actual quiet period duration is calculated based on the duration of the scheduled quiet period, propagation delays and synchronization errors.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Actual Quiet Period Duration

There are several factors that effect the duration of the actual quiet period at a WRAN station.  The first factor is the duration of the scheduled quiet period for the WRAN cell and its nearby cells.  The next factor is the propagation time from the farthest WRAN cell that could effect the quiet time.  The last factor is the synchronization error between WRAN cells.

The relationship between the scheduled quiet periods in two nearby WRAN cells and the actual quiet period at the WRAN station is illustrated in Figure 1.

Figure 1: Actual Quiet Period and two Scheduled Quiet Periods

The duration of the actual quiet period is less than the duration of the scheduled quiet periods due to the timing difference at the WRAN station due to the propagation time difference and the synchronization error.

First let us look at propagation time difference.  The speed of light is given by,

The propagation time for this distance d is,

Where d is in km.

The worst case situation is when the propagation time within the cell is small and the propagation time from the nearby cell is large.  So let us assume the propagation time within the cell is zero, which is the worst case.  The propagation from the nearby cell depends on propagation distance.  From this perspective the worst case situation is when there is a good propagation path between the current WRAN cell and the nearby cell.  So we need to specify how far away we need to be concerned about. Let us say that any WRAN cell that is far enough away that even if it does not quiet then the resulting interference is 4 dB below the noise floor.  With a typical noise floor of -96 dBm (assuming a 10 dB receiver noise figure) that corresponds to an interference level of -100 dBm.

If we use a Rural Hata model for our WRAN and a 36 dBm EIRP then the interfering WRAN must be 135 km away.  So we assume we have to synchronize WRANs that are closer than that distance.  Then the worst case propagation time difference is,

If we assume that the synchronization error between the two base stations is much smaller than this (by synchronizing to the GPS signal), then in order to obtain an actual quiet period of 5.1 ms (as required by the TG1 beacon) the scheduled quiet time must be at least 5.55 ms.

The scheduled quiet period must be a multiple of the symbol period.  For a 6 MHz bandwidth version of 802.22 the FFT period is 298.666 s.  If we assume a ¼ cyclic prefix then the total symbol period is 373.333 s.  So in order to schedule a sufficiently long quiet period we must schedule a quiet period of 15 symbols which gives us a scheduled quiet period of 5.6 s, which is longer that the required 5.55 s.

The PHY frame contains 26 symbols when a ¼ cyclic prefix is used.  These 26 symbols include the FCH, DS/US Map and the DCD/UCD symbols.  So that leaves 23 data symbols.  If 15 of the data symbols are used for the quiet period, that leaves 8 data symbols in the frame.  So some data traffic could be sent during the frame.