IEEE P802.22
Wireless RANs

| Actual Quiet Period Duration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | (858) 658-1874 | Shellhammer@ieee.org |

Abstract

The worst case duration of the actual quiet period duration is calculated based on the duration of the scheduled quiet period, propagation delays and synchronization errors.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Actual Quiet Period Duration

There are several factors that effect the duration of the actual quiet period at a WRAN station.  The first factor is the duration of the scheduled quiet period for the WRAN cell and its nearby cells.  The next factor is the propagation time from the farthest WRAN cell that could effect the quiet time. The last factor is the synchronization error between WRAN cells.

The relationship between the scheduled quiet periods in two nearby WRAN cells and the actual quiet period at the WRAN station is illustrated in Figure 1.

Figure 1: Actual Quiet Period and two Scheduled Quiet Periods

The duration of the actual quiet period is less than the duration of the scheduled quiet periods due to the timing difference at the WRAN station due to the propagation time difference and the synchronization error.

First let us look at propagation time difference.  The speed of light is given by,

The propagation time for this distance d is,

Where d is the distance in km.

The worst case situation is when the propagation time within the cell is small and the propagation time from the nearby cell is large.  This situation is at the base station with interference from a nearby base station.

We need to specify how far away we need to be concerned about. Let us say that any WRAN cell that is far enough away that even if it does not quiet then the resulting interference is 4 dB below the noise floor.  With a typical noise floor of -96 dBm (assuming a 10 dB receiver noise figure) that corresponds to an interference level of -100 dBm.

We use the ITU propagation model for our WRAN.  Each base station has an antenna height of 75 meters. The base station radiates at 36 dBm EIRP.  To model the 75 meter height of the receiving base station we need to use the correction factor in Section 9 of the ITU document [1].   We use the F(1,1) curves to ensure that the likelihood of receiving a signal above this level is small.  The ITU model for this case gives a distance of 85 km to ensure the interfering power level is below -100 dBm. So we assume we have to synchronize WRANs that are closer than that distance.

Then the worst case propagation time difference is,

The time accuracy that can be obtained using GPS is approximately 3 s.    So the maximum synchronization error between base stations would be twice that, 6 s.  So if we add that to the propagation time we get 289 s.  To simplify the analysis we can round this to 300 s.  So the scheduled quiet time must be 300 s longer than the required quiet time.  So if we need 5.1 ms (as required by the TG1 beacon) the scheduled quiet time must be at least 5.4 ms.

The number of symbols in a PHY frame depends on the length of the cyclic prefix.  Also, the duration of each symbol depends on the length of the cyclic prefix.  This effect how many symbols are required to be included in the scheduled quiet period.  In this analysis the receive transmit turnaround gap (RTTG) is not included in the quiet period since the system may want to transmit both downlink and uplink data.  So it is assumed that the quiet period is at the end of the frame after the completion of both the uplink and downlink data.

Table 1 summarizes the situation for different cyclic prefix lengths.  The number of required symbols is calculated to meet the 5.4 ms required scheduled quiet period.  The number of remaining symbols is the total number of symbols minus those that are used for the quiet period.  Finally, since the FCH, DS/US Map and the DCD/UCD are assumed to utilize a single symbol.

| Cyclic Prefix | Number of Symbols | Symbol period with CP (s) | Required symbols for QP | Remaining Symbols | Remaining data symbols |
| --- | --- | --- | --- | --- | --- |
| 1/4 | 26 | 373.33 | 15 | 11 | 10 |
| 1/8 | 29 | 336 | 15 | 13 | 12 |
| 1/16 | 30 | 317.33 | 17 | 13 | 12 |
| 1/31 | 31 | 308 | 18 | 13 | 12 |

Table 1: Summary of symbols used for quiet period and remaining data symbols

We see that of the original 26 to 31 symbols for each frame between 10 and 12 remain for data symbols once we account for the symbols used for the scheduled quiet period.

# References

- Method for point-to-area prediction for terrestrial services in the frequency range 30 MHz to 3000 MHz, ITU-R P.1546-1, October 11, 2005