IEEE P802.22
Wireless RANs

| Changes to Section 6.9 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution gives the changes that were proposed to the MAC sub-sections 6.9 up to 6.9.1.1 inclusive during the teleconference call of Wednesday, December 12th.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

  - Management Messages

As can be seen in Table 1, the MAC defines a collection of management messages to support and implement its basic functions. All these messages are carried in the payload of a MAC PDU, and share the same frame structure as depicted in Figure 1. Management messages begin with a Type field that uniquely identifies the message in question, while its payload varies according to the message type. As for transmission, management messages can only be transmitted in Initial Ranging, Basic, Primary, Multicast Management or Broadcast type of CIDs (see Error: Reference source not found). No other types of CIDs shall carry management messages.

In the following sections we describe each of the management messages shown in Table 1.

- —Management messages

| Type | Message | Description | Connection |
| --- | --- | --- | --- |
| 0 | DCD | Downstream Channel Descriptor | Broadcast |
| 1 | DS-MAP | Downstream Access Definition | Broadcast |
| 2 | UCD | Upstream Channel Descriptor | Broadcast |
| 3 | US-MAP | Upstream Access Definition | Broadcast |
| 4 | RNG-REQ | Ranging Request | Initial Ranging or Basic |
| 5 | RNG-RSP | Ranging Response | Initial Ranging or Basic |
| 6 | REG-REQ | Registration Request | Primary Management |
| 7 | REG-RSP | Registration Response | Primary Management |
| 8 |  | Reserved |  |
| 9 | PKM-REQ | Privacy Key Management Request | Primary Management |
| 10 | PKM-RSP | Privacy Key Management Response | Primary Management |
| 11 | DSA-REQ | Dynamic Service Addition Request | Primary Management |
| 12 | DSA-RSP | Dynamic Service Addition Response | Primary Management |
| 13 | DSA-ACK | Dynamic Service Addition Acknowledge | Primary Management |
| 14 | DSC-REQ | Dynamic Service Change Request | Primary Management |
| 15 | DSC-RSP | Dynamic Service Change Response | Primary Management |
| 16 | DSC-ACK | Dynamic Service Change Acknowledge | Primary Management |
| 17 | DSD-REQ | Dynamic Service Deletion Request | Primary Management |
| 18 | DSD-RSP | Dynamic Service Deletion Response | Primary Management |
| 19 |  | Reserved |  |
| 20 |  | Reserved |  |
| 21 | MCA-REQ | Multicast Assignment Request | Primary Management |
| 22 | MCA-RSP | Multicast Assignment Response | Primary Management |
| 23 | DBPC-REQ | Downstream Burst Profile Change Request | Basic |
| 24 | DBPC-RSP | Downstream Burst Profile Change Response | Basic |
| 25 | RES-CMD | Reset Command | Basic |
| 26 | CBC-REQ | CPE Basic Capability Request | Basic |
| 27 | CBC-RSP | CPE Basic Capability Response | Basic |
| 28 |  | Reserved |  |
| 29 | DREG-CMD | De/Re-register Command | Basic |
| 30 | DSX-RVD | DSx Receive Message | Primary Management |
| 31 | TFTP-CPLT | Config File TFTP Complete Message | Primary Management |
| 32 | TFTP-RSP | Config File TFTP Complete Response | Primary Management |
| 33 | ARQ-Feedback | Standalone ARQ Feedback | Basic |
| 34 | ARQ-Discard | ARQ Discard Message | Basic |
| 35 | ARQ-Reset | ARQ Reset Message | Basic |
| 36 | CPE-FPC | CPE Fast Power Control | Broadcast |
| 37 | DREG-REQ | CPE De-registration Message | Basic |
| 38 |  | Reserved |  |
| 39 | BLM-REQ | Bulk Measurement Request | Primary Management, Multicast Management or Broadcast |
| 40 | BLM-RSP | Bulk Measurement Response | Primary Management |
| 41 | BLM-REP | Bulk Measurement Report | Primary Management |
| 42 | BLM-ACK | Bulk Measurement Acknowledgement | Primary Management |
| 43 | CHT-REQ | Channel Terminate Request | Basic, Multicast Management or Broadcast |
| 44 | CHT-RSP | Channel Terminate Response | Basic |
| 45 | CHA-REQ | Channel Add Request | Primary Management, Multicast Management or Broadcast |
| 46 | CHA-RSP | Channel Add Response | Primary Management |
| 47 | CHS-REQ | Channel Switch Request | Basic, Multicast Management or Broadcast |
| 48 | CHS-RSP | Channel Switch Response | Basic |
| 49 | CHQ-REQ | Channel Quiet Request | Basic, Multicast Management or Broadcast |
| 50 | CHQ-RSP | Channel Quiet Response | Basic |
| 51 | CHO-UPD | Channel Occupancy Update | Basic, Multicast Management or Broadcast |
| 52 | TRC-REQ | Traffic Constraint Request | Primary Management |
| 53 | TRC-REP | Traffic Constraint Report | Primary Management |
| 54 | TMO-REQ | Timeout Request | Primary Management |
| 55 | TMO-RSP | Timeout Response | Primary Management |
| 56 | FSL-REQ | Frame Slide Request | Basic, Multicast Management or Broadcast |
| 57 | FSL-RSP | Frame Slide Response | Basic |
| 58 | AAS-CFB-REQ | AAS Channel Feedback Request | Basic |
| 59 | AAS-CFB-RSP | AAS Channel Feedback Response | Basic |

- —The general management frame structure

    - Downstream Channel Descriptor (DCD)

The format of a DCD message is shown in Table 2. This message shall be transmitted by the BS at a periodic interval (Error: Reference source not found) to define the characteristics of a downstream physical channel.

- —DCD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DCD_Message_Format() { |  |  |
| Management Message Type = 0 | 8 bits |  |
| Downstream Channel ID | 8 bits | The identifier of the downstream channel to which this message refers. This identifier is arbitrarily chosen by the BS and is unique only within the MAC domain. This acts as a local identifier for transactions such as ranging. |
| Configuration Change Count | 8 bits | Incremented by one (modulo 256) by the BS whenever any of the values of this channel descriptor change. If the value of this count in a subsequent DCD remains the same, the CPE can quickly decide that the remaining fields have not changed and may be able to disregard the remainder of the message. |
| Information Elements (IEs) for the overall channel | Variable | Table 3 |
| Begin PHY Specific Section { |  |  |
| for (i = 1; i  n; i++) { |  | “n” is the number of possible burst profiles contained in the downstream subframe [to be set in the CPEs or specified and sent from the base station? It can also be derived from the size of the DCD message as indicated in the DS-MAP sessage.][GC: since the DIUC is now defined by 6 bits and the DIUC table contains 64 burst types, should “n” be 64?] |
| Downstream_Burst_Profile | Variable | PHY specific (Error: Reference source not found) |
| } |  |  |
| } |  |  |
| } |  |  |

      - Channel IE

- —DCD channel information elements

| Name | Element ID (1 byte) | Length (bytes) | Value |
| --- | --- | --- | --- |
| Downstream_Burst_Profile | 1 |  | Value reserved for the burst profile (see Error: Reference source not found |
| BS EIRP | 2 | 1 | Unsigned in units of dBm in 0.25 dBm steps or signed in steps of 0.5 dBwith a range from –64 dBm to 63.3 dBm. [Action: Wendong to send the question to the PHY group.] |
| TTG | 3 | 1 | .[Should be constant at 210 usec for 30 km PHY buffer. Should this be left for more flexibility? Action: Wendong to send question to the PHY group] |
| RSSIR,max | 5 | 2 | Initial ranging maximum received signal strength at BS in units of 1dBm |
| Channel Action | 6 | 1 | Action to be taken by all CPEs in a cell. 0 = None 1 = Switch 3 = Remove [? Dave to verify.] |
| Action Mode [Possible merging with Channel Action: 0 = None 1 = Switch with no more transmission allowed until action is performed 2 = Switch with transmission allowed Action: Dave] | 7 | 1 | This is valid only for channel switch (Action = 1) or channel remove (Action = 3) Indicates any restrictions on transmission until the specified Channel Action is performed. The BS shall set the Action Mode field to either 0 or 1 on transmission. A value of 1 means that the CPE to which the frame containing the element is addressed shall transmit no further frames until the scheduled Channel Action is performed. An Action Mode set to 0 does not impose any requirement on the receiving CPE. |
| Action Superframe Number | 8 | 1 | The superframe number (modulo 255) at which Channel Action shall be performed. |
| Action Frame Number | 9 | 4 bits would be sufficient since only 16 frames in superframe] | Integer value grater than or equal to zero that indicates the starting frame number, within the Action Superframe Number, at which the Channel Action shall be performed by all CPEs. If this field is greater than or equal to 16, then Channel Action shall be set to zero. |
| Action Channel Number | 10 | 1 |  |
| Number of Backup channels | 11 | 4 bits | Number of channels to be kept clear as backup. |
| Number of Channels in the Backup and Candidate channels list. | 12 | 1 |  |
| For (i=0; i < Number Channels in the list; i++) { |  |  | List of backup channels in order of priority to be used by CPEs in case of loss of communication with the BS due to incumbents, followed by candidate channels in order of priority to be sensed when time permits. The number of backup channels is indicated in field no.11. The list shall be a disjoint set including the current operating channel. |
| Channel Number for Backup [i] | 13 | 1 |  |
| } |  |  |  |
| MAC version | 148 | 1 | Error: Reference source not found |