IEEE P802.22
Wireless RANs

| Supplement to Tables 76 – 87 in Section 6.9.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Huawei & HKUST | Department of ECE, HKUST, Clear Water Bay, Hong Kong | 852-2358-8027 | eeedward@ust.hk |
| Linjun Lv | Huawei Technologies | Shenzhen, China | 0086-755-28973119 | lvlinjun@huawei.com |

Abstract

This contribution provides the text description for Tables 76 – 87 in Section 6.9.7 (REG-REQ/RSP) of the Working Document (version 0.4.3).

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Background Information

Referring to Section 6.9.7.3 of the Working Document version 0.4.3, there is no text describing the functions of Tables 76 – 87, which are the information elementsd of the REG-REQ/RSP. In this contribution, we provide the text for these tables.

2. Table 76 (ARQ Parameters)

This field provides the fragmentation and ARQ parameters applied during the establishment of the secondary management connection. This field is related to the ARQ parameters described in 6.9.8.10.19.

3. Table 77 (CPE Management Support)

This field indicates whether or not the CPE is managed by standard-based IP messages over the secondary management connection. When the CPE indicates in the REG-REQ that it is managed, the BS and CPE shall perform the following three stages of the initial network entry process, namely establish IP connectivity, establish time of the day, and transfer operational parameters. Otherwise, these stages shall be skipped by both the BS and CPE.

4. Table 78 (IP Management Mode)

The IP management mode parameter dictates whether the provider intends to manage the SS on an ongoing basis via IP-based mechanisms.

5. Table 79 (IP Version)

This field indicates the version of IP used on the Secondary Management Connection.

6. Table 80 (Secondary Management CID)

This parameter contains the Secondary Management CID issued to a CPE.

7. Table 81 (Number of Upstream CID Supported)

This field shows the number of Uplink CIDs the CPE can support. The minimum value is three for managed CPEs and two for unmanaged CPEs. A CPE shall support a Basic CID, a Primary Management CID, and 0 or more Transport CIDs. A managed CPE shall also support a Secondary Management CID.

8. Table 82 (ARQ Support)

This field indicates the availability of CPE support for ARQ.

9. Table 83 (DSx Flow Control)

This field specifies the maximum number of concurrent DSA, DSC, or DSD transactions that may be outstanding.

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 11 | 1 | 0: no limit (default) 1-255: indicate maximum concurrent transactions | REG-REQ, REG-RSP |

10. Table 84 (MCA Flow Control)

This field specifies the maximum number of concurrent MCA transactions that may be outstanding.

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 12 | 1 | 0: no limit (default) 1–255: indicate maximum concurrent transactions | REG-REQ, REG-RSP |

11. Table 85 (Maximum Number of Multicast Groups Supported)

This field indicates the maximum number of simultaneous Multicast Polling Groups the CPE is capable of belonging to.

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 13 | 1 | 0-255 0 (default) | REG-REQ, REG-RSP |

12. Table 86 (PKM Flow Control)

This field specifies the maximum number of concurrent PKM transactions that may be outstanding.

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 14 | 1 | 0: no limit (default) 1–255: indicate maximum concurrent transactions | REG-REQ, REG-RSP |

13. Table 87 (Authorization Policy Supported)

This field indicates authorization policy that both CPE and BS need to negotiate and synchronize. A bit value of 0 indicates “not supported” while 1 indicates “supported.” If this field is omitted, then both CPE and BS shall use the IEEE 802.22 security as authorization policy.

References: