IEEE P802.22
Wireless RANs

| suggested changes on Table 49 53 61 64 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wu Yu-chun | Huawei Hisi | Xinxi Rd., Shangdi St, Haidian Dist, Beijing 100085 China | 86-10-8282-9018 | wuyuchun@huawei.com |
| Soo-Young Chang | Huawei Technologies | Davis, CA, U.S. | 1-916 278 6568 | sychang@ecs.csus.edu |
| Jianwei Zhang | Huawei Technologies | Shanghai, China | 86-21-68644808 | zhangjianwei@huawei.com |
| Liu Jinnan | Huawei Hisi | Xinxi Rd., Shangdi St, Haidian Dist, Beijing 100085 China | 86-10-8288-9148 | liujinnan@huawei.com |

Abstract

Since 6bits was used for both DIUC and UIUC now,  we can specify a one to one mapping between each DIUC/UIUC index and each combination of FEC type、FEC rate、Modulation type. Thus there is no need to include the information of FEC type、FEC rate、Modulation type in the DCD/UCD any more and wireless resouces can be saved somehow.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

With 6 bits of DIUC/UIUC index, we can represent up to 64 possibilities, so there’s no need to transfer the FEC type, FEC rate, and modulation type information in the DCD/UCD anymore. Thus we suggest changes on table 49, 53, 61, 64 as following:

- Table 49—Information elements

| Name | Element ID (1 byte) | Length (bytes) | Value |
| --- | --- | --- | --- |
| Frequency | 1 | 4 | Downstream frequency (kHz) |
| FEC code type and modulation type | 150 | 1 | Bits 0-2 have the following values: 000 = CC, 001 = DBTC, 010 = LDPC, 011 = STBC, 100-111 = reserved. Bits 3-5 have the following values: 000 = Spreading, 001 = reserved, 010 = QPSK, 011 = 16QAM, 100 = 64QAM. Bits 6-7 have the following values: 00 = rate 1/2, 01 = rate 2/3, 10 = rate 3/4, 11 = rate 5/6. |
| DIUC mandatory exit threshold | 151 | 1 | 0-63.75 dB CINR at or below where this DIUC can no longer be used and where change to a more robust DIUC is required (in 0.25 dB units) [0.5 dB with larger range???] |
| DIUC minimum entry threshold | 152 | 1 | 0-63.75 dB The minimum CINR required to start using this DIUC when changing from a more robust DIUC is required (in 0.25 dB units) [0.5 dB with larger range???] |

- Table 53—DIUC values

| DIUC | Usage |  |  |
| --- | --- | --- | --- |
| 0 | Self-Coexistence (Passive Mode) |  |  |
| 1 | PAPR Reduction |  |  |
| 2-12 | Reserved |  |  |
| 13 | Uncoded | NA | BPSK |
| 14 | Convolutional Code | FEC rate = 1/2 | QPSK |
| 15 | Convolutional Code | FEC rate = 2/3 | QPSK |
| 16 | Convolutional Code | FEC rate = 3/4 | QPSK |
| 17 | Convolutional Code | FEC rate = 5/6 | QPSK |
| 18 | Convolutional Code | FEC rate = 1/2 | 16-QAM |
| 19 | Convolutional Code | FEC rate = 2/3 | 16-QAM |
| 20 | Convolutional Code | FEC rate = 3/4 | 16-QAM |
| 21 | Convolutional Code | FEC rate = 5/6 | 16-QAM |
| 22 | Convolutional Code | FEC rate = 1/2 | 64-QAM |
| 23 | Convolutional Code | FEC rate = 2/3 | 64-QAM |
| 24 | Convolutional Code | FEC rate = 3/4 | 64-QAM |
| 25 | Convolutional Code | FEC rate = 5/6 | 64-QAM |
| 26 | CTC Code | FEC rate = 1/2 | QPSK |
| 27 | CTC Code | FEC rate = 2/3 | QPSK |
| 28 | CTC Code | FEC rate = 3/4 | QPSK |
| 29 | CTC Code | FEC rate = 5/6 | QPSK |
| 30 | CTC Code | FEC rate = 1/2 | 16-QAM |
| 31 | CTC Code | FEC rate = 2/3 | 16-QAM |
| 32 | CTC Code | FEC rate = 3/4 | 16-QAM |
| 33 | CTC Code | FEC rate = 5/6 | 16-QAM |
| 34 | CTC Code | FEC rate = 1/2 | 64-QAM |
| 35 | CTC Code | FEC rate = 2/3 | 64-QAM |
| 36 | CTC Code | FEC rate = 3/4 | 64-QAM |
| 37 | CTC Code | FEC rate = 5/6 | 64-QAM |
| 38 | LDPC Code | FEC rate = 1/2 | QPSK |
| 39 | LDPC Code | FEC rate = 2/3 | QPSK |
| 40 | LDPC Code | FEC rate = 3/4 | QPSK |
| 41 | LDPC Code | FEC rate = 5/6 | QPSK |
| 42 | LDPC Code | FEC rate = 1/2 | 16-QAM |
| 43 | LDPC Code | FEC rate = 2/3 | 16-QAM |
| 44 | LDPC Code | FEC rate = 3/4 | 16-QAM |
| 45 | LDPC Code | FEC rate = 5/6 | 16-QAM |
| 46 | LDPC Code | FEC rate = 1/2 | 64-QAM |
| 47 | LDPC Code | FEC rate = 2/3 | 64-QAM |
| 48 | LDPC Code | FEC rate = 3/4 | 64-QAM |
| 49 | LDPC Code | FEC rate = 5/6 | 64-QAM |
| 50 | SBTC Code | FEC rate = 1/2 | QPSK |
| 51 | SBTC Code | FEC rate = 2/3 | QPSK |
| 52 | SBTC Code | FEC rate = 3/4 | QPSK |
| 53 | SBTC Code | FEC rate = 5/6 | QPSK |
| 54 | SBTC Code | FEC rate = 1/2 | 16-QAM |
| 55 | SBTC Code | FEC rate = 2/3 | 16-QAM |
| 56 | SBTC Code | FEC rate = 3/4 | 16-QAM |
| 57 | SBTC Code | FEC rate = 5/6 | 16-QAM |
| 58 | SBTC Code | FEC rate = 1/2 | 64-QAM |
| 59 | SBTC Code | FEC rate = 2/3 | 64-QAM |
| 60 | SBTC Code | FEC rate = 3/4 | 64-QAM |
| 61 | SBTC Code | FEC rate = 5/6 | 64-QAM |
| 62 | Extended DIUC |  |  |
| 63 | End of Map |  |  |

- Table 61—Information elements

| Name | Element ID (1 byte) | Length (bytes) | Value |
| --- | --- | --- | --- |
| FEC code type and modulation type | 150 | 1 | Bits 0-2 have the following values: 000 = CC, 001 = DBTC, 010 = LDPC, 011 = STBC, 100-111 = reserved. Bits 3-5 have the following values: 000 = Spreading, 001 = reserved, 010 = QPSK, 011 = 16QAM, 100 = 64QAM. Bits 6-7 have the following values: 00 = rate 1/2, 01 = rate 2/3, 10 = rate 3/4, 11 = rate 5/6. |
| Ranging data ratio | 151 | 1 | Reducing factor, in units of 1 dB, between the power used for this burst and the power that should be used for CDMA Ranging. |
| Normalized C/N override | 152 | 5 | This is a list of numbers, where each number is encoded by one nibble, and interpreted as a signed integer. The nibbles are defined in the PHY spec. The number encoded by each nibble represents the difference in normalized C/N relative to the previous one. [Explain, reference?] |

- Table 64—UIUC values

| UIUC | Usage |  |  |
| --- | --- | --- | --- |
| 0 | Self-Coexistence (Active Mode) |  |  |
| 1 | Self-Coexistence (Passive Mode) |  |  |
| 2 | UCS Notification |  |  |
| 3 | BW Request |  |  |
| 4 | CDMA UCS Notification |  |  |
| 5 | CDMA BW Request |  |  |
| 6 | CDMA Ranging |  |  |
| 7 | CDMA Allocation IE (see Error: Reference source not found) |  |  |
| 8~12 | Reserved |  |  |
| 13 | Uncoded | NA | BPSK |
| 14 | Convolutional Code | FEC rate = 1/2 | QPSK |
| 15 | Convolutional Code | FEC rate = 2/3 | QPSK |
| 16 | Convolutional Code | FEC rate = 3/4 | QPSK |
| 17 | Convolutional Code | FEC rate = 5/6 | QPSK |
| 18 | Convolutional Code | FEC rate = 1/2 | 16-QAM |
| 19 | Convolutional Code | FEC rate = 2/3 | 16-QAM |
| 20 | Convolutional Code | FEC rate = 3/4 | 16-QAM |
| 21 | Convolutional Code | FEC rate = 5/6 | 16-QAM |
| 22 | Convolutional Code | FEC rate = 1/2 | 64-QAM |
| 23 | Convolutional Code | FEC rate = 2/3 | 64-QAM |
| 24 | Convolutional Code | FEC rate = 3/4 | 64-QAM |
| 25 | Convolutional Code | FEC rate = 5/6 | 64-QAM |
| 26 | CTC Code | FEC rate = 1/2 | QPSK |
| 27 | CTC Code | FEC rate = 2/3 | QPSK |
| 28 | CTC Code | FEC rate = 3/4 | QPSK |
| 29 | CTC Code | FEC rate = 5/6 | QPSK |
| 30 | CTC Code | FEC rate = 1/2 | 16-QAM |
| 31 | CTC Code | FEC rate = 2/3 | 16-QAM |
| 32 | CTC Code | FEC rate = 3/4 | 16-QAM |
| 33 | CTC Code | FEC rate = 5/6 | 16-QAM |
| 34 | CTC Code | FEC rate = 1/2 | 64-QAM |
| 35 | CTC Code | FEC rate = 2/3 | 64-QAM |
| 36 | CTC Code | FEC rate = 3/4 | 64-QAM |
| 37 | CTC Code | FEC rate = 5/6 | 64-QAM |
| 38 | LDPC Code | FEC rate = 1/2 | QPSK |
| 39 | LDPC Code | FEC rate = 2/3 | QPSK |
| 40 | LDPC Code | FEC rate = 3/4 | QPSK |
| 41 | LDPC Code | FEC rate = 5/6 | QPSK |
| 42 | LDPC Code | FEC rate = 1/2 | 16-QAM |
| 43 | LDPC Code | FEC rate = 2/3 | 16-QAM |
| 44 | LDPC Code | FEC rate = 3/4 | 16-QAM |
| 45 | LDPC Code | FEC rate = 5/6 | 16-QAM |
| 46 | LDPC Code | FEC rate = 1/2 | 64-QAM |
| 47 | LDPC Code | FEC rate = 2/3 | 64-QAM |
| 48 | LDPC Code | FEC rate = 3/4 | 64-QAM |
| 49 | LDPC Code | FEC rate = 5/6 | 64-QAM |
| 50 | SBTC Code | FEC rate = 1/2 | QPSK |
| 51 | SBTC Code | FEC rate = 2/3 | QPSK |
| 52 | SBTC Code | FEC rate = 3/4 | QPSK |
| 53 | SBTC Code | FEC rate = 5/6 | QPSK |
| 54 | SBTC Code | FEC rate = 1/2 | 16-QAM |
| 55 | SBTC Code | FEC rate = 2/3 | 16-QAM |
| 56 | SBTC Code | FEC rate = 3/4 | 16-QAM |
| 57 | SBTC Code | FEC rate = 5/6 | 16-QAM |
| 58 | SBTC Code | FEC rate = 1/2 | 64-QAM |
| 59 | SBTC Code | FEC rate = 2/3 | 64-QAM |
| 60 | SBTC Code | FEC rate = 3/4 | 64-QAM |
| 61 | SBTC Code | FEC rate = 5/6 | 64-QAM |
| 62 | Extended UIUC |  |  |
| 63 | End of Map |  |  |

References

- 《IEEE P802.22?/D0.4.4 Draft Standard for Wireless Regional Area Networks Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands》, December 2007,