IEEE P802.22
Wireless RANs

| MAC Management Message for Transmission of Maximum EIRP Array from BS to CPE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Huawei and HKUST | Department of ECE, HKUST, Clear Water Bay, Hong Kong | (852) 2358-8027 | eeedward@ust.hk |
| Dave Cavalcanti | Philips | 345 Scarborough Road, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada, K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Linjun Lv | Huawei | Shenzhen, China | 86-755-28973119 | lvlinjun@huawei.com |

Abstract

A MAC management message is proposed to be added to Section 6.9 for the transmission of maximum EIRP array from BS to CPE during initialization.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. MAC Managemenet Message

Referring to the CPE initialization figure in the contribution 22-07-0362-14-0000, the Base Station is required to transmit to CPE not only the available channel set, but also the respective maximum allowable EIRP array.

In this contribution, we propose to modify the channel management message CHO-UPD (Table 161 of Working Document v0.4.6) to incorporate this change. In other words, the function of CHO-UPD is not only restricted for sensing purpose, but also used for updating those CPEs their TPC cap. The modification is indicated by bold value as follows.

- Table 161 —CHO-UPD

| Syntax | Size | Notes |
| --- | --- | --- |
| CHO-UPD_Message_Format() { |  |  |
| Management Message Type = 51 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Number of Channels | 8 bits | Number of channels reported in the CHO-UPD message |
| for (i = 1; i  Number of Channels; i++) { |  |  |
| Channel number | 8 bits |  |
| Channel State | 3 bits | Table 162 |
| Priority | 2 bits | Take one of the following values: i) Low, ii) Medium, iii) High, vi) Undefined. We may not need this field because the priority will be indicated in the prioritized list of backup and candidate channels carried to all CPEs in the DCD message. |
| } |  |  |
| EIRP_Vector IE | Variable | The EIRP_Vector IE is added to the CHO-UPD message when the BS wants to indicate the maximum allowed EIRP in each channel. Since the max allowed EIRP may be different for each CPE, this EIRP_Vector IE is expected to be used mainly when the CHO-UPD message is transmitted as a unicast/multicast message to a specific CPE/group of CPEs which are closely resided. If the EIRP_Vector IE is included when the CHO-UPD message is a broadcast, the max EIRP values will apply to all CPEs. |
| Padding Nibble | 0-7 bits | To bring the PDU at the byte boundary. |
| } |  |  |

- Table 162 —Channel state information

| Value | State | Remarks |
| --- | --- | --- |
| 000 | Unmeasured |  |
| 001 | Vacant |  |
| 010 | Unavailable channel (from the TV Incumbent Database) | These two states are described in Gerald Chouinard’s contribution 07/0529r1. |
| 011 | Dissallowed channel at the base station |  |
| 100 | Occupied by TV service |  |
| 101 | Occupied by wireless microphones |  |
| 110 | Occupied by 802.22 |  |
| 111 | Reserved |  |

- Table x—EIRP Vector Information Element

| Syntax | Size | Notes |
| --- | --- | --- |
| EIRP_Vector_Message_Format() { |  |  |
| Element ID | 8 bits | There is only one possible IE. |
| Length | 8 bits |  |
| for (i = 1; i  Number of Channels; i++) { |  |  |
| Max_EIRP_Channel[i] | 8 bits | Maximum Allowed EIRP for the channel associated with channel number i. |
| } |  |  |
| } |  |  |

References: