IEEE P802.22
Wireless RANs

| CPE Basic Capability for Initialization |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Huawei & HKUST | Department of ECE, HKUST, Clear Water Bay, Hong Kong | (852) 2358-8027 | eeedward@ust.hk |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

Referring to the network entry and initialization process, CPE needs to negotiate its capabilities with the BS during the authorization and registration stages. This contribution discusses the need and pros & cons of negotiating the CPE capabilities with BS during the authorization stage. Suggested changes are presented in the last page of this document.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. CPE Capability Negotiation

Referring to the network entry and initialization process (22-07-0248-17-0000), CPE needs to negotiate its capabilities with the BS during the authorization (via CBC-REQ/RSP) and registration (via REG-REQ/RSP) stages.

The motivation of basic capability negotiation is to facilitate effective communication between BS and CPE during the reminder of the initialization protocols, e.g. key exchange, registration. The following shows the CBC-REQ and CBC-RSP message formats, along with their information elements and the physical parameters involved.

- Table 138 —CBC-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CBC-REQ_Message_Format() { |  |  |
| Management Message Type = 26 | 8 bits |  |
| Information Elements (IEs) | Variable | 6.9.7.3.7, 6.9.15.3.1, and 6.9.15.3.3 |
| } |  |  |

- Table 139 —CBC-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CBC-RSP_Message_Format() { |  |  |
| Management Message Type = 27 | 8 bits |  |
| Information Elements (IEs) | Variable | 6.9.15.3.1 and 6.9.15.3.3 |
| } |  |  |

1.1 Information Elements

For the information elements, they include (1) bandwidth allocation support (c.f. Table 140); and (2) capabilities for construction and transmission of MAC PDUs (c.f. Table 141).

- Table 140 (Bandwidth allocation support)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 1 | 1 | Bits 0-7: reserved (set to zero) | CBC-REQ, CBC-RSP |

Comments: The original intent of Table 140 is to indicate the properties of the SS in WiMax that the BS needs to know for bandwidth allocation purposes. In particular, as referred to Section 11.8.1 of IEEE 802.16d/2004,

Bit #0: 	Reserved; shall be set to zero

Bit #1 	= 0: Half-Duplex (FDD only)

Bit #1 	= 1: Full-Duplex (FDD only)

Bits #2–7: 	Reserved; shall be set to zero

Since our current consideration is TDD rather than FDD, all bits have been set to zero as shown in Table 140 of version 0.4.3 of the Working Document.

- Table 141 (Capabilities for construction and transmission of MAC PDUs)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 4 | 1 | Bit 0: Ability to receive requests piggybacked with data Bit 1: Specifies the maximum size of FSN values used when forming MAC PDUs on non-ARQ connections 0: Only 3-bit supported 1: Only 11-bit supported Bits 2-7: reserved (set to zero) | CBC-REQ, CBC-RSP |

Comments: In 802.16d, this information element is not included in the basic capability negotiation process. In 802.16e, it states that this information element may be included during the negotiation.

Note: FSN = Fragment Segement Number

1.2 Physical Parameters

For the physical parameters, they involve (1) maximum transmit power for QPSK, 16QAM, and 64QAM (c.f. Table 142); (2) current transmit power used for the burst which carried the message (c.f. Table 143); (3) CPE demodulator options supported by a CPE for downstream reception (c.f. Table 144); and (4) modulator options supported by a CPE for upstream transmission (c.f. Table 145).

- Table 142 (Maximum transmit power for QPSK, 16QAM, 64QAM)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 3 | 3 | Byte 0: Maximum transmitted power for QPSK Byte 1: Maximum transmitted power for 16-QAM Byte 2: Maximum transmitted power for 64-QAM. | CBC-REQ |

- Table 143 (Current transmit power used for the burst which carried the message)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 147 | 1 | Current transmit power (Error: Reference source not found) | CBC-REQ |

The original intent of Tables 142 – 143 is to allow the BS to assign optimal number of sub-channels and appropriate coding and modulation scheme to the SS in WiMax. In particular, as referred to IEEE 802.16d/2004, it states that

“when subchannelization is employed in WiMax, the SS shall maintain the same transmitted power density unless the maximum power level is reached. That is, when the number of active subchannels allocated to a user is reduced, the total transmitted power shall be reduced proportionally by the CPE without additional power control messages. When the number of subchannels is increased the total transmitted power shall also be increased proportionally. However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements. CPEs shall report the maximum available power, and the current transmitted power. These parameters may be used by the Base station for optimal assignment of coding schemes and modulations and also for optimal allocation of subchannels. The algorithm is vendor-specific. These parameters are reported in the CBC-REQ message. The current transmitted power shall also be reported in the REP-RSP message if the relevant flag in the REP-REQ message has been set.”

In WRAN, however, the maximum power is bounded by the EIRP and the BS will assign the operating channel (rather than some subchannels) to the CPE. Therefore, the necessarity of passing current transmit power of CPE and the maximum transmit power for QPSK/16QAM/64QAM to the BS for capability negotiation is questionable.

- Table 144 (CPE demodulator options for DS reception)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 151 | 1 | Bit 0: DBTC Bit 1: LDPC Bit 2: SBTC Bits 3-7: reserved (set to zero) | CBC-REQ, CBC-RSP |

- Table 145 (CPE modulator options for US reception)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 152 | 1 | Bit 0: DBTC Bit 1: LDPC Bit 2: SBTC Bits 3-7: reserved (set to zero) | CBC-REQ, CBC-RSP |
| 153 | 1 | The number of HARQ ACK channel | CBC-REQ, CBC-RSP |

Comments: Are these physical parameters necessary for effective communication between BS and CPE during the rest of initialization process?

2. Discussion / Proposed Change

The question here is if there is any necessarily of doing a CPE basic capability negotiation during the authorization process. There are three suggested options as follows.

2.1 Option 1 – Consolidate the CBC-REQ(RSP) and REG-REQ(RSP) into one single message

Move the capability IEs (Tables 141, 144 and 145) of the CPE capability negotiation to the registration stage, while deleting the others (Tables 140, 142 and 143).

Pros: Reduce transmissions by two messages, which are only exchanged one, and minimize interference to incumbents.

Cons: Potentially loss the flexibility to implement some optimization depending on the basic capabilities for the rest of initialization process. For example, do we need to negotiate some capabilities for key exchange, such as PKM flow control, authorization policy support, and the like (or if these security information can be negotiated during the key exchange?).

2.2 Option 2 – Keep it as is

No consolidation, no deletion of any capability IEs / physical parameters.

2.3 Option 3 – Somewhere in between

Move some of the capabilities IEs, for example those in Tables 144 and 145, to the registration stage; while deleting some of the others (e.g. Tables 142 and 143). Additionally, create some capability IEs for key exchange / security association, if there is more than one version of PKM flow control, and more than one authorization policy supported.

3. Latest Update

Based on the discussion on Wednesday AM2, Wednesday PM1, and Thursday AM2, the suggestion is to:

- Keep Table 140, Table 141, Table 142, and Table 143 for basic CPE capability negotiation.

- For Table 144 and Table 145, the contents will be revised according to the static DIUC and UIUC profiles as in the document IEEE802.22-08/0004r0, and the revised tables will be kept for basic CPE capability negotiation.

- Add three tables, related to PKM flow control and supported authorization policy, to the basic CPE capability negotiation.

Suggested Change #1:

- Table 144 (CPE demodulator options for DS reception)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 151 | 1 | For a particular mode being represented, see the corresponding index in Table 53 (DIUC values) | CBC-REQ, CBC-RSP |

Suggested Change #2:

- Table 145 (CPE modulator options for US reception)

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 152 | 1 | For a particular mode being represented, see the corresponding index in Table 64 (UIUC values) | CBC-REQ, CBC-RSP |

Suggested Change #3: Change the scope of Table 86 from REG-REQ and REG-RSP in Section 6.9.7.3.7.5 (PKM Flow Control) to CBC-REQ and CBC-RSP.

- Table 86 —Information elements

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 14 | 1 | 0: no limit (default) 1–255: indicate maximum concurrent transactions | CBC-REQ, CBC-RSP |

Suggested Change #4:

Change the scope of Table 87 from REG-REQ and REG-RSP in Section 6.9.7.3.7.6 (Authorization Policy Supported) to CBC-REQ and CBC-RSP.

- Table 87 —Information elements

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 15 | 1 | Bit #0: 802.22 privacy supported | CBC-REQ, CBC-RSP |

Suggested Change #5:

Change the scope of Table 88 from REG-REQ and REG-RSP in Section 6.9.7.3.7.7 (Maximum Number of Supported Security Associations) to CBC-REQ and CBC-RSP.

- Table 88 —Information elements

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 16 | 1 | 0-255 | CBC-REQ, CBC-RSP |

References: