IEEE P802.22
Wireless RANs

| Improving the Synchronization Procedure between PDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianwei Zhang | Huawei Technologies | 450 Jinyu Road, Pudong, Shanghai, P. R. China, 201206 | 86-21-50993036 | zhangjianwei@huawei.com |

Abstract

The improvement in the synchronization procedure is suggested in this document to patch the hole in IEEE 802.22.1/D2 [1].

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Problem Description:

In the current IEEE 802.22.1/D2 [1], the synchronization procedure has not been defined clearly. There is no synchronization mechanism in the MAC, i.e. between the MAC and the PHY.

The initialization search process in the D1/D2 just care about the incoming beacons. With the current design, the high layers just know the other devices' existence with the report of incoming beacons. That means that one PD identifies other existing PDs with the granularity of at least ONE beacon frame length.

Actually each device can always detect in time the Sync burst tranmitted in the air and synchronizes with that signal. The synchronization between the PHY and the MAC can be done in time.

- Suggested Remedy:

(1) Add a PLME-SYNC-BURST primitive

Table 7—PLME-SAP primitives

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| PLME-ANP-DECISION | 6.2.2.1 |  | 6.2.2.2 |
| PLME-ANP-RESPONSE |  | 6.2.2.3 |  |
| PLME-GET | 6.2.2.4 |  | 6.2.2.5 |
| PLME-INITIATE-RTS-BURST | 6.2.2.6 |  | 6.2.2.7 |
| PLME-NPD-ACTIVE |  | 6.2.2.8 |  |
| PLME-NPD-HEARTBEAT | 6.2.2.9 |  | 6.2.2.10 |
| PLME-SET | 6.2.2.11 |  | 6.2.2.12 |
| PLME-SET-RTX-STATE | 6.2.2.13 |  | 6.2.2.14 |
| PLME-SYNC-BURST |  | 6.2.2.x |  |

6.2.2.x PLME-SYNC-BURST.indication

The PLME-SYNC-BURST.indication primitive is generated by the PLME of the PD to tell the MAC in the case of the confirmed Sync burst appearing first time. At the same time, the PHY will try to synchronize with other device based on this alignment. Table yy specifies the parameter for the PLME-SYNC-BURST.indication primitive.

Table yy—PLME-SET-TRX-STATE.confirm parameters

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Index | Integer | (0…30) | The index number of receiving sync burst. |

During the serching procedure in the initialization stage, if the PHY detects one Sync burst appering, the PHY shall report this case immediately to the MAC. With the reception of this primitive, the MAC will know there has been at least one existing PD and can use it as its time alignment base. The MAC can wait to receive one possible beacon frame by, for example, letting the transceiver keeping listening.

References:

- Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands, P802.22.1/D2, October 2007.