IEEE P802.22
Wireless RANs

| Suggested text changes for convolutional coding |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document is the suggested text changes for convolutional coding. In particular, regarding the concatenation rule, we need to correct some typos and we should modify Table 257 and 258 according to the characteristics of WRAN system.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Introduction

Regarding the concatenation rule of convolutional coding, the basic unit is the OFDM slot, not subchannel. Here, the OFDM slot is defined by one subchannel and one OFDM symbol. Thus the subchannel concatenation should be corrected by slot concatenation.

Moreover, Table 258 describes the useful data payload size before encoding. While Table 257 describes the FEC block size after encoding. According to the rule of Table 257, the Table 258 presents the paylod size before encoding corresponding to the each FEC block size. Thus the Table 258 should be the useful data payload size per block, not subchannel. In addition, dislike the 802.16e, the number of subcarriers per subchannel is 24, not 48. Therefore, the useful data payload can be less than 6 Bytes, such as 3, 4, and 5 Bytes. We also should include this value in Table 258. That is, for the 802.22 WRAN system, the minimum useful data payload size is 3 Bytes, not 6 Bytes (refer to the doc. 22-08/0xxxr0-Useful data payload per block for convolutional coding).

For the other optional FEC schemes (CTC, LDPC, and STBC), we need to make a correction similarly.

2. Suggested text changes

We suggest the changes of normative text for convolutional coding as follows,

      - Convolutional code (CC) mode (mandatory)

- 8.6.2.1.1 Convolutional coding

The data burst is encoded using a rate – ½ binary convolutional encoder. The constraint length of this coder is equal to 7 and its generator polynomials are 171o and 133o for outputs A and B respectively.  is a graphical description of the generator polynomials. Output A and output B represent the first and second output bits respectively of this encoder.

The convolutional coder shall be initialized at the beginning of the control headers and at the beginning of each burst. Tail biting shall be used with control headers, while tail bits (6 in number) shall be used with the burst or PSDU data. This implies that for the case of control headers, the encoder memory is initialized with the last 6 bits in the header; and for the case of PSDU data, the encoder memory is initialized to an all zeros state.

- Figure 113 Rate – ½ convolutional coder with generator polynomials 171o, 133o. The delay element represents a delay of 1 bit.

- 8.6.2.1.2 Puncturing

Different coding rates can be obtained by puncturing the output of the convolutional coder.  shows the different rates that can be derived from the output of rate – ½ convolutional coder and the associated puncturing patterns.

Decoding by using Viterbi algorithm is recommended. At the receiver, zeros are inserted in the locations of the punctured bits before the Viterbi decoder.

- Table 255—Puncturing and bit-insertion for the different coding rates

| Code rate | ½ | 2/3 | ¾ | 5/6 |
| --- | --- | --- | --- | --- |
| Convolutional coder output | A1B1 | A1B1A2B2 | A1B1A2B2A3B3 | A1B1A2B2A3B3A4B4A5B5 |
| Puncturer output/bit-inserter input | A1B1 | A1B1B2 | A1B1B2A3 | A1B1B2A3B4A5 |
| Decoder input | A1B1 | A1B10B2 | A1B10B2A30 | A1B10B2A300B4A50 |

- 8.6.2.1.3 Subchannel Slot concatenation

The encoding block size shall depend on the number of subchannels slots allocated and the modulation specified

for the current transmission.  Concatenation of a number of subchannels slots shall be performed in order to make larger blocks of coding where it is possible, with the limitation of not passing exceeding the largest supported block under the same size for the applied modulation and coding rate (the block defined by 64-QAM modulation).  specifies the concatenation of subchannels slots for different allocations and modulations.

For any modulation and code rate, given an allocation of n subchannels slots, we define the following parameters:

— j: parameter dependent on the modulation and FEC rate

— n: number of allocated subchannels slots

— k: floor (n / j)

— m: n mod j

shows the rules used for subchannels slots concatenation.

- Table 256 —Encoding subchannels slots concatenation for different allocations and modulations

| Modulation and Rate | j |
| --- | --- |
| QPSK 1/2 | 12 |
| QPSK 2/3 | 9 |
| QPSK 3/4 | 8 |
| QPSK 5/6 | 7 |
| 16-QAM 1/2 | 6 |
| 16-QAM 2/3 | 4 |
| 16-QAM 3/4 | 4 |
| 16-QAM 5/6 | 3 |
| 64-QAM 1/2 | 4 |
| 64-QAM 2/3 | 3 |
| 64-QAM 3/4 | 2 |
| 64-QAM 5/6 | 2 |

- Table 257 — Subchannels Slots concatenation rule

| Number of subchannels slots | Subchannels Slots concatenated |
| --- | --- |
| n ≤ j | 1 block of n subchannels slots |
| n > j | If (n mod j =0) k blocks of j slots else (k-1) blocks of j subchannels slots 1 block of ceil((m+j)/2) subchannels slots 1 block of floor((m+j)/2) subchannels slots |

defines the basic sizes of the useful data payloads (in bytes) to be encoded in relation with the selected modulation type and encoding rate and concatenation rule.

- Table 258 —Useful data payload for a subchannel block

| QPSK |  |  |  | 16 QAM |  |  |  | 64 QAM |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R=1/2 | R=2/3 | R=3/4 | R=5/6 | R=1/2 | R=2/3 | R=3/4 | R=5/6 | R=1/2 | R=2/3 | R=3/4 | R=5/6 |
| 3 |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  | 6 |  |  |  |  |  |  |  |
|  | 8 |  |  |  | 8 |  |  |  |  |  |  |
| 9 |  | 9 |  |  |  | 9 |  | 9 |  |  |  |
|  |  |  | 10 |  |  |  | 10 |  |  |  |  |
| 12 | 12 |  |  | 12 |  |  |  |  | 12 |  |  |
| 15 |  |  | 15 |  |  |  |  |  |  |  | 15 |
|  | 16 |  |  |  | 16 |  |  |  |  |  |  |
| 18 |  | 18 |  | 18 |  | 18 |  | 18 |  |  |  |
|  | 20 |  | 20 |  |  |  | 20 |  |  |  |  |
| 21 |  |  |  |  |  |  |  |  |  |  |  |
| 24 | 24 |  |  | 24 | 24 |  |  |  | 24 |  |  |
|  |  |  | 25 |  |  |  |  |  |  |  |  |
| 27 |  | 27 |  |  |  | 27 |  | 27 |  | 27 |  |
|  | 28 |  |  |  |  |  |  |  |  |  |  |
| 30 |  |  | 30 | 30 |  |  | 30 |  |  |  | 30 |
|  | 32 |  |  |  | 32 |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 35 |  |  |  |  |  |  |  |  |
| 36 | 36 | 36 |  | 36 |  | 36 |  | 36 | 36 |  |  |

References:

[1] IEEE 802.22-08/0xxxr0, “Useful data payload per block for convolutional coding,” C. Kim et al., March 2008.