IEEE P802.22
Wireless RANs

| Suggested Text Changes for Sections 6.9.4.1.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Huawei & HKUST | Department of Electronic and Computer Engineering, HKUST, Clear Water Bay, Hong Kong | 852-2358-8027 | ks.au@ieee.org |
| Linjun Lv | Huawei Technologies | Shenzhen, China | 0086-755-28973119 | lvlinjun@huawei.com |

Abstract

This contribution provides suggested text changes for Section 6.9.4.1.2 and Section 6.9.15 of the Working Document v0.5.0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Text for Section 6.9.4.1.2 US-MAP Extended UIUC IE

A US-MAP IE entry with a UIUC value of 62 indicates that the IE carries special information and conforms to the structure shown in Table 64. A BS/CPE shall ignore an extended IE entry with an extended UIUC value for which it has no knowledge. In the case of a known extended UIUC value but with a length field longer than expected, it shall process information up to the known length and ignore the remainder of the IE.

- Table 64 —US-MAP extended IE general format

| Syntax | Size | Notes |
| --- | --- | --- |
| US_Extended_IE() { |  |  |
| Extended UIUC | 6 bits | Values specific to the Extended IE |
| Length | 4 bits | Length is the number of octets following this field – range = 0-15 octets |
| Unspecified Data | Variable |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

- Text for Section 6.9.4.1.2.1 US-MAP Power Control IE

When a power change for the CPE is needed, the extended UIUC is used with the subcode set to 0x00 as shown in Table 65. The power control value is an 8-bit signed integer expressing the change in power level (in 0.5 dB units) that the CPE should apply to correct its current transmission power. The CID used in the IE shall be the Basic CID of the CPE.

- Table 65 —US-MAP power control IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Power_Control_IE() { |  |  |
| Extended UIUC | 6 bits | Fast power control = 0x00 |
| Length | 4 bits | Length is the number of octets following this field=0x01 |
| Padding nibble | 6 bits | Padding to octet alignment – set all 6 bits to 0 |
| Power Control | 8 bits | Signed integer which expresses the change in power level (in 0.5 dB units) that the CPE should apply to correct its current transmission power |

- Text for Section 6.9.4.1.2.3 US-MAP Dummy IE

A CPE shall be able to decode the US-MAP Dummy IE. A BS shall not transmit this IE (unless under test). The Length field of Table 67 specifies the size of the Unspecified Data Field.

- Table 67—US-MAP dummy IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Dummy_IE() { |  |  |
| Extended UIUC | 6 bits | 0x03 |
| Length | 4 bits | Length is the number of octets following this field – range = 0-15 octets |
| Unspecified Data | Variable |  |
| If (!byte_boundary) |  |  |
| Padding Nibble | 0-7 bits |  |
| } |  |  |

- Text for Section 6.9.15.1 CBC-REQ

The CPE CBC-REQ shall be transmitted by the CPE during initialization. A CPE shall generate CBC-REQ messages in the form shown in Table 138. Basic Capability Requests contain those CPE capabilities IEs that are necessary for effective communication with the CPE during the remainder of the initialization protocols. Only the following parameters shall be included in the Basic Capabilities Request, namely the Physical Parameters Supported (6.9.15.3.1) and the Bandwidth Allocation Supported (Section 6.9.15.3.3). Capabilities for Construction and Transmission of MAC PDUs (Section 6.9.15.3.2) may be supported if needed.

- Table 138—CBC-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CBC-REQ_Message_Format() { |  |  |
| Management Message Type = 26 | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found |
| } |  |  |

- Text for Section 6.9.15.2 CBC-RSP

The CPE CBC-RSP shall be transmitted by the BS in response to a received CBC-REQ. The following parameters shall be included in the CBC-RSP if found in the CPE CBC-REQ, namely the Physical Parameters Supported (6.9.15.3.1), the Bandwidth Allocation Supported (Section 6.9.15.3.3), and the Capabilities for Construction and Transmission of MAC PDUs (Section 6.9.15.3.2). In addition, the BS responds to the subset of CPE capabilities present in the CBC-REQ message. The BS responds to the CPE capabilities to indicate whether they may be used. If the BS does not recognize a CPE capability, it may return this as “off” in the CBC-RSP. Only capabilities set to “on” in the CBC-REQ may be set “on” in the CBC-RSP, as this is the handshake indicating that they have been successfully negotiated.

- Table 139 —CBC-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| CBC-RSP_Message_Format() { |  |  |
| Management Message Type = 27 | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found and Error: Reference source not found |
| } |  |  |