IEEE P802.22
Wireless RANs

| Text on Pilot Tone Based OFDM Detector – For Informative Annex on Sensing Techniques |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hou-Shin Chen | Thomson | Two Independence Way Princeton, NJ 08540 |  | hou-shin.chen@thomson.net |
| Wen Gao | Thomson | Two Independence Way Princeton, NJ 08540 |  | wen.gao@thomson.net |

Abstract

This document contains the text on the pilot tone based OFDM detector for the informative Annex on sensing techniques.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

Spectrum sensing is one of the core technologies of Cognitive Radio (CR) systems which provide a viable solution to the problem of sparsity of wireless spectrum. Nowadays, OFDM techniques are adopted by many existing or progressing wireless communication standards. Thus, a robust spectrum sensing algorithm for OFDM modulated signals is highly desired to implement CR when the primary signal uses OFDM modulation. Motivated by this demand, a Time-Domain Symbol Cross-Correlation based spectrum sensing algorithm (TDSC method) is presented in this text. The algorithm makes use of the property that the mean of the TDSC function of two OFDM symbols is not zero if they have embedded the same frequency-domain pilot tones. We use the DVB-T Standard [1] as an application example to illustrate the proposed spectrum sensing algorithm. The simulated channel environments are the Ricean and Rayleigh channels defined in the DVB-T Standard and the additive white Gaussian noise (AWGN) channel. Four CP ratios defined in the DVB-T Standard are simulated for the probability of false alarm set equal to 0.01 and the sensing time set equal to 50 ms and 5 ms. Simulation results show that the TDSC method can achieve a misdetection probability of 0.1 when the SNR equals -20.5 dB and -11.5 dB for all four CP ratios when the sensing times are 50 ms and 5 ms respectively.

# Statistical Development of the Cross-Correlation of Two OFDM Symbols

Under the assumption that L, the length of the Cyclic Prefix (CP), is longer than the length of the time-invariant channel, the nth sample of the lth OFDM symbol can be modeled as

N−1H[k]Xl[k]ej2πknN+wl[n]	(1)

where fΔ is the carrier frequency offset normalized to the subcarrier spacing. The phase θl=2πfΔlMN+θ0 is the initial phase of the lth OFDM symbol where M=N+L is the length of an OFDM symbol. The parameter N is the number of subcarriers, and Xl[k] which is taken from a finite complex alphabet constellation denotes the data symbols at the kth subcarrier of the lth OFDM symbol. Moreover, H[k] is the complex channel gain of the kth subcarrier and wl[n] is a sample of a complex additive white Gaussian noise (AWGN) process. We will assume that wl[n] is a circularly symmetric complex Gaussian random variable which has zero-mean and a variance of σw2N. Most of the existing standards which adopt OFDM modulation [1][2][3] allocate pilot symbols in the frequency domain and these pilot symbols are called pilot tones. Let Pa, a=0,1,…A−1, denote the sets of all possible pilot tone positions for the transmitted OFDM symbols. Assume that Pa is the set of pilot tone positions of the lth OFDM symbol and Xl[k]=Pa[k] for k∈Pa. Here, we should note that the pilot symbols Pa[k] are predefined and have the same amplitude. For most cases, Pa[k] is a fixed constant and in some cases they change sign. Assume that the lth and mth OFDM symbols have the same pilot tone positions and define

N−1xl[n]xm[n]	(2)

which is the Time-Domain Symbol Cross-Correlation (TDSC) function of two OFDM symbols. After some calculations and reasonable approximations, it can be shown that

N−1wl[n]wm[n].	(3)

The function e(ϕ)=ej2πϕMfΔN represents a phase rotation caused by the carrier frequency offset. Note that from (3), R(l,m) simply consists of a constant term and a noise term. The fact that the mean value of R(l,m) is not zero makes it different from noise, and we are able to exploit this property to perform spectrum sensing.

# TDSC Based Spectrum Sensing Algorithm

Let ν=l−m be the symbol index difference of two OFDM symbols. Note that in all OFDM standards, any two OFDM symbols which have their symbol index difference equal to ν have the same pilot tone positions. Let’s further define C(ν) as the accumulated TDSC function, i.e.,

N−1wl[n]wm[n]	(4)

where Sν is the number of R(l,m) which are accumulated and added. Here Sν is selected to be an integer multiple of A. We can see from (4) that the mean of C(ν) is unchanged no matter how many TDSC functions are accumulated. However, the variance of the noise term (second term) in C(ν) is inversely proportional to Sν. Therefore, as long as Sν, the accumulated number of R(l,m) is large enough, the noise term in C(ν) will be significantly reduced. Due to this property, we are able to perform spectrum sensing in very low SNR environments. For the convenience of derivation and readability, we rewrite C(ν) as

C(ν)=e(ν)Λ+ξ(ν)	(5)

where

|H[k]|2	(6)

is the average received signal power in the pilot tone positions divided by N2 and

ξ(ν):CN(0,σw4SνN3)	(7)

is a circularly symmetric complex Gaussian random variable. In addition, ξ(ν) and ξ(μ) are independent for ν≠μ. Note that because of the carrier frequency offset, there is a phase term e(ν) in (5) which is a function of ν=m−l. As a result, we cannot linearly combine C(ν) for different ν. In order to solve this problem, let

Q(ν,ν+d)=C(ν)C(ν+d)  =e(−d)Λ2+Λ⋅(ξ(ν)+ξ(ν+d))+ ξ(ν)ξ(ν+d)	(8)

which is the conjugate product of two accumulated TDSC functions. Then the phase term embedded in Q(ν,ν+d) becomes a function of d, and hence, we can linearly combine Q(ν,ν+d) for different ν. As a result, let Γ be the linear combination of Q(ν,ν+d)

aνQ(ν,ν+d)	(9)

where aν is a combining ratio. The problem arises as to how the aν should be chosen so as to achieve the best detection performance. Here, we use an intuitive criterion. That is, we choose aν such that the Kullback-Leibler divergence [4] is maximized for two hypothesis H1 and H0. Under this criterion and when the SNR is very low, the combining ratio is obtained by

aν=SνSν+d.	(10)

Before defining the decision statistic used for performing spectrum sensing, we should note that the lack of symbol timing information has not been considered in our derivation. When symbol timing is lacking, the usual approach is to try all possible symbol timing instances in order to compute (9). Then use the resulting maximum amplitude as the decision statistic. Due to the CP nature of the OFDM signal, our previous derivations are valid as long as the starting sample time instance is taken from any point within an intersymbol interference (ISI) free region [5]. Suppose that the maximum channel delay is D, then the length of the ISI free region is L−D+1. Thus, if we search over N+LL−D+1 points which are equally spaced by L−D as the starting sample time instances, there must be at least one point in the ISI free region. The function b is the smallest integer which is larger than or equal to b. Typically, we don't know the maximum channel delay D when we are performing spectrum sensing. Consequently, let Z=N+LL, and then use the Z points which are separated by L−1 as starting sample time instances. Although this suboptimal approach will introduce some ISI effect when none of the Z points are in the ISI free region, the detection performance will not be degraded too much since the ISI introduced is small when the CP length is much larger than the root mean-square (RMS) delay-spread of the wireless channel. Consequently, we use these Z points as starting sample time instances to compute (9) and use the maximum amplitude as the decision statistic. Hence, the decision statistic is defined as

TTDSC=maxn0|Γ(n0)|	(11)

where Γ(n0) is given by (9) and we use n0 as the starting sample time instance. The approach of performing spectrum sensing by computing Time-Domain Symbol Cross-Correlation function R(l,m) can be easily applied to any OFDM system employing pilot tones. However, the pilot tone patterns used in various standards are different. Thus, the spectrum sensing algorithms might be slightly different. In the next section, we use the DVB-T Standard as an example and describe how to perform spectrum sensing for DVB-T OFDM systems. Through this example, the spectrum sensing algorithm for other OFDM systems which embed pilot tones can be easily developed.

# Spectrum Sensing for DVB-T OFDM Systems

According to [1], every transmitted OFDM symbol contains two kinds of pilot tones. One is that of acontinued pilot and the other is that of a scattered pilot. The positions of continued pilots are the same for all transmitted OFDM symbols. The scattered pilots are inserted every twelve subcarriers and their positions are shifted by three subcarriers for the next OFDM symbol so that the positions of scattered pilots are repeated every four OFDM symbols, hence we have that

Pa,scatter=k|k=12l+3(a+1)	(12)

for l=0,1,…, and a=0,1,2,3. Therefore, there are four sets of pilot tone patterns for DVB-T OFDM. We should note that the number of scattered pilots is much larger than the number of continued pilots. For a 2K-subcarrier mode, there are 45 continued pilot tones and 141 scattered pilot tones in an OFDM symbol. Therefore, we shall compute C(ν) for the case where ν is a multiple of four, except zero, because by doing so, the absolute mean value of C(ν) is maximized. The decision statistic is given by (9) where Γ(n0) is defined by

KS4kS4k+4Q(4k,4k+4)	(13)

and n0 is used as the starting sample time instance.

# Simulation Results

The performance of the spectrum sensor for the OFDM signals employing frequency-domain pilot tones is demonstrated by computer simulation. The simulation environments are AWGN, multipath Rayleigh fading, multipath Ricean channels specified in [1]. The performances on misdetection probability are evaluated for a false alarm probability equal to 0.01. The sensing time of 50 ms and 5 ms are simulated for four CP ratios defined in [1]. From Tables 1, we can see that the TDSC method can achieve a misdetection probability of 0.1 when SNR equals -20.5 dB for four CP ratios when the sensing time is set to 50 ms. As for a sensing time of 5 ms, the required SNR is -11.5 dB for achieving a misdetection probability of 0.1. Furthermore, the performance of the TDSC method is not related to the  CP ratios.

| Sensing Time/ CP Ratio | 1/4 | 1/8 | 1/16 | 1/32 |
| --- | --- | --- | --- | --- |
|  | Required SNR (dB) |  |  |  |
| 50 ms | -20.5 | -20.5 | -20.7 | -20.7 |
| 5 ms | -11.5 | -11.5 | -11.5 | -11.5 |

Table 1 Detection performance of TDSC method.

References

- ETSI, "Digital Video Broadcasting: Framing Structure, Channel Coding, and Modulation for Digital Terrestrial Television," European Telecommunication Standard EN300744, August 1997.

- ETSI, "Digital Video Broadcasting (DVB); Transmission System for Handheld Terminals (DVB-H)," European Telecommunication Standard EN302304 1.1.1, November 2004

- IEEE Standard, "IEEE Standard for Information Technology-Telecommunications and Information Exchange Between Systems-Local and Metropolitan Area Networks-Specific Requirements - Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications," June 2007.

- T. M. Cover and J. A. Thomas, "Elements of Information Theory," Wiley Series in Telecommunications, John Wiley & Sons, Inc., 1991.

- C. Sheu and C. Huang, "A Novel Guard Interval Based ISI-Free Sampling Region Detection Method for OFDM Systems,"  IEEE VTC, Vol. 1, pp. 515-519, September 2004.