IEEE P802.22
Wireless RANs

| Available TV Channel and WRAN Services List SME-MLME Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

The proposed Available TV Channel List and the Available WRAN Services List SME-MLME Primitives are presented in this document.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

  - 9.5  BS Reports the Resulting Available TV Channel List

The BS MLME SM occasionally sends the available TV channel list to its SME for additional channel classification.  The available TV channel list can be presented to its SME to have TV channels classified as disallowed.  The classification of an operating TV channel by the BS is also performed by its SME.  The SME-MLME-SAP is an interface that provides a means of exchange information between the SM and the SME in the BS.  Table 1 summarizes the primitives supported by the MLME to pass the available TV channel list and to receive disallowed TV channel classifications and the selected operating TV channel through the SME-MLME-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Available TV Channel List Primitives supported by the SME-MLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| SME-MLME-AVAIL-TV-CH-REPORT | 9.5.1 |  | 9.5.2 |
| SME-MLME-DISALLOWED-TV-CHS |  | 9.5.3 |  |
| SME-MLME-OPERATING-TV-CH |  | 9.5.4 |  |

    - SME-MLME-AVAIL-TV-CH-REPORT.request

The SME-MLME-AVAIL-TV-CH-REPORT.request primitive is sent by the BS SM in the MLME to request the SME for a selection of an operating TV channel based on the available TV channel list information provided this primitive.  Table 2 specifies the parameters for the SME-MLME-AVAIL-TV-CH-REPORT.request primitive.

- — SME-MLME-AVAIL-TV-CH-REPORT.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1 to Number of Channels Avaliable, i++) { Channel_Number Maximum Allowed EIRP } | List of available TV channels and their Maximum Allowed EIRP |  | List of available TV channels and corresponding Maximum Allowed EIRP. |
| Mode |  |  | The expected response from the SME 0 = Test 1 = Request for disallowed TV channel classification 2 = Request for selection of operating TV channel |

      - When generated

The SME-MLME-AVAIL-TV-CH-REPORT.request primitive is generated by the BS SM in the MLME and issued to the SME either (depending on the mode) to request disallowed TV channel classification or selection of an operating TV channel during BS initialization as described in Section 6.16.1.

      - Effect on receipt

When the SME receives the SME-MLME-AVAIL-TV-CH-REPORT.request primitive, it generates a SME-MLME-AVAIL-TV-CH-REPORT.confirm primitive to notify the MLME if the request was successfully received.

    - SME-MLME-AVAIL-TV-CH-REPORT.confirm

The SME-MLME-WRAN-SERVICE-REPORT.confirm primitive allows the SME to inform the SM if the receipt of the available TV channel list was successful. Table 3 specifies the parameters for the SME-MLME-AVAIL-TV-CH-REPORT.confirm primitive.

- — SME-MLME-WRAN-SERVICE-REPORT.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the request to select a WRAN service was successfully generated. |

      - When generated

The SME-MLME-AVAIL-TV-CH-REPORT.confirm primitive is generated by the SME and issued to its MLME when a SME-MLME-AVAIL-TV-CH-REPORT.request primitive is received to indicate whether the available TV channel list was successfully generated.

      - Effect on receipt

When the SM of a CPE receives the SME-MLME-AVAIL-TV-CH-REPORT.confirm primitive and depending on the mode, it expects the SME either to return nothing, an SME-MLME-DISALLOWED-TV-CHS.indication primitive with classified disallowed channels, or an SME-MLME-OPERATING-TV-CH.indication with the selected TV channel.

    - SME-MLME-DISALLOWED-TV-CHS.indication

The SME-MLME-DISALLOWED-TV-CHS.indication primitive is used by the SME to return the disallowed TV channels on the available TV channel list to the SM per its request.  Table 4 specifies the parameters for the SME-MLME-DISALLOWED-TV-CHS.indication primitive.

- — SME-MLME-DISALLOWED-TV-CHS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1 to Number of Channels Disallowed, i++) { Channel_Number } | List of disallowed TV channels |  | List of disallowed TV channels. |

      - When generated

The SME-MLME-DISALLOWED-TV-CHS.indication primitive is generated by the SME and issued to the MLME to indicate the disallowed channels from the available TV channel list.

      - Effect on receipt

When the SM receives the SME-MLME-DISALLOWED-TV-CHS.indication it will identify whether the response to its request for the SME to classify TV channels as disallowed from the available TV channel list was successfully received by the SME, in which case, the SM will obtain the classified disallowed TV channels and the BS will continue to the following steps of initialization and perform incumbent detection, described in Section 6.16.1.5. If the response is not successful the SM may decide to issue another request.

    - SME-MLME-OPERATING-TV-CH.indication

The SME-MLME-OPERATING-TV-CH.indication primitive is used by the SME to return the selected operating TV channel on the available TV channel list to the SM per its request.  Table 5 specifies the parameters for the SME-MLME-DISALLOWED-TV-CHS.indication primitive.

- — SME-MLME-OPERATING-TV-CH.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel_Number | The selected operating channel | 0-80 | The selected operating channel |

      - When generated

The SME-MLME-OPERATING-TV-CH.indication primitive is generated by the SME and issued to the MLME to indicate the selected operating TV channel from the available TV channel list.

      - Effect on receipt

When the SM receives the SME-MLME-OPERATING-TV-CH.indication it will identify whether the response to its request for the SME to select the operating TV channel from the available TV channel list was successfully received by the SME, in which case, the SM will obtain the selected operating TV channel and the BS will continue to commence operation on the selected channel. If the response is not successful the SM may decide to issue another request.

- 9.6  CPE Reports the Resulting Available WRAN Services List

The selection of WRAN service by the CPE is performed by its SME.  The SME-MLME-SAP is an interface that provides a means of exchange information between the SA and the SME.  Table 6 summarizes the primitives supported by the MLME to pass the available WRAN services list and the selcted WRAN service through the SME-MLME-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Available WRAN Sevices List Primitives supported by the SME-MLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| SME-MLME-WRAN-SERVICE-REPORT | 9.5.1 |  | 9.5.2 |
| SME-MLME-WRAN-SERVICE-RESPONSE |  | 9.5.3 |  |

    - SME-MLME-WRAN-SERVICE-REPORT.request

The SME-MLME-WRAN-SERVICE-REPORT.request primitive is sent by the CPE SA in the MLME to request the SME for a selection of a WRAN service based on the available WRAN service list information provided this primitive.  Table 7 specifies the parameters for the SME-MLME-WRAN-SERVICE-REPORT.request primitive.

- — SME-MLME-WRAN-SERVICE-REPORT.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1 to Number of Channels Avaliable, i++) { WRAN service Channel_Number RSSI } | List of available WRAN services, the TV channel, and the received signal strength |  | List of available WRAN services, corresponding TV channel, and received signal strength. |

      - When generated

The SME-MLME-WRAN-SERVICE-REPORT.request primitive is generated by the CPE SA in the MLME and issued to the SME to request a selection of a WRAN service during CPE initialization as described in Section 6.16.2.4.

      - Effect on receipt

When the SME receives the SME-MLME-WRAN-SERVICE-REPORT.request primitive, it generates a SME-MLME-WRAN-SERVICE-REPORT.confirm primitive to notify the MLME if the request was successfully received.

    - SME-MLME-WRAN-SERVICE-REPORT.confirm

The SME-MLME-WRAN-SERVICE-REPORT.confirm primitive allows the SME to inform the SA if the request to select an available WRAN service was successful. Table 38 specifies the parameters for the SME-MLME-WRAN-SERVICE-REPORT.confirm primitive.

- — SME-MLME-WRAN-SERVICE-REPORT.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the request to select a WRAN service was successfully generated. |

      - When generated

The SME-MLME-WRAN-SERVICE-REPORT.confirm primitive is generated by the SME and issued to the MLME when a SME-MLME-WRAN-SERVICE-REPORT.request primitive is received to indicate whether the request to select a WRAN service was successfully generated.

      - Effect on receipt

When the SA of a CPE receives the SME-MLME-WRAN-SERVICE-REPORT.confirm primitive, it expects the SME to return an SME-MLME-WRAN-SERVICE-RESPONSE.indication primitive with a selected WRAN service.

    - SME-MLME-WRAN-SERVICE-RESPONSE.indication

The SME-MLME-WRAN-SERVICE-RESPONSE.indication primitive is used by the SME to return a selected WRAN channel from the available WRAN service list to the SA per its request.  Table 9 specifies the parameters for the SME-MLME-WRAN-SERVICE-RESPONSE.indication primitive.

- — SME-MLME-WRAN-SERVICE-RESPONSE.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Selected Channel Number | Integer | 0-80 | The value identifies the selected channel of the WRAN service. |

      - When generated

The SME-MLME-WRAN-SERVICE-RESPONSE.indication primitive is generated by the SME and issued to the MLME to indicate the selected channel from the available WRAN service list.

      - Effect on receipt

When the SA receives the SME-MLME-WRAN-SERVICE-RESPONSE.indication it will identify whether the response to its request for the SME to select a channel from the available WRAN service list was successfully received by the SME, in which case, the SA will obtain the selected channel and CPE will continue to the following steps of initialization. If the response is not successful the SA may decide to issue another query.

References: