IEEE P802.22
Wireless RANs

| Text of Cooperative spectrum sensing inter cell |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jinnan Liu | Huawei Hisi | Beijing, China | 86-10-8282-9148 | liujinnan@huawei.com |
| Shulan Feng | Huawei Hisi | Beijing, China | 86-10-8282-9151 | fengsl@huawei.com |
| Edward Au | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68745666 | ks.au@ieee.org / ksau@i2r.a-star.edu.sg |

Abstract

The proposed document supports cooperative spectrum sensing inter cell according to comment #745.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Comment #745

Spectrum Manager or Spectrum Automatic shall control and coordinate spectrum sensing not only within the cell, but also inter-cell.

- Why should we do cooperative sensing inter-cell? Basic question why we need cooperative sensing? If this question is yes, this proposal is just to extend cooperative sensing from intra-cell to inter-cell. As we know, broadcast and multicast measurement request-response process permits cooperative measurement intra-cell. Furthermore, the overlap area measurements are useful for neighbor cells.

- For wireless microphone, one cell lack of CPE to do sensing in overlap area, and the neighbor cell has a valid measurement in overlap area.

- For saving resource on sensing, if one cell has a valid measurement in overlap area, the neighbor cell may request for cooperative measurement inter-cell, instead of doing measurement itself.

- 2. Recommend modified text in 9.2.1

3. Spectrum sensing: the SM shall use the MAC and PHY layer functionalities and management frames to control and coordinate spectrum sensing within the WRAN cell. Spectrum Manager or Spectrum Automatic shall control and coordinate spectrum sensing not only intra-cell, but also inter-cell. The measurements reports in overlap area of two cells may be transported by CBP IE (CM-REQ, CM-RSP, CM-REP, and CM-ACK) for cooperative measurement purpose.

- 3. Recommend adding text in 6.21.1.2

Figure 97  Measurement message flow between BS and CPE intra-cell

- XX.  Cooperative Measurement message flow inter-cell

- Figure xx    Cooperative Measurement (CM) message flow inter-cell

The CBP implements a cooperative measurement philosophy by four types of message, namely, CM-REQ, CM-RSP, CM-REP, and CM-ACK (see Table 6).

Cooperative WRAN1 entity (BS/CPE) should request cooperative measurement by CM_REQ, the cooperative WRAN2 entity (BS/CPE) may response by CM_RSP. If the cooperative WRAN2 entity (BS/CPE) accepted CM_REQ from cooperative WRAN2, the entity (BS/CPE) shall forward measurement reports in overlap area by CM_REP and the cooperative WRAN2 entity (BS/CPE) may confirm ACK/NACK by CM_ACK. The forwarding measurement reports according to CM_REQ are the valid measurement reports which are done by cooperative WRAN1 according to BLM_REQ.

- 4.  Recommend modified text in 6.7.1.2.1

- 6.7.1.2.1  CBP Information IE

- Table 6— CBP IEs

| Element ID | Name |
| --- | --- |
| 0 | Beacon IE |
| 1 | DS/US Boundary IE |
| 2 | BS IP Address IE |
| 3 | Inter-BS Capabilities IE |
| 4 | CC_REQ IE |
| 5 | CC_REP IE |
| 6 | CC_ACK IE |
| 7 | RR-REQ IE |
| 8 | RA-RSP IE |
| 9 | RA-ACK IE |
| 10 | RC-REQ IE |
| 11 | RC-RSP IE |
| 12 | RC-ACK IE |
| 13 | RE-REQ IE |
| 14 | RE-RSP IE |
| 15 | RE-ACK IE |
| 16 | RS-SEM IE |
| 17 | RS-ADV IE |
| 18 | BS Channel Parameter IE |
| 19 | CBP Location IE |
| 20 | CBP Geolocation IE |
| 21 | CM_REQ IE |
| 22 | CM_RSP IE |
| 23 | CM_REP IE |
| 24 | CM_ACK IE |

- xx.1 CM_REQ IE

The CM_REQ shall support two measurement report modes, zero indicates the compressed mode,one indicates the detail mode. The channel compress status indicates any signal is occupied or no signal is occupied, while channel detail status is which signal is occupied or unoccupied the channel.

- Table xxx  CM_REQ IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CM_REQ_Format(){ |  |  |
| Element ID | 8 |  |
| Channel Number | 8 |  |
| Measurement Report Mode | 1 | 0 indicates the channel compress status mode 1 indicates the Channel detail status mode |
| Confirmation Needed | 1 |  |
| } |  |  |

- xx.2 CM_RSP IE

The CM_REQ is sended according to CM_REQ comfirmation needed parameter.

- Table xxx  CM_RSP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CM_ACK_Format() { |  |  |
| Element ID | 8 bits |  |
| Confirmation Code | 8 bits |  |
| } |  |  |

- xx.3 CM_REP IE

The CM_REP may send the channel status in channel compress status as well as channel detail status.

- Table xxx  CM_REP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CM_REP_Format(){ |  |  |
| Element ID | 8 |  |
| Channel No. | 8 |  |
| Confirmation Code | 8 | Table 103 |
| Measurement Report Mode | 1 |  |
| If(Measurement Report Mode==0) { |  |  |
| channel compress status} | 1 | 0 indicates no any signal is found 1 indicates any signal is fournd |
| else{ |  |  |
| Channel detail status | 8 | Single present array (Table 277) |
| } |  |  |
| Duration | 16 | Measurement duration |
| } |  |  |

- xx.4 CM_ACK IE

The CM_ACK is sended according to CM_REP comfirmation needed parameter.

- Table xxx  CM_ACK IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CM_ACK_Format() { |  |  |
| Element ID | 8 bits |  |
| Confirmation Code | 8 bits | Table 103 |
| } |  |  |

References:

[1] Mishra, S.M.   Sahai, A.   Brodersen, R.W.  Cooperative Sensing among Cognitive Radios，ICC2006

[2]

” Exploiting cooperation among multiple users may be the only mechanism to achieve a target system-level probability of detection in the case when each Cognitive radio faces an SNRwall below which it is unable to reliably detect a Primary.”[1]