IEEE P802.22
Wireless RANs

| Supplement to Section 6.9.8 (Editorial) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-06-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68745666 | ks.au@ieee.org / ksau@i2r.a-star.edu.sg |
| Zander Lei | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68745686 | leizd@i2r.a-star.edu.sg |

Abstract

This contribution provides the resolution to the editorial comment #226 and the technical comment #227, which is the text description for Section 6.9.8 of the Draft v1.0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This contribution provides the resolution to the editorial comment #226. The additional texts are highlighted in blue.

      - DSA-RSP

- A DSA-RSP message shall be generated in response to a received DSA-REQ message. The format of a DSA-RSP message shall be as shown in Table 93. If the transaction is successful, the DSA-RSP message may contain the following: Service Flow Parameters (The complete specification of the service flow shall be included in the DSA-RSP if it includes a newly assigned CID or an expanded service class name) and CS Parameter Encodings (Specification of the service flow’s CS-specific parameters).

- Table 93 — DSA-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSA-RSP_Message_Format() { |  |  |
| Management Message Type = 12 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSA-ACK

A DSA-ACK message shall be generated in response to a received DSA-RSP message. The format of a DSA-ACK message shall be as shown in Table 94.

- Table 94 — DSA-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSA-ACK_Message_Format() { |  |  |
| Management Message Type = 13 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSC-REQ

A DSC-REQ message is sent by a CPE or BS to dynamically change the parameters of an existing service flow. A CPE or BS shall generate DSC-REQ messages in the form shown in Table 95. Note that a DSC-REQ message shall not carry parameters for more than one service flow. A DSC-REQ message shall contain Service Flow Parameters which specifies the service flow’s new traffic characteristics and scheduling requirements. The admitted and active QoS parameter sets currently in use by the service flow. If the DSC message is successful and it contains service flow parameters, but does not contain replacement sets for both admitted and active QoS parameter sets, the omitted set(s) shall be set to null. The service flow parameters shall contain a SFID.

- Table 95 — DSC-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSC-REQ_Message_Format() { |  |  |
| Management Message Type = 14 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSC-RSP

A DSC-RSP shall be generated in response to a received DSC-REQ. The format of a DSC-RSP shall be as shown in Table 96. If the transaction is successful, the DSC-RSP may contain the Service Flow Parameters in which the complete specification of the service flow shall be included in the DSC-RSP only if it includes a newly assigned CID or an expanded service class name. If a service flow parameter set contained an upstream admitted QoS parameter set and this service flow does not have an associated CID, the DSC-RSP shall include a CID. If a service flow parameter set contained a service class name and an admitted QoS parameter set, the DSC-RSP shall include the QoS parameter set corresponding to the named service class. If specific QoS parameters were also included in the classed service flow request, these QoS parameters shall be included in the DSC-RSP instead of any QoS parameters of the same type of the named service class. Additionally, the DSC-RSP may contain the CS Parameter Encodings which specifies the CS-specific parameters of the service flow.

- Table 96 — DSC-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSC-RSP_Message_Format() { |  |  |
| Management Message Type = 15 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSC-ACK

A DSC-ACK shall be generated in response to a received DSC-RSP. The format of a DSC-ACK shall be as shown in Table 97.

- Table 97 – DSC-ACK message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSC-ACK_Message_Format() { |  |  |
| Management Message Type = 16 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSD-REQ

A DSD-REQ is sent by a CPE or BS to delete an existing service flow. The format of a DSD-REQ shall be as shown in Table 98.

- Table 98 — DSD-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSD-REQ_Message_Format() { |  |  |
| Management Message Type = 17 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Service Flow ID | 32 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSD-RSP

A DSD-RSP shall be generated in response to a received DSD-REQ. The format of a DSD-RSP shall be as shown in Table 99.

- Table 99 — DSD-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSD-RSP_Message_Format() { |  |  |
| Management Message Type = 18 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| Service Flow ID | 32 bits |  |
| Information Elements (IEs) | Variable | Error: Reference source not found |
| } |  |  |

      - DSx-RVD

The DSX-RVD message shall be generated by the BS in response to a CPE-initiated DSx-REQ to inform the CPE that the BS has received the DSx-REQ message in a more timely manner than provided by the DSx-RSP message, which shall be transmitted only after the DSx-REQ is authenticated. The format of the DSX-RVD shall be as shown in Table 100.

- Table 100 — DSx-RVD message format

| Syntax | Size | Notes |
| --- | --- | --- |
| DSX-RVD_Message_Format() { |  | The DSX-RVD message shall be generated by the BS in response to a CPE-initiated DSx-REQ to inform the CPE that the BS has received the DSx-REQ message in a timelier manner than provided by the DSx-RSP message, which shall be transmitted only after the DSx-REQ is authenticated. |
| Management Message Type = 30 | 8 bits |  |
| Transaction ID | 16 bits |  |
| Confirmation Code | 8 bits |  |
| } |  |  |

      - Service Flow Encodings

Table 101 and the subsequent subclauses define the parameters associated with upstream/downstream scheduling for a service flow. Note that the encapsulated upstream and downstream flow classification configuration setting strings share the same subtype field numbering plan because many of the subtype fields defined are valid for both types of configuration settings except service flow encodings. Upstream encodings use the type 145. Downstream encodings use the type 146. Entries of the form [145/146] indicate the encoding can be applied to either an upstream or downstream service flow.

- Table 101 — Service flow encodings

| Type | Parameter |
| --- | --- |
| 1 | Service Flow Identifier |
| 2 | CID |
| 3 | Service Class Name |
| 4 | reserved |
| 5 | QoS Parameter Set Type |
| 6 | Traffic Priority |
| 7 | Maximum Sustained Traffic Rate |
| 8 | Maximum Traffic Burst |
| 9 | Maximum Reserved Traffic Rate |
| 10 | Minimum Tolerable Traffic Rate |
| 11 | Service Flow Scheduling Type |
| 12 | Request/Transmission Policy |
| 13 | Tolerated Jitter |
| 14 | Maximum Latency |
| 15 | Fixed-length versus Variable-length SDU Indicator |
| 16 | SDU Size |
| 17 | Target SAID |
| 18 | ARQ Enable |
| 19 | ARQ_WINDOW_SIZE |
| 20 | ARQ_RETRY_TIMEOUT – Transmitter Delay |
| 21 | ARQ_RETRY_TIMEOUT – Receiver Delay |
| 22 | ARQ_BLOCK_LIFETIME |
| 23 | ARQ_SYNC_LOSS |
| 24 | ARQ_DELIVER_IN_ONRDER |
| 25 | ARQ_PURGE_TIMEOUT |
| 26 | ARQ_BLOCK_SIZE |
| 27 | Reserved |
| 28 | CS Specification |
| 29 | Maximum Tolerable Packet Loss Rate |
| 30-98 | Reserved |
| 99-107 | Convergence Sublayer Types |
| 143 | Vendor-specific QoS Parameter |
| 144-xxx | Reserved |

The CC indicates the status for the dynamic service (DSx) messages. The value appears in the Confirmation Code field of a DSx message. The CC values are specified in Table 102.

- Table 102 — Confirmation Code (CC) values

| CC | Status |
| --- | --- |
| 0 | OK/success |
| 1 | reject-other |
| 2 | reject-unrecognized-configuration-setting |
| 3 | reject-temporary / reject-resource |
| 4 | reject-permanent / reject-admin |
| 5 | reject-not-owner |
| 6 | reject-service-flow-not-found |
| 7 | reject-service-flow-exists |
| 8 | reject-required-parameter-not-present |
| 9 | reject-header-suppression |
| 10 | reject-unknown-transaction-id |
| 11 | reject-authentication-failure |
| 12 | reject-add-aborted |
| 13 | reject-exceed-dynamic-service-limit |
| 14 | reject-not-authorized-for-the-request-SAID |
| 15 | reject-fail-to-establish-the-requested-SA |
| 16 | reject-not-supported-parameter |
| 17 | reject-not-supported-parameter-value |

References: