IEEE P802.22
Wireless RANs

| Recommended Text for Security in 802.22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |
| Tom Kiernan | US Army (CERDEC) | Ft Monmouth, NJ | - | Thomas.kiernan@us.army.mil |
| Matt Sherman | BAE Systems | Wayne, NJ | 973-633-6344 | Matthew.sherman@baesystems.com |

Abstract

This document provides the recommended normative and informative text to be included in Section 7 of the 802.22 Standard. Based on the proposed changes of Section 7, other sections of the 802.22 draft standard require modifications. These other sections, along with their proposed changes have been included here for completeness.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

6. MAC Common Part Sublayer

6.8.1 Authentication tuples

An authentication tuple shall be the last item in identified management messages. When no authorization is negotiated between a CPE and a BS as an authorization policy, all authentication tuples (i.e., CMAC-Tuple, HMAC-Tuple, and short HMAC-Tuple) shall be omitted from MAC management messages unless otherwise required by the negotiated authorization policy support (see 11.8.4.2).

      - HMAC

This parameter contains the HMAC Key Sequence Number concatenated with an HMAC-Digest used for message authentication. The HMAC Key Sequence Number is stored in the four least significant bits of the first byte of the HMAC Tuple, and the most significant four bits are reserved.

This parameter contains the HMAC Key Sequence Number concatenated with an HMAC Digest used for message authentication. The HMAC Key Sequence Number is stored in the 4 LSBs of the first byte of the HMAC Tuple, and the 4 MSBs are reserved. The HMAC-Tuple attribute format is shown in Table (HMAC Tuple Definition) and Table (HMAC Tuple Value Field). When SCM is disabled, the content of this field shall be ignored and the message considered authenticated.

Table 34 - HMAC definition

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 149 | 21 | See Table HMAC value field | DSx-REQ, DSx-RSP, DSx-ACK, REG-REQ, REQ-RSP, RES-CMD, DREG-CMD, RFTP-CPLT |

Table 35 — HMAC value field

| Field | Length (bits) | Notes |
| --- | --- | --- |
| Reserved | 4 |  |
| HMAC Key Sequence Number | 4 |  |
| HMAC-Digest | 160 bits | HMAC with SHA-1 |

6.8.1.2 CMAC Tuple

The CMAC Tuple attribute format is shown in Table (CMAC Tuple Definition and Table (CMAC Tuple Value). A message received, that contains an CMAC Tuple, shall not be considered authentic if the length field of the tuple is incorrect, or if the locally computed value of the digest does not match the digest in the message.

NOTE—It would be appropriate for a MIB to increment an error count on receipt of a non-authentic message so that management can detect an active attack.

Table – CMAC Tuple Definition

REMOVE ALL MOB_X_X messages – Messages such as Mobile Sleep Request (MOB_SLP_REQ) are specific to 802.16e

Table – CMAC Tuple Values

The CMAC tuple is added to the RNG-REQ message only during handover, secure location update or network re-entry from idle mode. This tuple shall be included in the messages if the CPE and the BS share a valid AK.

6.8.1.3 Short-HMAC tuple

This parameter contains the HMAC Key Sequence Number concatenated with an HMAC Digest used for message authentication. The HMAC Key Sequence Number is stored in the 4 LSBs of the first byte of the HMAC Tuple, and the 4 MSBs are reserved. The HMAC Tuple attribute format is shown in Table (Short HMAC Tuple Definition) and Table (Short HMAC Tuple Values).

Table – Short HMAC Tuple Definition

REMOVE ALL MOB_X_X messages – Messages such as Mobile Sleep Request (MOB_SLP_REQ) are specific to 802.16e

Table – Short HMAC Tuple Values

- 6.9.28   Security Control Management (SCM) Messages (SCM-REQ/RSP)

SCM employs two MAC message types: SCM Request (SCM-REQ) and SCM Response (SCM-RSP), as described in .

Table 208— SCM MAC messages

| Type Value | Message name | Message description |
| --- | --- | --- |
| 9 | SCM-REQ | Security Control Management Request [CPE -> BS] |
| 10 | SCM-RSP | Security Control Management Response [BS -> CPE] |

These MAC management message types distinguish between SCM requests (CPE–to–BS) and SCM responses (BS–to–CPE). Each message encapsulates one SCM message in the Management Message Payload.

SCM protocol messages transmitted from the CPE to the BS shall use the form shown in . They are transmitted on the CPEs Primary Management Connection.

Table 209 — SCM-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| SCM-REQ_Message_Format() { |  |  |
| Management Message Type = 9 | 8 bits |  |
| Code | 8 bits | Identifies the type of SCM packet. When a packet is received with an invalid Code, it shall be silently discarded. The code values are defined in . |
| SCM Identifier | 8 bits | A CPE uses the identifier to match a BS response to the CPE’s requests. The CPE shall increment (modulo 256) the Identifier field whenever it issues a new SCM message. A “new” message is an Authorization Request or Key Request that is not a retransmission being sent in response to a Timeout event. For retransmissions, the Identifier field shall remain unchanged. The Identifier field in Authentication Information messages, which are informative and do not effect any response messaging, shall be set to zero. The Identifier field in a BS’s SCM-RSP message shall match the Identifier field of the SCM-REQ message the BS is responding to. The Identifier field in TEK Invalid messages, which are not sent in response to SCM-REQs, shall be set to zero. The Identifier field in unsolicited Authorization Invalid messages shall be set to zero. On reception of a SCM-RSP message, the CPE associates the message with a particular state machine (the Authorization state machine in the case of Authorization Replies, Authorization Rejects, and Authorization Invalids; a particular TEK state machine in the case of Key Replies, Key Rejects, and TEK Invalids). A CPE shall keep track of the identifier of its latest, pending Authorization Request. The CPE shall discard Authorization Reply and Authorization Reject messages with Identifier fields not matching that of the pending Authorization Request. A CPE shall keep track of the identifiers of its latest, pending Key Request for each SA. The CPE shall discard Key Reply and Key Reject messages with Identifier fields not matching those of the pending Key Request messages. |
| Encoded Attributes | variable | SCM attributes carry the specific authentication, authorization, and key management data exchanged between client and server. Each SCM packet type has its own set of required and optional attributes. Unless explicitly stated, there are no requirements on the ordering of attributes within a SCM message. The end of the list of attributes is indicated by the Length field of the MAC PDU header. |
| } |  |  |

SCM protocol messages transmitted from the BS to the CPE shall use the form shown in . They are transmitted on the CPEs Primary Management Connection.

Table 210— SCM-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| SCM- RSP_Message_Format() { |  |  |
| Management Message Type = 10 | 8 bits |  |
| Code | 8 bits |  |
| SCM Identifier | 8 bits |  |
| Encoded Attributes | variable |  |
| } |  |  |

Table 211— SCM message codes

| Code | SCM message type | MAC Management Message Name |
| --- | --- | --- |
| 0-2 | reserved |  |
| 3 | SCM RSA-Request | SCM-REQ |
| 4 | SCM RSA-Reply | SCM-RSP |
| 5 | SCM RSA-Reject | SCM-RSP |
| 6 | SCM RSA-Acknowledgement | SCM-REQ |
| 7 | SCM EAP Start | SCM-REQ |
| 8 | SCM EAP-Transfer | SCM-REQ/SCM-RSP |
| 9 | SCM Authenticated EAP-Transfer | SCM-REQ/SCM-RSP |
| 10 | SCM SA TEK Challenge | SCM-RSP |
| 11 | SCM SA TEK Request | SCM-REQ |
| 12 | SCM SA TEK Response | SCM-RSP |
| 13 | SCM Key-Request | SCM-REQ |
| 14 | SCM Key-Reply | SCM-RSP |
| 15 | SCM Key-Reject | SCM-RSP |
| 16 | SCM SA-Addition | SCM-RSP |
| 17 | SCM TEK-Invalid | SCM-RSP |
| 18 | SCM Group-Key-Update-Command | SCM-RSP |
| 19 | SCM EAP Complete | SCM-RSP |
| 20 | SCM Authenticate EAP Start | SCM-REQ |
| 21 | SCM Auth Invalid | SCM-RSP |
| 22 | SCM Auth Info | SCM-REQ |
| 23—255 | reserved |  |

Formats for each of the SCM messages are described in the following subclauses.

      - SCM RSA-REQ

A client CPE sends a SCM RSA-Request message to the BS in order to request mutual authentication in the RSA-based authorization.

Code: 3

Table 212— SCM RSA-Request attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64-bit random number generated in the CPE |
| CPE_Certificate | Contains the CPE s X.509 user certificate |
| SAID | CPE s primary SAID equal to the Basic CID |
| SigCPE | An RSA signature over all the other attributes in the message |

The CPE-certificate attribute contains an X.509 CPE certificate (see 7.6 xxx) issued by the CPE s manufacturer. The CPE s X.509 certificate is as defined in 6.3.2.3.9.2 xxx.

The SigCPE indicates an RSA signature over all the other attributes in this message, and the CPE’s private key is used to make an RSA signature.

      - SCM RSA-Reply

Sent by the BS to a client CPE in response to a SCM RSA-Request message, the SCM RSA-Reply message contains an encrypted pre-PAK, the key s lifetime, and the key s sequence number. The pre-PAK shall be encrypted with the CPE s public key. The CPE_Random number is returned from the SCM RSA-Request message, along with a random number supplied by the BS, thus enabling assurance of key liveness.

Code: 4

Table 213— SCM RSA-Reply attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64-bit random number generated in the CPE |
| BS_Random | A 64-bit random number generated in the BS |
| Encrypted pre-PAK | RSA-OAEP-Encrypt(PubKey(CPE), pre-PAK ¦ CPE MAC Address) |
| Key Lifetime | PAK Aging timer |
| Key Sequence Number | PAK sequence number |
| BS_Certificate | Contains the BS’s X.509 certificate |
| SigBS | An RSA signature over all the other attributes in the message |

The SigBS indicates an RSA signature over all the other attributes in this message, and the BS’s private key is used to make an RSA signature.

      - SCM RSA-Reject

The BS responds to a CPE’s authorization request with an SCM RSA-Reject message if the BS rejects the CPE’s authorization request.

Code: 5

Table 214— SCM RSA-Reject attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64 bit random number generated in the CPE |
| BS_Random | A 64 bit random number generated in the BS |
| Error-Code | Error code identifying reason for rejection of authorization request |
| BS_Certificate | Contains the BS s X.509 certificate |
| Display-String (optional) | Display string providing reason for rejection of authorization request |
| SigBS | An RSA signature over all the other attributes in the message |

The Error-Code and Display-String attributes describe to the requesting CPE the reason for the RSA-based authorization failure.

The SigBS indicates an RSA signature over all the other attributes in this message, and the BS’s private key is used to make an RSA signature.

      - SCM RSA-Acknowledgement

The CPE sends the SCM RSA-Acknowledgement message to BS in response to a SCM RSA-Reply message or a SCM RSA-Reject message. Only if the value of Auth Result Code is failure, then the Error-Code and Display-String can be included in this message.

Code: 6

Table 215— SCM RSA-Acknowledgement attributes

| Attribute | Contents |
| --- | --- |
| BS_Random | A 64 bit random number generated in the BS |
| Auth Result Code | Indicates result (Success or Failure) of authorization procedure. |
| Error-Code | Error code identifying reason for rejection of authorization request |
| Display-String (optional) | Display string providing reason for rejection of authorization request |
| SigCPE | An RSA signature over all the other attributes in the message |

The SigCPE indicates an RSA signature over all the other attributes in this message, and the CPE’s private key is used to make an RSA signature.

      - SCM EAP Start

In the case of EAP re-authentication HMAC Digest/CMAC Digest and Key Sequence Number attributes shall be included. At initial EAP authentication, these attributes are omitted.

Code: 7

Table 216 — SCM EAP Start attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| HMAC Digest/CMAC Digest | Message digest calculated using AK |

      - SCM EAP Transfer

When a CPE has an EAP payload received from an EAP method for transmission to the BS or when a BS has an EAP payload received from an EAP method for transmission to the CPE, it encapsulates it in a SCM EAP Transfer message. In the case of re-authentication, HMAC Digest/CMAC Digest and Key Sequence Number attributes shall be included.

Code: 8

Table 217 — SCM EAP Transfer attributes

| Attribute | Contents |
| --- | --- |
| EAP Payload | Contains the EAP authentication data, not interpreted in the MAC |
| Key Sequence Number | AK sequence number |
| HMAC Digest/ CMAC Digest | Message Digest calculated using AK |

The EAP Payload field carries data in the format described in section 4 of RFC 3748.

      - SCM Authenticated EAP Transfer

This message is used for Authenticated EAP-based authorization (if this was specified by Authorization Policy Support negotiated in the SBC-REQ/RSP exchange). Specifically, when a CPE or BS has an EAP payload received from an EAP method for transmission after an authentication established EIK, it encapsulates the EAP payload in a SCM Authenticated EAP Transfer message.

Code: 9

Table 218 — SCM Authenticated EAP Transfer attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | PAK Sequence Number (optional) |
| EAP Payload | Contains the EAP authentication data, not interpreted in the MAC |
| HMAC/CMAC Digest | Message Digest calculated using EIK |

The EAP Payload field carries EAP data in the format described in RFC 3748

The CMAC-Digest s or HMAC Digest s attribute shall be the final attribute in the message s attribute list.

Inclusion of the CMAC or HMAC Digest allows the CPE and BS to cryptographically bind previous authorization and following EAP authentication by authenticating the EAP payload. The key for the CMAC Value or HMAC Digest is derived from the EIK.

PAK Sequence Number attribute carries PAK sequence number only if CPE and BS negotiate Authenticated EAP after RSA mode.

      - SCM SA-TEK-Challenge

The BS transmits the SCM SA-TEK-Challenge message as a first step in the 3-way SA-TEK handshake at initial network entry and at reauthorization. The BS shall send this message to the CPE after finishing authorization procedure(s) selected by the negotiated Authorization Policy Support included in the SBC-REQ/RSP messages. It identifies an AK to be used, and includes a random number challenge to be returned by the CPE in the SCM SA-TEK-Request message.

Code: 10

Table 219 — SCM SA-TEK-Challenge attributes

| Attribute | Contents |
| --- | --- |
| BS_Random | A freshly generated random number of 64 bits. |
| Key Sequence Number | AK sequence number |
| AKID | AKID of the AK (this is the AKID of the new AK in the case of re-authentication) |
| Key lifetime | PMK lifetime, this attribute shall include only follows EAP-based authorization or EAP-based re-authorization procedures |
| HMAC/CMAC Digest | Message authentication digest for this message |

The HMAC Digest attribute or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC-Digest allows the CPE and BS to authenticate a SCM SA-TEK-Challenge message. The HMAC or the CMAC authentication keys are derived from the AK.

      - SCM SA-TEK-Request

The CPE transmits the SCM SA-TEK-Request message after receipt and successful HMAC Digest or CMAC value verification of an SA-Challenge tuple or SCM SA-TEK-Challenge message from the BS. The SCM SA-TEK-Request proves liveliness of the CPE and its possession of the AK to the BS. If this message is being generated during initial network entry, then it constitutes a request for SA-Descriptors identifying the primary and static SAs and GSAs the requesting CPE is authorized to access and their particular properties (e.g., type, cryptographic suite).

If this message is being generated following HO, then it constitutes a request for establishment (in the target BS) of TEKs, GTEKs and GKEKs for the CPE and renewal of active primary, static and dynamic SAs and associated SAIDs used by the CPE in its previous serving BS.

Code: 11

Table 220 — SCM SA-TEK-Request attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64-bit number chosen by the CPE freshly for every new handshake. |
| BS_Random | The 64-bit random number used in the SCM SA-TEK-Challenge message or SA-Challenge Tuple |
| Key Sequence Number | AK sequence number |
| AKID | Identifies the AK that was used for protecting this message |
| Security-Capabilities | Describes requesting CPE s security capabilities |
| Security Negotiation Parameters | ConfirCPE requesting CPE s security capabilities (see 11.8.4 xxx) |
| SCM configuration settings | SCM configuration defined in 11.9.36 xxx. |
| HMAC/CMAC Digest | Message authentication digest for this message. |

Receipt of a new BS Random value in SA-TEK-Challenge or SA-Challenge tuple indicates the beginning of a new handshake.

      - SCM SA-TEK-Response

The BS transmits the SCM SA-TEK-Response message as a final step in the 3-way SA-TEK handshake.

Code: 12

Table 221 — SCM SA-TEK-Response attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | The number received from the CPE |
| BS_Random | The random number included in the SCM SA-TEK-Challenge message or SA-Challenge IE. |
| Key Sequence Number | AK sequence number |
| AKID | This identifies the AK to the CPE that was used for protecting this message. |
| SA_TEK_Update | A compound IE list each of which specifies an SA identifier (SAID) and additional properties of the SA that the CPE is authorized to access. This compound field may be present at the reentry only. For each active SA in previous serving BS, corresponding TEK, GTEK and GKEK parameters are included. |
| Frame Number | An absolute frame number in which the old PMK and all its associate AKs should be discarded. |
| (one or more) SA-Descriptor(s) | Each compound SA-Descriptor attribute specifies an SA identifier (SAID) and additional properties of the SA. This attribute is present at the initial net-work entry only. |
| Security Negotiation Parameters | ConfirCPE the authentication and message integrity parameters to be used (see 11.8.4) |
| HMAC Digest/CMAC Digest | Message authentication digest for this message. |

      - SCM Key-Request

A CPE sends a SCM Key-Request message to the BS to request new TEK and TEK-related parameters (GTEK and GTEK-related parameters for the multicast or broadcast service) or GKEK and GKEK-related parameters for the multicast or broadcast service.

Code: 13

Table 222 — SCM Key-Request attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier - GSAID for multicast or broadcast service |
| Nonce | A random number generated in a CPE |
| HMAC Digest/CMAC Digest | Message Digest calculated using AK |

The HMAC Digest attribute or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC digest allows the CPE and BS to authenticate the SCM Key-Request message. The HMAC Digest or the CMAC-Digest’s authentication key is derived from the AK.

      - SCM Key-Reply

The BS responds to a CPE’s SCM Key-Request message with a SCM Key-Reply message.

Code: 14

Table 223 — SCM Key-Reply attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier - GSAID for multicast or broadcast service. |
| TEK-Parameters | Older generation of key parameters relevant to SAID - GTEK-Parameters for the multicast or broadcast service |
| TEK-Parameters | Newer generation of key parameters relevant to SAID |
| GKEK-Parameters | Older generation of GKEK-related parameters for multicast or broadcast service. |
| GKEK-Parameters | Newer generation of GKEK-related parameters for multicast or broadcast service. |
| Nonce | A same random number included in the SCM Key Request message |
| HMAC/CMAC Digest | Message Digest calculated using AK |

The GKEK-Parameters attribute is a compound attribute containing all of the GKEK-related parameters corresponding to a GSAID. This would include the GKEK, the GKEK’s remaining key lifetime, and the GKEK’s key sequence number. The older generation of GKEK-Parameters is valid within the current lifetime and the newer generation of GKEK-Parameters is valid within the next lifetime.

The HMAC Digest or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC digest allows the CPE and BS to authenticate the SCM Key-Reply message. The HMAC Digest or the CMAC-Digest’s authentication key is derived from the AK.

      - SCM Key-Reject

The BS responds to a CPE’s SCM Key-Request message with a SCM Authorization-Reject message if the BS rejects the CPE’s traffic keying material request.

Code: 15

Table 224 — SCM Key-Reject attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier |
| Error-Code | Error code identifying reason for rejection of the SCM Key-Request message |
| Display-String (optional) | Display string containing reason for the SCM Key-Request message |
| Nonce | A same random number included in the SCM Key Request message |
| HMAC/CMAC Digest | Message Digest calculated using AK |

The HMAC Digest or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC digest allows the CPE and BS to authenticate the SCM Key-Reject message. The HMAC Digest or the CMAC-Digest’s authentication key is derived from the AK.

      - SCM SA-Addition

This message is sent by the BS to the CPE to establish one or more additional SAs.

Code: 16

Table 225 — SCM SA-Addition attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| (one or more) SA-Descriptor(s) | Each compound SA-Descriptor attribute specifies an SA identifier (SAID) and additional properties of the SA |
| HMAC/CMAC Digest | Message Digest calculated using AK |

The HMAC Digest or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC digest allows the CPE and BS to authenticate the SCM SA-Add message. The HMAC Digest or the CMAC-Digest’s authentication key is derived from the AK.

      - SCM TEK-Invalid

The BS sends a SCM TEK-Invalid message to a client CPE if the BS determines that the CPE encrypted an upstream PDU with an invalid TEK (i.e., an SAID’s TEK key sequence number), contained within the received packet’s MAC header, is out of the BS’s range of known, valid sequence numbers for that SAID.

Code: 17

Table 226 — SCM TEK-Invalid attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security Association Identifier |
| Error-Code | Error code identifying reason for SCM TEK-Invalid message |
| Display-String (optional) | Display string containing reason for the SCM TEK-Invalid message |
| HMAC/CMAC Digest | Message Digest calculated using AK |

The HMAC Digest or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC digest allows the CPE and BS to authenticate the SCM SA-Add message. The HMAC Digest or the CMAC-Digest’s authentication key is derived from the AK.

      - SCM Group-Key-Update-Command

This message is sent by BS to push the GTEK and/or GKEK parameters to CPEs served with the specific multicast service or broadcast service.

Code: 18

Table 227 — SCM Group-Key-Update-Command attributes

| Attribute | Contents |
| --- | --- |
| Key-Sequence-Number | AK sequence number |
| GSAID | Security Association ID |
| Key Push Modes | Usage code of Key Update Command message. |
| Key Push Counter | Counter one greater than that of older generation. |
| GTEK-Parameters | Newer generation of GTEK-related parameters relevant to GSAID. The GTEK-Parameters is the TEK-Parameters for multicast or broadcast service. |
| GKEK-Parameters | Newer generation of GKEK-related parameters for multicast or broadcast service. |
| HMAC/CMAC Digest | Message integrity code of this message. |

Key Sequence Number is the sequence number of the synchronized AK (Authorization Key) between a CPE and a BS.

GSAID is SAID for the multicast group or the broadcast group. The type and length of the GSAID is equal to ones of the SAID.

There are two types in a SCM Group Key Update Command message, GKEK update mode and GTEK update mode. The former is used to update GKEK and the latter is used to update GTEK for the multicast service or the broadcast service. Key Push Modes indicates the usage code of a SCM Group Key Update Command message. The SCM Group Key Update Command message for the GKEK update mode is carried on the Primary Management connection, but one for the GTEK update mode is carried on the Broadcast connection. A few attributes in a SCM Group Key Update Command message shall not be used according this Key Push Modes attribute’s value.

Key Push Counter is used to protect for replay attack. This value is one greater than that of older generation.

A SCM Group Key Update Command message contains only newer generation of key parameters, because this message inform a CPE next traffic key material. The GTEK-Parameters attribute is a com-pound attribute containing all of the keying material corresponding to a newer generation of a GSAID’s GTEK. This would include the GTEK, the GTEK’s remaining key lifetime, the GTEK’s key sequence number, the associated GKEK sequence number, and the cipher block chaining (CBC) initialization vector. The GTEK is TEK for the multicast group or the broadcast group. The type and length of the GTEK is equal to ones of the TEK. The GKEK (Group Key Encryption Key) can be randomly generated from a BS or a certain network node (i.e., an ASA server). The GKEK should be identically shared within the same multicast group or the broadcast group. The GTEK is encrypted with GKEK for the multicast service or the broadcast service. GKEK parameters contain the GKEK encrypted by the KEK, GKEK sequence number, and GKEK lifetime.

The HMAC/CMAC Digest attribute shall be the final attribute in the message’s attribute list. Inclusion of the keyed digest allows the receiving client to authenticate the Group Key Update Command message. The HMAC/CMAC Digest’s authentication key is derived from the AK for the GKEK update mode and GKEK for the GTEK update mode.

      - SCM EAP Complete

In double EAP mode (EAP after EAP), BS sends the SCM EAP Complete message to CPE with EAP-Success to inform CPE of completing 1st EAP conversation.

This message is used only if CPE and BS negotiate EAP in EAP mode.

The Key Sequence Number and HMAC/CMAC Digest attributes of this message appear only in re-authentication.

Code: 19

Table 228 — SCM EAP Complete attributes

| Attribute | Contents |
| --- | --- |
| EAP Payload | Contains the EAP authentication data, not interpreted in the MAC layer |
| Key Sequence Number | AK sequence number appear only if AK is available from previous double EAP |
| HMAC Digest/CMAC Digest | Message Digest calculated using AK only if AK is available from previ-ous double EAP Message Digest calculated using EIK when initial authentication |

      - SCM Authenticated EAP Start

In double EAP mode (EAP after EAP), CPE sends the SCM EAP Authenticated EAP Start message to BS in order to initiate 2nd round EAP. This message is signed by EIK which is generated by 1st EAP.

This message is used only for initial authentication of double EAP.

Code: 20

Table 229 — SCM Authenticated EAP Start atttributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | Random number generated by CPE. |
| HMAC Digest/CMAC Digest | Message Digest calculated using EIK |

      - SCM Authentication Invalid

The BS may send an Authorization Invalid message to a client CPE as:

- An unsolicited indication, or

- A response to a message received from that CPE.

In either case, the Authorization Invalid message instructs the receiving CPE to reauthorize with its BS.

The BS sends an Authorization Invalid in response to a Key Request if (1) the BS does not recognize the CPE as being authorized (i.e., no valid AK associated with the requesting CPE) or (2) verification of the Key Request’s keyed message digest (in HMAC-Digest attribute) failed, indicating a loss of AK synchronization between CPE and BS.

Code: 21

Table 230 — Authorization Invalid attributes

| Attribute | Contents |
| --- | --- |
| Error-Code | Error code identifying reason for Authorization Invalid. |
| Display-String (optional) | Display String describing failure condition. |

      - SCM Authentication Information (Auth Info)

The Auth Info message contains a single CA-Certificate attribute, containing an X.509 CA certificate for the manufacturer of the CPE. The CPE’s X.509 user certificate shall have been issued by the CA identified by the X.509 CA certificate.

Auth Info messages are strictly informative; while the CPE shall transmit Auth Info messages as indicated by the Authentication state model, the BS may ignore them.

Code: 22

Table 231 — Auth Info attributes

| Attribute | Contents |
| --- | --- |
| CA-Certificate | Certificate of manufacturer CA that issued CPE certificate. |

The CA-certificate attribute contains an X.509 CA certificate for the CA that issued the CPE’s X.509 user certificate. The external CA issues these CA certificates to CPE manufacturers.

7. Security sublayers

Traditional broadband communications systems such as 802.16 contain data, control and management functions which require protection. However, due to the unique characteristics of the 802.22 systems which include cognitive radio capability as well as long range, enhanced security mechanisms are needed. These security features provide protection for the 802.22 users, service providers and most importantly, the incumbents, who are the primary users of the spectrum. As a result, the protection mechanisms in 802.22 are divided into several security sublayers which target non-cognitive as well as cognitive functionality of the system and the interactions between the two.

The Security Sublayers 1 and 2 provide subscribers with privacy, authentication, or confidentiality (In security parlance, confidentiality = privacy + authenticity) across the broadband wireless network. It does this by applying cryptographic transforms to MAC PDUs carried across connections between CPE and BS. In addition, these security sublayers provide operators with strong protection from theft of service. In cognitive radio systems, confidentiality and privacy mechanisms need to protect not just the data but, also sensitive spectrum occupancy information from the competitors and the spectrum management information used by the BS to configure the operation of the CPEs. The BS protects against unauthorized access to these data transport services by securing the associated service flows across the network.

The security sublayers employ an authenticated client/server key management protocol in which the BS, the server, controls distribution of keying material to client CPE. Additionally, the basic security mechanisms are strengthened by adding digital-certificate-based CPE device-authorization to the key management protocol. If during capabilities negotiation, the CPE specifies that it does not support IEEE 802.22 security, step of authorization and key exchange shall be skipped. The BS, if provisioned so, shall consider the CPE authenticated; otherwise, the CPE shall not be serviced. Neither key exchange nor data encryption performed. The procedures defined for Security Sublayers 1 and 2 are based on the Privacy Key Management version 2 (PKMv2) protocol defined for IEEE 802.16 [insert reference here].

To enhance the security for the cognitive functionality in 802.22, Security Sublayers 3 is introduced. The security mechanisms for this layer include authentication and availability, authorization, confidentiality and privacy. The authentication mechanisms validate the availability of spectrum for the primary and the secondary users of the spectrum. This includes authentication of the incumbent sensing information to avoid Denial of Service (DoS) attacks such as replay and ghosting, authentication of the 802.22.1 beacon frame utilizing the security features that are already embedded in it, authentication of the geolocation and co-existence information, as well as detection and reporting of spurious transmissions. The authorization mechanisms attempt to make the spectrum manager functionality tamper proof by allowing only the authorized personnel and information to configure it.

7.1 Security Architecture for the Data / Control and Management Planes

Privacy has two component protocols as follows:

a) An encapsulation protocol for securing packet data across the network. This protocol defines a set of supported cryptographic suites, i.e., pairings of data encryption and authentication algorithms, and the rules for applying those algorithms to a MAC PDU payload.

b) A Security Control and Management (SCM) protocol providing the secure distribution of keying data from the BS to the CPE. Through this key management protocol, the CPE and the BS synchronize keying data; in addition, the BS uses the protocol to enforce conditional access to network services.

The protocol stack for the security components of the system are shown in Figure xxx.

Figure xxx — Security Sublayer 1

— SCM Control Management: This stack controls all security components. Various keys are derived and generated in this stack.

— Traffic Data Processing: This stack encrypts or decrypts the traffic data and executes the authentication function for the traffic data.

— Control Message Processing: This stack processes the various SCM-related MAC messages, and provides either authentication and/or encryption of such messages.

—RSA/ECC-based Authorization: This stack performs the RSA- or ECC-based authorization function using the CPE’s X.509 digital certificate and the BS’s X.509 digital certificate, when the RSA or ECC-based authorization is selected as an authorization policy between a CPE and a BS.

— Authorization/SA Control: This stack controls the authorization state machine and the traffic encryption key state machine.

7.1.1 Secure encapsulation of MAC PDUs

Encryption services are defined as a set of capabilities within the MAC security sublayer. MAC header information specific to encryption is allocated in the generic MAC header format.

Encryption is always applied to the MAC PDU payload when required by the selected ciphersuite; the generic MAC header is not encrypted. All MAC management messages shall be sent in the clear to facilitate registration, ranging, and normal operation of the MAC.

The format of MAC PDUs carrying encrypted packet data payloads is specified in 6.x.x.x

7.1.2 Key management protocol

The SCM protocol allows for both mutual authentication and unilateral authentication (e.g., where the BS authenticates CPE, but not vice versa). It also supports periodic reauthentication/reauthorization and key refresh. The key management protocol uses X.509 digital certificates [IETF RFC 3280] together with RSA public-key encryption algorithm [PKCS #1] or a sequence starting with RSA authentication or even with ECC public-key encryption algorithm. It uses strong encryption algorithms to perform key exchanges between a CPE and BS.

The SCM’s authentication protocol establishes a shared secret (i.e., the AK) between the CPE and the BS. The shared secret is then used to secure subsequent SCM exchanges of TEKs. This two-tiered mechanism for key distribution permits refreshing of TEKs without incurring the overhead of computation-intensive operations.

A BS authenticates a client CPE during the initial authorization exchange. Each CPE presents its credentials, which will be a unique X.509 digital certificate issued by the CPE’s manufacturer (in the case of RSA authentication) or an operator-specified credential (in the case of EAP-based authentication).

The BS associates a CPE’s authenticated identity to a paying subscriber and hence to the data services that subscriber is authorized to access.

Since the BS authenticates the CPE, it may protect against an attacker employing a cloned CPE that masquerades as a legitimate subscriber’s CPE.

The traffic key management portion of the SCM protocol adheres to a client/server model, where the CPE (a SCM “client”) requests keying material and the BS (a SCM “server”) responds to those requests. This model ensures that individual CPE clients receive only keying material for which they are authorized.

The SCM protocol uses MAC management messaging, i.e., SCM-REQ and SCM-RSP messages defined in 6.x.x.x. The SCM protocol is defined in detail in 7.x.

7.1.3 Authorization Overview

A CPE uses the SCM protocol to obtain authorization and traffic keying material from the BS and to support periodic reauthorization and key refresh.

SCM supports two distinct authentication protocol mechanisms: RSA and ECC-based authorization.

7.1.3.1 SCM RSA Authorization

The SCM RSA authorization protocol uses X.509 digital certificates [IETF RFC 3280], the RSA public-key encryption algorithm [PKCS #1] that binds public RSA encryption keys to MAC addresses of CPEs.

A BS authorizes a client CPE during the initial authorization exchange. Each CPE carries a unique X.509 digital certificate issued by the CPE’s manufacturer. The digital certificate contains the CPE’s Public Key and CPE MAC address. When requesting an AK, a CPE presents its digital certificate to the BS. The BS verifies the digital certificate, and then uses the verified Public Key to encrypt an AK, which the BS then sends back to the requesting CPE.

All CPEs using RSA authorization shall have factory-installed RSA private/public key pairs or provide an internal algorithm to generate such key pairs dynamically. If a CPE relies on an internal algorithm to generate its RSA key pair, the CPE shall generate the key pair prior to its first AK exchange, described in 7.2.1. All CPEs with factory-installed RSA key pairs shall also have factory-installed X.509 certificates. All CPEs that rely on internal algorithms to generate an RSA key pair shall support a mechanism for installing a manufacturer-issued X.509 certificate following key generation.

7.1.3.2 SCM ECC Authorization

In addition to the current RSA-based authorization within the SCM protocol,  Elliptic Curve Cryptography (ECC)-based authorization may be employed.  ECC authentication operates in the same way as RSA authentication. Requirements for ECC-based X.509 certificates are defined in Section 7.6.1.

7.1.4 Mapping of connections to SAs

The following rules for mapping connections to SAs apply:

- All transport connections shall be mapped to an existing SA.

- Multicast transport connections may be mapped to any Static or Dynamic SA.

- The secondary management connection shall be mapped to the Primary SA.

- The basic and the primary management connections shall not be mapped to an SA.

The actual mapping is achieved by including the SAID of an existing SA in the DSA-xxx messages together with the CID. No explicit mapping of secondary management connection to the Primary SA is required.

7.1.5 Cryptographic suite

A cryptographic suite is the SA’s set of methods for data encryption, data authentication, and TEK exchange. A cryptographic suite is specified as described in 11.x.x. The cryptographic suite shall be one of the ones listed in Table xxx.

7.2 SCM protocol

The Security Control Management (SCM) protocol supported in this standard is based on the Privacy Key Management version 2 (PKMv2) developed for IEEE 802.16 [insert proper reference here]. SCM differs from PKMv2 in that it supports features such as; mutual authorization using RSA and ECC, a modified key hierarchy, and AES-CCM is used for all message encryption and authentication.

7.2.1 Security associations (SAs)

A security association (SA) is the set of security information a BS and one or more of its client CPEs share in order to support secure communications across the IEEE 802.22 network. Three types of SAs are defined: Primary, Static, and Dynamic. Each CPE establishes a Primary SA during the CPE initialization process. Static SAs are provisioned within the BS. Dynamic SAs are established and eliminated, on the fly, in response to the initiation and termination of specific service flows. Both Static and Dynamic SAs may be shared by multiple CPEs.

An SA’s shared information shall include the cryptographic suite employed within the SA. The shared information may include TEKs. The exact content of the SA is dependent on the SA’s cryptographic suite.

SAs are identified using SAIDs. The Primary SA that a CPE establishes with the BS is exclusive, and shall be identified by an SAID that is equal to the Basic CID of that CPE.

Using the SCM protocol, a CPE requests from its BS an SA’s keying material.

The BS shall ensure that each client CPE only has access to the SAs it is authorized to access.

An SA’s keying material has a limited lifetime. When the BS delivers SA keying material to a CPE, it also provides the CPE with that material’s remaining lifetime. It is the responsibility of the CPE to request new keying material from the BS before the set of keying material that the CPE currently holds expires at the BS. Should the current keying material expire before a new set of keying material is received, the CPE shall perform network entry as described in x.x.x.

In certain cryptographic suites, key lifetime may be limited by the exhaustion rate of a number space, e.g., the PN of AES in CCM. In this case, the key ends either at the expiry of the key lifetime or the exhaustion of the number space, which ever is earliest. Note that in this case, security is not determined by the key lifetime.

7.2.1.1 Dynamic Creation of SA

The BS may dynamically establish SAs by issuing an SA Add message (via SCM-RSP). Upon receiving an SA Add message, the SS shall start a TEK state machine for each SA listed in the message.

7.2.1.2 Dynamic Mapping of SA

When creating a new service flow, an CPE may request an existing SA be used by passing the SAID of the SA in a DSA-REQ or DSC-REQ message. The BS checks the CPEs authorization for the requested SA and generates appropriate response using a DSA-RSP or DSC-RSP message correspondingly. With BS-initiated dynamic service creations, a BS may also map a new service flow to an existing SA that is supported by a specific CPE. The SAID of the SA shall be communicated to the CPE in a DSA-REQ or DSC-REQ message.

7.2.3 CPE Authorization Process

7.2.3.1 CPE authorization and AK exchange overview

CPE authorization, controlled by the Authorization state machine, is the process of the BS’s authenticating a client CPE’s identity:

a) The BS and CPE establish a shared AK by RSA/ECC from which a key encryption key (KEK) and message authentication keys are derived.

b) The BS provides the authorized CPE with the identities (i.e., the SAIDs) and properties of Primary and Static SAs for which the CPE is authorized to obtain keying information.

After achieving initial authorization, a CPE periodically reauthorizes with the BS; reauthorization is also managed by the CPE’s Authorization state machine. TEK state machines manage the refreshing of TEKs.

7.2.3.2 Mutual Authorization via RSA

A CPE begins authorization by sending an Authorization Information message to its BS. The Authorization Information message contains the CPE manufacturer’s X.509 certificate, issued by the manufacturer itself or by an external authority. The Authorization Information message is strictly informative; i.e., the BS may choose to ignore it. However, it does provide a mechanism for a BS to learn the manufacturer certificates of its client CPE.

The CPE sends an Authorization Request message to its BS immediately after sending the Authentication Information message. This is a request for an AK, as well as for the SAIDs identifying any Static SAs the CPE is authorized to participate in.

The Authorization Request includes

- A manufacturer-issued X.509 certificate.

- A description of the cryptographic algorithms the requesting CPE supports. A CPE’s cryptographic capabilities are presented to the BS as a list of cryptographic suite identifiers, each indicating a particular pairing of packet data encryption and packet data authentication algorithms the CPE supports.

- The CPE’s Basic CID. The Basic CID is the first static CID the BS assigns to a CPE during initial ranging. The primary SAID is equal to the Basic CID.

- A 64 bit random number generated by the SS

- RSA signature over all the attributes in the authorization request message, used to assure the authenticity of the SCM RSA-Request message

In response to an Authorization Request message, a BS validates the requesting CPE’s identity, determines the encryption algorithm and protocol support it shares with the CPE, activates an AK for the CPE, encrypts it with the CPE’s public key, and sends it back to the CPE in an Authorization Reply message. The authorization reply includes

- The BS’s X.509 certificate, used to verify the BS’s identity

- A pre-PAK encrypted with the CPEs public key

- A 4-bit PAK sequence number, used to distinguish between successive generations of AKs

- PAK lifetime

- The identities (i.e. SAID’s) and properties of the Primary SA and any of the Static or Dynamic SAs the CPE is authorized for. This also includes the cryptographic suite (See Table 7.4-1) that is to be applied to each SA.

- The 64 bit random number genereated by the CPE.

- A 64 bit random number generated by the BS, which, along with SS random number used to ensure key livness.

- RSA signature over all the attributes in the authorization reply message, used to assure teh authenticity of the SCM RSA-Reply messages.

While the Authorization Reply shall identify Static SAs in addition to the Primary SA whose SAID matches the requesting CPE’s Basic CID, the Authorization Reply shall not identify any Dynamic SAs.

The BS, in responding to a CPE’s Authorization Request, shall determine whether the requesting CPE, whose identity can be verified via the X.509 digital certificate, is authorized for basic unicast services, and what additional statically provisioned services (i.e., Static SAIDs) the CPE’s user has subscribed for. Note that the protected services a BS makes available to a client CPE can depend upon the particular cryptographic suites for which the CPE and the BS share support.

A CPE shall periodically refresh its AK by reissuing an Authorization Request to the BS. Reauthorization is identical to authorization with the exception that the CPE does not send Authorization Information messages during reauthorization cycles. The description of the authorization state machine in 7.2.1.5 clearly indicates when Authorization Information messages are sent. To avoid service interruptions during reauthorization, successive generations of the CPE’s AKs have overlapping lifetimes. Both the CPE and BS shall be able to support up to two simultaneously active AKs during these transition periods. The operation of the Authorization state machine’s Authorization Request scheduling algorithm, combined with the BS’s regimen for updating and using a client CPE’s AKs (see 7.3), ensures that the CPE can refresh.

When the RSA-based authorization is negotiated as authorization policy, the SCM RSA-Request, the SCM RSA-Reply, the SCM RSA-Reject, and the SCM RSA-Acknowledgement messages are used to share the pre-PAK.

7.2.3.3 Mutual Authorization via ECC

Initial and re-authorization is executed in the same way for ECC authorization, as it is executed for RSA authorization.

During initial and re-authorization, the CPE can format the Authorization Request in either one of two ways.  The first way is to make use of a manfucaturer-installed ECC certificate (see Section 7.6.2) and public key that is associated with the CPE that the CPE transmits in the intial Authorization Request.  The other method is that the CPE uses the elltiptic curve domain parameters defined in its certificate to generate an ephemeral key pair.

The initial or re-authorization request is transmitted with the ephemeral public key and just the domain parameters or entire manufacturer-installed certificate (domain parameters and manufacturer installed public key), the list of supported cryptographic suites, a 64bit randomon number generated by CPE, as well as a signature calculated over the request using the process defined in Section 7.3 of ANSI X9.62-2005.

Regardless of the method used, the BS then verifies the domain parameters (Section 5.1 of ANSI X9.63-2001), the public key (Section 5.2 of ANSI X9.63-2001), and the signature over the request (Section 7.4 ANSI X9.62-2005).  If any of these checks fail, the then Authorization Request is rejected.  When the BS responds, it can choose between either of two methods (similar to CPE initiation methods) when formatting the response.  The first way is to make use of a manfucaturer-installed ECC certificate (see Section 7.6) and public key that is associated with that BS.  The other method is that the BS uses the elltiptic curve domain parameters defined in its certificate to generate an ephemeral key pair.

The Authorization Reply is transmitted with the ephemeral public key and just the domain parameters or entire manufacturer-installed certificate (domain parameters and manufacturer installed public key), the assigned cryptographic suite, the 64bit randomon number generated by CPE, a 64bit random number generated by the BS, as well as a signature calculated over the reply using the process defined in Section 7.3 of ANSI X9.62-2005.

When the ECC-based authorization is negotiated as authorization policy, the SCM ECC-Request, the SCM ECC-Reply, the SCM ECC-Reject, and the SCM ECC-Acknowledgement messages are used to share the pre-PAK.

7.2.3.4 Authorization state machine

The Authorization state machine consists of six states and eight distinct events (including receipt of messages) that can trigger state transitions. The Authorization finite state machine (FSM) is presented below in a graphical format, as a state flow model (Figure xxx2), and in a tabular format, as a state transition matrix (Table xxx1).

The state flow diagram depicts the protocol messages transmitted and internal events generated for each of the model’s state transitions; however, the diagram does not indicate additional internal actions, such as the clearing or starting of timers, that accompany the specific state transitions. Accompanying the state transition matrix is a detailed description of the specific actions accompanying each state transition; the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

The following legend applies to the Authorization State Machine flow diagram depicted in Figure 160.

- Ovals are states.

- Events are in italics.

- Messages are in normal font.

- State transitions (i.e., the lines between states) are labeled with <what causes the transition>/<messages and events triggered by the transition>. So “timeout/Auth Request” means that the state received a “timeout” event and sent an Authorization Request (“Auth Request”) message. If there are multiple events or messages before the slash “/” separated by a comma, any of them can cause the transition. If there are multiple events or messages listed after the slash, all of the specified actions shall accompany the transition.

Figure xxx2. Authorization state machine flow diagram

The Authorization state transition matrix presented in Table xxx1 lists the six Authorization machine states in the topmost row and the eight Authorization machine events (includes message receipts) in the leftmost column. Any cell within the matrix represents a specific combination of state and event, with the next state (the state transitioned to) displayed within the cell. For example, cell 4-B represents the receipt of an Authorization Reply (Auth Reply) message when in the Authorize Wait (Auth Wait) state. Within cell 4-B is the name of the next state, “Authorized.” Thus, when an CPE’s Authorization state machine is in the Auth Wait state and an Auth Reply message is received, the Authorization state machine will transition to the Authorized state. In conjunction with this state transition, several protocol actions shall be taken; these are described in the listing of protocol actions, under the heading 4-B, in 7.2.1.5.5.

A shaded cell within the state transition matrix implies that either the specific event cannot or should not occur within that state, and if the event does occur, the state machine shall ignore it. For example, if an Auth Reply message arrives when in the Authorized state, that message should be ignored (cell 4-C). The CPE may, however, in response to an improper event, log its occurrence, generate an SNMP event, or take some other vendor-defined action. These actions, however, are not specified within the context of the Authorization state machine, which simply ignores improper events.

Table xxx – Authorization Finite State Machine State Transition Matrix

7.2.3.4.1 States

- Start: This is the initial state of the FSM. No resources are assigned to or used by the FSM in this state—e.g., all timers are off, and no processing is scheduled.

- Authorize Wait (Auth Wait): The CPE has received the “Communication Established” event indicating that it has completed basic capabilities negotiation with the BS. In response to receiving the event, the CPE has sent both an Authentication Information and an Auth Request message to the BS and is waiting for the reply.

- Authorized: The CPE has received an Auth Reply message that contains a list of valid SAIDs for this CPE. At this point, the CPE has a valid AK and SAID list. Transition into this state triggers the creation of one TEK FSM for each of the CPE’s privacy-enabled SAIDs.

- Reauthorize Wait (Reauth Wait): The CPE has an outstanding reauthorization request. The CPE was either about to expire (see Authorization Grace Time in Table xxx2) its current authorization or received an indication (an Authorization Invalid message from the BS) that its authorization is no longer valid. The CPE sent an Auth Request message to the BS and is waiting for a response.

- Authorize Reject Wait (Auth Reject Wait): The CPE received an Authorization Reject (Auth Reject) message in response to its last Auth Request. The Auth Reject’s error code indicated the error was not of a permanent nature. In response to receiving this reject message, the CPE set a timer and transitioned to the Auth Reject Wait state. The CPE remains in this state until the timer expires.

- Silent: The CPE received an Auth Reject message in response to its last Auth Request. The Auth Reject’s error code indicated the error was of a permanent nature. This triggers a transition to the Silent state, where the CPE is not permitted to pass subscriber traffic. The CPE shall, however, respond to management messages from the BS issuing the Perm Auth Reject.

7.2.3.4.2 Messages

Note that the message formats are defined in detail in 6.X.X.X.

- Authorization Request (Auth Request): Request an AK and list of authorized SAIDs. Sent from CPE to BS.

- Authorization Reply (Auth Reply): Receive an AK and list of authorized, static SAIDs. Sent from BS to CPE. The AK is encrypted with the CPE’s public key.

- Authorization Reject (Auth Reject): Attempt to authorize was rejected. Sent from the BS to the CPE.

- Authorization Invalid (Auth Invalid): The BS may send an Authorization Invalid message to a client CPE as follows:

- An unsolicited indication, or

- A response to a message received from that CPE. In either case, the Auth Invalid message instructs the receiving CPE to reauthorize with its BS. The BS responds to a Key Request with an Auth Invalid message if (1) the BS does not recognize the CPE as being authorized (i.e., no valid AK associated with CPE) or (2) verification of the Key. Request’s keyed message digest (in HMAC-Digest attribute) failed. Note that the Authorization Invalid event, referenced in both the state flow diagram and the state transition matrix, signifies either the receipt of an Auth Invalid message or an internally generated event.

- Authentication Information (Auth Info): The Auth Info message contains the CPE manufacturer’s X.509 Certificate, issued by an external authority. The Auth Info message is strictly an informative message the CPE sends to the BS; with it, a BS may dynamically learn the manufacturer certificate of client CPE. Alternatively, a BS may require out-of-band configuration of its list of manufacturer cer-tificates.

7.2.3.4.3 Events

- Communication Established: The Authorization state machine generates this event upon entering the Start state if the MAC has completed basic capabilities negotiation. If the basic capabilities negotia-tion is not complete, the CPE sends a Communication Established event to the Authorization FSM upon completing basic capabilities negotiation. The Communication Established event triggers the CPE to begin the process of getting its AK and TEKs.

- Timeout: A retransmission or wait timer timed out. Generally a request is resent.

- Authorization Grace Timeout (Auth Grace Timeout): The Authorization Grace timer timed out. This timer fires a configurable amount of time (the Authorization Grace Time) before the current authori-zation is supposed to expire, signalling the CPE to reauthorize before its authorization actually expires. The Authorization Grace Time takes the default value from Table 549 or may be specified in a configuration setting within the Auth Reply message.

- Reauthorize (Reauth): CPE’s set of authorized static SAIDs may have changed. This event may be generated in response to an SNMP set and meant to trigger a reauthorization cycle.

- Authorization Invalid (Auth Invalid): This event is internally generated by the CPE when there is a failure authenticating a Key Reply or Key Reject message, or externally generated by the receipt of an Auth Invalid message, sent from the BS to the CPE. A BS responds to a Key Request with an Auth Invalid if verification of the request’s message authentication code fails. Both cases indicate BS and CPE have lost AK synchronization. A BS may also send to an CPE an unsolicited Auth Invalid message, forcing an Auth Invalid event.

- Permanent Authorization Reject (Perm Auth Reject): The CPE receives an Auth Reject in response to an Auth Request. The error code in the Auth Reject indicates the error is of a permanent nature. What is interpreted as a permanent error is subject to administrative control within the BS. Auth Request processing errors that can be interpreted as permanent error conditions include the follow-ing:

- Unknown manufacturer (do not have CA certificate of the issuer of the CPE Certificate).

- Invalid signature on CPE certificate.

- ASN.1 parsing failure.

- Inconsistencies between data in the certificate and data in accompanying SCM data attribute.

- Incompatible security capabilities. When an CPE receives an Auth Reject indicating a permanent failure condition, the Authorization State machine moves into a Silent state, where the CPE is not permitted to pass subscriber traffic. The CPE shall, however, respond to management messages from the BS issuing the Perm Auth Reject. The managed CPE may also issue an SNMP Trap upon entering the Silent state.

- Authorization Reject (Auth Reject): The CPE receives an Auth Reject in response to an Auth Request. The error code in the Auth Reject does not indicate the failure was due to a permanent error condition. As a result, the CPE’s Authorization state machine shall set a wait timer and transition into the Auth Reject Wait State. The CPE shall remain in this state until the timer expires, at which time it shall reattempt authorization.

NOTE — The following events are sent by an Authorization state machine to the TEK state machine:

- [TEK] Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate the FSM and remove the corresponding SAID’s keying material from the CPE’s key table.

- [TEK] Authorized: Sent by the Authorization FSM to a nonactive (START state), but valid TEK FSM.

- [TEK] Authorization Pending (Auth Pend): Sent by the Authorization FSM to a specific TEK FSM to place that TEK FSM in a wait state until the Authorization FSM can complete its reauthorization operation.

- [TEK] Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational Reauthorize Wait (Op Reauth Wait) or Rekey Reauthorize Wait (Rekey Reauth Wait) states to clear the wait state begun by a TEK FSM Authorization Pending event.

7.2.3.4.4 Parameters

All configuration parameter values take the default values from Table CFX1 or may be specified in the Auth Reply message.

- Authorize Wait Timeout (Auth Wait Timeout): Timeout period between sending Authorization Request messages from Auth Wait state (see 11.9.19.2).

- Authorization Grace Timeout (Auth Grace Timeout): Amount of time before authorization is sched-uled to expire that the CPE starts reauthorization (see 11.9.19.3).

- Authorize Reject Wait Timeout (Auth Reject Wait Timeout): Amount of time an CPE’s Authorization FSM remains in the Auth Reject Wait state before transitioning to the Start state (see 11.9.19.7).

7.2.3.4.5 Actions

Actions taken in association with state transitions are listed by <event> (<rcvd message>) --> <state> below:

1-A Start (Communication Established) → Auth Wait

a) Send Auth Info message to BS

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Auth Wait Timeout

2-B Auth Wait (Auth Reject) → Auth Reject Wait

a) Clear Auth Request retry timer

b) Set a wait timer to Auth Reject Wait Timeout

2-D Reauth Wait (Auth Reject) → Auth Reject Wait

a) Clear Auth Request retry timer

b) Generate TEK FSM Stop events for all active TEK state machines

c) Set a wait timer to Auth Reject Wait Timeout

3-B Auth Wait (Perm Auth Reject) → Silent

a) Clear Auth Request retry timer

b) Disable all forwarding of CPE traffic

3-D Reauth Wait (Perm Auth Reject) → Silent

a) Clear Auth Request retry timer

b) Generate TEK FSM Stop events for all active TEK state machines

c) Disable all forwarding of CPE traffic

4-B Auth Wait (Auth Reply) → Authorized

a) Clear Auth Request retry timer

b) Decrypt and record AK delivered with Auth Reply

c) Start TEK FSMs for all SAIDs listed in Authorization Reply (provided the CPE supports the cryptographic suite that is associated with an SAID) and issue a TEK FSM Authorized event for each of the new TEK FSMs

d) Set the Authorization Grace timer to go off “Authorization Grace Time” seconds prior to the supplied AK’s scheduled expiration

4-D Reauth Wait (Auth Reply) → Authorized

a) Clear Auth Request retry timer

b) Decrypt and record AK delivered with Auth Reply

c) Start TEK FSMs for any newly authorized SAIDs listed in Auth Reply (provided the CPE supports the cryptographic suite that is associated with the new SAID) and issue TEK FSM Authorized event for each of the new TEK FSMs

d) Generate TEK FSM Authorization Complete events for any currently active TEK FSMs whose corresponding SAIDs were listed in Auth Reply

e) Generate TEK FSM Stop events for any currently active TEK FSMs whose corresponding SAIDs were not listed in Auth Reply

f) Set the Authorization Grace timer to go off “Authorization Grace Time” seconds prior to the supplied AK’s scheduled expiration

5-B Auth Wait (Timeout) → Auth Wait

a) Send Auth Info message to BS

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Auth Wait Timeout

5-D Reauth Wait (Timeout) → Reauth Wait

a) Send Auth Request message to BS

b) Set Auth Request retry timer to Reauth Wait Timeout

5-E Auth Reject Wait (Timeout) → Start

a) No protocol actions associated with state transition

6-C Authorized (Auth Grace Timeout) → Reauth Wait

a) Send Auth Request message to BS

b) Set Auth Request retry timer to Reauth Wait Timeout

7-C Authorized (Auth Invalid) → Reauth Wait

a) Clear Authorization Grace timer

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Reauth Wait Timeout

d) If the Auth Invalid event is associated with a particular TEK FSM, generate a TEK FSM Authorization Pending event for the TEK state machine responsible for the Auth Invalid event (i.e., the TEK FSM that either generated the event, or sent the Key Request message the BS responded to with an Auth Invalid message)

7-D Reauth Wait (Auth Invalid) → Reauth Wait

a) If the Auth Invalid event is associated with a particular TEK FSM, generate a TEK FSM Authorization Pending event for the TEK state machine responsible for the Auth Invalid event (i.e., the TEK FSM that either generated the event, or sent the Key Request message the BS responded to with an Auth Invalid message)

8-C Authorized (Reauth) → Reauth Wait

a) Clear Authorization Grace timer

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Reauth Wait Timeout

7.2.3.5 Security capabilities selection

As part of their authorization exchange, the CPE provides the BS with a list of all the cryptographic suites (pairing of data encryption and data authentication algorithms) the CPE supports. The TLVs that describe the cryptographic suite options are in section 11.9.x). The BS selects from this list a single cryptographic suite to employ with the requesting CPE’s primary SA. The Authorization Reply the BS sends back to the CPE includes a primary SA-Descriptor that, among other things, identifies the cryptographic suite the BS selected to use for the CPE’s primary SA. A BS shall reject the authorization request if it determines that none of the offered cryptographic suites are satisfactory.

The Authorization Reply also contains an optional list of static SA-Descriptors; each static SA-Descriptor identifies the cryptographic suite employed within the SA. The selection of a static SA’s cryptographic suite is typically made independent of the requesting CPE’s cryptographic capabilities. A BS may include in its Authorization Reply static SA-Descriptors identifying cryptographic suites the requesting CPE does not support; if this is the case, the CPE shall not start TEK state machines for static SAs whose cryptographic suites the CPE does not support.

If the SA defines use of authentication and encryption method, all MAC PDUs sent with CIDs linked to this SA must have EC bit set to ‘1’ in the Generic MAC Header. Otherwise, if only authentication is supported the EC bit must be set to ‘0’ in the Generic MAC Header. Other combinations are not allowed; MAC PDUs presenting other combinations should be discarded.

The capabilities defined in an SA are not applicable to management messages transmitted on the Initial Ranging, Basic, Multicast, as well as Broadcast CIDs. Only traffic assigned to an SA where “no authorization” is specificed, will be allowed to be transmitted without any authentication or encryption information.

7.2.4 TEK exchange overview

7.2.4.1 TEK exchange overview for PMP topology

If the CPE and BS decide “No authorization” as their authorization policy, the CPE and BS shall perform neither SA-TEK handshake nor Key Request/Key Reply handshake. In this case, target SAID value, which may be included in DSA-REQ/RSP messages, shall be Null SAID.

Upon achieving authorization, a CPE starts a separate TEK state machine for each of the SAIDs identified in the Authorization Reply message. Each TEK state machine operating within the CPE is responsible for managing the keying material associated with its respective SAID. TEK state machines periodically send Key Request messages to the BS, requesting a refresh of keying material for their respective SAIDs.

TEK state machines periodically cause the CPE to send Key Request messages to the BS, requesting a refresh of keying material for their respective SAIDs. The BS responds to a Key Request with a Key Reply message, containing the BS’s active keying material for a specific SAID.

The TEK is encrypted using appropriate KEK derived from the AK. For AES-CCM, the TEK is a 128 bit key and the KEK is derived from the AK using a 128 bit key and 128 bit block size.

Note that at all times the BS maintains two active sets of keying material per SAID. The lifetimes of the two generations overlap so that each generation becomes active halfway through the life of it predecessor and expires halfway through the life of its successor. A BS includes in its Key Replies both of an SAID’s active generations of keying material.

For SAs using a ciphersuite employing AES-CCM mode, the Key Reply provides the requesting CPE, in addition to the TEK, the remaining lifetime of each of the two sets of keying material. The receiving CPE uses these remaining lifetimes to estimate when the BS will invalidate a particular TEK and, therefore, when to schedule future Key Requests so that the CPE requests and receives new keying material before the BS expires the keying material the CPE currently holds. For AES-CCM mode, when more than half the available PN numbers in the 24-bit PN number space are exhausted, the CPE shall schedule a future Key Request in the same fashion as if the key lifetime was approaching expiry. The operation of the TEK state machine’s Key Request scheduling algorithm, combined with the BS’s regimen for updating and using an SAID’s keying material (see 7.3), ensures that the CPE will be able to continually exchange encrypted traffic with the BS.

The operation of the TEK state machine’s Key Request scheduling algorithm, combined with the BS’s regimen for updating and using an SAID’s keying material (see 7.4), ensures that the CPE will be able to continually exchange encrypted traffic with the BS.

A TEK state machine remains active as long as

a) The CPE is authorized to operate in the BS’s security domain, i.e., it has a valid AK, and

b) The CPE is authorized to participate in that particular SA, i.e., the BS continues to provide fresh key-ing material during rekey cycles.

The parent Authorization state machine stops all of its child TEK state machines when the CPE receives from the BS an Authorization Reject during a reauthorization cycle. Individual TEK state machines can be started or stopped during a reauthorization cycle if a CPE’s Static SAID authorizations changed between successive reauthorizations.

Communication between Authorization and TEK state machines occurs through the passing of events and protocol messaging. The Authorization state machine generates events (i.e., Stop, Authorized, Authorization Pending, and Authorization Complete events) that are targeted at its child TEK state machines. TEK state machines do not target events at their parent Authorization state machine. The TEK state machine affects the Authorization state machine indirectly through the messaging a BS sends in response to a CPE’s requests: a BS may respond to a TEK machine’s Key Requests with a failure response (i.e., Authorization Invalid message) to be handled by the Authorization state machine.

7.2.4.2 TEK state machine

The TEK state machine consists of seven states and eleven events (including receipt of messages) that may trigger state transitions. Like the Authorization state machine, the TEK state machine is presented in both a state flow diagram (Figure 134) and a state transition matrix (Table 134). As was the case for the Authorization state machine, the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

Shaded states in Figure 131 (Operational, Rekey Wait, Rekey Reauthorize Wait, and M&B Rekey InterimWait) have valid keying material and encrypted traffic may be sent.

The SAID may be replaced by the GSAID for the multicast service or the broadcast service. And, the TEK may be also replaced by the GTEK for the multicast service or the broadcast service.

The Authorization state machine starts an independent TEK state machine for each of its authorized SAIDs. As mentioned in 7.2.2, the BS maintains two active TEKs per SAID.

For the unicast service, the BS includes in its Key Replies both of these TEKs, along with their remaining lifetimes.

The BS encrypts downlink traffic with the older of its two TEKs and decrypts uplink traffic witheither the older or newer TEK, depending upon which of the two keys the CPECPE was using at the time. The CPECPE encrypts uplink traffic with the newer of its two TEKs and decrypts downlink traffic with either the older or newer TEK, depending upon which of the two keys the BS was using at the time. See 7.3 for details on CPECPE and BS key usage requirements.

For the multicast service or the broadcast service, the BS may include both of GTEKs in its Key Reply messages, when an CPE request traffic keying material. And, the BS may include the newer GTEK in the KeyUpdate Command message, when the BS transmits the new traffic keying material in key push mode. The BS encrypts downlink traffic with current GTEK. The CPE decrypts downlink traffic with either the older or newer GTEK, depending upon which of the two keys the BS is using at the time. See 7.9 for details on CPE and BS key usage requirements.

Through operation of a TEK state machine, the CPECPE attempts to keep its copies of an SAID’s TEKs synchronized with those of its BS. A TEK state machine issues Key Requests to refresh copies of its SAID’s keying material soon after the scheduled expiration time of the older of its two TEKs and before the expiration of its newer TEK. To accommodate for CPE/BS clock skew and other system processing and transmission delays, the CPECPE schedules its Key Requests a configurable number of seconds before the newer TEK’s estimated expiration in the BS. With the receipt of the Key Reply, the CPECPE shall always update its records with the TEK Parameters from both TEKs contained in the Key Reply message. TEK Parameters contained in the two Key Update Command messages for the multicast service or the broadcast service.

Figure. TEK State Machine Flow Diagram

Table – TEK State Transition Finite State Machine

7.2.4.2.1 States

Start: This is the initial state of the FSM. No resources are assigned to or used by the FSM in this state—e.g., all timers are off, and no processing is scheduled.

Operational Wait (Op Wait): The TEK state machine has sent its initial request (Key Request) for its SAID’s keying material (TEK), and is waiting for a reply from the BS.

Operational Reauthorize Wait (Op Reauth Wait): The wait state the TEK state machine is placed in if it does not have valid keying material while the Authorization state machine is in the middle of a reauthorization cycle.

Operational: The CPECPE has valid keying material for the associated SAID.

Rekey Wait: The TEK Refresh Timer has expired and the CPE has requested a key update for this SAID. Note that the newer of its two TEKs has not expired and may still be used for both encrypting and decrypting data traffic.

Rekey Reauthorize Wait (Rekey Reauth Wait): The wait state the TEK state machine is placed in if the TEKstate machine has valid traffic keying material, has an outstanding request for the latest keying material, and the Authorization state machine initiates a reauthorization cycle.

M&B Rekey Interim Wait (Multicast & Broadcast Rekey Interim Wait): This state is defined only for the

multicast service or the broadcast service. This state is the wait state the TEK state machine is placed in if the TEK state machine has valid traffic keying material and receives the new GKEK from the BS.

7.2.4.2.2 Messages

Note that the message formats are defined in detail in 6.3.2.3.9.

Key Request: Request a TEK for this SAID. Sent by the CPE to the BS and authenticated with keyed message digest. The message authentication key is derived from the AK.

Key Reply: Response from the BS carrying the two diversity sets of traffic keying material for this SAID. Sent by the BS to the CPE, it includes the SAID’s TEKs, encrypted with a KEK derived from the AK or the GSAID’s GTEK, encrypted with a GKEK randomly generated from the BS or the ASA server. The Key Reply message is authenticated with a keyed message digest; the authentication key is derived from the AK.

Key Reject: Response from the BS to the CPECPE to indicate this SAID is no longer valid and no key will be sent. The Key Reject message is authenticated with a keyed message digest; the authentication key is derived from the AK.

TEK Invalid: The BS sends an CPECPE this message if it determines that the CPECPE encrypted an uplink PDU with an invalid TEK, i.e., an SAID’s TEK key sequence number, contained within the received PDU’s MAC Header, is out of the BS’s range of known, valid sequence numbers for that SAID.

Key Update Command: Push a GTEK for this GSAID for the multicast service or the broadcast service. Sent by the BS to the CPECPE and authenticate with keyed message digest. The message authentication key is derived from the AK in the Key Update Command message for the GKEK update mode. The message authentication key is derived from the GKEK in the Key Update Command message for the GTEK update mode.

7.2.4.2.3 Events

Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate TEK FSM and remove the corresponding SAID’s keying material from the CPECPE key table. See Figure 130k.

Authorized: Sent by the Authorization FSM to a non-active (START state) TEK FSM to notify TEK FSM of successful authorization. See Figure 130k.

Authorization Pending (Auth Pend): Sent by the Authorization FSM to TEK FSM to place TEK FSM in a wait state while Authorization FSM completes reauthorization. See Figure 130k.

Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational

Reauthorize Wait or Rekey Reauthorize Wait states to clear the wait state begun by the prior Authorization Pending event. See Figure 130k.

TEK Invalid: This event is triggered by either an CPEs data packet decryption logic or by the receipt of a TEK Invalid message from the BS.

An CPECPEs data packet decryption logic triggers a TEK Invalid event if it recognizes a loss of TEK key synchronization between itself and the encrypting BS. For example, an SAID’s TEK key sequence number, contained within the received downlink MAC PDU header, is out of the CPECPEs range of known sequence numbers for that SAID.

A BS sends an CPECPEs a TEK Invalid message, triggering a TEK Invalid event within the CPE, if the BS’s decryption logic recognizes a loss of TEK key synchronization between itself and the CPE.

Timeout: A retry timer timeout. Generally, the particular request is retransmitted.

TEK Refresh Timeout: The TEK refresh timer timed out. This timer event signals the TEK state machine to issue a new Key Request in order to refresh its keying material. The refresh timer is set to fire a configurable duration of time (TEK Grace Time) before the expiration of the newer TEK the CPECPE currently holds. This is configured via the BS to occur after the scheduled expiration of the older of the two TEKs.

GKEK Updated: This event is triggered when the CPECPE receives the new GKEK through the Key Update Command message for the GKEK update mode.

GTEK Updated: This event is triggered when the CPECPE receives the new GTEK and traffic keying material through the Key Update Command message for the GTEK update mode.

7.2.4.2.4 Parameters

All configuration parameter values take the default values from Table 343 or may be specified in Auth Reply message.

Operational Wait Timeout: Timeout period between sending of Key Request messages from the Op Wait state (see 11.9.19.4).

Rekey Wait Timeout: Timeout period between sending of Key Request messages from the Rekey Wait state (see 11.9.19.5).

TEK Grace Time: Time interval, in seconds, before the estimated expiration of a TEK that the CPE starts rekeying for a new TEK. TEK Grace Time takes the default value from Table 343 or may be specified in a configuration setting within the Auth Reply message and is the same across all SAIDs (see 11.9.19.6).

M&B TEK Grace Time (Multicast & Broadcast TEK Grace Time): Time interval, in seconds, before the estimated expiration of an old distributed GTEK.

7.2.4.2.5 Actions

Actions taken in association with state transitions are listed by <event> (<rcvd message>) --> <state>:

1-B Op Wait (Stop) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

1-C Op Reauth Wait (Stop) Start

a) Terminate TEK FSM

1-D Operational (Stop) Start

a) Clear TEK refresh timer, which is timer set to go off “TEK Grace Time” seconds prior to the TEK’s scheduled expiration time

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-E Rekey Wait (Stop) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-F Rekey Reauth Wait (Stop) Start

a) Terminate TEK FSM

b) Remove SAID keying material from key table

2-A Start (Authorized) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

3-B Op Wait (Auth Pend) Op Reauth Wait

a) Clear Key Request retry timer

3-E Rekey Wait (Auth Pend) Rekey Reauth Wait

a) Clear Key Request retry timer

4-C Op Reauth Wait (Auth Comp) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

4-F Rekey Reauth Wait (Auth Comp) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

5-D Operational (TEK Invalid) Op Wait

a) Clear TEK refresh timer

b) Send Key Request message to BS

c) Set Key Request retry timer to Operational Wait Timeout

d) Remove SAID keying material from key table

5-E Rekey Wait (TEK Invalid) Op Wait

a) Clear TEK refresh timer

b) Send Key Request message to BS

c) Set Key Request retry timer to Operational Wait Timeout

d) Remove SAID keying material from key table

5-F Rekey Reauth Wait (TEK Invalid) Op Reauth Wait

a) Remove SAID keying material from key table

6-B Op Wait (Timeout) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

6-E Rekey Wait (Timeout) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

7-D Operational (TEK Refresh Timeout) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

7-G M&B Rekey Interim Wait (TEK Refresh Timeout) -> Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

8-B Op Wait (Key Reply) Operational

a) Clear Key Request retry timer

b) Process contents of Key Reply message and incorporate new keying material into key database

c) Set the TEK refresh timer to go off “TEK Grace Time” seconds prior to the newer key’s scheduled expiration

8-E Rekey Wait (Key Reply) Operational

a) Clear Key Request retry timer

b) Process contents of Key Reply message and incorporate new keying material into key database

c) Set the TEK refresh timer to go off “TEK Grace Time” seconds prior to the newer key’s scheduled expiration

9-B Op Wait (Key Reject) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

9-E Rekey Wait (Key Reject) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

c) Remove SAID keying material from key table

10-D Operational (GKEK Updated) -> M&B Rekey Interim Wait

a) Process contents of Key Update Command message for the GKEK update mode and incorporate new GKEK into key database

11-G M&B Rekey Interim Wait (GTEK Updated) -> Operational

a) Clear Key Request retry timer

b) Process contents of Key Update Command message for the GTEK update mode and incorporate new traffic keying material into key database

c) Set the TEK refresh timer to go off “TEK Grace Time” seconds prior to the key’s scheduled expiration

7.2.5 Key derivation

The SCM key hierarchy defines what keys are present in the system and how the keys are generated. IEEE 802.22 systems shall use either one of two, either ECC-based or RSA based, authentication schemes. The keys used to protect management message integrity and transport the TEKs are derived from source key material generated by the authentication and authorization processes. The ECC/RSA-based authorization process yields the pre-Primary AK (pre-PAK. All SCM key derivations are based on the Dot22KDF algorithm as defined in 7.5.4.6.1.

7.2.5.1 AK derivation

The AK will be derived by the BS and the CPE from the PAK (from ECC/RSA-based authorization procedure). Note that PAK can be used according to the value of Authorization Policy Support field included in the CBC-REQ/RSP messages.

After the authorization procedure has been performed, the CPE and BS will both posses the PAK. The derivation of the AK varies based on which keys are possessed.

The AK shall be generated as follows:

AK ⇐ Dot22KDF (PAK, CPE MAC Address | BSID | PAK | “AK”, 160)

7.2.5.2 KEK derivation

The KEK is derived directly from the AK. The KEK is defined in 7.2.2.2.9. It is used to encrypt the TEKs, GKEK and all other keys sent by the BS to CPE in unicast message.

7.2.5.3 GKEK derivation

GKEK is randomly generated at the BS or a network entity (for example, an ASA server) and transmitted to the CPE encrypted with the KEK. There is one GKEK per Group Security Association. GKEK is used to encrypt the GTEKs sent by the BS to the CPEs in the same multicast.

7.2.5.4 Traffic encryption key (TEK)

The TEK is generated as a random number in the BS and is encrypted using the corresponding TEK encryption algorithm (e.g., AES key wrap for SAs with TEK encryption algorithm identifier in the cryptographic suite is equal to 0x01-0x04), keyed with the KEK and transferred between BS and CPE in the TEK exchange.

7.2.5.5 Group traffic encryption key (GTEK)

The GTEK is used to encrypt data packets of the multicast service and it is shared among all CPEs that belong to the multicast group. There are two GTEKs per GSA. The GTEK is randomly generated at the BS or at certain network node and is encrypted using same algorithms applied to encryption for TEK and transmitted to the CPE in broadcast or unicast messages. The GTEK in a SCM Key-Reply message shall be encrypted by the KEK. Also, the GTEK in a PKMv2 Group Key Update Command message will be encrypted by the GKEK.

7.2.5.6 Management Message Protection key and KEK derivation

7.2.5.6.1 MMP_PN management

The CPE shall maintain a MMP_PN counter for each AK. The BS is assumed to maintain a MMP_PNcounter for each AK context as well. This is done to keep the MMP_PN value synchronized with the corresponding counter at the CPE. The value of this counter maintained by the CPE is denoted as MMP_PNC and the value maintained by the BS is denoted as MMP_PNB.

7.2.5.6.1.1 Maintenance of MMP_PNC by the CPE

Upon successful completion of the SCM Authorization or Re-authorization, and establishment of a new AK, the CPE shall instantiate a new MMP_PN counter and set its value to 1. The CPE shall initiate re-authorization before the MMP_PNC expires, e.g. reaches the half of the counter space (0x7FFFFF)., The AK Lifetime (See 7.2.10.1) shall not be set to a value less than the expected amount of time required to for the MMP_PN to be exhausted. The CPE shall manage a separate MMP_PNC counter for every active AK context. Specifically, during re-authorization, but before the activation of the new AK, the old MMP_PNC (corresponding to the old AK) shall be used signing and/or encryption of MAC control messages, while the new MMP_PNC shall be used for signing and/or encrypting of SCM Key Request messages. If CPE should lose its AK context, then CPE shall restart the authentication process.

7.2.5.6.1.2 Processing of MMP_PNB by the BS

The BS may possess one or more AK contexts associated with the CPE, each of which includes the value of MMP_PNB. This value shall be maintained as specified in subsequent paragraphs of this section.

Upon successful completion of the SCM Authorization and Re-authorization, and establishment of a new AK context, the BS shall set MMP_PNB of the corresponding newly instantiated AK context to 1. The BS shall manage a separate MMP_PN_U/DB for every AK context it is maintaining. Specifically, during re-authorization, but before the activation of the new AK, the old MMP_PNB (corresponding to the old AK context) shall be used for signing and/or encryptionof MAC control messages, while the new MMP_PNB shall be used for signing and/or encrypting of SCM Key Replymessages.

During periodic ranging, the CPE can optionally transmit the RNG-REQ containing the MMP_PN parameter, the BS shall compare the received MMP_PN value, which is MMP_PNC, with MMP_PNB (i.e., the value of MMP_PN counter maintained by the BS for the corresponding AK context). If MMP_PNC < MMP_PNB, the BS shall process the message as being invalid and send a RNG-RSP message requesting re-authorization. This can happen when either the BS or CPE loses the AK context.

7.2.5.6.2 Derivation of Management Message Protection (MMP) keys and KEKs

MMP keys are used to sign and/or encrypt management messages in order to validate the authenticity and protect the contents of these messages. The level of protection to be used for management messages is negotiated at CPE Basic Capabilities negotiation.

There is a different key for UL and DL messages. Also, a different message authentication key is generated for a broadcast message (this is DL direction only) and for a unicast message.

The keys used for CMAC key and for KEK are as follows:

MMP_PREKEY | KEK ⇐ Dot22KDF(AK, CPE MAC Address | BSID | “MMP_KEY+KEK”, 256)

MMP_KEY_GD ⇐ Dot22KDF(GKEK, “GROUP MMP KEY”,128) (Used for broadcast MAC message such as a PKMv2 Group-Key-Update-Command message)

MMP_KEY ⇐ AESMMP_PREKEY (MMP_PN)

For a fixed CPE, the MMP_PN shall be set to 0 in the derivation of the MMP_KEY at the BS and the CPE. Specifically, the preprocessed value of MMP_PREKEY is treated as the Cipher Key of the Advanced Encryption Standard (AES) algorithm AES128 (FIPS197). The MMP_PN is treated as the Input Block Plain Text of this algorithm. The AES128 algorithm is executed once. The Output Block Cipher Text of this algorithm is treated as the resulting MMP_KEY. When MMP_PN is used as an input of AES128 algorithm, 104 zero bits are prepadded before the 24-bit MMP_PN where the MMP_PN is regarded as most-significant-bit first order.

7.2.5.6.3 Key Derivation Function

The Dot22KDF algorithm is a CTR mode construction that may be used to derive an arbitrary amount of keying material from source keying material.

Dot22KDF(key, astring, keylength)

{

result = null;

Kin = Truncate (key, 128);

for (i = 0; i < ceil((keylength-1)/128); i++) {

result = result | AESKin(i | astring | keylength | result);

}

return Truncate (result, keylength);

}

7.2.6 Key hierarchy

Figure 162 outlines the process to calculate the AK when the RSA/ECC-based authorization process has taken place.

Figure 162. AK from PAK Only (from RSA based authorization)

Figure 165 outlines the unicast key hierarchy starting from the AK.

Figure 165. MMP / KEK derivation from AK

7.2.7 Maintenance of AK

The BS and CPE maintain cached AK as follows:

- Successful completion of the SCM ECC/RSA Authorizaiton and establishment of a PAK, causes the activation of all the AK associated with the new PAK.

- If the packet counter belonging to MMP key reaches its maximum value, the associated AK becomes permanently deactivated.

- The BS and CPE must maintain the AK context (i.e., replay counters etc.) as long as they retain the AK.

7.2.8 AK switching methods

Once the SCM SA-TEK 3-way handshake begins, the BS and CPE shall use the new AK for the 3-way handshake messages. Other messages shall continue to use the old AK until the 3-way handshake completes successfully. Upon successful completion of the 3-way handshake, all messages shall use the new AK. The old AK may be used for receiving packets before the “frame number” attribute specified in SCM SA-TEK-response message.

7.2.9 Associations

Keying material is held within associations. There are two types of association: The security associations (SA) that maintain keying material for unicast connections; and group security associations (GSA) that hold keying material for multicast groups. If CPEand BS decide “No authorization” as their authorization policy, they do not have any security association. In this case, Null SAID shall be used as the target SAID field in DSA-REQ/RSP messages.

7.2.9.1 Security associations

A security association contains keying material that is used to protect unicast connections. The contents of an SA are as follows:

- The SAID, a 16-bit identifier for the SA. The SAID shall be unique within a BS.

- The KEK, a 128-bit key encryption key, derived from the AK.

- MMP Keys for encryption of upstream and downstream management traffic, MMP_KEY

- MMP_PN, 24 bit packet numbers for use by link cipher to protect management messages

- RxMMP_PN, 24-bit receive sequence counter, for use by link cipher to protect management messages

- TEK0 and TEK1, 128-bit traffic encryption keys, generated within the BS and transferred from the BS to the CPE using a secure key exchange.

- The TEK Lifetimes TEK0 and TEK1, a key aging lifetime value.

- PN0 and PN1, 24-bit packet numbers for use by the link cipher.

- RxPN0 and RxPN1, 24-bit receive sequence counter, for use by the link cipher.

7.2.9.2 Group Security Association

The Group Security Association (GSA) contains keying material used to secure multicast or broadcast transmissions. These are defined separately from SAs since GSA offer a lower security bound than unicast security associations, since keying material is shared between all members of the group, allowing any member of the group to forge traffic as if it came from any other member of the group.

The contents of a GSA are as follows:

- The Group Key Encryption Key (GKEK). Serves the same function as an SA KEK but for a GSA.

- The Group Traffic Encryption Key (GTEK). Served the same function as an SA TEK but for a GSA.

7.2.10 Security context

The security context is a set of parameters linked to a key in each hierarchy that defines the scope while the key usage is considered to be secure. Examples of these parameters are key lifetime and counters ensuring the same encryption will not be used more than once. When the context of the key expires, a new key should be obtained to continue working.

The purpose of this subclause is to define the context that belongs to each key, how it is obtained and the

scope of its usage.

7.2.10.1 AK context

The AK key has two phases of lifetime: the first begins at AK creation and the second begins after validation by the 3-way handshake.

The phases ensure that when the AK is created it will be defined with the PAK pre-handshake lifetime and after successful 3-way handshake.

If the cached AK and associated context is lost by either BS or CPE, no new TEKs can be derived from this AK on handover. Cached AKs that were derived from the PMK can continue to be used in HO. Reauthentication is required to obtain a new AK so as to derive new TEKs.

The AK context is described in Table 133a.

Table – AK Context in SCM

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| AK | 160 | The authorization key, calculated as defined in 7.2.2.2.3. |
| AKID | 4 | Sequence number of root key (PAK) for the AK. This value is the most significant 2-bit of PAK sequence number, AK SN = PAK SN |
| AK Lifetime | --- | This is the time this key is valid; it is calculated AK lifetime = PAK lifetime. This this expires, re-authentication is needed. |
| AK Sequence Number | 4 | The sequence number of the PAK from which this AK is derived. |
| KEK | 160 | Used to encrypt transport keys from the BS to the CPE. |
| MMP_KEY_U | 128 | The key which is used for signing and/or encrypting UL management messages |
| MMP_KEY_D | 128 | The key which is used for signing and/or encrypting DL management messages. |
| MMP_PN_U | 24 | Used to avoid UL replay attack on management connection. When this expires re-authentication is needed. |
| MMP_PN_D | 24 | Used to avoid DL replay attack on management connection. When this expires re-authentication is needed. |

7.2.10.2 GKEK context

The GKEK is the head of the group key hierarchy. There is a separate GKEK for each group (each GSA).

This key is randomly generated by the BS and transferred to the CPEencrypted with KEK. It is used to encrypt group TEKs (GTEK) when broadcasting them to all CPEs. The GKEK context is described in Table 133b.

Table – GKEK Context

7.2.10.3 PAK context

The PAK context includes all parameters associated with the PAK. This context is created when RSA

Authentication completes.

The PAK context is described in Table 133d.

Table – PAK Context

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| PAK | 160 | A key yielded from ECC/RSA-based authentication |
| PAK Lifetime | 4 | PAK lifetime, from when the ECC/RSA-based authorization is achieved. The value of the PAK lifetime is initially set to a default value. The 3-way handshake may subsequently change this value. |
| PAK Sequence Number | 4 | PAK sequence number, when the ECC/RSA-based authorization is achieved and a key is generated. The most significant 2 bits are the sequence counter, and the least significant bits are set to o. |

7.3 Key Usage

7.3.1 BS Key Usage

The BS is responsible for maintaining keying information for all SAs. The SCM protocol defined in this specification describes a mechanism for synchronizing this keying information between a BS and its client CPE.

7.3.1.1 AK Lifetime

At initial network entry, if the security is enabled during the basic capabilities negotiation, the authorization procedure shall be initiated. The authorization procedure activates a new AK. This AK shall remain active until it expires according to its predefined AK Lifetime, a BS system configuration parameter. In SCM, AK lifetime is determined the PAK lifetime or by the expiration of the MMP_PN_U or MMP_PN_D.

A CPE will initiate re-authorization before the expiration of it’s current AK. This leads the the CPE to have two active sets of keying material. If an CPE fails to reauthorize before the expiration of its current AK, the BS shall hold no active AKs for the CPE and shall consider the SS unauthorized. A BS shall remove from its keying tables all TEKs associated with an unauthorized CPEs SA.

7.3.1.2 AK Transition Period on BS

The BS shall always be prepared to start re-authentication upon request. The BS shall be able to support two simultaneously active AKs for each client SS. The BS has two active AKs during an AK transition period; the two active keys have overlapping lifetimes.

In SCM, an AK transition period begins when the BS receives an Auth Request message from an CPE and the BS has a single active AK for that CPE. In response to this Auth Request, the BS activates a second AK [see point (a) and (d) in Figure 166], which shall have a key sequence number one greater (modulo 16) than that of the existing AK and shall be sent back to the requesting CPE in an Auth Reply message. The BS shall set the active lifetime of this second AK to be the remaining lifetime of the first AK [between points (a) and (c) in Figure 166], plus the predefined AK Lifetime; thus, the second, “newer” key shall remain active for one AK Lifetime beyond the expiration of the first, “older” key. The key transition period shall end with the expiration of the older key. This is depicted on the right-hand side of Figure 166.

As long as the BS is in the midst of an CPE AK transition period, and thus is holding two active AKs for that CPE, it shall respond to Auth Request messages with the newer of the two active keys. Once the older key expires, an Auth Request shall trigger the activation of a new AK, and the start of a new key transition period.

Figure 166

7.3.1.3 BS usage of AK

The BS shall make use of keys derived from the CPE’s AK for the following purposes:

- Verify MAC-Digests and/or decrypt (See 7.5.x) of SCM Key Request messages as well as all other non-SCM related management messages received from CPE

- Calculate MAC-Digests and/or encrypt (See 7.5.x) for SCM Key Reply messages as well as other non-SCM related management messages sent to CPE

- Encrypting TEK when it is sent to CPE in the Key Reply message

The MMP_KEY is used to verify MAC Digests and/or decrypt (see 7.5.x) of SCM Key Request messages as well as other non-SCM related management messages received from CPE. The PN used in this operation is MMP_PN (See 7.2.5.6.1.2). When a CPE issues an SCM Key Request message, it shall include the AK Sequence Number, to indicate that it is using the newer of the two AKs that it is assigned.

The MMP_KEY_D is used to verify MAC Digests and/or decrypt (see 7.5.x) of SCM Key Reply messages as well as other non-SCM related management messages sent to the CPE. The PN used in this operation is MMP_PN_D (See 7.2.5.6.1.1). If the BS has been informed by the CPE that the Key Request issued by the CPE was issued with the newer AK, then BS shall use the MMP_KEY_D derived from the new AK and initialize MMP_PN_D associated with the new AK to 0. If the BS has no knowledge that the CPE is using the new AK, then it will use the MMP_KEY_D derived from the old AK and use the current value of MMP_PN_D associated with the older AK. The CPE indicates which AK is being used by including the AK Sequence Number of that AK in the Key Request message.

Prior to expiration of the MME_KEY_COUNT_U/D, when half the key space (2^23) has been used up, the CPE conduct re-authorization.

7.3.1.4 TEK lifetime

The BS maintains two, active TEKs per SA. Both TEKs will have overlapping lifetimes. TEK Lifetime is a system parameter that determines the length of time that the TEK is valid for. TEK Lifetime is only invalidated when the PN associated with that TEK is about to expire. The TEK Lifetime is communicated to the CPE, along with both generations of TEKs, from the BS in the SCM Key Reply message.

In the GMH, the usage of the newer TEK is indicated in the value of the EKS field. For the newer TEK, the EKS field is 1 greater (modulo 4) than that of the older TEK.

7.3.1.5 BS usage of TEK

Two generations of TEKs shall be maintained per SA. Transitioning between both generations of TEKs and how they are used are dependent on whether or not the TEK is used for DS or US traffic.

The BS transitions from using the older generation to the newer generation (for each of its’ SAs) based on the following rules:

- At the expiration of the older TEK, the BS shall immediately transition to using the newer TEK for encryption.

- The transitional period for the encrypting US traffic begins when the BS issues the newer TEK to the CPE in the SCM Key Reply message and concludes that the older TEK has expired (i.e. it’s PN has expired)

The BS makes use of both generations of TEKs, based on the following rules:

- BS shall use the older TEK for encrypting DS traffic.

- BS shall be able to decrypt US traffic using either of the two TEK

It is the responsibility of the SS to update its keys in a timely fashion; the BS shall transition to a new DL encryption key regardless of whether a client SS has retrieved a copy of that TEK.

Note that the BS encrypts with a given TEK for only the second half of that TEK’s total lifetime (See Figure XXX). The BS is able, however, to decrypt with a TEK for the TEK’s entire lifetime.

Figure. TEK Management in BS and CPE

7.3.2 CPE Key Usage

CPEs shall maintain an active AK and stay authorized with the BS. This requires the CPE to initiate re-aurhorization periodically. Upon completing authorization and re-authorization, the CPE shall also be responsible for maintaining active keying material (e.g. TEKs) for encrypting DS and US traffic.

7.3.2.1 CPE Reauthorization

AKs have a limited lifetime and shall be periodically refreshed. In SCM, a CPE refreshes its AK by reissuing an Authorization Request to the BS. The Authorization State Machine (See XXXX) manages the scheduling of Authorization Requests for refreshing AKs. In SCM RSA-based authorization, the CPE refreshes its AK by issuing a SCM RSA-Request message. In ECC-based authorization the CPE refreshes its AK by issuing a SCM ECC-Request message.

In SCM, an CPE’s Authorization state machine schedules the beginning of reauthorization a configurable duration of time, the Authorization Grace Time, [see points (x) and (y) in Figure 166], before the CPE’s latest AK is scheduled to expire. The Authorization Grace Time is configured to provide an CPE with an authorization retry period that is sufficiently long to allow for system delays and provide adequate time for the SS to successfully complete an Authorization exchange before the expiration of its most current AK. The Authorization Grace Time should be set at a value that doesn’t expire for the MMP_PN_D/U expires.

Note that the BS does not require knowledge of the Authorization Grace Time. The BS, however, shall track the lifetimes of its AKs and shall deactivate a key once it has expired.

7.3.2.2 CPE usage of AK

A CPE shall use the MMP_KEY_U derived from the newer of the two AKs it has when calculating MAC Digests and/or encryption of SCM Key Reqeust, and other non-SCM related management messages that are transmitted to the BS.

The CPE shall be capable of using the MMP_KEY_D derived from either one of its AKs to verify and/or decrypt SCM Key Reply, SCM TEK Invalid, and other non-SCM related management messagests thare are received from the BS.

A CPE shall use the MMP_KEY_U derived from the newer of its two most recent AKs when calculating the MAC-Digests of management messages..

7.3.2.3 CPE usage of TEK

Through operation of its TEK state machines, a CPE shall maintain two, successive sets of keying material for encrypting traffic per SA. The CPE schedules requests for a new set of traffic keying material, based a configurable amount of time, the TEK Grace Time [see points (x) and (y) in Figure 167], before the CPE’s latest TEK is scheduled to expire.

For each of its authorized SAIDs, the SS

- Shall use the newer of its two TEKs to encrypt UL traffic

- Shall be able to decrypt DL traffic encrypted with either of the TEKs.

The left-hand side of Figure 167 illustrates the CPE’s maintenance and usage of an SA’s TEKs, where the shaded portion of a TEK’s lifetime indicates the time period during which that TEK shall be used to encrypt MAC PDU payloads.

7.4 Cryptographic Methods

This subclause specifies the cryptographic algorithms and key sizes used by the SCM protocol. All CPE and BS implementations shall support the method of packet data encryption and authentication defined in 7.5.1, and encryption of the TEK as specified in 7.5.2

All inputs to key derivation and other supporting functions shall be byte aligned. Furthermore, each byte shall be in canonical form as defined in IEEE Std. 802-2001 where the leftmost bit in each byte is the most significant bit and the rightmost bit is the least significant bit.

### 7.4.1 Selection of Data Encryption and Authentication Methods

Only one data encryption and authentication algorithm is supported in 802.22, AES in CCM mode, therefore no specific configuration item is required to define the use of AES CCM. The parameters (see Table 318) associated with SCM Authorization Request/Reply define how AES CCM is to be applied in order to provide authentication and/or encryption for MAC PDU payloads. The cryptographic suite configuration is made up of selecting an Authentication Method, and Encryption Method, and a TEK Encryption method.

Possible values for Authentication method are:

- No Authentication

- Authentication for Unicast

- Authentication for Multicast/Broadcast

Possible values for Encryption method are:

- No Encryption

- Encryption for Unicast

- Encryption for Multicast/Broadcast

Possible values for TEK/GTEK Encryption Method are:

- AES-128 key wrap of TEK/GTEK using KEK/GKEK

The cryptographic suite list is defined in Table 7.4.-1 is a 1-byte construct defined in the following table:

Table 7.4-1

| Value | Cryptographic Suite |
| --- | --- |
| 0x00 | No Authentication, No Encryption |
| 0x01 | Authentication for Unicast, No Encryption, AES-128 key wrap of TEK using KEK |
| 0x02 | Authenticaion and Encryption for Unicast, AES-128 key wrap of TEK using KEK |
| 0x03 | Authentication for Multicast/Broadcast, No Encryption, AES-128 key wrap of GTEK with GKEK |
| 0x04 | Authentication and Encryption for Multicast/Broadcast, AES-128 key wrap of GTEK with GKEK |
| 0x05-0xFF | Reserved |

In Table 318, a configuration parameter for the list of Cryptopgrahic Suites supported will be transmitted to the BS by the CPE in the SCM RSA/ECC Authorization Information, Request and Authorization Reply messages.

Implentation shall support, as a default

7.4.2 Data Encryption and Authentication with AES CCM

If the cryptographic suite selected (see Table 7.4-1 for operation during the Authorization process is 0x01-0x04 than AES in CCM (NIST Special Publication 800-38B and FIPS 197) mode to provide authentication and/or encryption (known as CCM* in IEEE 802.15.4-2006) of MAC PDUs.

7.4.2.1 PDU Format

7.4.2.1.1 Packet Number

The MAC PDU payload shall be prepended with a 3-byte PN (Packet Number). The PN shall be encoded in the MAC PDU least significant byte first. The PN shall not be encrypted.

The PN associated with an SA shall be set to 1 when the SA is established and when a new TEK is installed. Upon completion of Authorization/Re-Authorization and after the MMP_KEY has been derived has been derived, the MMP_PN is set to 1. After each PDU transmission, the PN and MMP_PN shall be incremented by 1.

On UL connections, the PN shall be XORed with 0x800000 prior to encryption and transmission. This effectively splits the PN space into two ranges for DL (0x000000-0x7FFFFF) and UL (0x800001-0xFFFFFF), thereby avoiding collision of PN values when using a single PN for UL and DL. On DL connections, the PN shall be used without such modification.

Any tuple value of {PN, KEY} shall not be used more than once for the purposes of transmitting data. This measure is known a protection against replay attacks.

The CPE shall ensure that a new TEK is requested and transferred before the PN on either the CPE or BS reaches 0x7FFFFFFF. If the PN in either the CPE or BS reaches 0x7FFFFFFF without new keys being installed, transport communications on that SA shall be halted until new TEKs are installed. In the case of the MMP_KEY, if MMP_PN expires, then current AK is invalidated and must start Re-Authorization.

7.4.2.1.2 PDU Format – Authentication Only

The cyphersuites allow for authentication and/or encryption of MAC PDUs. If the suites 0x01 or 0x03 is assigned to the SA, then only authentication is provided for any MAC PDUs transmitted on service flows that are mapped to these SAs.

The AES in CCM protocol is applied in the following manner:

- The Plaintext Payload is processed, generating an Integrity Check Value (ICV) that is 8 bytes long.

- Only the ICV is encrypted using the active TEK/GTEK, generating the Ciphertext ICV

- The Authenticated PDU is formed by appending the Ciphertext ICV to the Plaintext Payload form the authenticated PDU

This requires the EC bit in the GMH to be set to 0. If EC bit is not set to 0, the the PDU shall be discarded, as this would indicate a conflict between the configured cryptographic suite and how it is being applied.

Figure 7.4-1 illustrates how MAC PDUs are processed and formatted when suite 0x01 or 0x03 is configured and the EC bit in GMH is set to 0. The Ciphertext ICV is transmitted so that byte index 0 (as enumerated in NIST Special Publication 800-38) is transmitted first and byte index 7 is transmitted last (i.e., LSB first).

Figure 7.4-1 – Authentication Only PDU Format

7.4.2.1.3 PDU Format – Authentication and Encryption

The cyphersuites allow for authentication and/or encryption of MAC PDUs. If the suites 0x02 or 0x04 is assigned to the SA, then authentication and encryption is provided for any MAC PDUs transmitted on service flows that are mapped to these SAs.

The AES in CCM protocol is applied in the following manner:

- The Plaintext Payload is processed, generating an Integrity Check Value (ICV) that is 8 bytes long.

- Then the ICV is encrypted using the active TEK/GTEK, generating the Ciphertext ICV

- Then the Plaintext Payload is then encrypted with AES using the active TEK/GTEK, generating a Ciphertext Payload

- The encrypted PDU is formed by appending the Ciphertext ICV to the Ciphertext Payload

This requires the EC bit in the GMH to be set to 1. If EC bit is not set to 1, the the PDU shall be discarded, as this would indicate a conflict between the configured cryptographic suite and how it is being applied.

Figure 7.4-2 illustrates how MAC PDUs are processed and formatted when suite 0x02 or 0x04 is configured and the EC bit in GMH is set to 1. The Ciphertext ICV is transmitted so that byte index 0 (as enumerated in NIST Special Publication 800-38) is transmitted first and byte index 7 is transmitted last (i.e., LSB first).

Table 7.4-2 – Encrypted PDU Format

7.4.2.2 CCM Algorithm Constraints

The CCM specification (NIST SP 800-38B) defines specific values for several parameters.

The Tlen parameters specifices then length of the ICV (otherise known as Message Authentication Code, MAC) in bits. This value, as stated in section 7.4.2.1, shall be 64 bits (8bytes).

Consistent with the CCM specification, the 3-bit binary encoding [(t–2)/2)]3 of bits 5, 4, and 3 of the Flags byte in B0 shall be 011.

The size q of the Length field Q shall be set to 2. Consistent with the CCM specification, the 3-bit binary encoding [q-1]3 of the q field in bits 2, 1, and 0 of the Flags byte in B0 shall be 001.

The length a of the associated data string A shall be set to 0.

The nonce shall be 13 bytes long as shown in Figure 7.4-3. Bytes 0 through 3 shall be set to the first 4 bytes of the generic MAC header (thus excluding the HCS). The HCS of the generic MAC header is not included in the nonce since it is redundant. Bytes 4 through 9 are reserved and shall be set to 0x000000000000. Bytes 10 through 12 shall be set to the value of the PN. The PN bytes shall be ordered so that byte 10 shall take the least significant byte and byte 12 shall take the most significant byte.

| Byte | 0 4 | 5 9 | 10 12 |
| --- | --- | --- | --- |
| Field | GMH | Reserved | PN |
| Contents | GMH (without HCS) | 0x0000000000 | PN field from Payload |

Figure 7.4-3 – Nonce Construction

Consistent with the CCM specification, the initial block B0 is formatted as shown in Figure 7.4-4.

| Byte | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte Significance |  |  | MSB LSB |
| Number of Bytes | 1 | 13 | 2 |
| Field | Flag | Nonce | L |
| Contents | 0x19 | As specificed in Figure 7.4.2-3 | Length of plaintext payload |

Figure 7.4-4 – Initial Block, B0, construction for CCM

Consistent with the NIST CCM specification, the counter blocks Ctrj are formatted as shown in Figure 7.4.2-5.

| Byte | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte Significance |  |  | MSB LSB |
| Number of Bytes | 1 | 13 | 2 |
| Field | Flag | Nonce | Counter |
| Contents | 0x1 | As specificed in Figure 7.4.2-3 | i |

Figure 7.4-5 – Counter Block, Ctrj, construction for CCM

7.4.2.3 Receive Processing Rules

On receipt of a PDU the receiving CPE or BS shall decrypt and authenticate the PDU consistent with the NIST CCM specification configured as specified in 7.4.2.2.

Packets that are found to be not authentic shall be discarded.

Receiving BS or CPEs shall maintain a record of the highest value PN and MMP_PN received for each SA. The receiver shall maintain a PN window whose size is specified by the PN_WINDOW_SIZE parameter for SAs and management connections as defined in Table 318.

Any received PDU with a PN lower than the beginning of the PN window shall be discarded as a replay attempt. The receiver shall track PNs within the PN window. Any PN that is received more than once shall be discarded as a replay attempt. Upon reception of a PN, which is greater than the end of the PN window, the PN window shall be advanced to cover this PN.

## 7.5 Certificate Profile

### 7.5.1 Certificate Format

This subclause describes the X.509 Version 3 certificate format and certificate extensions used in IEEE 802.22-compliant CPEs.  The X.509 Version 3 format is defined in IETF RFC 2459.  ASN.1 encoding of algorithm identifiers are also further described in IETF RFC 3279.  The basic X.509 Version 3 certificate format is retained from the reference system. Table 7.6.1-1 highlights the fields of a X.509v3 certificate.

Table 7.6.1-1- Fields of X.509 Version 3 Certificate

| X.509 Version 3 Field | Description |
| --- | --- |
| tbsCertificate.version | Indicates X.509 certificate version. Always set to 3 |
| tbsCertificate.serialNumber | Unique integer the issuing CA assigns to the certificate. |
| tbsCertificate.signature | Object identifier (OID) and optional parameters defining algorithm used to sign the certificate. This field shall contain the same algorithm identifier as the signatureAlgorithm field. |
| tbsCertificate.issuer | Distinguished Name of the CA that issued the certificate. |
| tbsCertificate.validity | Specifies when the certificate becomes active and when it expires. |
| tbsCertificate.subject | Distinguished Name identifying the entity whose public key is certified in the subjectpublic key information field. |
| tbsCertificate.subjectPublicKeyInfo | Field contains the public key material (public key and parameters) and the identifier of the algorithm with which the key is used |
| tbsCertificate.issuerUniqueID | Optional field to allow reuse of issuer names over time. |
| tbsCertificate.subjectUnique ID | Optional field to allow reuse of subject names over time. |
| tbsCertificate.extensions | The extension data. |
| signatureAlgorithm | OID and optional parameters defining algorithm used to sign the certificate. This field shall contain the same algorithm identifier as the signature field in tbsCertificate. |
| signatureValue | Digital signature computed upon the ASN.1 DER encoded tbsCertificate. |

All certificates described in this specification shall be based on RSA of ECC. With RSA, the RSA signature algorithm SHA-1 is used as the one-way hash function. The RSA signature algorithm is described in PKCS #1; SHA-1 is described in FIPS 180-1.

For ECC certificates elliptic curve domain parameters can be generated according to procedures defined in Section A.3 of ANSI X9.62-2005.  Example parameters sets of parameters can be found in FIPS 186-3 and ANSI X9.63-2001.  Domain parameters sets that are selected will produce keys of no less than 160 and no greater than 256 bits in length.

Restrictions posed on the certificate values are described in 7.6.1.x through 7.6.1.x.

#### 7.5.1.1 tbsCertificate.validity.notBefore and tbsCertificate.validity.notAfter

CPE certificates shall not be renewable and shall thus have a validity period greater than the operational lifetime of the CPE. A Manufacturer/ServiceProvider CA certificate’s validity period should exceed that of the CPE certificates it issues. The validity period of an CPE certificate shall begin with the date of generation of the device’s certificate; the validity period should extend out to at least 10 years after that manufacturing date. Validity periods shall be encoded as UTCTime. UTCTime values shall be expressed Greenwich Mean Time (Zulu) and shall include seconds (i.e., times are YYMMDDHHMMSSZ), even where the number of seconds is zero.

#### 7.5.1.2 tbsCertificate.serialNumber

Serial numbers for CPE certificates signed by a particular issuer shall be assigned by the manufacturer in increasing order. Thus, if the tbsCertificate.validity.notBefore field of one certificate is greater than the tbsCertificate.validity.notBefore field of another certificate, then the serial number of the first certificate shall be greater than the serial number of the second certificate.

#### 7.5.1.3 tbsCertificate.signature and signatureAlgorithm

Certificates can be signed with the RSA or ECDSA (ANSI X9.62-2005) algorithms.  ECDSA is the elliptic curve analog of the DSA signature algorithm.  The following two sub-sections define OIDs that represent values for tbsCertificate.signature and signatureAlgorithm fields of the X.509v3 certificate to describe each algorithm.

##### 7.5.1.3.1 RSA signature & signatureAlgorithm

The RSA signature algorithm (PKCS #1, IETF RFC 2313), which makes use of SHA-1 is described in FIPS 180-1) as the one-way hash algorithm.  The ASN.1 OID used to describe the RSA signature algorithm using SHA-1 is as follows:

sha-1WithRSAEncryption OBJECT IDENTIFIER ::=

{ iso(1) member-body(2) us(840) rsadsi(113549) pkcs(1) pkcs-1(1) 5}

The calculation and encoding of the signature value is described in PKCS #1 (IETF RFC 2313) specification.  The signature value is calculated over the rest of ASN.1 encoded certificate,  then inserted signatureValue field of the certificate.

##### 7.5.1.3.2 ECC signature & signatureAlgorithm

ECDSA itself is identified by OIDs arranged in the following manner:

ansi-X9-62  OBJECT IDENTIFIER ::= {

iso(1) member-body(2) us(840) 10045 }

id-ecSigType OBJECT IDENTIFIER  ::=  {

ansi-X9-62 signatures(4) }

ECDSA also uses SHA-1 as the one-way hash function.  The ASN.1 OID used to described the ECDSA signature algorithm using SHA-1 is as follows:

ecdsa-with-SHA1  OBJECT IDENTIFIER ::= {

id-ecSigType 1 }

The calculation and encoding of the signature value is described in ANSI X9.62-2005.  This process outputs two values (r and s).  The signature value is calculated over the rest of ASN.1 encoded certificate,  then inserted signatureValue field of the certificate using the following ASN.1 encoded data structure:

Ecdsa-Sig-Value  ::=  SEQUENCE  {

r     INTEGER,

s     INTEGER  }

#### 7.5.1.4 tbsCertificate.issuer and tbsCertificate.subject

X.509 Names are SEQUENCES of RelativeDistinguishedNames, which are in turn SETs of AttributeTypeAndValue. AttributeTypeAndValue is a SEQUENCE of an AttributeType (an OBJECT IDENTIFIER) and an AttributeValue. The value of the countryName attribute shall be a two-character PrintableString, chosen from ISO 3166; all other AttributeValues shall be encoded as either T.61/TeletexString or PrintableString character strings. The PrintableString encoding shall be used if the character string contains only characters from the PrintableString set. Specifically:

abcdefghijklmnopqrstuvwxyz

ABCDEFGHIJKLMNOPQRSTUVWXYZ

0123456789

’()+,–./:=? and space

The T.61/TeletexString shall be used if the character string contains other characters. The following OIDs are needed for defining issuer and subject Names in SCM certificates:

id-at OBJECT IDENTIFIER ::= {joint-iso-ccitt(2) ds(5) 4}

id-at-commonName OBJECT IDENTIFIER ::= {id-at 3}

id-at-countryName OBJECT IDENTIFIER ::= {id-at 6}

id-at-localityName OBJECT IDENTIFIER ::= {id-at 7}

id-at-stateOrProvinceName OBJECT IDENTIFIER ::= {id-at 8}

id-at-organizationName OBJECT IDENTIFIER ::= {id-at 10}

id-at-organizationalUnitName OBJECT IDENTIFIER ::= {id-at 11}

The following subclauses describe the attributes that comprise the subject Name forms for each type of SCM certificate. Note that the issuer name form is the same as the subject of the issuing certificate. Additional attribute values that are present but unspecified in the following forms should not cause a device to reject the certificate.

##### 7.5.1.4.1 Manufacturer/ServiceProvider certificate

countryName=<Country of Manufacturer/ServiceProvider>

[stateOrProvinceName=<state/province>]

[localityName=<City>]

organizationName=<Company Name>

organizationalUnitName=WirelessRAN

[organizationalUnitName=<Manufacturing Location>]

commonName=<Company Name> <Certification Authority>

The countryName, organizationName, and commonName attributes shall be included and shall have the values shown. The organizationalUnitName having the value “WirelessMAN” shall be included. The organizationalUnitName representing manufacturing location should be included. If included, it shall be preceded by the organizationalUnitName having value “WirelessMAN.” The stateOrProvinceName and localityName may be included. Other attributes are not allowed and shall not be included.

##### 7.5.1.4.2 CPE Certificate

countryName=<Country of Manufacturer/ServiceProvider>

organizationName=<Company Name>

organizationalUnitName=<manufacturing location>

commonName=<Serial Number>

commonName=<MAC Address>

The MAC address shall be the CPE’s MAC address. It is expressed as six pairs of hexadecimal digits separated by colons (:), e.g., “00:60:21:A5:0A:23.” The Alpha HEX characters (A–F) shall be expressed as uppercase letters.

The organizationalUnitName in an CPE certificate, which describes the modem’s manufacturing location, should be the same as the organizationalUnitName in the issuer Name describing a manufacturing location.

The countryName, organizationName, organizationalUnitName, and commonName attributes shall be included. Other attributes are not allowed and shall not be included.

##### 7.5.1.4.3 BS Certificate

countryName=<Country of Operation>

organizationName=< Name of Infrastructure Operator>

organizationalUnitName=<WirelessRAN>

commonName=<Serial Number>

commonName=<BS Id>

The BS Id field shall contain the operator-defined BSId.1 It is expressed as six pairs of hexadecimal digits separated by colons (:), e.g., “00:60:21:A5:0A:23.” The Alpha HEX characters (A-F) shall be expressed as uppercase letters.

#### 7.5.1.5 tbsCertificate.subjectPublicKeyInfo

The tbsCertificate.subjectPublicKeyInfo field contains the public key and the public key algorithm identifier. The following two subsections describe OIDs used to encode this information for RSA public keys (7.6.1.5.1) and ECDSA/ECDH public keys (7.6.1.5.2).

##### 7.5.1.5.1 RSA Public Keys

The OID that identify RSA encryption for a certificate are defined as follows:

pkcs-1 OBJECT IDENTIFIER ::= { iso(1) member-body(2) us(840)

rsadsi(113549) pkcs(1) 1}

rsaEncryption OBJECT IDENTIFIER ::= { pkcs-1 1}

The OID that identifies RSA public keys in a certificate is defined as follows:

RSAPublicKey ::= SEQUENCE {

modulus            INTEGER,    -- n

publicExponent     INTEGER  }  -- e

##### 7.5.1.5.2 ECDSA/ECDH Public Keys

ECDH (ANSI X9.63-2001) is the elliptic curve analog of Diffie-Hellman agreement.  The OIDs and parameters used to encode information are similar for ECDSA signatures and ECDH encryption, and arranged in the following manner:

ansi-X9-62 OBJECT IDENTIFIER ::=

{ iso(1) member-body(2) us(840) 10045 }

id-public-key-type OBJECT IDENTIFIER  ::= { ansi-X9.62 2 }

id-ecPublicKey OBJECT IDENTIFIER ::= { id-publicKeyType 1 }

Each elliptic curve public key is associated with a set of elliptic curve parameters.  The OIDs that define the elliptic curve parameters are arranged as follows:

EcpkParameters ::= CHOICE {

ecParameters  ECParameters,

namedCurve    OBJECT IDENTIFIER,

implicitlyCA  NULL }

If ecParameters are inherited from the certificate authority, then the implicitlyCA value is included in EcpkParameters and is set to NULL.  ecParameters and its components are defined as follows:

ECParameters ::= SEQUENCE {

version   ECPVer,          -- version is always 1

fieldID   FieldID,         -- identifies the finite field over

-- which the curve is defined

curve     Curve,           -- coefficients a and b of the

-- elliptic curve

base      ECPoint,         -- specifies the base point P

-- on the elliptic curve

order     INTEGER,         -- the order n of the base point

cofactor  INTEGER OPTIONAL -- The integer h = #E(Fq)/n

}

ECPVer ::= INTEGER {ecpVer1(1)}

Curve ::= SEQUENCE {

a         FieldElement,

b         FieldElement,

seed      BIT STRING OPTIONAL }

FieldElement ::= OCTET STRING

ECPoint ::= OCTET STRING

ECPoint represents the base point of an elliptic curve and can take on two forms, compressed and uncompressed (defined in ANSI X9.62-2005).  For certificates the encoding of ECPoint shall be supported by the uncompressed form.  The compressed form may (optionally) be used instead.

The elliptic curve domain parameters can be generated according to procedures defined in Section A.3 of ANSI X9.62-2005.  Example parameters sets of parameters can be found in FIPS 186-3 and ANSI X9.63-2001.  Domain parameters sets that are selected will produce keys of no less than 160 and no greater than 224 bits in length.

#### 7.5.1.6 tbsCertificate.issuerUniqueID and tbsCertificate.subjectUniqueID

The issuerUniqueID and subjectUniqueID fields shall be omitted for all of the SCM’s certificate types.

#### 7.5.1.7 tbsCertificate.extensions

##### 7.5.1.7.1 CPE Certificates

CPE certificates may contain noncritical extensions; they shall not contain critical extensions. If the KeyUsage extension is present, the keyAgreement and keyEncipherment bits shall be turned on, keyCertSign and cRLSign bits shall be turned off, and all other bits should be turned off.

##### 7.5.1.7.2 BS Certificates

Manufacturer/ServiceProvider certificates may contain the Basic Constraints extension. If included, the Basic Constraints extension may appear as a critical extension or as a noncritical extension. Manufacturer/ServiceProvider certificates may contain noncritical extensions; they shall not contain critical extensions other than, possibly, the Basic Constraints extension. If the KeyUsage extension is present in a Manufacturer/ServiceProvider certificate, the keyCertSign bit shall be turned on and all other bits should be turned off.

#### 7.5.1.8 signatureValue

In all three SCM certificate types, the signatureValue can contain either the RSA (with SHA-1) or ECDSA signature computed over the ASN.1 DER encoded tbsCertificate. The ASN.1 DER encoded tbsCertificate is used as input to the RSA signature function. The resulting signature value is ASN.1 encoded as a bit string and included in the Certificate’s signatureValue field.

### 7.5.2 Certificate Storage and Management

#### 7.5.2.1 Certificate Storage and Management in CPE

Manufacturer/ServiceProvider-issued CPE certificates shall be stored in CPE permanent, write-once memory. SSs that have factory-installed RSA private/public key pairs shall also have factory-installed CPE certificates. SSs that rely on internal algorithms to generate an RSA key pair shall support a mechanism for installing a manufacturer-issued CPE certificate following key generation. The CA certificate of the Manufacturer/ServiceProvider CA that signed the CPE certificate shall be embedded into the CPE software. If a manufacturer issues CPE certificates with multiple Manufacturer/ServiceProvider CA certificates, the CPE software shall include ALL of that manufacturer’s CA certificates. The specific Manufacturer/ServiceProvider CA certificate installed by the CPE [i.e., advertised in Authentication Information messages and returned by the management information base (MIB) object] shall be that identifying the issuer of that modem’s CPE certificate.

#### 7.5.2.2 Certificate Storage and Management in the BS

SCM employs digital certificates to allow BSs to verify the binding between an CPE’s identity (encoded in an X.509 digital certificate’s subject names) and its public key. The BS does this by validating the CPE certificate’s certification path or chain. Validating the chain means verifying the Manufacturer/ServiceProvider CA Certificate through some means.

7.10 Security sublayer Architecture for the Cognitive Plane (Informative)

Unlike 802.11 and 802.16 which usually operate in the licensed spectrum, cognitive radio based WRANs need to operate in the unlicensed spectrum or White spaces as secondary occupiers of the spectrum.  Mechanisms have been defined in various sections of the 802.22 standard to protect the incumbents who are the primary occupiers of the spectrum (e.g. spectrum sensing, geolocation, spectrum management, spectrum etiquette etc.). This section provides further details on how to protect the incumbents as well as the 802.22 sytems against various types of Denial of Service (DoS) attacks targeted at the cognitive functions of the 802.22 systems. As a result, the Security Sub-layer 2 has been introduced in the cognitive plane of the 802.22 entity. The cognitive plane consists of a Spectrum Sensing Function (SSF), a Geolocation Function and a Spectrum Manager (SM) at the BS or the Spectrum Automaton at the CPE.

Figure XCP1 (a) shows the 802.22 Protocol Reference Model (PRM) with its Cognitive Plane functions containing Security Sublayer 2 at the BS. Figure XCP1 (b) shows the 802.22 PRM with its Cognitive Plane functions containing Security Sublayer 2 at the CPE. Details on the PRM itself can be found in Section 6.2.

Organization - Sections 7.10 defines the detailed functionalities that 802.22 systems need to provide to ensure the security at the cognitive plane. Section 7.11 defines the operations / mechanisms of the Security Sublayer 2 to provide the functionalities defined in 7.10.

(a)

(b)

Figure XCP1 (a) PRM of the 802.22 BS showing Security Sublayer 2 which provides protection for the cognitive functions at the BS (b) PRM of the 802.22 CPE with Security Sublayer 2 which provides protection for the cognitive functions at the CPE

Some of basic security functions of this layer and the remediation measures required to protect these functions include:

7.10.1 Availability

This is the primary function of any network.  If there is a perception, real or not, that a particular type of network is unreliable due to service or security issues then that network type will not be utilized by WRAN operators or end-users.  In the case of cognitive networks availability refers to ability of the BSs to properly sense the available spectrum and make it available to the CPEs.   This means that the BSs must have built-in security mechanisms that can:

  - Ensure the availability of the spectrum for the primary (incumbents) and the secondary (WRAN) users

  - Mitigate any DoS-type attacks against BS, CPE, and other supporting devices such as the ones used to generate 802.22.1 beacons.

7.10.2 Authentication

This functionality provides assurance that the communicating parties, sender and receiver, are who they purport to be.  In cognitive networks there is the added problem of distinguishing between the valid incumbents of the spectrum and the secondary users.  WRAN operators must be able to:

  - Validate incumbent TV signals and the wireless microphone beacons

  - Detect and counter man-in-the-middle and similar type attacks that attempt to steal available spectrum space.

  - Detect and counter any spoofing and similar type attacks

  - Authenticate geolocation information

  - Authenticate co-existence information of neighboring WRAN systems

  - Detection and reporting of spurious transmissions from other CPEs

7.10.3 Authorization

Different network entities will have different privilege levels.  For example, a BS may be authorized to forcibly remove an interfering CPE from the network but the adjacent CPE reporting the interference will not.  In cognitive networks the ability of the BS spectrum manager to sense the available spectrum, make decisions regarding its use and enforce those decisions at the CPE level is an important authorization example.  For a cognitive network to properly function:

  - Only the authorized parties are allowed to configure the spectrum manager at the BS and the spectrum automaton at the CPE

  - Configuration information is identified and protected

7.10.4 Identification

Identification works hand-in-hand with authentication in assuring both incumbent and secondary spectrum users that the communicating entities are known.  To that end it is necessary that cognitive networks provide mechanisms that will:

  - Positively identify transmitting/receiving BS and CPE equipment

  - Ensure that the identification methods employed cannot be compromised through spoofing or similar type attacks.

  - Protect against replay-type attacks which employ previously transmitted valid identifiers.

7.10.5 Integrity

Integrity is the assurance that the information transmitted over the medium arrives at its destination unaltered. Integrity provides write protection for the content. This is especially difficult in wireless networks because of the uncontrolled nature of the medium.  Also the fact that certain portions of the data must be altered to ensure proper transmission and delivery (timestamps, source, destination etc.) compounds the problem.  Cognitive networks must:

  - Protect against Co-Existence Beaconing (“CBP”) falsification

  - Protect against replay-type attacks using previously transmitted valid data

7.10.6 Confidentiality/Privacy

Confidentiality works closely with integrity to ensure read as well as write protection for data. This is usually carried out using encryption and ciphers that can operate at the link and higher layers.  One must account for the fact that the wireless medium is more sensitive to transmission errors due to propagation effects such as shadowing, fading as well as un-intentional interference.  This sensitivity can wreak havoc with complex ciphers and cause numerous re-transmissions resulting in wasted bandwidth.  With cognitive networks this sensitivity is especially troublesome because of the opportunistic nature of spectrum use by the secondary users and the fact that use of the spectrum is not guaranteed.  Cognitive networks, therefore, must provide:

  - Support for robust ciphers and encryption methods

  - Mechanisms to safeguard WRAN operator’s spectrum availability information from eavesdropping by competitors or would-be hackers.

7.11 Security Mechanisms for the Cognitive Plane

7.11.1 Introduction

The essential functions of the Security Sublayer 2 at the cognitive plane are to provide protection for the incumbents as well as protection to the 802.22 systems against DoS attacks of various types. Threat model for the 802.22 systems has been discussed in  [5 – PRM and Security Enhancements in 802.22]. Figure [Security_Sublayer_Cognitive_Plane] shows the security sublayer architecture for the Cogitive Plane. One of the most important mechanisms employed by this security sublayer is that of collaborative sensing and correlation with geolocation information from various sensors (CPEs) located in the network. This information from various sensors needs to be combined in a certain way which is referred to as information fusion followed by decision making. Figure [Security_Sublayer_Cognitive_Plane] represents these steps which enable the 802.22 systems to provide protection against some types of DoS attacks.

Figure Security_Sublayer_Cognitive_Plane Security Sublayer 2 at the Cognitive Plane

7.11.1 Collaborative Sensing Mechanisms

Co-operative or Collaborative sensing is a scheme wherein more than one Radio Frequency (RF) sensors are used to sense the spectrum. The local sensing information may be combined with information from the incumbent database to see if any of the subscribers (CPEs) lie in the protected contour of an incumbent. This local sensing information is then passed on to a central node such as the BS which makes a decision on whether to make opportunistic usage of the spectrum.

Appendix E shows the benefits of collaborative sensing and information fusion to improve security in a network consisting of cognitive radios such as the one being considered for 802.22 systems. In Appendix E, various rules were considered for information fusion where it was shown that rather than using simple OR or AND type rules, Voting based rules provide flexibility and accuracy needed to provide security protection in cognitive radio systems.

Collaborative sensing takes all it strength when the information fusion described above takes into account the knowledge on the geographic location of the specific sensing devices providing sensing reports.  The correlation of the reports of the sensing devices that may have sent an “Urgent Coexistence Situation (UCS)” message to the BS and their respective geographic location will provide powerful means to identify the characteristics of the sensed signal. This will, in turn improve the decision making as to the presence of an incumbent and its type versus presence of other devices based on their expected extent of coverage.

In the case of DTV detection, the area where correlation will exist between sensing devices will be large and will be in the direction of the DTV transmitter.  In the case of a wireless microphone, the correlarion will exist over a more limited area.  This limited area correlation will also be used in the case of the detection of an 802.22.1 beacon and will help increasing the level of conficence of the decision that it is indeed a TG1 beacon based on its expected area of coverage, and possibly reducing the need to rely on the full PPDU decoding.

Since all the information will be available at the base station, it will be up to the manufacturers to implement various levels of complexity in fusing the sensing reports and the geographic location of the sensing devices to come up with a reliable assessment of the presence of incumbents, that is true positives, while minimizing the probability of false positives. Artificial intelligence algorithms could be taken advantage of to improve this data fusion and decision process.  Since incumbents will need to be protected in all cases and in case of doubt, action will need to be taken to avoid interference to incumbents, more advanced data fusion and decision processes will have the advantage of avoiding false positives and false alarms and therefore reducing cases where WRAN systems would have to change frequency while it is not necessary.

Colloborative sensing and information fusion provides protection to the primary users of the spectrum since the probability of missed detection reduces with the number of sensors. Collaborative sensing provides protection to the secondary users of the spectrum against DoS attacks since it is easy to fool one sensor  into believing the state of the spectrum, however, it is difficult to fool many sensors located in proximal, but disparate geographical locations.

Hence, collaborative sensing, correlation with incumbent database, information fusion and decision making can provide protection to not just the primary users, but also to the secondary users of the spectrum.

7.11.1.1 Collaborative Sensing Operation and Flow Diagram

Figure CSD - Collaborative Sensing and Decision Making Flow Diagram

Figure [CSD] above, shows the typical steps involved in enhancing security in 802.22 systems by the use of collaborative sensing, information fusion, correlation with geolocation information and decision making. The flow diagram is representative of the state when the 802.22 system has already gone through the process of finding the right channel to operate on, and the network has been initialized.

Once the network has been initialized, the BS performs following steps related to collaborative sensing and decision making to enhance security in 802.22 systems.

- The BS shall collect local sensing and location information from various CPEs. The BS shall also obtain its own sensing and location information.

- The BS shall obtain the incumbent type and location information from incumbent location database that may be resident on the network, or on the internet.

- The BS shall correlate the location information of the CPEs with the location information of the incumbents to see if any of the CPEs fall within the protected contours of the incumbents.

- If any of the CPEs or the BS itself, are located within the protected contour of the incumbents, then the BS shall send an Urgent Co-existence Situation (UCS) message, and either move to another / back-up channel, and / or de-authorize the CPE from transmission and joining the network on that channel.

- If any of the CPEs or the BS itself, are not located within the protected contour of the incumbents, then the BS shall check the sensing information collected from its own sensor, as well as from various CPEs.

- If any of the CPEs or the BS itself, have detected the presence of incumbents through their sensing, the BS shall initiate a voting based rule for collaborative sensing.

- If the Voting threshold (See Appendix E) is exceeded, then the BS shall send an Urgent Co-existence Situation (UCS) message, and move to another / back-up channel. The BS may also de-authorize the CPEs reporting the incumbent detection, and other CPEs located within [TBD] mile radius from them.

- If the the Voting threshold is not exceeded, then the BS shall continue to collect sensing and location information from the CPE within its network.

7.11.2 CBP Authentication Mechanisms

This section discusses a method to provide protection over CBP.  This method provides authentication and integrity protection for CBPs by using ECC-based implicit certificates to calculate a signature over the CBPs.  This signature is then added to the CBPs as an IE.  This method was derived from the method used to protect beacon messages transmitted by low powered, licensed devices operating in television broadcast bands as defined in the current IEEE P802.22 TG1 draft document (IEEE P802.22.1/D1).  The receiving station (CPE or BS) uses the certificate information to verify the signature.  CBP Protection can be provided using one of four options.  Figure 7.11.2-1 describes the CBP Protection scheme options.  Sections 7.11.2.1-7.11.2.4 describe the options in detail.

In Figure 7.11.2-1 the following is depicted:

- We are depicting tx of CBPs from Bss to another BS (over-the-air or through backhaul) or received by a CPE at cell-edge or w/in over-lapping area.

- If Rx station is CPE:

  - It can validate cert/signature or received CBP, then strip out that information before relaying CBP to its current serving BS, or

  - Just blindly relay the CBP to its serving BS

- Process by which this certificates are generated is outside the scope of the standard.  Definition of this process is left for the duty of an industry consortium formed to develop profiles for products based on this standard.

Figure 7.11.2-1 CBP Authentication Mechanisms

##### 7.11.2.1 CBP Protection Mode 1

Mode 1 of CBP Protection entails BS ECC implicit certificates and signatures (calculated using CMAC) to be transmitted in each CBP.  The format of the of the BS certificate data that is pre-distributed shall follow Table 7.11.2-1. The format of the signature is defined in Table 7.11.2-2. Option 1 is executed in the following manner:

1) Certificate authority provides certificate (w/ public key) and private key to each BS individually.

- BS adds certificate to CBP as an IE.  This may happen only via a periodic broadcast or during a certificate request/response (CERT-REQ/RSP). In these cases, it calculates the signature over the CBP IE's (CERT-REQ/RSP) and adds signature to CBP into the Signature IE of the CBP MAC PDU.

- BS certificate public key is not used directly to sign the CBP.  Instead it is fed into a Key Derivation Function (KDF), defined in Section 7.x.x.x (TBD).  The output, or CBP Signature Key is then used  by the signature function.  The KDF is called by the following manner; 		CBP_SIGNATURE_KEY_i = KDF(BS Implicit Certificate Public Key, BS ID | i | “CBP Signature Key”, 128). “i” in the formula refers to the current time stamp that is part of the signature IE (Table 7.11.2-4).

4) Signature will be calculated via CMAC (e.g. IETF RFC 4493).   CMAC output is 8 bytes.

4) Receiving station validates certificate & signature.  The receiving station then validates the BS implicit certificate.  If this validation fails, the receiving station drops the CBP.  If successful, the receiving stations uses the BS implicit certificate public key and time stamp from the signature IE to derive the CBP Signature Key, then recomputes the signature, and compares it to signature value in signature IE.  If there is a mismatch, then validation of CBP signature has failed, and receiving station will drop the CBP.

##### 7.11.2.2 CBP Protection Mode 2

Mode 2 of CBP Protection entails pre-distribution of BS ECC implicit certificates to receiving station.  This can be done via a SIM card, when CPE is activated, or via NCMS (for periodic certificate updates).  BS calculates signature (using CMAC) of CBP using its implicit certificate and adds the signature as an IE to each CBP. This option is applicable when sharing BS implicit certificates between BSs that belong to the same operator. The format of the of the BS certificate data that is pre-distributed shall follow Table 7.11.2-1. The format of the signature is defined in Table 7.11.2-2. Mode 2 is executed in the following manner:

1) Certificate Authority generates certificate (w/ public key) and private key for BSs.

2) Certificate Authority then distributes keys to:

a)  a service provider, who then would distribute certificates to all BSs within its network, possibly through NCMS

b) central database that it or service provider consortium provides, that houses certificates for all BSs in a given geo-graphical area, & individual providers use NCMS to gain certs for their own and competitor's  BSs

3) BS certificate public key is not used directly to sign the CBP.  Instead it is fed into a Key Derivation Function (KDF), defined in Section 7.x.x.x (TBD).  The output, or CBP Signature Key is then used  by the signature function.  The KDF is called by the following manner; 		CBP_SIGNATURE_KEY_i = KDF(BS Implicit Certificate Public Key, BS ID | i | “CBP Signature Key”, 128). “i” in the formula refers to the current time stamp that is part of the signature IE (Table 7.11.2-4).

4) BS calculates signature over each CBPs, adds signature to each CBPs via an IE

5) Signature will be calculated via CMAC.  CMAC output is 8 bytes.

6) Receiving station validates certificate & signature.  The receiving station then validates the BS implicit certificate.  If this validation fails, the receiving station drops the CBP.  If successful, the receiving stations uses the BS implicit certificate public key and time stamp from the signature IE to derive the CBP Signature Key, then recomputes the signature, and compares it to signature value in signature IE.  If there is a mismatch, then validation of CBP signature has failed, and receiving station will drop the CBP.

##### 7.11.2.3 CBP Protection Certificate and Signature Structures

The following subsections describe the structures of the certificate and signature information.  For Mode 2, the BS ECC implicit certificate is not transmitted over the air.

Table 7.11.2-1: Mode 1 and Mode 2 BS Implicit Certificate (BSIC)

| Item | Size | Description |
| --- | --- | --- |
| CA ID | 8 bits | Id of CA that issued implicit certificate to BS |
| Key Validity Date (Not Before) | 41 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Date (Not After) | 3 bits | # of years after Key Validity Date (Not Before) date that certificate expires. Encoded as follows: - 000 – 1 year - 001 – 2 years - 010 – 3 years - 011 – 4 years - 100 – 5 years - 101 – 10 years - 110 – 15 years - 111 – 20 years |
| Public Key Reconstruction Data | 33 byte | Key data used to reconstruct public key: - 33 bytes for 256 bit ECC keys |

Table 7.11.2-2: Mode 1 and Mode 2 Signature (added to 2nd Symbol of Self Coexistence Window)

| Item | Size | Description |
| --- | --- | --- |
| Key ID/Serial # | 10 bits | Serial # of key associated with BS implicit certificate. This is generated by the CA. |
| Time Stamp | 54 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - ss: seconds, 00-59, 6 bits - .ss: 10 ms boundary, .000-.99, 7 bits - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Signature | 8 bytes | CMAC output is 128 bits (16bytes) signature is a truncation of this value to no less than 8 bytes |

##### 7.11.2.4 Certificate Generation, Processing, and Validation Requirements

###### 7.11.2.4.1 Certificate Generation Requirements

- Infrastructure as described in Figure 7.11.2-1 for certificate generation and distribution

- Recommended EC domain parameters to be used can be read from ANSI X9.63-2001 and FIPS 186-3

  - Domain parameters that provide 256 bit primes for ECs are to be used

- BS shall be identified by its 48bit MAC Address

- ‘to-be-signed certificate data’, IU construction differs between options 1/2 and options 3/4 (IU data used by CA in requirement 4a/4b below)

  - For option 3/4 IU = KeyID/Serial # || BS MAC Address || CA ID || Key Validity Date (Not Before) || Key Validity Date (Not After) ; these are fields from the Option 3/4 Implicit Certificate IE

  - For option 1/2 IU = KeyID/Serial # || BS MAC Address || CA ID || Key Validity Date (Not After) ; these are fields from the Option 1/2 Implicit Certificate IE

- If each operator is allowed to maintain it’s its’ own BS implicit certificates (i.e. act as its’ own Certificate Authority)

  - Operator will register its’ Certificate Authority ID when registering its’ Operator ID. Mechanism by registration is executed is outside the scope of the standard.

  - Operator’s shall share their own CA Root certificate with other operators that have BSs that border or overlap with BSs in their own network. The mechanism for CA Root Certificate sharing is outside the scope of the standard.

###### 7.11.2.4.2 Certificate Generation Process

- In order to start certificate generation process (see Figure 7.11.2-1) a request from the BS (or on behalf of BS) must be made to CA.

  - This process requires that BS generate its own ‘ephemeral’ key pair using established ECC domain parameters.  ECC domain parameters used for 'ephemeral' key pair generation shall be the same parameters used by CA for its own certificate and BS implicit certificate generation

  - Certificate generation initiation follows process as defined in Section 4.1 “Initiator Transformation” in SEC 4

- CA executes process as defined in Section 4.2 of SEC 4 (“Standards for Efficient Cryptography 4: Elliptic Curve Cryptography), with some exceptions.

###### 7.11.2.4.3 Certificate Validation Process

- Whether certificate information for a BS is received in Certificate IE or is distributed through the NCMS, a BS received that certificate shall validate it.

  - If certificate is received in Certificate IE & validation of certificate fails, then packet is dropped

  - BS or NCMS operator may check a CA’s CRL during installation and validation of other BSs implicit certificates

  - Verification of a BS Implicit Certificate is defined in Section 5 “ECQV Implicit Certificate Processing Transformation” of SEC 4

- The Certificate/License authority shall have its own certificate with it’s own EC key pair, that is generated from the EC domain parameters that are selected

  - ECC domain parameters that CA uses for generation of its' own certificate must be same as parameters used for BS implict certs and 'ephemeral' key pair generation

- The CA certificate and its associated KeyId shall be delivered to BS through NCMS, and stored in TBD MIB entry, this happens offline and not part of WRAN operation

- Certificates for all CA's and a CA's associated ECC domain parameters shall be shared between operator’s and distributed to all BSs within an operator’s network.. The mechanism by which is done is outside the scope of the standard.

##### 7.11.2.5 Signature Generation, Processing, and Validation Requirements

###### 7.11.2.5.1 Signature Generation Requirements

- Key Derivation Function (KDF) is used to derive the key used to sign each CBP transmission. KDF is defined in Section 7.x.x.x.

- For CMAC-based signature calculation use RFC 4493 (AES-CMAC) Output of AES-CMAC algorithm shall be truncated to 64 bits.

###### 7.11.2.5.2 Signature Generation Process

- BS uses system clock (synchronized to GPS), to get time (to millisecond) for "Time" field of Signature data in 2nd symbol of Self-coexistence window

- Message input parameter into CMAC calculation shall be all of the non-signature data in 2nd symbol of self-coexistence window and all IE data in 3rd symbol of self-coexistence window (if 3rd symbol is transmitted)

- Key input parameter is the public key associated with the certificate of the BS that transmitted the CBP

- Signature algorithm outputs:

  - For CMAC a digest that is 16bytes or 128bits long.  The MAC value shall be truncated to no less than 64bits, as recommended by NIST Special Publication 800-38B.  NIST 800-38B suggests taking 64 MSB.

##### 7.11.2.6 BS Implicit Certificate Exchange

CBP Protection Mode 1 allows for pre-distribution of CA Root and BS Implicit certificates between all BSs within an operator’s network. This would be accomplished using the NCMS. This may not be possible when BSs from different operators border/overlap with each other. This section defines a certificate exchange process to handle this case.

If there is a single CA, then the CA Root certificate and single set of EC domain parameters are available to all operators. If each operator is allow to sign/create their own certificates, then operators shall make available their CA Root Certificate and EC domain parameters available to other operators.

If a BS receives a CBP from a BS, for whom it does not have the BS implicit certificate, it cannot verify the signature in the 2nd symbol (1st data symbol) of CBP and must drop the CBP packet. Upon doing this the BS (Certificate Requestor) will initiate a Certificate Request directed towards the source BS (Certificate Responder) of the unverifiable CBP. The Certificate Requestor BS does this by adding a CERT-REQ IE (Section 6.7.1.2.x), with its’ own BS implicit certificate, as an IE in the CBP MAC PDU.

When the Certificate Responder receives a CBP with a CERT-REQ IE from a Certificate Requestor it verifies the certificate and Signature data in the Signature IE of the CBP MAC PDU. If both BSs are under the same CA then the Certificate Responder already has the CA Root certificate and EC domain parameters it needs to verify the BS implicit certificate from Certificate Requestor. If they are not, e.g. each operator is it’s own CA or CAs exist on a regional basis, than the Certificate Responder shall use a mechanism to request the CA Root certificate and EC domain parameters of another CA. This mechanism shall be provided, definition of this mechanism is outside the scope of the system.

After verifying the Certificate Requestor’s certificate and signature over CBP in which CERT-REQ IE was received, the Certificate Responder initiates a Certificate Response directed towards the Certificate Requestor. The Certificate Responder does this by adding a CERT-RSP IE (Section 6.7.1.2.x), with its’ own BS implicit certificate to the CBP MAC PDU.

When Certificate Requestor receives the CBP with the Certificate Response, it verifies it in the same manner that Certificate Responder verifies the Certificate Reqeust. After this, any pending coexistence signalling (e.g. Channel Contention) can proceed. Figure XXXX illustrates the BS Implicit Certificate Exchange.

Figure XXXX – BS Implicit Certificate Exchange

[Editor’s Note: Timers and/or some random backoff should be used to control re-transmission of certificate requests and/or responses. If working group deems this necessary then timers and the logic for controlling the retransmissions will be introduced at a later date.]

- Cognitive Radio Capability

  - Introduction

  - Spectrum Manager

The Spectrum Manager, as shown in the 802.22 reference model (Figure 6), is part of the management plane. Although the reference model in Figure 6 applies to both BS and CPEs, the spectrum manager at the WRAN BS is responsible for the most important tasks, such as maintaining spectrum availability information, channel selection, channel management and self-coexistence. On the other hand, the spectrum manager entity at the CPE is a much simpler entity including only essential features to allow proper CPE operation when it is not under the control of a BS, such as during initialization (before association with the BS), and basic functionalities to respond/react to the BS’s requests and commands. Throughout this document, we use the term Spectrum Manager (SM) to refer to the spectrum manager entity at the WRAN BS, whereas the corresponding entity at the CPE is called CPE Spectrum Automaton (SA). The following clauses describe the SM functionalities. The CPE SA entity is described in Section XXX.

The SM is a central entity part of the WRAN BS, which is responsible for ensuring protection of incumbents and efficient spectrum utilization while complying with regulatory policies. For that, the SM centralizes all the decisions within the WRAN cell with respect to spectrum availability and utilization. In summary, the key functions of the SM are the following:

- Maintain Spectrum Availability Information;

- Channel Classification and Selection;

- Association control;

- Channel Management;

- Self-coexistence with other WRANs.

These functions are described in the following clauses. It is important to note that this standard does not specify any particular SM implementation, but instead, it describes the mandatory behavior for any SM implementation in order to ensure proper protection of incumbents, compliance with regulatory policies, and interoperability amongst different WRAN implementations.

    - Maintain Spectrum Availability Information

The SM shall maintain the status of the spectrum (i.e. TV channels) available for WRAN operation at its location within a regulatory domain according to the policies and rules established for that domain (e.g. regulatory rules established by the FCC in the US for use of TV channels). The SM shall define the channel status with respect to the presence of incumbents and other WRANs in the area, and it shall use this information as input for its decisions with respect to channel selection, channel management and self-coexistence mechanisms.

To maintain the status of the channels available for operation, the SM shall be able to aggregate information from at least the following sources:

- Incumbent database: The SM shall access incumbent databases through the higher layers if such incumbent databases exist and are relevant for the WRAN’s regulatory domain;

- Geolocation: the SM shall be able to access geolocation information available at the BS to identify its own location, and it shall also be able to obtain location information from all CPEs associated with or that are requesting association with the BS.

- Spectrum sensing: the SM shall use the MAC and PHY layer functionalities and management frames to control and coordinate spectrum sensing within the WRAN cell. The SM shall trigger the requests for CPEs to perform sensing and collect sensing reports from CPEs. The SM shall control spectrum sensing performed locally by the BS and it shall combine the local results with the results collected from CPEs.

The SM shall define the status of the channels with respect to the presence of incumbents by combining geolocation information, information fromincumbent databases, spectrum sensing results and constraints programmed on the basis of regulatory rules. If an incumbent database does not exist that is relevant to the regulatory domain, all channels are initially assumed to be available. The SM shall define the status of the available channels based on spectrum sensing information and other constraints programmed on the basis of regulatory rules.

The channel availability information shall be defined during the network initialization and it shall be periodically updated during the network operation.

For example, in the US, the spectrum sensing information used to determine the channel availability status shall be updated every 2 sec for the operating channel and every 6 sec for backup channels. In addition, before the SM declares a given channel available for operation based on spectrum sensing, it shall ensure that the channel has been sensed at least every 6 sec within the last 30 sec.

    - Channel Classification and Selection

The SM is responsible for selecting the operating channel and assigning it to the MAC/PHY modules in the WRAN. The SM is also responsible for defining the backup channel(s) and their corresponding priorities. The rest of the channels that are potentially available for operation, but that are not selected as the operating channel or as backup channel(s), may be classified as candidate, occupied or disallowed channel(s). The channels may be classified using the following categories:

Available: channels available for considerationfor potential WRAN operation at a given location according the incumbent databases, if such databases exist. Channels not deemed available by the databases are precluded for use by WRANs. If no incumbent database exists, all channels are considered available. Available channels must be further classified into one and only one of the following categories:

- Disallowed: channels that are precluded from use by the operator due to operational or local regulatory constraints.

- Operating: the current channel used for communication between BS and CPEs within a WRAN cell. In order to ensure protection of incumbents, the operating channel must be sensed at least every 2 seconds.

- Backup: channels that can become the operating channel right away in case the WRAN needs to switch to another channel. The BS may maintain multiple backup channels at any given time and  shall ordered them according to their relative priorities. Backup channels shall be sensed for incumbent detection at least once every 6 seconds.

- Candidate: channels that are candidates to become a backup channel. These are channels that the BS may request the CPEs to sense to evaluate the possibility of elevating them to a backup channel status. Although sensing of candidate channels could be infrequent, before a candidate channel is elevated to backup channel, it must be sensed as incumbent-free at least every 6 seconds for no less than 30 seconds.

- Occupied: channels in which incumbent or WRAN operation has been detected through sensing. Occupied channels may be moved to the candidate channel set in the event the incumbent is found to have left the channel. Sensing is needed to determine whether the incumbent is still present on the channel, but sensing could be infrequent in these channels. An occupied channel may also become a backup channel, but before an occupied channel is elevated to backup channel, it must be sensed as incumbent-free at least every 6 seconds for no less than 30 seconds. The SM should, when possible, identify the type of signal occupying every occupied channel (see ).

- Unclassified: channels that have not been sensed. These channels may be sensed according to the SM implementation. Once an unclassified channel has been sensed, it may be re-classified as occupied or candidate channel depending on the sensing results.

The specific algorithms for selecting the operating channel and defining how the backup and candidate channels are prioritized is outside the scope of this standard as long as these implementations meet the sensing requirements. However, any implementation of these algorithms shall use as input current channel availability information (as described in Section 9.4.1) and combine the channel status information with predefined rules applicable to the specific regulatory domain. Furthermore, other criteria may also be taken into account by the implementation, such as traffic requirements, location information, and coexistence with neighboring WRANs.

    - Association Control

When CPEs request association with a WRAN BS (see CPE initialization procedure described in Error: Reference source not found), the SM is responsible for granting or denying association rights to the requesting CPEs. For that, the SM shall consider location information, and capabilities of each requesting CPE. The SM shall access the incumbent databases, if existent, to obtain the list of available channels and corresponding maximum EIRP limits at the CPE’s location, and based on the received information, the SM shall decide whether to grant association rights to the CPE in its current operating channel and indicate the maximum transmit power allowed for the CPE.  [Isn’t the BS supposed to send the vector of maximum EIRP on all channels to each CPE so that they know what EIRP they can work with after a channel switch?  See section 9.3.7: SME-MLME-DB-RESPONSE] It is the responsibility of the SM to ensure that granting association rights to the requesting CPE will not cause harmful interference to incumbents.

    - Channel Management

The SM is responsible for triggering channel management actions within the cell in order to guarantee the required protection of incumbents while supporting QoS for the WRAN users. The channel management actions shall  include:

- Switch the entire cell to a new operating channel;

- Direct a single CPE or a group of CPEs to a different operating channel when possible, when the two base stations are in the same general direction as seen from the CPEs;

- Terminate operation in a given channel for a single CPE, a group of CPEs or the entire cell;

- Inform CPEs of channels status updates (e.g. changes in backup channels, channel status information).

The SM shall use one of the channel management mechanisms defined in the 802.22 MAC layer (see Error: Reference source not found) to execute the appropriate action.

Each channel management action may be triggered by one or more events. For instance, the action of switching channels for the entire cell may be triggered by the detection of an incumbent on the operating channel, by degradation of the QoS due to interference, or traffic load in the current channel. Although different trigger events may be supported depending on the implementation, the trigger events and corresponding channel management actions shall ensure protection of incumbents as required by regulatory policies applicable within the regulatory domain. A list of possible trigger events and actions needed to protect incumbents is given in .

—  Trigger Events and Corresponding Actions

| Trigger Event | Action |
| --- | --- |
| If the SM confirms the presence of a TV Incumbent signal above the detection threshold on the operating channel through the BS spectrum sensing function or through a combination of sensing results from multiple CPEs. | Switch the entire cell to a new operating channel within the next 2 seconds, which should be the highest priority backup channel. |
| If the SM confirms the presence of a Wireless Microphone signal above the detection threshold or decode a legitimate TG1 beacon on the operating channel through the BS spectrum sensing function. | Switch the entire cell to a new operating channel within the next 2 seconds, which should be the highest priority backup channel. |
| If the SM obtains information from an incumbent database that indicates the current operating channel will become unavailable at a specific time in the future. | Schedule a channel switch for the entire cell to a new operating channel before the expected time (as obtained from the incumbent database) at which its current channel will become unavailable. |
| If the SM confirms the presence of a Wireless Microphone signal above the detection threshold or a legitimate TG1 beacon in the current channel that was reported by a CPE. | Direct all CPEs within the interfering range of the decoded beacon to a different operating channel or terminate the operation of those CPEs in their current channel within 2 seconds. |
| If a TV incumbent is confirmed on the operating channel by the BS spectrum sensing function and there is no backup channel available. | Terminate the operation of the entire cell in the current operating channel within 2 seconds. Note that this action will interrupt all the services to users, and therefore it should only be executed if no other channel becomes available for operation before the action is executed. |

    - Self-Coexistence with other WRANs

The SM is responsible for managing the channel selection for self-coexistence with other WRANs by using the self-coexistence mechanisms defined in the 802.22 MAC (6.20.2).

  - CPE spectrum sensing automaton

The BS normally controls the sensing behavior of the CPE. However the CPE shall control its sensing behavior locally under the following conditions:

- at the initial turn-on of the CPE before association is established with the base station;

- when the CPE looses contact with its base station; and

- during idle time when the base station has not attributed any specific task to either the CPE sensing signal path or the WRAN signal path, or both.

The functionality of the CPE local autonomous spectrum sensing for these three specific cases is covered in the following sub-clauses.  A more detailed description using an example of a possible implementation is given in .

    - Initial turn-on

The functionality of the CPE local autonomous spectrum sensing when the CPE is initially switched on is summarized below and a description of an example of implementation is given in .  At initial turn-on and self-test, the CPE shall detect WRAN operation first. Then, if a list of.  operating channels is known,channels N and N+/-1 corresponding to each potential channel on the list  shall be sensed. [still need discussion] If the operating channel is not known, all the TV channels that are within the range of operation of the CPE shall be sensed plus one more channel at both ends unless it goes beyond the extent of the relevant TV band.

For each channel, an RSSI measurement shall be performed on the WRAN signal path and attempt shall be made to capture a WRAN superframe header or a CBP packet. If an SCH is captured and the level of the RF signal is sufficiently high, the WRAN signal path shall attempt to acquire the frame header, the broadcast PDU’s sent by the BS to advertise the WRAN service for CPE initialization and the management packet providing the channel occupancy from the base station; the CHO-UPD MAC message.  If an SCH can be acquired but the signal level is insufficient or a CBP packet can be captured, the presence of a WRAN signal shall be recorded along with the channel number and the measured RSSI.  If an SCH or a CBP packet cannot be detected, the sensing path RSSI will be evaluated whether the level of the RF signal is sufficient to carry out a fast signal classification.

For those TV channels where the RF energy is found to be insufficient to allow for fast signal classification or even provide for a reliable RSSI measurement, fine sensing shall be applied to determine the presence of broadcast incumbents and their signal type.  The result of the measurement and the signal classification shall be stored locally so that it can later be sent to the base station when association is established or later on upon request from the BS.

The channel shall then be incremented to repeat the above initial sensing.  The order in which the channels are to be sensed will be implementation dependent.  Note that the information acquired from the local WRAN base stations through the CHO-UPD MAC message during initialization could be used to skip the non-available channels. It could also be used to skip the signal classification and low level sensing on channels belonging to the “occupied” set to save time although, since time is less critical at initialization, it may be useful that the CPE confirms that the incumbents indicated by the BS really exist in these ‘occupied’ channels at its location.

Once all the TV channels to be scanned have been sensed, the information on the local WRAN channel occupancy that would not create interference to local incumbents on channels N and N+/-1 shall be made available to the higher layers at the CPE.  If there is no WRAN channel that can be used, the CPE initialization shall be aborted.  Depending on the CPE implementation, this information may be presented to the local interface of the CPE so that it could ultimately be displayed on the screen of the user terminal to allow for an informed choice among available local WRAN networks (similar to the Access Point selection in Wi-Fi).  Local algorithms could also be implemented in the CPE to automate the process for choosing the WRAN network.

Once the choice of a WRAN service on TV channel N0 has been made by the user or a local routine, a process to verify that the WRAN TX/RX antenna is properly oriented for the selected channel could be carried out based on the fact that the RSSI can be obtained from both the WRAN directional and the sensing omni-directional antenna.  The best alignment will be when the RSSI obtained from the WRAN antenna can be maximized relative to that obtained from the sensing antenna.  Although useful, this process will not necessarily be successful especially in presence of high-level multipath.

A second round of spectrum sensing shall then take place on the selected channel (N0) and its adjacent channels (N0+/-1).  Since, by definition, a WRAN service is present on the selected channel, the WRAN signal path shall acquire the SCH or the CBP packet to determine the timing of the quiet periods in this channel.  Low-power signal sensing and signal classification shall then be carried out in channel N0 by the sensing path during the quiet periods to try to identify whether an incumbent service is present underneath the WRAN service. The findings shall be recorded locally.

The sensing process shall then move to sense the two adjacent channels during the quiet periods by measuring the RSSI and, depending on whether the measured signal level is high or low, sense the presence of an incumbent signal and classify its type using a fast algorithm or a low-level sensing scheme.  If the channel belongs to the “occupied” set as signaled by the CHO-UPD message, the signal classification could be skipped since it is already known by the base station. The findings shall be recorded locally and if incumbents are found in these channels, the selected channel shall be eliminated from the list of available WRAN services and the updated list will be presented to the higher layers at the CPE for the selection of another WRAN service.  If no more WRAN service exists, the CPE initialization shall be aborted. If no incumbent is present in the three channels sensed, the CPE initialization process shall continue as described in section 6.16.2.

If the authorization is refused by the currently selected base station, a new shortened list of available WRAN services shall be presented to the higher layers at the CPE for a new channel selection to be made.  Then, the next round of sensing process shall be repeated with the alignment of the azimuth of the antenna toward the newly selected WRAN base station and the sensing of the newly selected channel (N0) and its adjacent channels (N0+/-1) as described above.  Upon successful sensing results, the CPE association process will continue as described in section  Error: Reference source not found.

    - Loss of contact with the base station

If the CPE looses contact with its base station, the CPE automaton local intelligence shall make sure that a reasonable number of attempts are made to re-connect with the base station, while avoiding any potential interference to licensed incumbents.  The functionality of the CPE local automaton is summarized below for the loss of contact with the base station.

The CPE shall first identify whether or not a WRAN signal is still present on the selected channel by trying to capture the SCH.  If successful within 2 seconds, attempts to re-associate shall be made through the BW Request opportunistic burst or upon specific invitation by the BS.  If this does not work, the re-association shall start from an earlier stage with the CDMA ranging burst.  If re-association cannot be achieved within 2 seconds, then the CPE shall execute the second round of initial sensing for the co-channel and first adjacent channels cases to protect the broadcast incumbents.

If the WRAN signal is no longer present in the channel, then the CPE shall select the next TV channel on its back-up list and try to capture the superframe header to synchronize with the base station on this new channel and acquire the frame header.  If successful within 2 seconds, attempts to re-associate shall be made through the BW Request opportunistic burst or upon specific invitation by the BS, or through the earlier stage of the CDMA ranging burst.

If the new superframe capture is not successful, then the CPE shall select the next TV channel on its backup list and repeat the process until a successful superframe capture is achieved.  If re-association on all the valid channels in the backup list has failed, the CPE shall re-start its entire initialization process.

    - CPE idle time

In addition to being able to carry out all the in-band and out-of-band sensing requested by the base station through the specific messages described in section Error: Reference source not found, the CPE shall have the necessary routines to autonomously sense its operating channel, the TV channels in its backup list and, as many channels as possible in its candidate channel list in the proper order of priority during its idle time.

Since the channels adjacent to the one being sensed (N+/-1) will also need to be verified, the CPE will need to have the capability of re-tuning its sensing RF path and WRAN signal path to the channels adjacent to those contained in its lists of channels to be sensed.  A RSSI measurement shall first take place on the sensing path and RSSI will also be acquired through the WRAN signal path along with an acquisition of the SCH or CBP if WRAN operation is detected on the channel.

The CPE shall begin by sensing the operating channel (N0) and its adjacent channels (N0+/-1) and it shall inform the BS of any urgent finding using the UCS Notification.  If the last sensing has been carried out less than 2 seconds earlier, this sensing can be skipped.  The next channels to be sensed shall be the backup channels (NB) in the order given in the list and their respective adjacent channels (NB+/-1).  Again, if the sensing on a specific backup channel has been carried out less than 6 seconds earlier, this sensing can be skipped.  The following channels to be sensed are the channels appearing in the candidate list (NC) in the order given in the list.  Any channel sensed less than 6 seconds before can be skipped.  The CPE shall try to go as deep as possible in the candidate channel list given the sensing periods available and the interruptions that will come up.

If there is no WRAN operation on the channel being sensed, the sensing process shall verify whether there is WRAN operation on the adjacent channels to be able to schedule its sensing during the quiet intervals of these WRAN operation.  Since signal sensing has to be done on the adjacent channels, if no WRAN operation could be detected on N and N+/-1, the sensing process shall verify whether there is WRAN operation on N+/-2 to be able to sense N+/-1 during the quiet intervals of N+/-2 because of possible adjacent channel leakage that could mask the presence of incumbents on N+/-1.

The sensing process shall be re-initiated on the operating channel (N0) at each time. Due to the nature of the algorithm, this process will be directed to any channel reaching the end of its period of validity in order of priority from the operating channel (2 seconds) to the backup list and candidate list (6 seconds).  Also, any sensing request coming from the base station will be executed as soon as the operating and the backup channels have been cleared within their period of validity.

    - Sensing information representation at the CPE

Once association has been achieved, the base station may request the CPE to send the complete results of the initial sensing or any update thereof at anytime using the BLM-REQ PDU.  The CPE will therefore need to keep the latest information stored by the sensing process in its local registers at all times.   gives a representation of the information that needs to be stored in these registers.  The CPE shall be capable of reporting the information in all rows of the table except for the two last rows that contain information obtained from the base station.

— Sensing registers at the CPE

| Channel number | … | 25 | 26 | 27 | 28 | 29 | 30 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time of last sensing |  |  |  |  |  |  |  |  |
| Time of last positive |  |  |  |  |  |  |  |  |
| Sensing path RSSI |  |  |  |  |  |  |  |  |
| WRAN path RSSI |  |  |  |  |  |  |  |  |
| Signal type |  |  |  |  |  |  |  |  |
| WRAN service advertisement |  |  |  |  |  |  |  |  |
| Sensing path RSSI under WRAN |  |  |  |  |  |  |  |  |
| Signal type under WRAN |  |  |  |  |  |  |  |  |
| Channel occupancy from BS (CHO-UPD) |  |  |  |  |  |  |  |  |
| Max EIRP at CPE (SME-MLME-DB-RESPONSE) (9.4.6) |  |  |  |  |  |  |  |  |

  - Incumbent Database Services

The SME provides access to external incumbent databases services to the SM accessed through the SME-MLME-SAP. The SME-MLME-SAP is an interface that provides a means of exchange information between the SM and the SME. Table 2 summarizes the primitives supported by the MLME to access incumbent database services through the SME-MLME-SAP interface. The primitives are discussed in the subclauses referenced in the table.

— Incumbent Database Primitives supported by the SME-MLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| SME-MLME-DB-EXISTS | 9.3.1 |  | 9.3.2 |
| SME-MLME-DB-AVAILABLE | 9.3.3 |  | 9.3.4 |
| SME-MLME-DB-QUERY | 9.3.5 |  | 9.3.6 |
| SME-MLME-DB-RESPONSE |  | 9.3.7 |  |

    - SME-MLME-DB-EXISTS.request

The SME-MLME-DB-EXISTS.request primitive allows the SM to identify what incumbent database services exist through the SME in order to obtain channel availability information. The SME-MLME- DB-EXISTS.request primitive has no attributes.

      - When generated

The SME-MLME- DB-EXISTS.request primitive is generated by the SM and issued to its SME to identify the types of incumbent database services that exist and can be accessed through the SME.

      - Effect on receipt

When the SME of a BS receives the SME-MLME-DB-EXISTS.request primitive, it generates a SME-MLME-DB-EXISTS.confirm primitive to indicate the existent types of incumbent database services.

    - SME-MLME-DB-EXISTS.confirm

The SME-MLME- EXISTS.confirm primitive allows the SME to inform the SM of the types of incumbent database services that exist and are accessible through the SME.  specifies the parameters for the SME-MLME-DB-EXISTS.confirm primitive.

— SME-MLME-DB-EXISTS.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TV Incumbent Database |  | 0-1 | The value indicates whether a TV Incumbent Database exists. 0 = database does not exist 1= database exists |
| Part 74 Incumbent Database |  | 0-1 | The value indicates whether a Part 74 Incumbent Database exists. 0 = database does not exist 1= database exists |

      - When generated

The SME-MLME-DB-EXISTS.confirm primitive is generated by the SME and issued to its MLME when a SME-MLME-DB-EXISTS.request primitive is received to indicate the types of incumbent database services that exist and can be accessed through the SME.

      - Effect on receipt

When the SM of a BS receives the SME-MLME-DB-EXISTS.confirm primitive, it identifies the types of incumbent database services existent.

    - SME-MLME-DB-AVAILABLE.request

The SME-MLME-DB-AVAILABLE.request primitive allows the SM to identify what incumbent database services are accessible through the SME in order to obtain channel availability information. The SME-MLME-DB-AVAILABLE.request primitive has no attributes.

      - When generated

The SME-MLME-DB-AVAILABLE.request primitive is generated by the SM of a BS and issued to its SME to identify the types of incumbent database services that can be accessed through the SME.

      - Effect on receipt

When the SME of a BS receives the SME-MLME-DB-AVAILABLE.request primitive, it generates a SME-MLME-DB-AVAILABLE.confirm primitive to indicate the types of incumbent database services available.

    - SME-MLME-DB-AVAILABLE.confirm

The SME-MLME-DB-AVAILABLE.confirm primitive allows the SME to inform the SM of the types of incumbent database services that are accessible through the SME.  specifies the parameters for the SME-MLME-DB-AVAILABLE.confirm primitive.

— SME-MLME-DB-AVAILABLE.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| TV Incumbent Database |  | 0-1 | The value indicates whether a TV Incumbent Database is available. 0 = database is not available 1= database is available |
| Part 74 Incumbent Database |  | 0-1 | The value indicates whether a Part 74 Incumbent Database is available. 0 = database is not available 1= database is available |

      - When generated

The SME-MLME-DB-AVAILABLE.confirm primitive is generated by the SME and issued to its MLME when a SME-MLME-DB-AVAILABE.request primitive is received to indicate the types of incumbent database services that can be accessed through the SME.

      - Effect on receipt

When the SM of a BS receives the SME-MLME-DB-AVAILABLE.confirm primitive, it identifies the types of incumbent database services available.

    - SME-MLME-DB-QUERY.request

The SME-MLME-DB-QUERY.request primitive allows the SM to request the SME to access an incumbent database in order to obtain channel availability information.  specifies the parameters for the SME-MLME-QUERY-BD.request primitive.

— SME-MLME-DB-QUERY.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Database_type | Integer | 0-8 | The value identifies the type of database for which the query is directed. 0 = TV Incumbent Database 1 = Part 74 Incumbent Database |
| Latitude | TBD |  |  |
| Longitude | TBD |  |  |

      - When generated

The SME-MLME-DB-QUERY.request primitive is generated by the SM of a BS and issued to its SME to request the SME to query an external incumbent database.

      - Effect on receipt

When the SME of a BS receives the SME-MLME-DB-QUERY.request primitive, it generates a query to the external database available corresponding to the type indicated in the Database_type attribute.

On receipt of a the SME-MLME-DB-QUERY.request with the Database_type corresponding to a type of database which is not available or which is not accessible through the SME, the SME of the BS shall issue a SME-MLME-DB-QUERY.confirm primitive to the MLME with status value of INVALID_REQUEST.

    - SME-MLME-DB-QUERY.confirm

The SME-MLME-DB-QUERY.confirm primitive is used to inform the SM whether its request to query an external incumbent database service was successfully generated by the SME.  specifies the parameters for the SME-MLME-DB-QUERY.confirm primitive.

— SME-MLME-DB-QUERY.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Database_type | Integer | 0-8 | The value identifies the type of database for which the query request was directed. 0 = TV Incumbent Database 1 = Part 74 Incumbent Database |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the requested query was successfully generated. |

      - When generated

The SME-MLME-DB-QUERY.confirm primitive is generated by the SME and issued to the MLME to indicate whether a recieved SME-MLME-DB-QUERY.request was valid, in which case a query to an external database was generated and sent to the upper layers.

      - Effect on receipt

When the SM receives the SME-MLME-DB-QUERY.confirm it will identify whether its request to access the external database was successfully received by the SME and a query to the database was generated. The status parameter indicates the appropriate error code from Table XX in case the requested database is not available.

    - SME-MLME-DB-RESPONSE.indication

The SME-MLME-DB-RESPONSE.indication primitive is used to inform the SM when a response to a previously issued query to an external incumbent database service was successfully received by the SME.  specifies the parameters for the SME-MLME-DB-RESPONSE.indication primitive.

— SME-MLME-DB-RESPONSE.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Database_type | Integer | 0-8 | The value identifies the type of database for which the query request was directed. 0 = TV Incumbent Database 1 = Part 74 Incumbent Database |
| Status | Enumeration | SUCCESS, INVALID_REQUEST, TRANSACTION_EXPIRED | The value indicates whether a response to query was successfully received. |
| Latitude | TBD |  |  |
| Longitude | TBD |  |  |
| For (i=1 to Number of Channels Avaliable, i++) { Channel_Number Max_Transmit_EIRP } | List of available channels and max EIRP allowed per channel |  | List of Available Channel Numbers and corresponding maximum transmit power allowed. |

      - When generated

The SME-MLME-DB-RESPONSE.indication primitive is generated by the SME and issued to the MLME to indicate the reception of a response to a query previously issued to an incumbent database service.

      - Effect on receipt

When the SM receives the SME-MLME-DB-RESPONSE.indication it will identify whether the response to its query to an incumbent database was successfully received by the SME, in which case, the SM will obtain the list of available channels and corresponding maximum EIRP for the requested latitude and longitude. If the response is not successful the SM may decide to issue another query.

  - PHY Spectrum Sensing Services

The 802.22 PHY layer provides local spectrum sensing services through its SSF accessed through the MLME-PLME-SAP.  summarizes the spectrum sensing primitives supported by the PLME through the MLME-PLME-SAP interface. The primitives are discussed in the sub-clauses referenced in the table.

— Spectrum Sensing Primitives supported by the MLME-PLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| MLME-PLME-SAP-CHANNEL-SENSING | 9.4.1 |  | 9.4.2 |
| MLME-PLME-SAP-SENSING-RESULTS |  | 9.4.3 |  |

    - MLME-PLME-SAP-CHANNEL-SENSING.request

The MLME-PLME-SAP-CHANNEL-SENSING.request primitive allows the MLME to request the local PHY SSF unit to perform spectrum sensing.   specifies the parameters for the MLME-PLME-SAP-CHANNEL-SENSING.request primitive.

— MLME-PLME-SAP-CHANNEL-SENSING.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The channel number which is to be sensed by the SSF |
| Channel Bandwidth | Integer |  | The bandwidth of the channel to be sensed by the SSF |
| Sensing Mode | Integer | 0-2 | The sensing mode specifies which SSF outputs are valid as specified in Section 9.3.1.1. |
| Signal Type Array | Array |  | A Array indicating which signal types the SSF is to sense for as specified in Section 9.3.1.1. |
| Sensing Window Specification Array | Array |  | A Array of sensing window specifications as specified in Section 9.3.1.1. Each element in the Sensing Window Specification consists of: NumSensingPeriods SensingPeriodDuration SensingPeriodInterval |
| Maximum Probability of False Alarm Array | Array |  | This value is valid only for sensing modes 0 and 1. Each element specifies the maximum probability of false alarm for the corresponding signal type decision in the sensing present Array. |

      - When generated

The MLME-PLME-SAP-CHANNEL-SENSING.request primitive is generated by the MLME and issued to its PLME to request the local PHY SSF to perform spectrum sensing.

      - Effect on receipt

When the PLME receives the MLME-PLME-SAP-CHANNEL-SENSING.request primitive, it requests the local PHY SSF to perform spectrum sensing.

On receipt of the MLME-PLME-SAP-CHANNEL-SENSING.request the PLME shall issue a MLME-PLME-SAP- CHANNEL-SENSING.confirm primitive to the MLME with a status value.

    - MLME-PLME-SAP-CHANNEL-SENSING.confirm

The MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive is used to inform the MLME whether its request to the local PHY SSF was successfully generated by the MLME.   specifies the parameters for the MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive.

— MLME-PLME-SAP-CHANNEL-SENSING.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The channel number which is to be sensed by the SSF |
| Sensing Mode | Integer | 0-2 | The sensing mode specifies which SSF outputs are valid as specified in Section 9.3.1.1. |
| Status | Enumeration | SUCCESS, INVALID_REQUEST, INVALID_SIGNAL_TYPES | The value indicates whether the sensing request was successfully generated. |
| Invalid Signal Type Array | Array |  | This attribute is valid only if the Status = INVALID_SIGNAL_TYPEs A Array indicating the requested signal types the SSF will not be able to sense for. This Array is defined similarly to the Signal Type Array (see 9.3.1.1). |

      - When generated

The MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive is generated by the PLME and issued to its MLME to indicate whether the received MLME-PLME-SAP-CHANNEL-SENSING.request was valid and whether the SSF is able to perform sensing for the signal types as requested. If the SSF is able to perform the sensing in the requested sensing mode and with the requested probability of false alarm for all types of signals requested, the Status code shall be set to SUCCESS. If the SSF is do not support the requested sensing mode, the status value should be INVALID_REQUEST. If one or more of the signal types in the request is not valid or the SSF does not have the capability to sensing a requested signal type, the status code should be set to INVALID_SIGNAL_TYPE and the corresponding invalid signal types shall be indicated in the Invalid Signal Type Array.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive, it will identify whether its request to the local PHY SSF was successfully received by the PLME.  The status parameter indicates the appropriate error code from Table XX in case the request is invalid.

    - MLME-PLME-SAP-SENSING-RESULTS.indication

The MLME-PLME-SAP-SENSING-RESULTS.indication primitive is used to inform the MLME when the results of a previously issued request to the local PHY SSF were successfully generated by the SSF.   specifies the parameters for the MLME-PLME-SAP-SENSING-RESULTS.indication primitive.

— MLME-PLME-SAP-SENSING-RESULTS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The number of the channel sensed |
| Sensing Mode | Integer | 0-2 | The sensing mode used by the SSF to perform the measurements as specified in Section 9.3.1.1. |
| Signal Type Array | Array |  | A Array indicating which signal types the SSF sensed for as specified in Section 9.3.1.1. |
| Signal Present Array | Array |  | Each element in the Array is a signal present decision. Each decision can take on three possible values, which are given in 9.3.1.2. |
| Confidence Array | Array |  | Each element in the confidence Array is a confidence metric for the sensing result for the corresponding signal type as defined in 9.3.1.2. The Confidence Array is only valid for Sensing Mode 1. |
| Field Strength Estimate Array | Array |  | Each element in the Array is an estimate of the field strength of the corresponding signal type. (see 9.3.1.2). The Field Strength Estimate Array is only valid for Sensing Mode 2. |
| Error Standard Deviation Array | Array |  | Each element in the Array is the standard deviation of the corresponding field strength estimate in the field strength estimate Array. The Error Standard Deviation Array is only valid for Sensing Mode 3. |

      - When generated

The MLME-PLME-SAP-SENSING-RESULTS.indication primitive is generated by the PLME and issued to the MLME to indicate the results of a previously issued request to the local PHY SSF have been generated.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-SENSING-RESULTS.indication it will obtain the sensing results to its request to the local PHY SSF.

  - Geo-location Services

The PHY layer provides local geolocation services through its satellite-based location acquisition unit to the SM/SA through the MLME-PLME-SAP.  summarizes the geolocation primitives supported by the PLME through the MLME-PLME-SAP interface. The primitives are discussed in the subclauses referenced in the table.

— Geolocation Primitives supported by the MLME-PLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| MLME-PLME-SAP-GEOLOCATION | 9.5.1 |  | 9.5.2 |
| MLME-PLME-SAP-GEOLOCATION-RESULTS |  | 9.5.3 |  |

    - MLME-PLME-SAP-GEOLOCATION.request

The MLME-PLME-SAP-GEOLOCATION.request primitive allows the MLME to request the local PHY geolocation unit to perform geolocation.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION.request primitive.

— MLME-PLME-SAP-GEOLOCATION.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NMEA Sentence Request | String | (length 6 octets) | NMEA 0183 ASCII string (e.g. $GPGGA) |

      - When generated

The MLME-PLME-SAP-GEOLOCATION.request primitive is generated by the MLME and issued to its PLME to request the local PHY geolocation service to perform geolocation.

      - Effect on receipt

When the PLME receives the MLME-PLME-SAP-GEOLOCATION.request primitive, it requests the local PHY geolocation service to perform geolocation.

On receipt of the MLME-PLME-SAP-GEOLOCATION.request the PLME shall issue a MLME-PLME-SAP-GEOLOCATION.confirm primitive to the MLME with a status value.

    - MLME-PLME-SAP-GEOLOCATION.confirm

The MLME-PLME-SAP-GEOLOCATION.confirm primitive is used to inform the MLME whether its request to the local PHY geolocation service was successfully generated by the MLME.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION.confirm primitive.

— MLME-PLME-SAP-GEOLOCATION.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the requested query was successfully generated. |

      - When generated

The MLME-PLME-SAP-GEOLOCATION.confirm primitive is generated by the PLME and issued to its MLME to indicate whether the received MLME-PLME-SAP-GEOLOCATION.request was valid, in which case the PLME acquires the requested NMEA sentence from the local PHY geolocation service.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-GEOLOCATION.confirm primitive, it will identify whether its request to the local PHY geolocation service was successfully received by the PLME.  The status parameter indicates the appropriate error code from Table XX in case the local PHY geolocation service was not available.

    - MLME-PLME-SAP-GEOLOCATION-RESULTS.indication

The MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive is used to inform the MLME when a response to a previously issued request to the local PHY geolocation service was successfully received by the PLME.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive.

— MLME-PLME-SAP-GEOLOCATION-RESULTS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Length | Integer | 0-128 | Length of the location data string in octets (0 to 128 characters) |
| Location Data String | Char |  | NMEA 0183 ASCII string |

      - When generated

The MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive is generated by the PLME and issued to the MLME to indicate the reception of a response to a query previously issued to the local PHY geolocation service.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-GEOLOCATION-RESULTS.indication it will identify whether the response to its request to the local PHY service was successfully received by the PLME, in which case, the MLME will obtain NMEA string containing the latitude and longitude information. If the response is not successful the MLME may decide to issue another request.

  - Spectrum Sensing

Spectrum sensing is the process of observing the RF spectrum of a television channel to to determine its occupancy (by either incumbents or other WRANs).

The base station and all CPEs shall implement the spectrum sensing function

The spectrum sensing function observes the RF spectrum of a television channel and reports the results of that observation to the SM (at the BS) or the SA (at a CPE).  The spectrum sensing function is described in Sub-clauses 9.6 and 9.7.

    - The Spectrum Sensing Function

The spectrum sensing function observes the RF spectrum of a television channel for a set of signal types and reports the results of this observation.  The spectrum sensing function is implemented in both the base station and the CPEs.  There are MAC management frames that allow the base station to control the operation of the spectrum sensing function within each of the CPEs.   illustrates the inputs and outputs of the spectrum sensing function.

The inputs to the spectrum sensing function come from the spectrum manager (SM) in the case of the BS and/or the SA in the case of a CPE. The inputs to the spectrum sensing function are described in Sub-clause 9.1.1. The outputs from the spectrum sensing function are returned to the SM and/or the SA. The outputs of the spectrum sensing function are described in Sub-clause 9.1.2.  The behaviour of the spectrum sensing function is described in Sub-clause 9.1.3.

Some of the possible sensing techniques that can be used to realize the spectrum sensing function are described in Annex A.  The use of any specific sensing technique is optional, as long as the inputs, outputs and behaviour meet the specification of this sub-clause.

Spectrum Sensing Function

(

SSF

)

Channel Bandwidth

Signal Type Vector

Sensing Window

Specification Vector

Sensing Mode

Maximum  Probability

of False Alarm

Signal Present Vector

Confidence Vector

Channel Number

RF

Field Strength

Estimate Vector

Error Standard

Deviation Vector

Signal Type Vector

Sensing Mode

The Spectrum Sensing Function

      - SSF Inputs

A summary of the spectrum sensing inputs is given in .

— The Spectrum Sensing Function Input Signals

| Input Name | Input Description |
| --- | --- |
| RF | Radio Frequency |
| Channel Number | The channel number which is to be sensed by the SSF |
| Channel Bandwidth | The bandwidth of the channel to be sensed by the SSF |
| Signal Type Array | An array indicating the signal types for which the SSF is to sense |
| Sensing Window Specification Array | An array of sensing window specifications. Each SFS specifies the details of the sensing window for a given signal type being sensed |
| Sensing Mode | The sensing mode specifies which SSF outputs are valid and in some cases it specifies the behaviour of the SSF |
| Maximum Probability of False Alarm (could be array to allow different values for different signal types) | In sensing modes 0 and 1 this value specifies the maximum probability of false alarm for each sensing mode decision in the signal present array |

The RF input shall be connected to the output of a series of RF amplifiers which are themselves connected to the WRAN sensing antenna.

The channel number is the television channel number that the SSF is to sense.  The center frequency for each channel number is given in Table XYZ.

The channel bandwidth is the bandwidth of the television channel that the SSF is to sense.

The signal type array (STV) indicates which signal types are to be sensed for by the SSF.  The array is a one-dimensional array of length STVLength, indexed from 0 to STVLength -1.  The STV is a binary array whose elements are either zero or 1.  The i-th element in the array specifies whether the SSF is to sense for i-th signal type.  The mapping of STV index to signal type is given in .

The value of STVLength is 2 octets.

— Signal Type Array Indices

| STV Index | Signal Type |
| --- | --- |
| 0 | Any Signal Type |
| 1 | IEEE 802.22 WRAN |
| 2 | IEEE 802.22.1 Sync Burst |
| 3 | IEEE 802.22.1 PPDU |
| 4 | ATSC |
| 5 | NTSC |
| 6 | Wireless Microphone |
| 7 | DVB-T |
| 8-15 | Reserved |

A one in index zero of the STV indicates sensing for any signal type, with no distinction between signal types.  A one in index one of the STV indicates the SSF should sense for an 802.22 WRAN.

As an example, if the STV is given as follows, [edit figure to represent 2 octet size]

Then the SSF shall sense for an 802.22.1 Sync Burst, an ATSC signal, and NTSC signal and a wireless microphone.

The sensing window specification array (SWSV) is a one-dimensional array of length STVLength.  Each element in the SWSV is a sensing window specification (SWS).  If the i-th element of the STV is one (1) then the i-th element of the SWSV shall be set to a valid sensing window specification.  If the i-th element of the STV is set to zero (0) then the i-th element of the SWSV does not need to be set to a valid SWS since it will be ignored by the SSF.

A sensing window specification (SWS) consists of two parameters.  These two parameters specify the window of time over which the SSF shall sense for specified signal type.   illustrates a sensing window.

A sensing window consists of NumSensingPeriods number of sensing periods. The minimum number of sensing periods is one and the maximum number is TBD.

Each sensing period is of duration SensingPeriodDuration symbols and adjacent sensing periods are separated by SensingPeriodInterval symbols as shown in Figure 126.

The parameters in a sensing window specification are given in .

The details of when a sensing window starts and the scheduling of the, possibly multiple, quiet times are detailed in the MAC.

Sensing Window [fix a bit to harmonize w/text]

— Bounds of Sensing Window Specifications

| Parameter Name | Range | Units | Value |
| --- | --- | --- | --- |
| NumSensingPeriods | 0 to 63 | integer | comes from SM or SA |
| SensingPeriodDuration | 0 to 1023 | symbols | comes from SM or SA |
| SensingPeriodInterval | 0 to 255 | frames | comes from SM or SA |

[Need text here to refer to appropriate parts of MAC section to allow flexible definition of sensing windows and assure interoperability and performance and need to define bounds for quiet periods in some part of this major section on the SSF]

Quiet times are scheduled by the MAC layer.  Quiet periods can be scheduled using the superframe control header as described in Clause Error: Reference source not found.  Also, quiet periods can be scheduled using the channel quiet request message, as described in Clause Error: Reference source not found.  A general discussion on quiet periods can found in Clause Error: Reference source not found

The sensing mode specifies which outputs of the SSF are valid and in some cases the behaviour of the SSF.   summarizes the valid SSF outputs for each of the sensing modes.  The table also includes a description of each sensing mode.

— Summary of the Sensing Modes

| Sensing Mode | Valid SSF Outputs | Description |
| --- | --- | --- |
| 0 | Signal Present Array | For each signal type the SSF generates a binary decision as to whether the signal is present in the television channel |
| 1 | Signal Present Array Confidence Array | Same as sensing mode 0 with the addition of a confidence metric for binary decision |
| 2 | Field Strength Estimate Array | For each signal type the SSF generates an estimate of the field strength of that signal |
| 3 | Error Standard Deviation Array | The standard deviation of the field strength estimate from Mode 2 if implemented |

Mode 0 is mandatory at the BS and the CPE.

Mode 1 is optional at the BS and the CPE.

Mode 2 is optional at the BS and the CPE.

Mode 3 is optional at the BS and the CPE.

The maximum probability of false alarm input specifies the behaviour of the elements of the signal present array when no signal is present on the RF input.  The details of the behaviour of the SSF, and its dependence on this input parameter, are given in Clause 9.1.3.

      - SSF Outputs

The sensing mode and the signal type array are passed through the SSF.  These parameters indicate which of the other SSF outputs are valid and hence are useful for subsequent processing.  The MAC messages for specifying the sensing measurement report structure are covered in Clause Error: Reference source not found.  There is a general description of sensing measurement reporting in Clause Error: Reference source not found.

The signal present array is a one dimensional array of length STVLength.  Each element in the array is a signal present decision.  Each decision can take on three possible values, which are given in .  The i-th signal present decision corresponds to the i-th signal type as listed in .

— Values of the Signal Present Decision

| Value of Signal Present Decision | Description |
| --- | --- |
| TRUE | SSF decided that the signal is present in the channel |
| FALSE | SSF decided that the signal is not present in the channel |
| NODECISION | The SSF makes no decision regarding the presence of the signal in the channel. This is the output when the SSF was not instructed to sense for the signal type. |

The confidence array is a one dimensional array of confidence metrics.  The i-th confidence metric indicated the confidence in the i-th signal present decision.  A confidence metric varies between a minimum of zero (0) indicating no confidence in the signal present decision and a maximum of one (1) indicating total confidence in the signal present decision.  In practice since it is unlikely the SSF will have no confidence in the SPD or have total confidence in the SPD, the confidence metric is typically larger than zero and smaller than one.  The range of a confidence metric is given in .  [unsigned int 0-15 or 0-255] (nibble or byte]]

— Range of a Confidence Metric

| Limit | Value | Description |
| --- | --- | --- |
| Minimum confidence metric | 0 | No confidence in SPD |
| Maximum confidence metric | 1 | Total confidence in SPD |

The field strength estimate array is a one dimensional array of field strength estimates.  The i-th field strength estimate is an estimate of the i-th signal type measured at the location of the station (base station or CPE) in which the SSF resides.  The range of a field strength estimate is given in  for a 8-bit resolution and an increment of 0.5 dB.

— Range of a Field Strength Estimate

| Limit | Value | Units | Description |
| --- | --- | --- | --- |
| FSMin | 10 | dBV/m | Minimum Field Strength Estimate |
| FSMax | 137.5 | dBV/m | Maximum Field Strength Estimate |

The error standard deviation array is a one dimensional array with each element in the array is the standard deviation of the corresponding estimate in the field strength estimate array.  The range of an error standard deviation of a field strength estimate is given in .

— Range of the Standard Deviation of a Field Strength Estimate Error

| Limit | Value | Units | Description |
| --- | --- | --- | --- |
| ESDMin | 0 | dB | Minimum Standard Deviation of the Field Strength Estimate Error |
| ESDMax | TBD | dB | Maximum Standard Deviation of the Field Strength Estimate Error |

      - SSF Behavior

In this clause all references to signal power refer to the signal power measured as the input to the sensing receiver.

        - Sensing Mode 0

When the sensing mode is set to zero (0) the only valid SSF outputs are listed in .

— Valid outputs in Sensing Mode 0

Sensing Mode

Signal Type Array

Signal Present Array

The values of Sensing Mode and Signal Type Array are the same as their input values.

In sensing mode zero, if the i-th element of the signal type array is zero then the i-th element of the SPV shall be set to NODECISION.

— Summary of SPV for Sensing Mode Zero with STV(i) set to 0

| Sensing Mode | STV Index i | Signal Type | STV(i) | SPV(i) |
| --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 0 | NODECISION |
| 0 | 1 | IEEE 802.22 WRAN | 0 | NODECISION |
| 0 | 2 | IEEE 802.22.1 Sync Frame | 0 | NODECISION |
| 0 | 3 | IEEE 802.22.1 PPDU | 0 | NODECISION |
| 0 | 4 | ATSC | 0 | NODECISION |
| 0 | 5 | NTSC | 0 | NODECISION |
| 0 | 6 | DVD | 0 | NODECISION |
| 0 | 7 | Wireless Microphone: Spectral characteristic match (center frequency, bandwidth) | 0 | NODECISION |
| 0 | 8 | Wireless Microphone: Spreading code detection | 0 | NODECISION |
| 0 | 9 | Wireless Microphone: Sync word detection | 0 | NODECISION |
| 0 | 10 | Wireless microphone: Index properly decodes | 0 | NODECISION |
| 0 | 11 | Wireless Microphone: Payload passes CRC | 0 | NODECISION |
| 0 | 12 | Wireless Microphone: Payload authenticates | 0 | NODECISION |

In sensing mode zero if the i-th element of the signal type array is set to one and there is no signal present at the sensing antenna then the i-th element of the SPV shall be FALSE with probability greater than or equal to 1-MPFA , where MPFA is the maximum probability of false alarm [KAY1]. And the i-th element of the SPV is set to TRUE with probability less than or equal to MPFA.

The behavior of the SSF in sensing mode 0 with the i-th element of STV set to one and with no signal present at the sensing antenna is summarized in .

— Summary of SPV for Sensing Mode Zero with STV(i) set to 1 with no Signal Present

| Sensing Mode | STV Index i | Signal Type | STV(i) | SPV(i) | Probability (need or not?) |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 1 | FALSE |  |
| 0 | 1 | IEEE 802.22 WRAN | 1 | FALSE |  |
| 0 | 2 | IEEE 802.22.1 Sync Frame | 1 | FALSE |  |
| 0 | 3 | IEEE 802.22.1 PPDU | 1 | FALSE |  |
| 0 | 4 | ATSC | 1 | FALSE |  |
| 0 | 5 | NTSC | 1 | FALSE |  |
| 0 | 6 | Wireless Microphone | 1 | FALSE |  |
| 0 | 7 | DVB | 1 | FALSE |  |

In sensing mode zero with the i-th element of the STV set to one and with the signal present in the channel at the specified signal power level given in , the i-th output of the SPV shall be TRUE with probability greater than or equal to the value specified in .

The behaviour of the SSF output SPV is specified in .

— Summary of SSF Outputs for Sensing Mode Zero

| Sensing Mode | STV Index i | Signal Type | STV(i) | Signal Power | SPV(i) | Probability (need or not?) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | Any Signal Type | 1 | TBD | TRUE |  |
| 0 | 1 | IEEE 802.22 WRAN | 1 | TBD | TRUE |  |
|  | 2 | IEEE 802.22.1 Sync Burst | 1 | TBD | TRUE |  |
| 0 | 3 | IEEE 802.22.1 PPDU | 1 | TBD | TRUE |  |
| 0 | 4 | ATSC | 1 | -116 dBm | TRUE |  |
| 0 | 5 | NTSC | 1 | -94 dBm | TRUE |  |
| 0 | 6 | Wireless Microphone | 1 | -107 dBm | TRUE |  |
| 0 | 7 | DVB-T | 1 | TBD | TRUE |  |

        - Sensing Mode 1

When the sensing mode is set to one, the only valid SSF outputs are listed in .

— Valid outputs in Sensing Mode 1

Sensing Mode

Signal Type Array

Signal Present Array

Confidence Array

The values of sensing mode, signal type array and signal present array are the same as in sensing mode 0.

If the i-th element of the signal type array is set to zero then the i-th element of the confidence array is set to 0.

If the i-th element of the signal type array is set to one then the i-th element of the confidence array is a confidence metric indicating the confidence the SSF has in the i-th element of the signal present array.  A confidence metric is a measure of the confidence of a decision.  The range of a confidence metric is given in .

If the i-th element of the SPV is TRUE and the signal power of the i-th signal type is increased then the average value of the confidence metric should increase.

If the i-th element of the SPV is FALSE and the signal power of the i-th signal type is decreased then the average value of the confidence metric should increase.

        - Sensing Mode 2

When the sensing mode is set to two the only valid SSF outputs are listed in .

— Valid outputs in Sensing Mode 2

Sensing Mode

Signal Type Array

Field Strength Estimate Array

Error Standard Deviation Array

The values of sensing mode and signal type array are the same as their input values.

If the i-th element of the STV is set to zero then the i-th element of the FSEV shall be invalid since the i-th signal type is not sensed.

If the i-th element of the STV is set to one then the i-th element of the FSEV shall be an estimate [KAY2] of the field strength of the i-th signal type at the location of the antenna connected to the station in which the SSF resides.

If the i-th element of the STV is set to zero then the i-th element of the ESDV shall be invalid since the i-th signal type is not sensed.

If the i-th element of the STV is set to one then the i-th element of the ESDV shall be the average standard deviation of the i-th FSEV.  This average standard deviation is the square root of the average variance of the field strength estimate [KAY2].

  - PHY Spectrum Sensing Services

The 802.22 PHY layer provides local spectrum sensing services through its SSF accessed through the MLME-PLME-SAP.  summarizes the spectrum sensing primitives supported by the PLME through the MLME-PLME-SAP interface. The primitives are discussed in the sub-clauses referenced in the table.

— Spectrum Sensing Primitives supported by the MLME-PLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| MLME-PLME-SAP-CHANNEL-SENSING | 9.4.1 |  | 9.4.2 |
| MLME-PLME-SAP-SENSING-RESULTS |  | 9.4.3 |  |

    - MLME-PLME-SAP-CHANNEL-SENSING.request

The MLME-PLME-SAP-CHANNEL-SENSING.request primitive allows the MLME to request the local PHY SSF unit to perform spectrum sensing.   specifies the parameters for the MLME-PLME-SAP-CHANNEL-SENSING.request primitive.

— MLME-PLME-SAP-CHANNEL-SENSING.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The channel number which is to be sensed by the SSF |
| Channel Bandwidth | Integer |  | The bandwidth of the channel to be sensed by the SSF |
| Sensing Mode | Integer | 0-2 | The sensing mode specifies which SSF outputs are valid as specified in Section 9.3.1.1. |
| Signal Type Array | Array |  | An array indicating which signal types the SSF is to sense for as specified in Section 9.3.1.1. |
| Sensing Window Specification Array | Array |  | An array of sensing window specifications as specified in Section 9.3.1.1. Each element in the Sensing Window Specification consists of: NumSensingPeriods SensingPeriodDuration SensingPeriodInterval |
| Maximum Probability of False Alarm Array | Array |  | This value is valid only for sensing modes 0 and 1. Each element specifies the maximum probability of false alarm for the corresponding signal type decision in the sensing present array. |

      - When generated

The MLME-PLME-SAP-CHANNEL-SENSING.request primitive is generated by the MLME and issued to its PLME to request the local PHY SSF to perform spectrum sensing.

      - Effect on receipt

When the PLME receives the MLME-PLME-SAP-CHANNEL-SENSING.request primitive, it requests the local PHY SSF to perform spectrum sensing.

On receipt of the MLME-PLME-SAP-CHANNEL-SENSING.request the PLME shall issue a MLME-PLME-SAP- CHANNEL-SENSING.confirm primitive to the MLME with a status value.

    - LME-PLME-SAP-CHANNEL-SENSING.confirm

The MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive is used to inform the MLME whether its request to the local PHY SSF was successfully generated by the MLME.   specifies the parameters for the MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive.

— MLME-PLME-SAP-CHANNEL-SENSING.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The channel number which is to be sensed by the SSF |
| Sensing Mode | Integer | 0-2 | The sensing mode specifies which SSF outputs are valid as specified in Section 9.3.1.1. |
| Status | Enumeration | SUCCESS, INVALID_REQUEST, INVALID_SIGNAL_TYPES | The value indicates whether the sensing request was successfully generated. |
| Invalid Signal Type Array | Array |  | This attribute is valid only if the Status = INVALID_SIGNAL_TYPEs A Array indicating the requested signal types the SSF will not be able to sense for. This Array is defined similarly to the Signal Type Array (see 9.3.1.1). |

      - When generated

The MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive is generated by the PLME and issued to its MLME to indicate whether the received MLME-PLME-SAP-CHANNEL-SENSING.request was valid and whether the SSF is able to perform sensing for the signal types as requested. If the SSF is able to perform the sensing in the requested sensing mode and with the requested probability of false alarm for all types of signals requested, the Status code shall be set to SUCCESS. If the SSF is do not support the requested sensing mode, the status value should be INVALID_REQUEST. If one or more of the signal types in the request is not valid or the SSF does not have the capability to sensing a requested signal type, the status code should be set to INVALID_SIGNAL_TYPE and the corresponding invalid signal types shall be indicated in the Invalid Signal Type Array.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-CHANNEL-SENSING.confirm primitive, it will identify whether its request to the local PHY SSF was successfully received by the PLME.  The status parameter indicates the appropriate error code from Table XX in case the request is invalid.

    - MLME-PLME-SAP-SENSING-RESULTS.indication

The MLME-PLME-SAP-SENSING-RESULTS.indication primitive is used to inform the MLME when the results of a previously issued request to the local PHY SSF were successfully generated by the SSF.   specifies the parameters for the MLME-PLME-SAP-SENSING-RESULTS.indication primitive.

— MLME-PLME-SAP-SENSING-RESULTS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Channel Number | Integer |  | The number of the channel sensed |
| Sensing Mode | Integer | 0-2 | The sensing mode used by the SSF to perform the measurements as specified in Section 9.3.1.1. |
| Signal Type Array | Array |  | A Array indicating which signal types the SSF sensed for as specified in Section 9.3.1.1. |
| Signal Present Array | Array |  | Each element in the Array is a signal present decision. Each decision can take on three possible values, which are given in 9.3.1.2 (). |
| Confidence Array | Array |  | Each element in the confidence Array is a confidence metric for the sensing result for the corresponding signal type as defined in 9.3.1.2. The Confidence Array is only valid for Sensing Mode 1. |
| Field Strength Estimate Array | Array |  | Each element in the Array is an estimate of the field strength of the corresponding signal type. (see 9.3.1.2). The Field Strength Estimate Array is only valid for Sensing Mode 2. |
| Error Standard Deviation Array | Array |  | Each element in the Array is the standard deviation of the corresponding field strength estimate in the field strength estimate Array. The Error Standard Deviation Array is only valid for Sensing Mode 3. |

      - When generated

The MLME-PLME-SAP-SENSING-RESULTS.indication primitive is generated by the PLME and issued to the MLME to indicate the results of a previously issued request to the local PHY SSF have been generated.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-SENSING-RESULTS.indication it will obtain the sensing results to its request to the local PHY SSF.

  - Geolocation/Database

Two modes of geolocation can be used with the 802.22 standard. Satellite-based geolocation is mandatory.  The MAC requirements are described below.  Terrestrial-based geolocation using the CDMA ranging and the coexistence beacon protocol is also described below.

    - Satellite-based Geolocation

The WRAN system shall use its satellite-based geolocation capability to determine the latitude and longitude of the BS transmitting antenna within a radius of 15 m and its altitude above mean sea level.

The BS geolocator shall use NMEA strings provided by each CPE’s satellite-based geolocation subsystem to determine the location of the CPE and to determine the distance between the CPE location and each nearby incumbent protected contour.

    - Terrestrially-based Geolocationd Geolocation

Geolocation is achieved in a two-step process. First, the range between the CPEs and their BS shall be determined. Second, the distance between various CPEs within a cell can be determined to enhance geolocation resolution. The goal of this process is to allow the geolocator to build a graphic representaion of the set of CPEs that form a cell under the BSs control. An abstract entity, called the geolocator, shall  send ranging requests (to the BS), receive the responses (from the BS) and perform the calculus to derive this representation from a combination of ranging data and waypoints.

      - Geolocator

This entity requests and stores ranging information collected by a BS and its CPEs as well as waypoint information and computes the distance between the BS and each of its CPEs (6.9.28.2) as well as the distance between selected CPEs (6.9.28.3) within a cell.

When the geolocator wants to locate a CPE, it determines which benchmark devices will be used. It then signals the BS or BSs it has selected that it wants to locate a CPE and which benchmark devices are to be used in the process, using a specific Fine Ranging Symbol Set (FRSS).

The BS now sends a Fine Ranging Arm request to its assocaited benchmark CPEs that are to capture the FRSS. It then sends a Fine Ranging Burst Transmission Request to the CPE to be ranged or located.

Following this request, the CPE to be ranged captures and stores the next DS-SCH-FRSS amplitudes and phases before correction. In the following CBP interval, the CPE emits the requested FRSS (US-FRSS) along with the data describing the captured DS-SCH FRSS amplitudes and phases. The BS and its CPE's capture the US-FRSS tone amplitudes and phases along with the DS-SCH-FRSS data. The BS forwards this raw data to the Geolocator. The BS then querries its reference CPEs. The reference CPEs then send their DS-SCH-FRSS and US-FRSS data to the BS that relays this to the Geolocator.

Using all this information, the Geolocator computes the location of the CPE to be located and returns a NMEA longitude-latitude string to the BS.

      - BS to CPE

The superframe preambule will be used for CPE synchronisation in time and frequency. It shall be followed by a message containing information (as requested by the geolocator) to trigger the acquisition and storage at the CPE of the symbol transmitted by the BS as received by the CPE and returned to the BS by the CPE in the same (or a subsequent) superframe as scheduled by the BS. This « Timeout » message (see section Error: Reference source not found) is sent by the BS to the CPE to the exclusion of any other communication with the BS in this interval. During this interval, de BS will retain the CPEs CID such as to allow it to reconnect to the BS without requiring a resynchronization at the end of this interval.

At the end of this interval, the CPE, already having a scheduled reserved response time window, shall respond to the BS with a 'US PHY PDU'; the BCH shall contain the identification of both the BS and CPE entities and this CPE shall transmit the symbol (formed of the set of previously acquired DS pilot measurements) to the BS in the payload of the 'MAC PDU', and the BS shall relay these with a minimum resolution of 3 ns, less the scheduling delays it introduced, back to the geolocator. The geolocator shall store this measured data and use it at a later time to compute the geographic location.

      - CPE to CPE

Inter-CPE communication is possible by the means provided by the ‘CBP packets’. These are used, amongst other things, for self-coexistence between WRAN systems. The superframe ‘self-coexistence windows’ are under the BSs' control. They shall allow a given CPE to transmit a « CBP packet » while indicating that this packet is destined for geolocation ranging purposes.  A CPE to CPE ranging initiative shall come from a BS throught a ‘BLM-REQ’ management message and shall request a CPE to communicate to another CPE within a cell. The BS shall specify the CPE response interval and the means to be used. The ‘BLM-REP’ is the means for the CPE to communicate its response to the BS at the end of the sepcified delay interval.

The BS sends the preamble followed by the 'SCH' with the 'CT=1' field to indicate that the following transmission will be a gelolcation ‘CBP MAC PDU’. To this effect, the CBP beacon shall carry the CBP Geolocation IE (see Error: Reference source not found), which contains the MAC address of the CPE transmitting the ‘Beacon MAC PDU’ .  By carrying the CBP Geolocation IE, these beacons shall be distinguished as geolocation messages. The 'Payload' shall contain scheduling information necessary to precisely determine the « Timeout » interval, interval after which the receiving CPE shall return the acquired measurements from the requesting CPE. The second CPE shall then forward its aquired measurements to the BS at the specified interval. The BS shall then deduct the scheduling intervals and report the flight times back to the geolocator. The geolocator may then extract the flight time from the BS to the CPE and the inter-CPE reported flight time and calculate the precise geographic location of each CPE.]

  - Geo-location Services

The PHY layer provides local geolocation services through its satellite-based location acquisition unit to the SM/SA through the MLME-PLME-SAP.  summarizes the geolocation primitives supported by the PLME through the MLME-PLME-SAP interface. The primitives are discussed in the subclauses referenced in the table.

— Geolocation Primitives supported by the MLME-PLME-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| MLME-PLME-SAP-GEOLOCATION | 9.5.1 |  | 9.5.2 |
| MLME-PLME-SAP-GEOLOCATION-RESULTS |  | 9.5.3 |  |

    - MLME-PLME-SAP-GEOLOCATION.request

The MLME-PLME-SAP-GEOLOCATION.request primitive allows the MLME to request the local PHY geolocation unit to perform geolocation.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION.request primitive.

— MLME-PLME-SAP-GEOLOCATION.request parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| NMEA Sentence Request | String | (length 6 octets) | NMEA 0183 ASCII string (e.g. $GPGGA) |

      - When generated

The MLME-PLME-SAP-GEOLOCATION.request primitive is generated by the MLME and issued to its PLME to request the local PHY geolocation service to perform geolocation.

      - Effect on receipt

When the PLME receives the MLME-PLME-SAP-GEOLOCATION.request primitive, it requests the local PHY geolocation service to perform geolocation.

On receipt of the MLME-PLME-SAP-GEOLOCATION.request the PLME shall issue a MLME-PLME-SAP-GEOLOCATION.confirm primitive to the MLME with a status value.

    - MLME-PLME-SAP-GEOLOCATION.confirm

The MLME-PLME-SAP-GEOLOCATION.confirm primitive is used to inform the MLME whether its request to the local PHY geolocation service was successfully generated by the MLME.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION.confirm primitive.

— MLME-PLME-SAP-GEOLOCATION.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Status | Enumeration | SUCCESS, INVALID_REQUEST | The value indicates whether the requested query was successfully generated. |

      - When generated

The MLME-PLME-SAP-GEOLOCATION.confirm primitive is generated by the PLME and issued to its MLME to indicate whether the received MLME-PLME-SAP-GEOLOCATION.request was valid, in which case the PLME acquires the requested NMEA sentence from the local PHY geolocation service.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-GEOLOCATION.confirm primitive, it will identify whether its request to the local PHY geolocation service was successfully received by the PLME.  The status parameter indicates the appropriate error code from Table XX in case the local PHY geolocation service was not available.

    - MLME-PLME-SAP-GEOLOCATION-RESULTS.indication

The MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive is used to inform the MLME when a response to a previously issued request to the local PHY geolocation service was successfully received by the PLME.   specifies the parameters for the MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive.

— MLME-PLME-SAP-GEOLOCATION-RESULTS.indication parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Length | Integer | 0-128 | Length of the location data string in octets (0 to 128 characters) |
| Location Data String | Char |  | NMEA 0183 ASCII string |

      - When generated

The MLME-PLME-SAP-GEOLOCATION-RESULTS.indication primitive is generated by the PLME and issued to the MLME to indicate the reception of a response to a query previously issued to the local PHY geolocation service.

      - Effect on receipt

When the MLME receives the MLME-PLME-SAP-GEOLOCATION-RESULTS.indication it will identify whether the response to its request to the local PHY service was successfully received by the PLME, in which case, the MLME will obtain NMEA string containing the latitude and longitude information. If the response is not successful the MLME may decide to issue another request.

10 Configuration

[FRD

The WRAN firmware SHALL meet regulatory requirements to be tamper-proof to prevent unauthorized modification to firmware and/or functionalities (e.g., CPE MAC address, cognitive functionality, RF sensing, DFS, TPC, tuning). Any attempt to load unapproved firmware into a WRAN device shall render it inoperable.

]

# Annex B (informative)
 Example of the functionality of the CPE Spectrum Automaton

## Introduction

There are three instances where the CPE will need to control its sensing behavior locally without immediate control of the base station.  These are:

a)	at the initial turn-on of the CPE before association is established with the base station;

b)	when the CPE looses contact with its base station; and

c)	during idle time when the base station has not attributed any specific task to either the CPE sensing signal path or the WRAN signal path, or both.

These are the only cases where local autonomous intelligence related to spectrum sensing, akin to a ‘spectrum management’ function at the base station, will be required at the CPE.  The CPE local spectrum sensing should be considered more as a set of simple CPE MAC routines (sensing automaton) than complex ‘spectrum manager’ functions that will reside at the base station.  The functionality of the CPE local autonomous spectrum sensing for these three specific cases is covered in the following sub-sections.

## Initial turn on

The functionality of the CPE local autonomous spectrum sensing when the CPE is initially switched on is described below for an example of implementation and illustrated in Figure 1.  At initial turn-on and self-test, the CPE will need to sweep all the TV channels that are within its range of operation plus one more channel at both ends unless it goes beyond the extent of the relevant TV band.

For each channel, an RSSI measurement will be performed on the WRAN signal path and attempt shall be made to capture a WRAN superframe header or a CBP packet. An RSSI measurement shall also be performed on the sensing signal path.  If an SCH is captured and the level of RF signal is sufficiently high, the WRAN signal path will attempt to acquire the frame header, the broadcast PDU’s sent by the BS to advertise the WRAN service for CPE initialization and the management packet providing the channel occupancy from the base station; the CHO-UPD MAC message.  If an SCH can be acquired but the signal level is insufficient or a CBP packet can be captured, the presence of a WRAN signal will be recorded along the channel number and the measured RSSI.  If an SCH or a CBP packet cannot be detected, the sensing path RSSI will be evaluated whether the level of the RF signal is sufficient to carry out a fast signal classification.  The result of the measurement will be stored locally so that it can later be sent to the base station when association is established or later on upon request from the BS.

For those TV channels where the RF energy is found to be insufficient to allow for fast signal classification or even provide for a reliable RSSI measurement, fine sensing will be applied to determine the presence of broadcast incumbents and their signal type.  Again, the information on whether there is a broadcast incumbent in a channel or not and its type will be stored locally so that it can later be sent to the base station when association is established or later upon request from the BS.

The channel will then be incremented to repeat the above initial sensing.  The order in which the channels are to be sensed will be implementation dependent but a simple sweep going from the lowest channel of the range to the highest will likely be the most straight-forward.  Note that the information acquired from the local WRAN base stations through the CHO-UPD MAC message during initialization could be used to skip the non-available channels and the signal classification and low level sensing on channels belonging to the “occupied” set to save time, but since time is less critical at initialization, it will be more useful for the CPE to document all the channels in the range and confirm that the incumbents indicated by the BS really exist in these ‘occupied’ channels and at what signal level at the CPE location.

Once all the TV channels to be scanned have been sensed, the information on the local WRAN channel occupancy that would not create interference to local incumbents on channels N and N+/-1 shall be made available to the higher layers at the CPE.  Note that if there is no WRAN channel that can be used, the CPE initialization will be aborted.  Depending on the CPE implementation, this information may be presented to the local interface of the CPE so that it could ultimately be displayed on the screen of the user terminal to allow for an informed choice among available local WRAN networks (similar to the Access Point selection in Wi-Fi).  Local algorithms could also be implemented in the CPE to automate the process for choosing the WRAN network.

Once the choice of a WRAN service on TV channel N0 has been made by the user or a local routine, a process to verify that the WRAN TX/RX antenna is properly oriented for the selected channel can be carried out based on the fact that the RSSI can be obtained from both the WRAN directional and the sensing omni-directional antennas.  If these two levels are not equal (within a 1 dB tolerance), indication would be sent to the higher layer that the azimuth of the antenna needs to be aligned for proper association with the selected WRAN service.  (The signal differential could be sent to provide a variable indicating the goodness of the azimuth alignment for easy setup.)  Once the antenna azimuth has been changed, a new RSSI sensing is performed on the sensing and WRAN signal paths and a new comparison is made.  The process would loop until adequate azimuth alignment is achieved with the base station of the selected WRAN service (i.e., the two RSSI’s are within the 1 dB tolerance).  Although useful, this process will need to be used with care by the installer because it may not always result in a successful alignment of the WRAN antenna if high level multipath is present at the CPE location.

A second round of spectrum sensing will then take place on the selected channel (N0) and its adjacent channels (N0+/-1) to make sure that the sensing results are valid within a period of 2 seconds before the CPE can transmit.  Since, by definition, a WRAN service is present on the selected channel, the WRAN signal path will need to re-acquire the SCH or the CBP packet to determine the timing of the quiet periods in this channel since sufficient time will likely have passed since the first channel acquisition which was part of the systematic sweep of all the channels and that the timing of the quiet periods would likely have changed.  Low-power signal sensing and signal classification will then be carried out in channel N0 by the sensing path during these quiet periods to try to identify whether an incumbent service is present underneath the WRAN service. The findings will be recorded locally.

- Flow diagram for CPE sensing during initialization

The sensing process will then move to sense the two adjacent channels during the quiet periods by measuring the RSSI and, depending on whether the measured signal level is high or low, sense the presence of an incumbent signal and classify its type using a fast algorithm or a low-level sensing scheme.  If the channel belongs to the “occupied” set as signaled by the CHO-UPD message, the signal classification could be skipped since it is already known by the base station. The findings will be recorded locally and if incumbents are found in these channels, the selected channel will be eliminated from the list of available WRAN services and the updated list will be presented to the higher layers at the CPE for the selection of another WRAN service.  If no more WRAN service exists, the CPE initialization will be aborted. If no incumbent is present in the three channels sensed, the CPE initialization process will continue as described in section Error: Reference source not found.

If the authorization is refused by the currently selected base station, a new shortened list of available WRAN services will be presented to the higher layers at the CPE for a new channel selection to be made as illustrated in Figure 1 by the entry point “A”.  Then, the next round of sensing process will be repeated with the alignment of the azimuth of the antenna toward the newly selected WRAN base station and the sensing of the newly selected channel (N0) and its adjacent channels (N0+/-1) as described above.  Upon successful sensing results, the CPE association process will continue as described in section  Error: Reference source not found.

## Loss of contact with the base station

If the CPE looses contact with its base station, the CPE automaton local intelligence will make sure that a reasonable number of attempts are made to re-connect with the base station, while avoiding any potential interference to licensed incumbents.  The functionality of the CPE local automaton for this loss of contact with the base station is described below and illustrated in Figure 2.

The CPE shall first identify whether or not a WRAN signal is still present on the selected channel by trying to capture the SCH.  If successful within 2 seconds, attempts to re-associate should be made through the BW Request opportunistic burst or upon specific invitation by the BS.  If this does not work, the re-association should start from an earlier stage with the CDMA ranging burst (entry point C in Figure 1).  If re-association cannot be achieved within 2 seconds., then the CPE should execute the second round of initial sensing for the co-channel and first adjacent channels cases to protect the broadcast incumbents (entry point B in Figure 1).

If the WRAN signal is no longer present in the channel, then the CPE will select the next TV channel on its back-up list and try to capture the superframe header to synchronize with the base station on this new channel and acquire the frame header.  If successful within 2 seconds, attempts to re-associate should be made through the BW Request opportunistic burst or upon specific invitation by the BS, or through the earlier stage of the CDMA ranging burst (entry point C in Figure 1).

If the new superframe capture is not successful, then the CPE should select the next TV channel on its backup list and repeat the process until a successful superframe capture is achieved.  If re-association on all the valid channels in the backup list has failed, the CPE will re-start its initialization process from the entry point “D” in Figure 1.

- Flow diagram for CPE sensing during loss of contact with the base station

## Sensing during CPE idle time

In addition to being able to carry out all the in-band and out-of band sensing requested by the base station through the specific messages described in section 6.9.22.1, the CPE will need to have the necessary routines to autonomously sense its operating channel, the TV channels in its backup list and, as many channels as possible in its candidate channel list in the proper order of priority during its idle time.

Since the channels adjacent to the one being sensed (N+/-1) will also need to be verified, the CPE will need to have the capability of re-tuning its sensing RF path and WRAN signal path to the channels adjacent to those contained in its lists of channels to be sensed.  A RSSI measurement will first take place on the sensing path and RSSI will also be acquired through the WRAN signal path along with an acquisition of the SCH or CBP if WRAN operation is detected on the channel.

A large variation of management traffic due to sensing can be expected depending on how much local intelligence can be relied upon at the CPE to carry out the necessary sensing.  At one extreme of the range, all the sensing requests would come from the base station and a relatively sizeable management traffic will be generated from these sensing requests and their reports.  At the other extreme of the range, all the necessary sensing could be initiated autonomously at the CPE to meet the sensing requirements to clear the operating channel within the required 2 seconds and the channels in the backup channel list within the required 6 seconds.  In this case, the base station would only need to keep the backup channel list and the candidate channel list up-to-date by sending any variation there to the CPEs as a bulk message.  The CPE would only need to send a UCS notification to the base station if an incumbent appears on the operating channel, or a warning using the opportunistic BW Request mechanism if an incumbent appears on one of the backup channels, as well as the number of the channels that the CPE has been able to clear in its candidate channel list.  The advantage here is that if successful sensing can be done by CPEs during their idle periods, the need for more formal sensing with the scheduled quiet periods is reduced.

This way, the base station would still be able to move out of the channel in a timely fashion, remove any occupied channel from its backup list, re-order it and possibly augment it with a candidate channel that has been cleared by all the needed CPEs.

The reality will likely be in between these two extremes where some specific sensing requests will come from the base station while the rest will be carried out by the sensing automaton at the CPE during its idle time.  The functionality of the automaton will be described for the case where all the sensing activities are initiated at the CPE. Figure 3 depicts this process.  Any sensing request from the base station will be scheduled in the local process once the operating channel and the backup channels have been cleared within their respective maximum lapse time.

Note that if a common RF path is used for WRAN operation and sensing, all interruptions on the WRAN signal path will equally affect the sensing signal path.  As a minimum, the CPE needs to acquire all superframe headers and all frame headers from its operating channel unless an inter-frame sensing period is signaled by the superframe control header.  Sensing through the WRAN signal path can be interrupted during the following intervals:

- Superframe headers

- Frame headers

- CPE receiving data during the DS subframe as signaled by the DS-MAP;

- CPE transmitting data during the US subframe as signaled by the US-MAP;

- CPE transmitting data during the opportunistic ranging/UCS notification/BW request window;

- CPE monitoring activity as requested by the base station for CBP packet capture;

- CPE transmitting activity as requested by the base station for CBP packet transmission.

If the CPE is implemented with two separate RF chains for WRAN operation and sensing, sensing through the sensing signal path would only need to be interrupted during the transmission periods on the WRAN signal path due to the limited RF isolation between the WRAN transmit path and the sensing receive path.  This additional time for sensing could then be used to explore more TV channels and more potential interference situations over a shorter time while the CPE is in normal operation.  However, when sensing is to be done on channels where WRAN transmissions are involved or on their respective adjacent channels, the WRAN signal path will be needed to acquire the RSSI and, if large enough, the SCH to identify the timing of the quiet periods so that the presence of incumbents can be sensed underneath the WRAN operation. It is assumed that these quiet periods will be aligned among overlapping WRAN cells operating on N, N+/-1 and N+/-2 in the same area.

The sensing of the operating channel will be carried out during the quiet period signaled by the SCH from the base station for inter-frame sensing and by the maps of the frame header for the intra-frame sensing.  If a coexistence window at the end of the frame is signaled by the base station in the US-MAP, the CPE will shorten its sensing window accordingly and its WRAN receiving path will be enabled during that period to either transmit or acquire a CBP packet.  Upon successful acquisition of a CBP packet, the CPE will send a bandwidth request using the BW-Request opportunistic window and, when scheduled by the BS, send the information extracted from the CBP packet.

- Flow diagram for CPE sensing during idle time

Two types of basic sensing and data gathering will be supported: a) sensing during the idle time of the CPE and b) sensing during specific quiet periods during the idle time of the CPE.  These two basic sensing and data gathering mechanisms are depicted in Figure 4 and Figure 5 respectively.  These mechanisms are used as simple blocks in Figure 3 to simplify the diagram.  If sufficient RF signal is available, a fast RF signal type identification can be used.  If insufficient RF signal is found, then an intra-frame sensing scheme will need to be used in presence of WRAN operation on the channel to be sensed or on its adjacent channels to identify possible broadcast incumbents. Inter-frame sensing could also be used as long as the CPE is not being asked to transmit during such sensing interval.

- Basic sensing algorithm (Sensing, record results and note time)

- Basic quiet period sensing algorithm (Sensing during quiet period, record results and note time)

The sensing process at the CPE automaton will begin by sensing the operating channel (N0) and its adjacent channels (N0+/-1) and it will inform the BS of any urgent finding using the UCS Notification.  If the last sensing has been carried out less than 2 seconds earlier, this sensing will be skipped.  The next channels to be sensed will be the backup channels (NB) in the order given in the list and their respective adjacent channels (NB+/-1).  Again, if the sensing on a specific backup channel has been carried out less than 6 seconds earlier, this sensing will be skipped.  The following channels to be sensed will be the channels appearing in the candidate list (NC) in the order given in the list.  Any channel sensed less than 6 seconds before will be skipped.  The CPE will try to go as deep as possible in the candidate channel list given the sensing periods available and the interruptions that will come up.

If there is no WRAN operation on the channel being sensed, the sensing process will need to verify whether there is WRAN operation on the adjacent channels to be able to schedule its sensing during the quiet intervals of these WRAN operation.  Since signal sensing has to be done on the adjacent channels, if no WRAN operation could be detected on N and N+/-1, the sensing process will need to verify whether there is WRAN operation on N+/-2 to be able to sense N+/-1 during the quiet intervals of N+/-2 because of possible adjacent channel leakage that could mask the presence of incumbents on N+/-1.

The sensing process will be re-initiated on the operating channel (N0) at each time. Due to the nature of the algorithm, this process will be directed to any channel reaching the end of its period of validity in order of priority from the operating channel (2 seconds) to the backup list and candidate list (6 seconds).  Also, any sensing request coming from the base station will be executed as soon as the operating and the backup channels have been cleared within their period of validity.

## Local representation of sensing results

Once association has been achieved, the base station may request the CPE to send the complete results of the initial sensing or any update thereof at anytime using the BLM-REQ PDU.  The CPE will therefore need to keep the latest information stored by the sensing process in its local registers at all times.  34 gives a representation of the information that needs to be stored in these registers.  The CPE shall be capable of reporting the information in all rows of the table except for the two last rows that contain information obtained from the base station.

- — Sensing report registers at the CPE

| Channel number | … | 25 | 26 | 27 | 28 | 29 | 30 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Time of last sensing |  |  |  |  |  |  |  |  |
| Time of last positive |  |  |  |  |  |  |  |  |
| Sensing path RSSI |  |  |  |  |  |  |  |  |
| WRAN path RSSI |  |  |  |  |  |  |  |  |
| Signal type |  |  |  |  |  |  |  |  |
| WRAN service advertisement |  |  |  |  |  |  |  |  |
| Sensing path RSSI under WRAN |  |  |  |  |  |  |  |  |
| Signal type under WRAN |  |  |  |  |  |  |  |  |
| Channel occupancy from BS (CHO-UPD) |  |  |  |  |  |  |  |  |
| Max EIRP at CPE (SME-MLME-DB-RESPONSE) (9.3.7) |  |  |  |  |  |  |  |  |

Annex E.

Collaborative Spectrum Sensing and Decision Making to Provide Protection Against Spurious Signals

Let

H0: Hypothesis such that the incumbent is not occupying the channel being sensed

H1:  Hypothesis such that the incumbent is occupying the channel being sensed

Let x(n) be the received signal at the input of the SSF, where n is the running index of the samples

x(n)=w(n)→H0s(n)+w(n)→H1        (C.1),

where s(n) be the authentic incumbent signal, and w(n) be the noise.  Hence the goal of the SSF is to decide between hypotheses H0 and H1.

Let y(n) be the decision statistic, and  be some threshold used to choose between the Hypotheses H0 and H1 respectively. Then the detection probability (Pd) and the false alarm probability (Pf) can be denoted by

Pd=Py>λ|H1Pf=Py>λ|H0                  (C.2)

Consider a group of N localized sensors, monitoring a specific area, collaboratively trying to distinguish between Hypotheses H0 and H1. If the group collectively decides that the incumbent signal is detected when the detection statistic at, at least one of the sensors exceeds the threshold, then the sensors are said to follow the OR rule of the collaborative sensing. Under these circumstances, the network detection and false alarm probabilities are denoted by Qd and Qf respectively, and given by

QdOR=1−1−PdNQfOR=1−1−PfN                  (C.3)

Under the OR rule of collaborative sensing, the net detection as well as false alarm probabilities increase.

However, consider another signal c(n), which statistically looks similar to the authentic signal s(n), such that, when transmitted, it may result in false detection of the signal s(n). Assume that c(n) is being transmitted at a lower power than s(n) and hence, will be detected by L sensors where (L<N). Let PC be the probability that c(n) is transmitted (ON) at a particular instant of time. If the OR rule of the collaborative sensing is followed, then the probability of detection of an authentic incumbent signal is give by

QdC,OR=PC+1−PC1−1−PdN=1−1−PC1−PdN           (C.4)

and the false alarm probability = probability that Hypothesis H1 is selected when H0 is true is given similarly by

QfC,OR=1−1−PC1−PfN                     (C.5)

This means that if the other signal is present along with the true signal, it helps detection. For instance, if c(n) is ON all the time (that is PC = 1), then, QdC,OR=QfC,OR=1, which means that a particular channel always appears to be occupied, whether or not an authentic incumbent signal is present. If c(n) is OFF all the times then QdC,OR=1−1−PdN and probability of authentic incumbent detection is the same as in C.3. Note, that the false alarm probability increases with an increasing value of PC.

Now consider a case where instead of the OR rule of collaborative sensing, the AND rule is followed. Under these circumstances, the incumbent signal is said to be detected if and ONLY if all the N localized sensors detect the signal. In absence of c(n), the detection and false alarm probabilities are given by

QdAND=PdNQfAND=PfN            (C.6),

which means that both, the detection as well as the false alarm probabilities decrease with increasing N.

Let PC be the probability that c(n) is transmitted at a particular instant of time and some L<N sensors are able to detect it. If the AND rule of the collaborative sensing is followed, then the probability that an authentic incumbent signal is detected is given by,

QdC,AND=PdN−L              (C.7).

which means that even in presence of a false signal c(n), the probability that an authentic incumbent signal is detected increases, but is not automatic. Similarly, the false alarm probability = probability that Hypothesis H1 is selected when H0 is true, and is given by

QfC,AND=PfN−L             (C.8).

As an example, let Pd = 0.9, let Pf = 0.1, let N = 2, L = 1, and PC = 0.5, then

| Pd= 0.9 | QdOR = 0.99 | QdC,OR = 0.995 | QdAND = 0.81 | QdC,AND = 0.9 |
| --- | --- | --- | --- | --- |
| Pf= 0.1 | QfOR = 0.19 | QfC,OR = 0.595 | QfAND = 0.01 | QfC,AND = 0.1 |

Rather than using simple OR or AND based decision making, it makes sense to use some form of a VOTING based rule for decision making which provides a finer granularity, flexibility and protection against low powered signals c(n) which may have statistical properties that are similar to s(n) – the signal of interest.

Given N collaborative detectors, let k be the number of these radios that detect a transmitter present in a band. To decide when we declare a transmitter present, we define the collaborative sensing voting rule. The rule has one parameter, the voting threshold T, a number between 0 and 1. It can be expressed as a percentage, e.g.  T = 0.5 is a threshold of 50%.

The rule is that we declare the signal s(n) to be present if  d  TN nodes detect its presence.

To evaluate the rule, suppose that all detectors can detect the signal of interest s(n) with probability Pd, independent of each other. Then the probability of k successes from N transmitters is given by the binomial distribution,

B(k,N;Pd)=N!k!(N−k)!Pdk1−PdN−k.      (C.9)

The probability the voting rule declares that the signal of interest s(n) is present if

QDcollaborative=k≥TNNB(k,N;Pd).         (C.10)

The probability of the rule declaring no transmitter present is 1 − QDcollaborative.

Under this formulation, the AND rule that everyone must detect to declare a transmitter present is T = 1. The OR rule that if anyone detects we declare a transmitter present is 0 < T ≤ 1/N.

If a transmitter is actually present, then QD represents the true detection rate and 1 − QD represents the missed detection rate. If a transmitter is not present then QD represents the false positive rate and 1 − QD represents the true negative rate.

Figure C.1 OR Rule of Collaborative Sensing and Signal Detection

Figure C.1 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the OR rule of collaborative sensing with Pd = 0.9 and Pf = 0.1. Simulations assume the sensors are independent. It can be seen that with the OR rule, false positives increase with the number of detectors. Hence the OR rule of collaborative sensing fails to provide robust detection of the signal of interest.

Figure C.2 AND Rule of Collaborative Sensing and Signal Detection

Figure C.2 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the AND rule of collaborative sensing. It can be seen that the AND rule performs much better than the OR rule for false positives, however, the true positive performance suffers as the number of independent observations from the detectors increases. Hence once again the AND rule of collaborative sensing fails to provide robust detection of the signal of interest s(n).

Figure C.3 Voring Based Rule of Collaborative Sensing and Signal Detection where the signal is said to be detected if half the number of sensors detect the signal.

Finally, Figure C.3 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the VOTING based rule of collaborative sensing with voting threshold set to 0.5: that is half the number of independent detectors (N/2) must confirm the presence of the signal. It can be seen that the VOTING based rule performs much better than the OR as well as the AND rules for true positives as well as the false positives. Probability of detection for true and false positives reaches an optimal value with observations from around 6 sensors.

The voting rule can also perform well in the presence of a false signal, c(n), that affects a limited number, L, of detectors. As the number of independent detectors increases, these L detectors will be overruled by the detection from other sensors.

Hence collaborative sensing with information fusion and decision making based on a VOTING rule can help in enhancing the network detection performance of the true signal s(n) in presence of a false signal c(n). Based on the simulations results, information fusion and decision making from approximately 6 to 10 independent sensors is likely to be sufficient to obtain satisfactory performance results.

Correlation of the voting results with the geographical location of the sensing CPEs

Collaborative sensing takes all it strength when the information fusion described above takes into account the knowledge on the geographic location of the specific sensing devices providing sensing reports.  The correlation of the reports of the sensing devices that will have sent an “Urgent Coexistence Situation (UCS)” message to the base station and their respective geographic location which is, by definition, known at the base station from the time of their registration on the network will provide powerful means to identify the characteristics of the sensed signal and help improve the decision making as to the presence of an incumbent and its type versus presence of other devices based on their expected extent of coverage.

In the case of DTV detection, the area where correlation will exist between sensing devices will be large and will be in the direction of the DTV transmitter.  In the case of a wireless microphone, the correlarion will exist over a more limited area.  This limited area correlation will also be used in the case of the detection of an 802.22.1 beacon and will help increasing the level of conficence of the decision that it is indeed a TG1 beacon based on its expected area of coverage, and possibly reducing the need to rely on the full PPDU decoding.

Since all the information will be available at the base station, it will be up to the manufacturers to implement various levels of complexity in fusing the sensing reports and the geographic location of the sensing devices to come up with a reliable assessment of the presence of incumbents, that is true positives, while minimizing the probability of false positives. Artificial intelligence algorithms could be taken advantage of to improve this data fusion and decision process.  Since incumbents will need to be protected in all cases and in case of doubt, action will need to be taken to avoid interference to incumbents, more advanced data fusion and decision processes will have the advantage of avoiding false positives and false alarms and therefore reducing cases where WRAN systems would have to change frequency while it is not necessary.

## Annex F (Informative) CBP Protection Method Variant Overhead Values

The following table shows the total amount of certificate and signature data in the for both CBP Protection Methods.

Table XX-1: Overhead Values from CBP Protection Method Option Variants

| Variant Name | Certificate Public Key Type & Size | Signature Type & Size | Overhead (bits) |
| --- | --- | --- | --- |
| 1 | 256 bit ECC | 8 byte CMAC | 456 |
| 2 | -- | 8 byte CMAC | 128 |

References:

- A. Mody, T. Kiernan, “Meeting Minutes of the Security Ad-Hoc Group in 802.22” https://mentor.ieee.org/802.22/file/08/22-08-0163-07-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Recommended text for Section 7 on Security in 802.22” - https://mentor.ieee.org/802.22/file/08/22-08-0174-07-0000-recommended-text-for-section-7-on-security-in-802-22.doc was presented and discussed

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Table of Contents for Section 7”. https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Scope and the Workplan for the Security Ad-Hoc Group” –  https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, DJ Shyy, “Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis” https://mentor.ieee.org/802.22/file/08/22-08-0083-07-0000-security-and-prm-enhancements-in-80222-v3.ppt

- A. Mody, R. Reddy, M. Sherman, T. Kiernan, DJ Shyy, “Security and Protocol Reference Model Enhancements in 802.22,” https://mentor.ieee.org/802.22/file/08/22-08-0083-04-0000-security-and-prm-enhancements-in-80222-v3.ppt

- IEEE 802.16 Draft Standard for Broadband Wireless Access – P802.16Rev2/D5 (June 2008) – Section 7 – Security Sublayer

- Security Enhancement for 802.16e - A SDD Proposal for 802.16m -www.wirelessman.org/tgm/contrib/C80216m-08_046.doc

- http://www.navigadget.com/index.php/2007/01/29/gps-and-gsm-jammer/

- M. Barbeau, “WiMAX Threat Analysis”, Proceedings of the ACM, Q2SWinet’05, October 13, 2005, Montreal, Quebec, Canada.

- S. Xu,  M. Matthews, “Security Issues in Privacy and Key Management Protocols of 802.16,” Proceedings of the ACM SE’06, March 10-12, 2006, Melbourne, Florida, USA

- D. Johnston and J. Walker, “Overview of IEEE 802.16 Security,” IEEE Security and Privacy, Magazine Published by the IEEE Computer Society, 2004

- Y. Xiao, X. Shen and D. Du, Wireless Network Security, Springer Series on Signals and Communications Technology, 2006

- Qusay H. Mahmoud, Cognitive Networks, Towards Self Aware Networks, Wiley, Sept. 2007 – Chapter 11, Security Issues in Cognitive Radio Networks

- Amita Sethi, Potential Denial of Service Threat Assessment for Cognitive Radios, MS Thesis, University of Colorado at Boulder, 2008.

The RSSI measurements will generate more useful information than a simple signal detection and classification.  When sufficiently high-level signals are present, the signal classification schemes developed for low signal levels may be replaced by simpler, faster and more effective signal classification schemes.  Such fast incumbent signal classification schemes will be dependent on implementation by the manufacturers. Even though 3rd order intermodulation is not to be considered by the sensing process, documenting the levels of the high power signals received at the CPE would be useful for the base station to enquire the incumbent database about possible channels to be excluded to avoid 3rd order intermodulation.  In such case, the RSSI measurement using the omnidirectional sensing antenna should be done on all channels of the TV band to cover for all possible intermodulation beat patterns.  The actual high signal levels present at the TV receiver would be very similar to those measured at the CPE because the worst case intermodulation will occur when the CPE is located close to the TV receiver (limit case considered is 10 m separation distance.

The CPE should be calibrated by the manufacturer so that the RSSI measured on the WRAN signal path is normalized to compensate for the gain difference between the maximum WRAN receive antenna gain (on-axis) and the gain of the omni-directional sensing antenna, that is when a signal is received on-axis on the WRAN signal path, it appears to have the same level as the signal received by the sensing RF path.  Such calibration should be done at the same time as the 4 Watt EIRP calibration for the CPE so that the TPC process actually controls the EIRP of the CPE and not only the power going to the antenna.

It is assumed that the quiet periods of the local WRAN systems operating on channels N0, N0+/-1 and N0+/-2 will all be synchronized since the sensing on N0+/-1 could be impacted by WRAN transmissions operating on channels adjacent to the channels being sensed because of possible large signal differential and out-of-band emission from WRAN transmission in adjacent channels.  In the case where the WRAN transmission on N0+/-2are weak because of their distance to the CPE, the quiet periods would not need to be synchronized beause the out-of-band emission would then be negligible.

A mechanism has to be provided to prevent a CPE from trying to re-associate continuously if the BS refuses to re-associate with it.  A positive refusal should be sent from the base station or a limited number of attempts should be specified if the BS can not be reached.

The sensing information on the affected TV channels may have become obsolete with the appearance of a wireless microphone within 2 seconds.

The base station will need to send the information about the 2 second maximum lapse time for sensing the operating channel and the 6 second for the backup channels during the CPE association process since these values may be different in different administrations.