IEEE P802.22
Wireless RANs

| Recommended Text for Security in 802.22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-05-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |
| Tom Kiernan | US Army (CERDEC) | Ft Monmouth, NJ | - | Thomas.kiernan@us.army.mil |

Abstract

This document provides the proposed text for a complete revision of Section 7 on Security in 802.22. Based on the proposed changes of Section 7, other sections of the 802.22 draft standard require modifications. These other sections, along with their proposed changes have been included or referred to for completeness. This document also introduces a new annex on some theoretical results related to collaborative sensing and decision making to provide potection against spurious signals.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# I. Modifcations to Clause 2

[------------------------------------------------Start of Text Modification--------------------------------------------]

2. Normative References

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments or corrigenda) applies.

FIPS 180-1, Secure Hash Standard (SHS), April 1995.

IEEE 100™, The Authoritative Dictionary of IEEE Standards Terms, Seventh Edition.

IEEE 802.22 Working Group on Wireless Regional Area Networks, http://www.ieee802.org/22/.

IEEE 802.22 Working Group on Wireless Regional Area Networks, ―IEEE 802.22 Functional

Requirements Document, September 2005, doc.: 22-05-0007-47-0000_RAN_Requirements.doc

IEEE 802.22 Working Group on Wireless Regional Area Networks, ―WRAN Reference Model.doc.: IEEE 802.22-04/0002r15.

IETF RFC 3280, Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile, April 2002.

NIST Special Publication 800-38B - Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication, May 2005.

NIST Special Publication 800-38C - Recommendation for Block Cipher Modes of Operation: The CCM Mode for Authentication and Confidentiality, May 2004.

NIST Special Publication 800-38A -Recommendation for Block Cipher Modes of Operation: Methods and Techniques.

PKCS #1: RSA Cryptography Standard, June 2002.

FIPS 197, Advanced Encryption Standard, November 2001.

AES Key Wrap Specification, November 2001, http://csrc.nist.gov/groups/ST/toolkit/documents/kms/AES_Key_Wrap.pdf

IETF RFC 3279, Algorithms and Identifiers for the Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile, April 2002.

IETF RFC 4493, The AES-CMAC Algorithm, June 2006.

FIPS 186-2, Digital Signature Standard (DSS), January 2000.

SEC1, Standards for Efficient Cryptography – SEC1: Elliptic Curve Cryptography, September 2000.

IEEE Std. 802-2001TM, IEEE Standard for Local and Metropolitan Area Networks: Overview and Architecture, March 2002.

IEEE Std.802.15.4-2006TM, IEEE Standard for Information Technology – Telecommunications and information exchange between systems – Local and metropolitan area networks – Specific requirements – Part 15.4: Wireless Medium Access Control (MAC) and Physical Layer (PHY) Specfication for Low-Rate Wireless Personal Area Networks (WPANs), September 2006.

ANSI X9.63-2001, Public Key Cryptography for the Financial Services Industry: Key Agreement and Key Transport Using Elliptic Curve Cryptography, November 2001.

ANSI X9.62-2005, Public Key Crytopgraphy for the Financial Services Industry: The Elliptic Curve Digital Signature Algorithm (ECDSA), November 2005.

IEEE Std. 802.16-2009, IEE Standard for Local and Metropolitan Area Networks – Part 16: Air Interface for Broadband Wireless Access Systems, 2009.

# II. Modifications to Clause 6

[------------------------------------------------Start of Text Modification--------------------------------------------]

6. MAC Common Part Sublayer

[add the following IEs to Table 6]

| XX1 | Signature IE |
| --- | --- |
| XX2 | CERT-REQ IE |
| XX3 | CERT-RSP IE |
| XX4 | BS Implicit Certificate IE |

[add the definition of the following IEs to 6.7.1.2.1]

6.7.1.2.1.A CBP Protection IEs

The CBP protection method (Section 7.6.8.1) is an optional security procedure that can be used to provide authentication of CBP MAC PDU transmissions. If it is enabled then three new IEs are required to be defined; CBP Signature IE, Certificate Request (CERT-REQ) IE, and Certifcate Response (CERT-RSP) IE. The Signature IE is used to provide a signature that is calculated over the CBP MAC PDU, and is verified by the receiving BS. If verification fails, then the CBP MAC PDU is not processed. Signature verification failure can also occur if a BS receives a CBP from a BS that is run by a different operator and each operator signs its own certificates. This is a problem, and will affect coexistence performance. The suggestion then, is that a BS periodically broadcast its own certificate and/or make a request for a neighbor BS’s (that belongs to a different operator) ECC implicit certificate using a dedicated IE (e.g. using the BS Implicit Certificate IE). When the CBP Protection is enabled, the Signature shall be transmitted in every CBP MAC PDU.

[add a new subsection 6.7.1.2.1.A.1 Signature IE]

6.7.1.2.1.A.1 Signature IE

Table 11 – Signature IE

| Syntax | Size | Description |
| --- | --- | --- |
| Signature_IE_Format { |  |  |
| Element ID | 8 bits | 0x04 |
| Key ID | 10 bits | Serial # of key associated with BS implicit certificate. This is generated by the CA. |
| Time Stamp | 54 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - ss: seconds, 00-59, 6 bits - .ss: 10 ms boundary, .000-.99, 7 bits - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Signature | 64 bit | CMAC output is 128 bits (16bytes) signature is a truncation of this value to no less than 8 bytes |
| } |  |  |

[add new subsection 6.7.1.2.1.A.2 Certificate Request IE]

6.7.1.2.1.A.2 Certificate Request (CERT-REQ) IE

Table 12 – Certificate Request (CERT-REQ) IE

| Syntax | Size | Description |
| --- | --- | --- |
| CERT-REQ_IE_Format { |  |  |
| Element ID | 8 bits | 0x05 |
| Destination BS ID | 48 bits | ID of BS that request is being directed to. |
| CA ID | 8 bits | ID of Certificate Authority that issued certificate to BS that is initiating the certificate request. |
| Key Validity Date (Not Before) | 41 bits | Date that signifies start of period in which certificate of BS that is making request, is valid. Derived from ZDA NMEA 0183 string (each letter represents a digit, encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Time Period | 7 bits | Amount of time, in 6 month increments, that certificate is valid for. |
| Public Key Reconstruction Data | 264 bits | Key data used to reconstruct public key. 33 bytes for 256 bit ECC keys |
| } |  |  |

[add new subsection 6.7.1.2.1.A.3 Certificate Response IE]

6.7.1.2.1.A.3 Certificate Response (CERT-RSP) IE

Table 13 – Certificate Response (CERT-RSP) IE

| Syntax | Size | Description |
| --- | --- | --- |
| CERT-RSP_IE_Format { |  |  |
| Element ID | 8 bits | 0x06 |
| Source BS ID | 48 bits | ID of BS that Certificate Response is being directed towards |
| CA ID | 8 bits | ID of Certificate Authority that issued certificate to BS that is transmitting the certificate response. |
| Key Validity Date (Not Before) | 41 bits | Date that signifies start of period in which certificate of BS, that is transmitting the certificate response, is valid. Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Time Period | 7 bits | Amount of time, in 6 month increments, that certificate is valid for. |
| Public Key Reconstruction Data | 264 bits | Key data used to reconstruct public key. 33 bytes for 256 bit ECC keys |
| Time Stamp | 54 bits | Copied from Signature IE of CBP MAC PDU in which the CERT-REQ IE was received in. |
| Reserved | 10 bits | Set to 1111111111 |
| } |  |  |

[remove section 6.8.1]

HMAC definition

— HMAC value field

[modify Table 43, pg 39, starting on line 27 as follows]

Table xx – Management Messages

| Type | Message | Description | Connection |
| --- | --- | --- | --- |
| 0 | DCD | Downstream Channel Descriptor | Broadcast |
| 1 | DS-MAP | Downstream Access Definition | Broadcast |
| 2 | UCD | Upstream Channel Descriptor | Broadcast |
| 3 | US-MAP | Upstream Access Definition | Broadcast |
| 4 | RNG-REQ | Ranging Request | Initial Ranging or Basic |
| 5 | RNG-RSP | Ranging Response | Initial Ranging or Basic |
| 6 | REG-REQ | Registration Request | Primary Management |
| 7 | REG-RSP | Registration Response | Primary Management |
| 8 |  | Reserved |  |
| 9 | PKM-REQ | Privacy Key Management Request | Primary Management |
| 10 | PKM-RSP | Privacy Key Management Response | Primary Management |
| 11 | DSA-REQ | Dynamic Service Addition Request | Primary Management |
| 12 | DSA-RSP | Dynamic Service Addition Response | Primary Management |
| 13 | DSA-ACK | Dynamic Service Addition Acknowledge | Primary Management |
| 14 | DSC-REQ | Dynamic Service Change Request | Primary Management |
| 15 | DSC-RSP | Dynamic Service Change Response | Primary Management |
| 16 | DSC-ACK | Dynamic Service Change Acknowledge | Primary Management |
| 17 | DSD-REQ | Dynamic Service Deletion Request | Primary Management |
| 18 | DSD-RSP | Dynamic Service Deletion Response | Primary Management |
| 19 |  | Reserved |  |
| 20 |  | Reserved |  |
| 21 | MCA-REQ | Multicast Assignment Request | Primary Management |
| 22 | MCA-RSP | Multicast Assignment Response | Primary Management |
| 23 | DBPC-REQ | Downstream Burst Profile Change Request | Primary Management |
| 24 | DBPC-RSP | Downstream Burst Profile Change Response | Primary Management |
| 25 | RST-CMD | Reset Command | Primary Management |
| 26 | CBC-REQ | CPE Basic Capability Request | Primary Management |
| 27 | CBC-RSP | CPE Basic Capability Response | Basic |
| 28 |  | Reserved |  |
| 29 | DREG-CMD | De/Re-register Command | Basic |
| 30 | DSx-RVD | DSx Receive Message | Primary Management |
| 31 | TFTP-CPLT | Config File TFTP Complete Message | Primary Management |
| 32 | TFTP-RSP | Config File TFTP Complete Response | Primary Management |
| 33 | ARQ-Feedback | Standalone ARQ Feedback | Primary Management |
| 34 | ARQ-Discard | ARQ Discard Message | Primary Management |
| 35 | ARQ-Reset | ARQ Reset Message | Primary Management |
| 36 | CPE-FPC | CPE Fast Power Control | Broadcast, Multicast Management |
| 37 | DREG-REQ | CPE De-registration Message | Primary Management |
| 38 |  | Reserved |  |
| 39 | BLM-REQ | Bulk Measurement Request | Primary Management, Multicast Management, or Broadcast |
| 40 | BLM-RSP | Bulk Measurement Response | Primary Management |
| 41 | BLM-REP | Bulk Measurement Report | Primary Management |
| 42 | BLM-ACK | Bulk Measurement Acknowledgement | Primary Management |
| 43 | CHT-REQ | Channel Terminate Request | Primary Management, Multicast Management or Broadcast |
| 44 | CHT-RSP | Channel Terminate Response | Basic |
| 45 | CHA-REQ | Channel Add Request | Primary Management, Multicast Management, or Broadcast |
| 46 | CHA-RSP | Channel Add Response | Primary Management |
| 47 | CHS-REQ | Channel Switch Request | Primary Management, Multicast Management or Broadcast |
| 48 | CHS-RSP | Channel Switch Response | Primary Management |
| 49 | CHQ-REQ | Channel Quiet Request | Primary Management, Multicast Management or Broadcast |
| 50 | CHQ-RSP | Channel Quiet Response | Primary Management |
| 51 | CHO-UPD | Channel Occupancy Update | Primary Management, Multicast Management or Broadcast |
| 52 | TRC-REQ | Traffic Constraint Request | Primary Management |
| 53 | TRC-RSP | Traffic Constraint Report | Primary Management |
| 54 | TMO-REQ | Timeout Request | Primary Management |
| 55 | TMO-RSP | Timeout Response | Primary Management |
| 56 | FSL-REQ | Frame Slide Request | Primary Management, Multicast Management or Broadcast |
| 57 | FSL-RSP | Frame Slide Response | Primary Management |
| 58 | AAS-CFB-REQ | AAS Channel Feedback Request | Primary Management |
| 59 | AAS-CFG-RSP | AAS Channel Feedback Response | Primary Management |

[remove Section 6.9.7.3.7.5 “PKM Flow Control” (including Table 85), Section 6.9.7.3.7.6 “Authorization Policy Supported” (including Table 86), and Section 6.9.7.3.7.7 “Maximum Number of Supported Security Association” (including Table 87)]

[Editor’s Note: These sections are to be removed because it’s better to assume some default values for initialization, and then let the BS adjust them as necessary during Authorization exchange. This also has some benefit, if minimal, of reducing the size of the REG-REQ/RSP messages”]

- [make the following modifications to Section 6.9.28]

- 6.9.28   Security Control Management (SCM) Messages (SCM-REQ/RSP)

SCM employs two MAC message types: SCM Request (SCM-REQ) and SCM Response (SCM-RSP), as described in .

Table 208— SCM MAC messages

| Type Value | Message name | Message description |
| --- | --- | --- |
| 9 | SCM-REQ | Security Control Management Request [CPE -> BS] |
| 10 | SCM-RSP | Security Control Management Response [BS -> CPE] |

These MAC management message types distinguish between SCM requests (CPE–to–BS) and SCM responses (BS–to–CPE). Each message encapsulates one SCM message in the Management Message Payload.

SCM protocol messages transmitted from the CPE to the BS shall use the form shown in . They are transmitted on the CPEs Primary Management Connection.

Table 209 — SCM-REQ message format

| Syntax | Size | Notes |
| --- | --- | --- |
| SCM-REQ_Message_Format() { |  |  |
| Management Message Type = 9 | 8 bits |  |
| Element ID | 8 bits | Identifies the type of SCM packet. When a packet is received with an invalid Code, it shall be silently discarded. The code values are defined in . |
| SCM Identifier | 8 bits | A CPE uses the identifier to match a BS response to the CPE’s requests. The CPE shall increment (modulo 256) the Identifier field whenever it issues a new SCM message. A “new” message is an Authorization Request or Key Request that is not a retransmission being sent in response to a Timeout event. For retransmissions, the Identifier field shall remain unchanged. The Identifier field in Authentication Information messages, which are informative and do not effect any response messaging, shall be set to zero. The Identifier field in a BS’s SCM-RSP message shall match the Identifier field of the SCM-REQ message the BS is responding to. The Identifier field in TEK Invalid messages, which are not sent in response to SCM-REQs, shall be set to zero. The Identifier field in unsolicited Authorization Invalid messages shall be set to zero. On reception of a SCM-RSP message, the CPE associates the message with a particular state machine (the Authorization state machine in the case of Authorization Replies, Authorization Rejects, and Authorization Invalids; a particular TEK state machine in the case of Key Replies, Key Rejects, and TEK Invalids). A CPE shall keep track of the identifier of its latest, pending Authorization Request. The CPE shall discard Authorization Reply and Authorization Reject messages with Identifier fields not matching that of the pending Authorization Request. A CPE shall keep track of the identifiers of its latest, pending Key Request for each SA. The CPE shall discard Key Reply and Key Reject messages with Identifier fields not matching those of the pending Key Request messages. |
| Encoded Attributes | variable | SCM attributes carry the specific authentication, authorization, and key management data exchanged between client and server. Each SCM packet type has its own set of required and optional attributes. Unless explicitly stated, there are no requirements on the ordering of attributes within a SCM message. The end of the list of attributes is indicated by the Length field of the MAC PDU header. |
| } |  |  |

SCM protocol messages transmitted from the BS to the CPE shall use the form shown in . They are transmitted on the CPEs Primary Management Connection.

Table 210— SCM-RSP message format

| Syntax | Size | Notes |
| --- | --- | --- |
| SCM- RSP_Message_Format() { |  |  |
| Management Message Type = 10 | 8 bits |  |
| Element ID | 8 bits |  |
| SCM Identifier | 8 bits |  |
| Encoded Attributes | variable |  |
| } |  |  |

Table 211— SCM message codes

| Element ID | SCM message type | MAC Management Message Name |
| --- | --- | --- |
| XX | SCM Auth-Request | SCM-REQ |
| XX | SCM Auth-Reply | SCM-RSP |
| XX | SCM Auth-Reject | SCM-RSP |
| XX | SCM Auth-Acknowledgement | SCM-REQ |
| XX | SCM Key-Request | SCM-REQ |
| XX | SCM Key-Reply | SCM-RSP |
| XX | SCM Key-Reject | SCM-RSP |
| XX | SCM GSA-Add | SCM-RSP |
| XX | SCM TEK-Invalid | SCM-RSP |
| XX | PKM Auth Invalid | SCM-RSP |

Formats for each of the SCM messages are described in the following subclauses.

[modify Section 6.9.28.1 as follows]

      - SCM Auth-REQ

A client CPE sends a SCM RSA-Request message to the BS in order to request mutual authentication in the RSA-based authorization.

Table 212— SCM Auth-Request attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64-bit random number generated in the CPE |
| CPE_Certificate | Contains the CPE s X.509 user certificate |
| SAID | CPE s primary SAID equal to the Basic CID |
| Cryptographic Suite List | List of cryptopgrahic suites (Table 7.4-1) that the CPE supports |
| SigCPE | An RSA or ECC signature over all the other attributes in the message |

The CPE-certificate attribute contains an X.509 CPE certificate (see 7.5) issued by the CPE s manufacturer.

The SigCPE indicates an RSA or ECC signature over all the other attributes in this message, and the CPE’s private key is used to make an RSA/ECC signature.

[modify Section 6.9.28.2 as follows]

      - SCM Auth-Reply

Sent by the BS to a client CPE in response to a SCM Auth-Request message, the SCM Auth-Reply message contains an encrypted pre-PAK, the key’s lifetime, and the key’s sequence number. The pre-PAK shall be encrypted with the CPEs public key. The CPE_Random number is returned from the SCM Auth-Request message, along with a random number supplied by the BS, thus enabling assurance of key liveness.

The SCM Configuration Settings Indicator denotes the presence of new values of parameters for Authorization State Machine (Section 7.2.3.4.4) and the TEK State Machine (Section 7.2.4.2.4), as awell as parameters to adjust the behavior of the SCM protocol. The bit order determines the order these variables as they fall in the SCM Configuration Settings Field:

- Bit #0: Auth Wait Timeout

- Bit #1: Auth Grace Timeout

- Bit #2: Auth Reject Wait Timeout

- Bit #3: Operation Wait Timeout

- Bit #4: Rekey Wait Timeout

- Bit #5: GTEK/TEK Grace Time

- Bit #6: SCM Flow Control

- Bit #7: Number of Supported Security Associations

The SCM Configuration Settings Field is a variable length field, whose length is determined by which parameters the BS has selected new values for witht the SCM Configuration Settings Indicator field.

Table 213— SCM Auth-Reply attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64-bit random number generated in the CPE, received in the SCM Auth-Request message. |
| BS_Random | A 64-bit random number generated in the BS |
| Encrypted pre-PAK | RSA-OAEP-Encrypt(PubKey(CPE), pre-PAK ¦ CPE MAC Address) or ECIES(PubKey(CPE), pre-PAK, CPE MAC Address) |
| PAK Lifetime | Lifetime of PAK |
| PAK Sequence Number | PAK sequence number |
| BS_Certificate | Contains the BS’s X.509 certificate |
| CPE SA List | List of SAs that CPE is authorized for each SA, the SAID and Cryptographic suite configured for the SA is provided. |
| SCM Configuration Settings Indicator | Indicates existence of new settings for SCM Parameters, 1 byte long. Bit order represents order that new configuration values are presented in. |
| SCM Configuration Settings Field | Variable |
| SigBS | An RSA or ECC signature over all the other attributes in the message |

If the “SCM Flow Control” or “Number of Supported Security Associations” are to be added to the SCM Configuration Settings Field, they shall be 1 octet long.

The SigBS indicates an RSA or ECC signature over all the other attributes in this message, and the BS’s private key is used to make an RSA or ECC signature.

[modify Seciton 6.9.28.3 as follows]

      - SCM Auth-Reject

The BS responds to a CPE’s authorization request with an SCM Auth-Reject message if the BS rejects the CPE’s authorization request.

Table 214— SCM Auth-Reject attributes

| Attribute | Contents |
| --- | --- |
| CPE_Random | A 64 bit random number generated in the CPE, same value as receive in Auth Request |
| BS_Random | A 64 bit random number generated in the BS |
| Message Response Code | Error code identifying reason for rejection of authorization request. Possible code values are listed in Section 6.9.28.10. |
| BS_Certificate | Contains the BS s X.509 certificate |
| SigBS | An RSA or ECCsignature over all the other attributes in the message |

The Error-Code and Display-String attributes describe to the requesting CPE the reason for the RSA/ECC-based authorization failure.

The SigBS indicates an RSA or ECC signature over all the other attributes in this message, and the BS’s private key is used to make an RSA signature.

[remove Section 6.9.28.4 through 6.9.28.10]

-Acknowledgement attributes

The SigCPE indicates an RSA signature over all the other attributes in this message, and the CPE’s private key is used to make an RSA signature.

— PKM EAP Start attributes

      - PKM EAP Transfer

      - — PKM EAP Transfer attributes

The EAP Payload field carries data in the format described in section 4 of RFC 3748.

      - PKM Authenticated EAP Transfer

      - — PKM Authenticated EAP Transfer attributes

The EAP Payload field carries EAP data in the format described in RFC 3748

The CMAC-Digest s or HMAC Digest s attribute shall be the final attribute in the message s attribute list.

Inclusion of the CMAC or HMAC Digest allows the CPE and BS to cryptographically bind previous authorization and following EAP authentication by authenticating the EAP payload. The key for the CMAC Value or HMAC Digest is derived from the EIK.

PAK Sequence Number attribute carries PAK sequence number only if CPE and BS negotiate Authenticated EAP after RSA mode.

— PKM SA-TEK-Challenge attributes

The HMAC Digest attribute or the CMAC-Digest attribute shall be the final attribute in the message’s attribute list.

Inclusion of the HMAC Digest or the CMAC-Digest allows the CPE and BS to authenticate a PKM SA-TEK-Challenge message. The HMAC or the CMAC authentication keys are derived from the AK.

      - PKM SA-TEK-Request

The CPE transmits the PKM SA-TEK-Request message after receipt and successful HMAC Digest or CMAC value verification of an SA-Challenge tuple or PKM SA-TEK-Challenge message from the BS. The PKM SA-TEK-Request proves liveliness of the CPE and its possession of the AK to the BS. If this message is being generated during initial network entry, then it constitutes a request for SA-Descriptors identifying the primary and static SAs and GSAs the requesting CPE is authorized to access and their particular properties (e.g., type, cryptographic suite).

If this message is being generated following HO, then it constitutes a request for establishment (in the target BS) of TEKs, GTEKs and GKEKs for the CPE and renewal of active primary, static and dynamic SAs and associated SAIDs used by the CPE in its previous serving BS.

— PKM SA-TEK-Request attributes

Receipt of a new BS Random value in SA-TEK-Challenge or SA-Challenge tuple indicates the beginning of a new handshake.

      - PKM SA-TEK-Response

The BS transmits the PKM SA-TEK-Response message as a final step in the 3-way SA-TEK handshake.

— PKM SA-TEK-Response attributes

- [renumber Section 6.9.28.11 to 6.9.28.4, and modify as follows]

      - SCM Key-Request

A CPE sends a SCM Key-Request message to the BS to request new TEK and TEK-related parameters or GTEK and GTEK-related parameters for the multicast or broadcast service.

Table 222 — SCM Key-Request attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier - GSAID for multicast or broadcast service |
| Group Key Indicator | 1 bit flag to indicate if this Key-Request message is for a GKEK or GTEK. Only applicable if SAID refers to a GSA. |
| CPE_Random | A 64 bit random number generated in the CPE |

Once a CPE has completed authorization, it has keying material (MMP_KEY) that can be used to sign and/or encrypt further MAC management messages. If SCM Key-Request is only to be signed, then CPE will use MMP_KEY derived from the AK identified by AK Sequence Number in Key-Request will be used to generate the Ciphertext ICV (see 7.4.2.1.2). If SCM Key-Request is to be encrypted, then CPE will use the MMP_KEY derived from the most current of it’s AKs to generate the Ciphertext ICV and encrypt the message (see 7.4.2.1.3).

[renumber Section 6.9.28.12 to 6.9.28.5, and modify as follows]

      - SCM Key-Reply

The BS responds to a CPE’s SCM Key-Request message with a SCM Key-Reply message.

Table 223 — SCM Key-Reply attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier - GSAID for multicast or broadcast service. |
| Older TEK/GTEK-Parameters | Older generation of TEK parameters relevant to SAID, GTEK-Parameters for the multicast or broadcast service of GSAID |
| Newer TEK/GTEK-Parameters | Newer generation of key parameters relevant to SAID or GSAID |
| GKEK-Parameters | GKEK-related parameters for multicast or broadcast service. |
| CPE Random | A same random number included in the SCM Key Request message |

The GKEK-Parameters attribute is a compound attribute containing all of the GKEK-related parameters corresponding to a GSAID. This would include the GKEK, the GKEK’s remaining key lifetime, and the GKEK’s key sequence number.

Once a BS has completed authorization with a particular CPE, both have keying material (MMP_KEY) that can be used to sign and/or encrypt further MAC management messages. If SCM Key-Reply is only to be signed, then BS will use MMP_KEY derived from the AK identified by AK Sequence Number in Key-Reply will be used to generate the Ciphertext ICV (see 7.4.2.1.2). If SCM Key-Reply is to be encrypted, then BS will use the MMP_KEY derived from the most current of it’s AKs to generate the Ciphertext ICV and encrypt the message (see 7.4.2.1.3).

GKEK parameters shall only be included in Key-Reply message, when responding to Key-Request that is conducted immediately after completion of Authorization exchange. To update GKEK parameters for an existing GSA, the BS shall add new GKEK parameters to the list of SA descriptors in an SA Add message.

[renumber Section  6.9.28.13 to 6.9.28.6, and modify as follows]

      - SCM Key-Reject

The BS responds to a CPE’s SCM Key-Request message with a SCM Key-Reject message if the BS rejects the CPE’s traffic keying material request.

Table 224 — SCM Key-Reject attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security association identifier |
| Message Response Code | Error code identifying reason for rejection of the SCM Key-Request message. Possible values are listed in Section 6.9.28.10 |
| CPE Random | A same random number included in the SCM Key Request message |

Once a BS has completed authorization with a particular CPE, both have keying material (MMP_KEY) that can be used to sign and/or encrypt further MAC management messages. If SCM Key-Reject is only to be signed, then BS will use MMP_KEY derived from the AK identified by AK Sequence Number in Key-Reject will be used to generate the Ciphertext ICV (see 7.4.2.1.2). If SCM Key-Reject is to be encrypted, then BS will use the MMP_KEY derived from the most current of it’s AKs to generate the Ciphertext ICV and encrypt the message (see 7.4.2.1.3).

[renumber Section 6.9.28.14 to 6.9.28.7 and modify as follows]

      - SCM SA-Addition

This message is sent by the BS to the CPE to establish one or more additional SAs, after completion of the Authorization exchange. BS shall use this method to update material (e.g. GKEKs and associated parameters) for exisiting GSAs

Table 225 — SCM SA-Add attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| (one or more) SA-Descriptor(s) | Each compound SA-Descriptor attribute specifies an SA identifier (SAID) and additional properties of the SA |

Once a BS has completed authorization with a particular CPE, both have keying material (MMP_KEY) that can be used to sign and/or encrypt further MAC management messages. If SCM SA-Add is only to be signed, then BS will use MMP_KEY derived from the AK identified by AK Sequence Number in SA-Add will be used to generate the Ciphertext ICV (see 7.4.2.1.2). If SCM SA-Add is to be encrypted, then BS will use the MMP_KEY derived from the most current of it’s AKs to generate the Ciphertext ICV and encrypt the message (see 7.4.2.1.3).

[renumber Section 6.9.28.15 to 6.9.28.8, and modify as follows]

      - SCM TEK-Invalid

The BS sends a SCM TEK-Invalid message to a client CPE if the BS determines that the CPE encrypted an upstream PDU with an invalid TEK (i.e., an SAID’s TEK key sequence number), contained within the received packet’s MAC header, is out of the BS’s range of known, valid sequence numbers for that SAID.

Table 226 — SCM TEK-Invalid attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| SAID | Security Association Identifier |
| Message Response Code | Error code identifying reason for SCM TEK-Invalid message. Possible values are list in Section 6.9.28.10. |

Once a BS has completed authorization with a particular CPE, both have keying material (MMP_KEY) that can be used to sign and/or encrypt further MAC management messages. If SCM TEK-Invalid is only to be signed, then BS will use MMP_KEY derived from the AK identified by AK Sequence Number in TEK-Invalid will be used to generate the Ciphertext ICV (see 7.4.2.1.2). If SCM TEK-Invalid is to be encrypted, then BS will use the MMP_KEY derived from the most current of it’s AKs to generate the Ciphertext ICV and encrypt the message (see 7.4.2.1.3).

[remove Sections 6.9.28.16, 6.9.28.17, and 6.9.28.18]

— PKM Group-Key-Update-Command attributes

Key Sequence Number is the sequence number of the synchronized AK (Authorization Key) between a CPE and a BS.

GSAID is SAID for the multicast group or the broadcast group. The type and length of the GSAID is equal to ones of the SAID.

There are two types in a PKM Group Key Update Command message, GKEK update mode and GTEK update mode. The former is used to update GKEK and the latter is used to update GTEK for the multicast service or the broadcast service. Key Push Modes indicates the usage code of a PKM Group Key Update Command message. The PKM Group Key Update Command message for the GKEK update mode is carried on the Primary Management connection, but one for the GTEK update mode is carried on the Broadcast connection. A few attributes in a PKM Group Key Update Command message shall not be used according this Key Push Modes attribute’s value.

Key Push Counter is used to protect for replay attack. This value is one greater than that of older generation.

A PKM Group Key Update Command message contains only newer generation of key parameters, because this message inform a CPE next traffic key material. The GTEK-Parameters attribute is a com-pound attribute containing all of the keying material corresponding to a newer generation of a GSAID’s GTEK. This would include the GTEK, the GTEK’s remaining key lifetime, the GTEK’s key sequence number, the associated GKEK sequence number, and the cipher block chaining (CBC) initialization vector. The GTEK is TEK for the multicast group or the broadcast group. The type and length of the GTEK is equal to ones of the TEK. The GKEK (Group Key Encryption Key) can be randomly generated from a BS or a certain network node (i.e., an ASA server). The GKEK should be identically shared within the same multicast group or the broadcast group. The GTEK is encrypted with GKEK for the multicast service or the broadcast service. GKEK parameters contain the GKEK encrypted by the KEK, GKEK sequence number, and GKEK lifetime.

The HMAC/CMAC Digest attribute shall be the final attribute in the message’s attribute list. Inclusion of the keyed digest allows the receiving client to authenticate the Group Key Update Command message. The HMAC/CMAC Digest’s authentication key is derived from the AK for the GKEK update mode and GKEK for the GTEK update mode.

— PKM EAP Complete attributes

      - PKM Authenticated EAP Start

In double EAP mode (EAP after EAP), CPE sends the PKM EAP Authenticated EAP Start message to BS in order to initiate 2nd round EAP. This message is signed by EIK which is generated by 1st EAP.

This message is used only for initial authentication of double EAP.

Code: 20

— PKM Authenticated EAP Start atttributes

- [renumber Section 6.9.28.19 to 6.9.28.9, and modify as follows]

      - SCM Authorization Invalid

The BS may send an Authorization Invalid message to a client CPE as:

- An unsolicited indication, or

- A response to a message received from that CPE.

In either case, the Authorization Invalid message instructs the receiving CPE to reauthorize with its BS.

The BS sends an Authorization Invalid in response to a Key Request if (1) the BS does not recognize the CPE as being authorized (i.e., no valid AK associated with the requesting CPE) or (2) verification of the Key Request’s keyed message digest (in HMAC-Digest attribute) failed, indicating a loss of AK synchronization between CPE and BS.

Code: 21

Table 230 — Authorization Invalid attributes

| Attribute | Contents |
| --- | --- |
| Error-Code | Error code identifying reason for Authorization Invalid. |

— Auth Info attributes

The CA-certificate attribute contains an X.509 CA certificate for the CA that issued the CPE’s X.509 user certificate. The external CA issues these CA certificates to CPE manufacturers.

[insert Section 6.9.28.10 Message Response Code]

      - Message Response Code

The Message response code values are list in Table 6.9.28.10-1. The Messages column in Table 6.9.28.10-1 indicates which SCM messages that this code is applicable to.

Table 6.9.28.10-1 – Message Response Code

| Message Response Code | Messages | Description |
| --- | --- | --- |
| 0x00 | All | No Information |
| 0x01 | Auth Reject, Auth Invalid | Unauthorized CPE |
| 0x02 | Auth Reject, Key Reject | Unauthorized SAID |
| 0x03 | Auth Invalid | Unsolicited |
| 0x04 | Auth Invalid, TEK Invalid | Invalid Key Sequence Number |
| 0x05 | Auth Invalid | Message (Key Request) authentication failure |
| 0x06 | Auth Reject | Permanent Authorization failure |
| 0x07 | Reserved | -- |

# [--------------------------------------------End of Text Modification-------------------------------------------------]

# III. Modifications to Clause 7

[--------------------------------------------Start of Text Modification------------------------------------------------]

[replace the text  for Clause 7 with the following]

7. Security Mechanisms in 802.22

Traditional broadband communications systems such as 802.16 contain data, control and management functions which require protection. However, due to the unique characteristics of the 802.22 systems which include cognitive radio capability as well as long range, enhanced security mechanisms are needed. These security features provide protection for the 802.22 users, service providers and most importantly, the incumbents, who are the primary users of the spectrum. As a result, the protection mechanisms in 802.22 are divided into several security sublayers which target non-cognitive as well as cognitive functionality of the system and the interactions between the two.

The Security Sublayer 1 provides subscribers with privacy, authentication, or confidentiality (In security parlance, confidentiality = privacy + authentication) across the broadband wireless network. It does this by applying cryptographic transforms to MAC PDUs carried across connections between CPE and BS. In addition, these security sublayers provide operators with strong protection from theft of service.

The security sublayers employ an authenticated client/server key management protocol in which the BS, the server, controls distribution of keying material to client CPE. Additionally, the basic security mechanisms are strengthened by adding digital-certificate-based CPE device-authorization to the key management protocol. If during capabilities negotiation, the CPE specifies that it does not support IEEE 802.22 security, step of authorization and key exchange shall be skipped. The BS, if provisioned so, shall consider the CPE authenticated and authorize the CPE to access the network; otherwise, the CPE shall not be serviced and network entry of the CPE shall be de-authorized and neither key exchange nor data encryption shall be performed. The procedures defined for Security Sublayer 1 are based on the Privacy Key Management version 2 (PKMv2) protocol defined for IEEE 802.16 [IEEE 802.16-2009].

In cognitive radio systems, confidentiality and privacy mechanisms need to protect not just the data but, also sensitive spectrum occupancy information from the competitors and the spectrum management information used by the BS to configure the operation of the CPEs. The BS protects against unauthorized access to these data transport services by securing the associated service flows across the network. The security attributes and mechanisms for cognitive functionality include availability, authentication, authorization, identification, integrity, confidentiality and privacy. Explanations of these terms have been provided in Section 7.6.

To enhance the security for the cognitive functionality in 802.22, Security Sublayer 2 is introduced. The security mechanisms validate the availability of spectrum for the primary and the secondary users by employing mechanisms such as collaborative sensing and decision making. This includes authentication of the incumbent sensing information to avoid Denial of Service (DoS) attacks, authentication of the 802.22.1 beacon frame utilizing the security features that are already embedded in it, authentication of the geolocation and co-existence information, etc. Some cognitive plane security related mechanisms are an integral part of other cognitive functions required for the 802.22 system implementation such as Spectrum Sensing Function, Geolocation, Spectrum Manager, Spectrum Automaton, Management Plane procedures and functions etc. Since these functions are described in sections other than Section 7, Section 7.6 briefly describes the required security features and provides references as to where in IEEE 802.22 standard the mechanisms or the messaging for the same can be found.

7.1 Security Architecture for the Data / Control and Management Planes

Privacy has two component protocols as follows:

a) An encapsulation protocol for securing packet data across the network. This protocol defines a set of supported cryptographic suites, i.e., pairings of data encryption and authentication algorithms, and the rules for applying those algorithms to a MAC PDU payload.

b) A Security for Control and Management (SCM) protocol providing the secure distribution of keying data from the BS to the CPE. Through this key management protocol, the CPE and the BS synchronize keying data; in addition, the BS uses the protocol to enforce conditional access to network services.

The protocol stack for the security components of the system are shown in Figure xxx.

Figure xxx — Security Sublayer 1

— SCM Control Management: This stack controls all security components. Various keys are derived and generated in this stack.

— Traffic Data Processing: This stack encrypts or decrypts the traffic data and executes the authentication function for the traffic data.

— Control Message Processing: This stack processes the various SCM-related MAC messages, and provides either authentication and/or encryption of such messages.

—RSA/ECC-based Authorization: This stack performs the RSA- or ECC-based authorization function using the CPE’s X.509 digital certificate and the BS’s X.509 digital certificate, when the RSA or ECC-based authorization is selected as an authorization policy between a CPE and a BS.

— Authorization/SA Control: This stack controls the authorization state machine and the traffic encryption key state machine.

7.1.1 Secure encapsulation of MAC PDUs

Encryption services are defined as a set of capabilities within the MAC security sublayer. MAC header information specific to encryption is allocated in the generic MAC header format.

Encryption is always applied to the MAC PDU payload when required by the selected ciphersuite; the generic MAC header is not encrypted. All MAC management messages shall be sent in the clear to facilitate registration, ranging, and normal operation of the MAC.

The format of MAC PDUs carrying encrypted or un-encrypted packet data payloads is specified in 6.7.

7.1.2 Key management protocol

The SCM protocol allows for both mutual authentication and unilateral authentication (e.g., where the BS authenticates CPE, but not vice versa). It also supports periodic reauthentication/reauthorization and key refresh. The key management protocol uses X.509 digital certificates [IETF RFC 3280] together with RSA public-key encryption algorithm [PKCS #1] or a sequence starting with RSA authentication or even with ECC public-key encryption algorithm. It uses strong encryption algorithms to perform key exchanges between a CPE and BS.

The SCM’s authentication protocol establishes a shared secret (i.e., the AK) between the CPE and the BS. The shared secret is then used to secure subsequent SCM exchanges of TEKs. This two-tiered mechanism for key distribution permits refreshing of TEKs without incurring the overhead of computation-intensive operations.

A BS authenticates a client CPE during the initial authorization exchange. Each CPE presents its credentials, which will be a unique X.509 digital certificate issued by the CPE’s manufacturer (in the case of RSA authentication) or an operator-specified credential (in the case of EAP-based authentication).

The BS associates a CPE’s authenticated identity to a paying subscriber and hence to the data services that subscriber is authorized to access.

Since the BS authenticates the CPE, it may protect against an attacker employing a cloned CPE that masquerades as a legitimate subscriber’s CPE.

The traffic key management portion of the SCM protocol adheres to a client/server model, where the CPE (a SCM “client”) requests keying material and the BS (a SCM “server”) responds to those requests. This model ensures that individual CPE clients receive only keying material for which they are authorized.

The SCM protocol uses MAC management messaging, i.e., SCM-REQ and SCM-RSP messages defined in 6.9.28. The SCM protocol is defined in detail in 7.2.

7.1.3 Authorization Overview

A CPE uses the SCM protocol to obtain authorization and traffic keying material from the BS and to support periodic reauthorization and key refresh.

SCM supports two distinct authentication protocol mechanisms: RSA and ECC-based authorization.

7.1.3.1 SCM RSA Authorization

The SCM RSA authorization protocol uses X.509 digital certificates [IETF RFC 3280], the RSA public-key encryption algorithm [PKCS #1] that binds public RSA encryption keys to MAC addresses of CPEs.

A BS authorizes a client CPE during the initial authorization exchange. Each CPE carries a unique X.509 digital certificate issued by the CPE’s manufacturer. The digital certificate contains the CPE’s Public Key and CPE MAC address. When requesting an AK, a CPE presents its digital certificate to the BS. The BS verifies the digital certificate, and then uses the verified Public Key to encrypt an AK, which the BS then sends back to the requesting CPE.

All CPEs using RSA authorization shall have factory-installed RSA private/public key pairs or provide an internal algorithm to generate such key pairs dynamically. If a CPE relies on an internal algorithm to generate its RSA key pair, the CPE shall generate the key pair prior to its first AK exchange, described in 7.2.3.2. All CPEs with factory-installed RSA key pairs shall also have factory-installed X.509 certificates. All CPEs that rely on internal algorithms to generate an RSA key pair shall support a mechanism for installing a manufacturer-issued X.509 certificate following key generation.

Requirements for the RSA X.509-certificate are defined in section 7.5.

7.1.3.2 SCM ECC Authorization

In addition to the current RSA-based authorization within the SCM protocol, Elliptic Curve Cryptography (ECC)-based authorization may be employed.  ECC authentication operates in the same way as RSA authentication. ECC-based authorization makes use of the Elliptic Curve Integrated Encryption Scheme (ECIES) public-key encryption method defined in [SEC1]. Requirements for ECC-based X.509 certificates are defined in Section 7.5.

7.1.4 Mapping of connections to SAs

The following rules for mapping connections to SAs apply:

- All transport connections shall be mapped to an existing SA.

- Multicast transport connections may be mapped to any Static or Dynamic SA.

- The secondary management connection shall be mapped to the Primary SA.

- The basic and the primary management connections shall not be mapped to an SA.

The actual mapping is achieved by including the SAID of an existing SA in the DSA-xxx messages together with the CID. No explicit mapping of secondary management connection to the Primary SA is required.

7.1.5 Cryptographic suite

A cryptographic suite is the SA’s set of methods for authorization, data encryption, data authentication, and TEK exchange. The available cryptographic suites are specified as described in 7.4.1. The cryptographic suite shall be one of the ones listed in Table 7.4-1.

7.2 SCM protocol

The Security Control Management (SCM) protocol supported in this standard is based on the Privacy Key Management version 2 (PKMv2) developed for IEEE 802.16 [IEEE 802.16-2009]. SCM differs from PKMv2 in that it supports features such as; mutual authorization using RSA and ECC, a modified key hierarchy, and AES-CCM is used for all message encryption and authentication.

7.2.1 Security associations (SAs)

A security association (SA) is the set of security information a BS and one or more of its client CPEs share in order to support secure communications across the IEEE 802.22 network. There are two basic types of SAs, Unicast and Group, that can be defined.

Each CPE establishes one Unicast SA (that is unique to self) that is known as it’s Primary SA, during the CPE authorization process. All of the unicast management message and data traffic to/from the CPE will be protected by the keying material provided by the Primary SA.

Establishment of Group SAs (GSAs) are optional. Group SAs are to be used for providing keying material for multicast/broadcast transmission of management message and data traffic. Upon completion of the authorization process, if a CPE is authorized for one or more GSAs, it will initiate a MCA-REQ to join each multicast group that it was authorized for. If the GSA is for broadcast traffic, then the CPE doesn’t have to initiate the MCA-REQ.

An SA’s shared information shall include the cryptographic suite employed within the SA. The shared information may include TEKs. The exact content of the SA is dependent on the SA’s cryptographic suite.

SAs are identified using SAIDs. The Primary SA shall be identified by an SAID that is equal to the Basic CID of that CPE. The SAID of a GSA will be the Mutlicast CID or Broadcast CID of the group that the CPE is assigned to.

Using the SCM protocol, a CPE requests from its BS an SA’s keying material.

The BS shall ensure that each client CPE only has access to the SAs it is authorized to access.

An SA’s keying material has a limited lifetime. When the BS delivers SA keying material to a CPE, it also provides the CPE with that material’s remaining lifetime. It is the responsibility of the CPE to request new keying material from the BS before the set of keying material that the CPE currently holds expires at the BS. Should the current keying material expire before a new set of keying material is received, the CPE shall perform re-authorization as described in 7.2.3.

In certain cryptographic suites, key lifetime may be limited by the exhaustion rate of a number space, e.g., the PN of AES in CCM. In this case, the key ends either at the expiry of the key lifetime or the exhaustion of the number space, which ever is earliest. Note that in this case, security is not determined by the key lifetime.

7.2.1.1 Dynamic Creation of GSA

The BS may dynamically establish GSAs by issuing a GSA Add message (via SCM-REQ/RSP). Upon receiving an GSA Add message, the CPE shall start a TEK state machine to establish and maintain GTEKs for each GSA listed in the message.

7.2.1.2 Dynamic Mapping of GSA

When creating a new DS multicast/broadcast service flow, the BS may request an existing GSA (configured at the CPE) be used by passing the SAID of the GSA in a DSA-REQ or DSC-REQ message sent to the CPE. The BS checks the CPEs authorization for the requested GSA and generates appropriate response using a DSA-RSP or DSC-RSP message correspondingly.

7.2.3 CPE Authorization Process

7.2.3.1 CPE authorization and AK exchange overview

CPE authorization, controlled by the Authorization state machine, is the process of the BS’s authenticating a client CPE’s identity:

a) The BS and CPE establish a shared AK by RSA/ECC from which a key encryption key (KEK) and message authentication keys are derived.

b) The BS provides the authorized CPE with the identities (i.e., the SAIDs) and properties of Primary and Static SAs for which the CPE is authorized to obtain keying information.

After achieving initial authorization, a CPE periodically reauthorizes with the BS; reauthorization is also managed by the CPE’s Authorization state machine. TEK state machines manage the refreshing of TEKs.

7.2.3.2 Mutual Authorization via RSA

A CPE begins authorization an Authorization Request message to its BS immediately after sending the Authentication Information message. This is a request for an AK, as well as for the SAIDs identifying any Static SAs the CPE is authorized to participate in.

The Authorization Request includes

- A manufacturer-issued X.509 certificate.

- A description of the cryptographic algorithms the requesting CPE supports. A CPE’s cryptographic capabilities are presented to the BS as a list of cryptographic suite identifiers, each indicating a particular pairing of packet data encryption and packet data authentication algorithms the CPE supports.

- The CPE’s Basic CID. The Basic CID is the first static CID the BS assigns to a CPE during initial ranging. The primary SAID is equal to the Basic CID.

- A 64 bit random number generated by the CPE

- RSA signature over all the attributes in the authorization request message, used to assure the authenticity of the SCM RSA-Request message

In response to an Authorization Request message, a BS validates the requesting CPE’s identity, determines the encryption algorithm and protocol support it shares with the CPE, activates an AK for the CPE, encrypts it with the CPE’s public key, and sends it back to the CPE in an Authorization Reply message. The authorization reply includes

- The BS’s X.509 certificate, used to verify the BS’s identity

- A pre-PAK encrypted with the CPEs public key, using [PKCS#1] (see section 7.5.3.1)

- A 4-bit PAK sequence number, used to distinguish between successive generations of AKs

- PAK lifetime

- The identities (i.e. SAID’s) and properties of the Primary SA and any of the GSAs the CPE is authorized for. This also includes the cryptographic suite (See Table 7.4-1) that is to be applied to each SA.

- The 64 bit random number genereated by the CPE.

- A 64 bit random number generated by the BS, which, along with CPE random number used to ensure key liveliness.

- RSA signature over all the attributes in the authorization reply message, used to assure the authenticity of the SCM RSA-Reply messages.

While the Authorization Reply shall identify Static SAs in addition to the Primary SA whose SAID matches the requesting CPE’s Basic CID, the Authorization Reply shall not identify any Dynamic SAs.

The BS, in responding to a CPE’s Authorization Request, shall determine whether the requesting CPE, whose identity can be verified via the X.509 digital certificate, is authorized for basic unicast services, and what additional statically provisioned services (i.e., Static SAIDs) the CPE’s user has subscribed for. Note that the protected services a BS makes available to a client CPE can depend upon the particular cryptographic suites for which the CPE and the BS share support.

A CPE shall periodically refresh its AK by reissuing an Authorization Request to the BS. Reauthorization is identical to authorization. To avoid service interruptions during reauthorization, successive generations of the CPE’s AKs have overlapping lifetimes. Both the CPE and BS shall be able to support up to two simultaneously active AKs during these transition periods. The operation of the Authorization state machine’s Authorization Request scheduling algorithm, combined with the BS’s regimen for updating and using a client CPE’s AKs (see 7.3), ensures that the CPE can be refreshed periodically.

When the RSA-based authorization is negotiated as authorization policy, the SCM Auth-Request, the SCM Auth-Reply, the SCM Auth-Reject, and the SCM Auth-Acknowledgement messages are used to share the pre-PAK.

7.2.3.3 Mutual Authorization via ECC

Initial and re-authorization is executed in the same way for ECC authorization, as it is executed for RSA authorization.

There are two exceptions in the process. When calculating the signature for either the Authorizaiton Request or Authorization Reply, the process defined in Section 7.3 of ANSI X9.62-2005 will be used. Encryption of the pre-PAK shall be done according to the specification of the ECIES algorithm specified in Section 5 of [SEC1]. The specifics configuration of this process is detailed in section 7.5.3.2.

Regardless of the method used, the BS then verifies the domain parameters (Section 5.1 of ANSI X9.63-2001), the public key (Section 5.2 of ANSI X9.63-2001), and the signature over the request (Section 7.4 ANSI X9.62-2005).  If any of these checks fail, the then Authorization Request is rejected.  When the BS responds, it can choose between either of two methods (similar to CPE initiation methods) when formatting the response.  The first way is to make use of a manfucaturer-installed ECC certificate (see Section 7.5) and public key that is associated with that BS.  The other method is that the BS uses the elltiptic curve domain parameters defined in its certificate to generate an ephemeral key pair.

The Authorization Reply is transmitted with the ephemeral public key and just the domain parameters or entire manufacturer-installed certificate (domain parameters and manufacturer installed public key), the assigned cryptographic suite, the 64bit randomon number generated by CPE, a 64bit random number generated by the BS, as well as a signature calculated over the reply using the process defined in Section 7.3 of ANSI X9.62-2005.

SCM Auth-Request, the SCM Auth-Reply, the SCM Auth-Reject, and the SCM Auth-Acknowledgement messages are used to share the pre-PAK.

7.2.3.4 Authorization state machine

The Authorization state machine consists of six states and eight distinct events (including receipt of messages) that can trigger state transitions. The Authorization finite state machine (FSM) is presented below in a graphical format, as a state flow model (Figure 7.2-1), and in a tabular format, as a state transition matrix (Table 7.2-1).

The state flow diagram depicts the protocol messages transmitted and internal events generated for each of the model’s state transitions; however, the diagram does not indicate additional internal actions, such as the clearing or starting of timers, that accompany the specific state transitions. Accompanying the state transition matrix is a detailed description of the specific actions accompanying each state transition; the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

The following legend applies to the Authorization State Machine flow diagram depicted in Figure 7.2-1.

- Ovals are states.

- Events are in italics.

- Messages are in normal font.

- State transitions (i.e., the lines between states) are labeled with <what causes the transition>/<messages and events triggered by the transition>. So “timeout/Auth Request” means that the state received a “timeout” event and sent an Authorization Request (“Auth Request”) message. If there are multiple events or messages before the slash “/” separated by a comma, any of them can cause the transition. If there are multiple events or messages listed after the slash, all of the specified actions shall accompany the transition.

Figure 7.2-1. Authorization state machine flow diagram

The Authorization state transition matrix presented in Table 7.2-1 lists the six Authorization machine states in the topmost row and the eight Authorization machine events (includes message receipts) in the leftmost column. Any cell within the matrix represents a specific combination of state and event, with the next state (the state transitioned to) displayed within the cell. For example, cell 4-B represents the receipt of an Authorization Reply (Auth Reply) message when in the Authorize Wait (Auth Wait) state. Within cell 4-B is the name of the next state, “Authorized.” Thus, when an CPE’s Authorization state machine is in the Auth Wait state and an Auth Reply message is received, the Authorization state machine will transition to the Authorized state. In conjunction with this state transition, several protocol actions shall be taken; these are described in the listing of protocol actions, under the heading 4-B, in 7.2.3.4.5.

A shaded cell within the state transition matrix implies that either the specific event cannot or should not occur within that state, and if the event does occur, the state machine shall ignore it. For example, if an Auth Reply message arrives when in the Authorized state, that message should be ignored (cell 4-C). The CPE may, however, in response to an improper event, log its occurrence, generate an SNMP event, or take some other vendor-defined action. These actions, however, are not specified within the context of the Authorization state machine, which simply ignores improper events.

Table 7.2-1 – Authorization Finite State Machine State Transition Matrix

| State Event or Recvd Msg | (A) Start | (B) Auth Wait | (C) Authorized | (D) Reauth Wait | (E) Auth Reject Wait | (F) Silent |
| --- | --- | --- | --- | --- | --- | --- |
| (1) Communication Established | Auth Wait |  |  |  |  |  |
| (2) Auth Reject |  | Auth Reject Wait |  | Auth Reject Wait |  |  |
| (3) Perm Auth Reject |  | Silent |  | Silent |  |  |
| (4) Auth Reply |  | Authorized |  | Authorized |  |  |
| (5) Timeout |  | Auth Wait |  | Reauth Wait | Start |  |
| (6) Auth Grace Timeout |  |  | Reauth Wait |  |  |  |
| (7) Auth Invalid |  |  | Reauth Wait | Reauth Wait |  |  |
| (8) Reauth |  |  | Reauth Wait |  |  |  |

7.2.3.4.1 States

- Start: This is the initial state of the FSM. No resources are assigned to or used by the FSM in this state—e.g., all timers are off, and no processing is scheduled.

- Authorize Wait (Auth Wait): The CPE has received the “Communication Established” event indicating that it has completed basic capabilities negotiation with the BS. In response to receiving the event, the CPE has sent both an Authentication Information and an Auth Request message to the BS and is waiting for the reply.

- Authorized: The CPE has received an Auth Reply message that contains a list of valid SAIDs for this CPE. At this point, the CPE has a valid AK and SAID list. Transition into this state triggers the creation of one TEK FSM for each of the CPE’s privacy-enabled SAIDs.

- Reauthorize Wait (Reauth Wait): The CPE has an outstanding reauthorization request. The CPE was either about to expire (see Authorization Grace Time in Table 7.2-1) its current authorization or received an indication (an Authorization Invalid message from the BS) that its authorization is no longer valid. The CPE sent an Auth Request message to the BS and is waiting for a response.

- Authorize Reject Wait (Auth Reject Wait): The CPE received an Authorization Reject (Auth Reject) message in response to its last Auth Request. The Auth Reject’s error code indicated the error was not of a permanent nature. In response to receiving this reject message, the CPE set a timer and transitioned to the Auth Reject Wait state. The CPE remains in this state until the timer expires.

- Silent: The CPE received an Auth Reject message in response to its last Auth Request. The Auth Reject’s error code indicated the error was of a permanent nature. This triggers a transition to the Silent state, where the CPE is not permitted to pass subscriber traffic. The CPE shall, however, respond to management messages from the BS issuing the Perm Auth Reject.

7.2.3.4.2 Messages

Note that the message formats are defined in detail in 6.9.28.

- Authorization Request (Auth Request): Request an AK and list of authorized SAIDs. Sent from CPE to BS.

- Authorization Reply (Auth Reply): Receive an AK and list of authorized, static SAIDs. Sent from BS to CPE. The AK is encrypted with the CPE’s public key.

- Authorization Reject (Auth Reject): Attempt to authorize was rejected. Sent from the BS to the CPE.

- Authorization Invalid (Auth Invalid): The BS may send an Authorization Invalid message to a client CPE as follows:

- An unsolicited indication, or

- A response to a message received from that CPE. In either case, the Auth Invalid message instructs the receiving CPE to reauthorize with its BS. The BS responds to a Key Request with an Auth Invalid message if (1) the BS does not recognize the CPE as being authorized (i.e., no valid AK associated with CPE) or (2) verification of the Key. Request’s keyed message digest failed. Note that the Authorization Invalid event, referenced in both the state flow diagram and the state transition matrix, signifies either the receipt of an Auth Invalid message or an internally generated event.

7.2.3.4.3 Events

- Communication Established: The Authorization state machine generates this event upon entering the Start state if the MAC has completed basic capabilities negotiation. If the basic capabilities negotia-tion is not complete, the CPE sends a Communication Established event to the Authorization FSM upon completing basic capabilities negotiation. The Communication Established event triggers the CPE to begin the process of getting its AK and TEKs.

- Timeout: A retransmission or wait timer timed out. Generally a request is resent.

- Authorization Grace Timeout (Auth Grace Timeout): The timer Authorization Grace Time expired. This timer fires a configurable amount of time (the Authorization Grace Time) before the current authori-zation is supposed to expire, signalling the CPE to reauthorize before its authorization actually expires. The Authorization Grace Time takes the default value from Table 318 or may be specified in a configuration setting within the Auth Reply message.

- Reauthorize (Reauth): CPE’s set of authorized static SAIDs may have changed. This event may be generated in response to an SNMP set and meant to trigger a reauthorization cycle.

- Authorization Invalid (Auth Invalid): This event is internally generated by the CPE when there is a failure authenticating a Key Reply or Key Reject message, or externally generated by the receipt of an Auth Invalid message, sent from the BS to the CPE. A BS responds to a Key Request with an Auth Invalid if verification of the request’s message authentication code fails. Both cases indicate BS and CPE have lost AK synchronization. A BS may also send to an CPE an unsolicited Auth Invalid message, forcing an Auth Invalid event.

- Permanent Authorization Reject (Perm Auth Reject): The CPE receives an Auth Reject in response to an Auth Request. The error code in the Auth Reject indicates the error is of a permanent nature. What is interpreted as a permanent error is subject to administrative control within the BS. Auth Request processing errors that can be interpreted as permanent error conditions include the follow-ing:

- Unknown manufacturer (do not have CA certificate of the issuer of the CPE Certificate).

- Invalid signature on CPE certificate.

- ASN.1 parsing failure.

- Inconsistencies between data in the certificate and data in accompanying SCM data attribute.

- Incompatible security capabilities. When an CPE receives an Auth Reject indicating a permanent failure condition, the Authorization State machine moves into a Silent state, where the CPE is not permitted to pass subscriber traffic. The CPE shall, however, respond to management messages from the BS issuing the Perm Auth Reject. The managed CPE may also issue an SNMP Trap upon entering the Silent state.

- Authorization Reject (Auth Reject): The CPE receives an Auth Reject in response to an Auth Request. The error code in the Auth Reject does not indicate the failure was due to a permanent error condition. As a result, the CPE’s Authorization state machine shall set a wait timer and transition into the Auth Reject Wait State. The CPE shall remain in this state until the timer expires, at which time it shall reattempt authorization.

NOTE — The following events are sent by an Authorization state machine to the TEK state machine:

- [TEK] Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate the FSM and remove the corresponding SAID’s keying material from the CPE’s key table.

- [TEK] Authorized: Sent by the Authorization FSM to a nonactive (START state), but valid TEK FSM.

- [TEK] Authorization Pending (Auth Pend): Sent by the Authorization FSM to a specific TEK FSM to place that TEK FSM in a wait state until the Authorization FSM can complete its reauthorization operation.

- [TEK] Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational Reauthorize Wait (Op Reauth Wait) or Rekey Reauthorize Wait (Rekey Reauth Wait) states to clear the wait state begun by a TEK FSM Authorization Pending event.

7.2.3.4.4 Parameters

All configuration parameter values take the default values from Table 318 or may be specified in the Auth Reply message. These values are 4 octets long.

- Authorize Wait Timeout (Auth Wait Timeout): Timeout period between sending Authorization Request messages from Auth Wait state (see Table 318).

- Authorization Grace Time (Auth Grace Timeout): Amount of time before authorization is sched-uled to expire that the CPE starts reauthorization (see Table 318).

- Authorize Reject Wait Timeout (Auth Reject Wait Timeout): Amount of time an CPE’s Authorization FSM remains in the Auth Reject Wait state before transitioning to the Start state (see Table 318).

7.2.3.4.5 Actions

Actions taken in association with state transitions are listed by <event> (<rcvd message>) --> <state> below:

1-A Start (Communication Established) → Auth Wait

a) Send Auth Info message to BS

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Auth Wait Timeout

2-B Auth Wait (Auth Reject) → Auth Reject Wait

a) Clear Auth Request retry timer

b) Set a wait timer to Auth Reject Wait Timeout

2-D Reauth Wait (Auth Reject) → Auth Reject Wait

a) Clear Auth Request retry timer

b) Generate TEK FSM Stop events for all active TEK state machines

c) Set a wait timer to Auth Reject Wait Timeout

3-B Auth Wait (Perm Auth Reject) → Silent

a) Clear Auth Request retry timer

b) Disable all forwarding of CPE traffic

3-D Reauth Wait (Perm Auth Reject) → Silent

a) Clear Auth Request retry timer

b) Generate TEK FSM Stop events for all active TEK state machines

c) Disable all forwarding of CPE traffic

4-B Auth Wait (Auth Reply) → Authorized

a) Clear Auth Request retry timer

b) Decrypt and record AK delivered with Auth Reply

c) Start TEK FSMs for all SAIDs listed in Authorization Reply (provided the CPE supports the cryptographic suite that is associated with an SAID) and issue a TEK FSM Authorized event for each of the new TEK FSMs

d) Set the Authorization Grace timer to go off “Authorization Grace Time” seconds prior to the supplied AK’s scheduled expiration

4-D Reauth Wait (Auth Reply) → Authorized

a) Clear Auth Request retry timer

b) Decrypt and record AK delivered with Auth Reply

c) Start TEK FSMs for any newly authorized SAIDs listed in Auth Reply (provided the CPE supports the cryptographic suite that is associated with the new SAID) and issue TEK FSM Authorized event for each of the new TEK FSMs

d) Generate TEK FSM Authorization Complete events for any currently active TEK FSMs whose corresponding SAIDs were listed in Auth Reply

e) Generate TEK FSM Stop events for any currently active TEK FSMs whose corresponding SAIDs were not listed in Auth Reply

f) Set the Authorization Grace timer to go off “Authorization Grace Time” seconds prior to the supplied AK’s scheduled expiration

5-B Auth Wait (Timeout) → Auth Wait

a) Send Auth Info message to BS

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Auth Wait Timeout

5-D Reauth Wait (Timeout) → Reauth Wait

a) Send Auth Request message to BS

b) Set Auth Request retry timer to Reauth Wait Timeout

5-E Auth Reject Wait (Timeout) → Start

a) No protocol actions associated with state transition

6-C Authorized (Auth Grace Timeout) → Reauth Wait

a) Send Auth Request message to BS

b) Set Auth Request retry timer to Reauth Wait Timeout

7-C Authorized (Auth Invalid) → Reauth Wait

a) Clear Authorization Grace timer

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Reauth Wait Timeout

d) If the Auth Invalid event is associated with a particular TEK FSM, generate a TEK FSM Authorization Pending event for the TEK state machine responsible for the Auth Invalid event (i.e., the TEK FSM that either generated the event, or sent the Key Request message the BS responded to with an Auth Invalid message)

7-D Reauth Wait (Auth Invalid) → Reauth Wait

a) If the Auth Invalid event is associated with a particular TEK FSM, generate a TEK FSM Authorization Pending event for the TEK state machine responsible for the Auth Invalid event (i.e., the TEK FSM that either generated the event, or sent the Key Request message the BS responded to with an Auth Invalid message)

8-C Authorized (Reauth) → Reauth Wait

a) Clear Authorization Grace timer

b) Send Auth Request message to BS

c) Set Auth Request retry timer to Reauth Wait Timeout

7.2.3.5 Security capabilities selection

As part of their authorization exchange, the CPE provides the BS with a list of all the cryptographic suites (pairing of data encryption and data authentication algorithms) the CPE supports. The parameters that describe the cryptographic suite options are in Table 7.4-1. The BS selects from this list a single cryptographic suite to employ with the requesting CPE’s primary SA. The Authorization Reply the BS sends back to the CPE includes a primary SA-Descriptor that, among other things, identifies the cryptographic suite the BS selected to use for the CPE’s primary SA. A BS shall reject the authorization request if it determines that none of the offered cryptographic suites are satisfactory.

The Authorization Reply also contains an optional list of static SA-Descriptors; each static SA-Descriptor identifies the cryptographic suite employed within the SA. The selection of a static SA’s cryptographic suite is typically made independent of the requesting CPE’s cryptographic capabilities. A BS may include in its Authorization Reply static SA-Descriptors identifying cryptographic suites the requesting CPE does not support; if this is the case, the CPE shall not start TEK state machines for static SAs whose cryptographic suites the CPE does not support.

If the SA defines use of authentication and encryption method, all MAC PDUs sent with CIDs linked to this SA must have EC bit set to ‘1’ in the Generic MAC Header. Otherwise, if only authentication is supported the EC bit must be set to ‘0’ in the Generic MAC Header. Other combinations are not allowed; MAC PDUs presenting other combinations should be discarded.

The capabilities defined in an SA are not applicable to management messages transmitted on the Initial Ranging, Basic, as well as Broadcast CIDs. Only traffic assigned to an SA where “no authorization” is specificed, will be allowed to be transmitted without any authentication or encryption information.

7.2.4 TEK exchange overview

7.2.4.1 TEK exchange overview for PMP topology

If the CPE and BS decide “No authorization” as their authorization policy, the CPE and BS shall not perform the Key Request/Key Reply handshake. In this case, target SAID value, which may be included in DSA-REQ/RSP messages, shall be Null SAID.

Upon achieving authorization, a CPE starts a separate TEK state machine for each of the SAIDs identified in the Authorization Reply message. Each TEK state machine operating within the CPE is responsible for managing the keying material associated with its respective SAID. TEK state machines periodically send Key Request messages to the BS, requesting a refresh of keying material for their respective SAIDs.

TEK state machines periodically cause the CPE to send Key Request messages to the BS, requesting a refresh of keying material for their respective SAIDs. The BS responds to a Key Request with a Key Reply message, containing the BS’s active keying material for a specific SAID.

The TEK is encrypted using appropriate KEK derived from the AK. For AES-CCM, the TEK is a 128 bit key and the KEK is derived from the AK using a 128 bit key and 128 bit block size.

Note that at all times the BS maintains two active sets of keying material per SAID. The lifetimes of the two generations overlap so that each generation becomes active halfway through the life of it predecessor and expires halfway through the life of its successor. A BS includes in its Key Replies both of an SAID’s active generations of keying material.

For SAs using a ciphersuite employing AES-CCM mode, the Key Reply provides the requesting CPE, in addition to the TEK, the remaining lifetime of each of the two sets of keying material. The receiving CPE uses these remaining lifetimes to estimate when the BS will invalidate a particular TEK and, therefore, when to schedule future Key Requests so that the CPE requests and receives new keying material before the BS expires the keying material the CPE currently holds. For AES-CCM mode, when more than half the available PN numbers in the 24-bit PN number space are exhausted, the CPE shall schedule a future Key Request in the same fashion as if the key lifetime was approaching expiry. The operation of the TEK state machine’s Key Request scheduling algorithm, combined with the BS’s regimen for updating and using an SAID’s keying material (see 7.3), ensures that the CPE will be able to continually exchange encrypted traffic with the BS.

The operation of the TEK state machine’s Key Request scheduling algorithm, combined with the BS’s regimen for updating and using an SAID’s keying material (see Figure 7.2-2 and Table 7.2-2), ensures that the CPE will be able to continually exchange encrypted traffic with the BS.

A TEK state machine remains active as long as

a) The CPE is authorized to operate in the BS’s security domain, i.e., it has a valid AK, and

b) The CPE is authorized to participate in that particular SA, i.e., the BS continues to provide fresh key-ing material during rekey cycles.

The parent Authorization state machine stops all of its child TEK state machines when the CPE receives from the BS an Authorization Reject during a reauthorization cycle. Individual TEK state machines can be started or stopped during a reauthorization cycle if a CPE’s Static SAID authorizations changed between successive reauthorizations.

Communication between Authorization and TEK state machines occurs through the passing of events and protocol messaging. The Authorization state machine generates events (i.e., Stop, Authorized, Authorization Pending, and Authorization Complete events) that are targeted at its child TEK state machines. TEK state machines do not target events at their parent Authorization state machine. The TEK state machine affects the Authorization state machine indirectly through the messaging a BS sends in response to a CPE’s requests: a BS may respond to a TEK machine’s Key Requests with a failure response (i.e., Authorization Invalid message) to be handled by the Authorization state machine.

7.2.4.2 TEK state machine

The TEK state machine consists of seven states and eleven events (including receipt of messages) that may trigger state transitions. Like the Authorization state machine, the TEK state machine is presented in both a state flow diagram (Figure 7.2-2) and a state transition matrix (Table 7.2-2). As was the case for the Authorization state machine, the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

Shaded states in Figure 7.2-2 (Operational, Rekey Wait, Rekey Reauthorize Wait, and M&B Rekey InterimWait) have valid keying material and encrypted traffic may be sent.

The SAID may be replaced by the GSAID for the multicast service or the broadcast service. And, the TEK may be also replaced by the GTEK for the multicast service or the broadcast service.

The Authorization state machine starts an independent TEK state machine for each of its authorized SAIDs. As mentioned in 7.2.2, the BS maintains two active TEKs/GTEKs per SAID.

For the unicast service, the BS includes in its Key Replies both of these TEKs, along with their remaining lifetimes. For the multicast/broadcast service, the BS includes in its Key Replies both of the GTEKs, along with their remaining lifetimes.

The BS encrypts downlink traffic with the older of its two TEKs and decrypts uplink traffic witheither the older or newer TEK, depending upon which of the two keys the CPE was using at the time. The CPE encrypts uplink traffic with the newer of its two TEKs and decrypts downlink traffic with either the older or newer TEK, depending upon which of the two keys the BS was using at the time. See 7.3 for details on CPE and BS key usage requirements.

For the multicast service or the broadcast service, the BS may include both of GTEKs in its Key Reply messages, when an CPE request traffic keying material. The BS signs and/or encrypts downlink traffic with current GTEK. The CPE verifies and/or decrypts downlink traffic with either the older or newer GTEK, depending upon which of the two keys the BS is using at the time. See 7.3 for details on CPE and BS key usage requirements.

Through operation of a TEK state machine, the CPE attempts to keep its copies of an SAID’s TEKs synchronized with those of its BS. A TEK state machine issues Key Requests to refresh copies of its SAID’s keying material soon after the scheduled expiration time of the older of its two TEKs and before the expiration of its newer TEK. To accommodate for CPE/BS clock skew and other system processing and transmission delays, the CPE schedules its Key Requests a configurable number of seconds before the newer TEK’s estimated expiration in the BS. With the receipt of the Key Reply, the CPE shall always update its records with the TEK Parameters from both TEKs/GTEKs contained in the Key Reply message.

Figure 7.2-2 - TEK State Machine Flow Diagram

Table 7.2-2 – TEK State Transition Finite State Machine

| State Event or Recvd Msg | (A) Start | (B) Op Wait | (C) Op Reauth Wait | (D) Op | (E) Rekey Wait | (F) Rekey Reauth Wait | (G) M&B Rekey Interm Wait |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (1) Stop |  | Start | Start | Start | Start | Start |  |
| (2) Authorized | Op Wait |  |  |  |  |  |  |
| (3) Auth Pend |  | Op Reauth Wait |  |  | Rekey Reauth Wait |  |  |
| (4) Auth Comp |  |  | Op Wait |  |  | Rekey Wait |  |
| (5) TEK Invalid |  |  |  | Op Wait | Op Wait | Op Reauth Wait |  |
| (6) Timeout |  | Op Wait |  |  | Rekey Wait |  |  |
| (7) TEK/GTEK Refresh Timeout |  |  |  | Rekey Wait |  |  | Rekey Wait |
| (8) Key Reply |  | Operational |  |  | Operational |  |  |
| (9) Key Reject |  | Start |  |  | Start |  |  |
| (10) GKEK Updated |  |  |  | M&B Rekey Interim Wait |  |  |  |
| (11) TEK/GTEK Updated |  |  |  |  | Operational |  | Operational |

7.2.4.2.1 States

Start: This is the initial state of the FSM. No resources are assigned to or used by the FSM in this state—e.g., all timers are off, and no processing is scheduled.

Operational Wait (Op Wait): The TEK state machine has sent its initial request (Key Request) for its SAID’s keying material (TEK), and is waiting for a reply from the BS.

Operational Reauthorize Wait (Op Reauth Wait): The wait state the TEK state machine is placed in if it does not have valid keying material while the Authorization state machine is in the middle of a reauthorization cycle.

Operational: The CPE has valid keying material for the associated SAID.

Rekey Wait: The TEK Refresh Timer has expired and the CPE has requested a key update for this SAID. Note that the newer of its two TEKs has not expired and may still be used for both encrypting and decrypting data traffic.

Rekey Reauthorize Wait (Rekey Reauth Wait): The wait state the TEK state machine is placed in if the TEKstate machine has valid traffic keying material, has an outstanding request for the latest keying material, and the Authorization state machine initiates a reauthorization cycle.

M&B Rekey Interim Wait (Multicast & Broadcast Rekey Interim Wait): This state is defined only for the

multicast service or the broadcast service. This state is the wait state the TEK state machine is placed in if the TEK state machine has valid traffic keying material and receives the new GKEK from the BS.

7.2.4.2.2 Messages

Note that the message formats are defined in detail in 6.9.28.

Key Request: Request a TEK for this SAID. Sent by the CPE to the BS and authenticated with keyed message digest. The message authentication key is derived from the AK.

Key Reply: Response from the BS carrying the two diversity sets of traffic keying material for this SAID. Sent by the BS to the CPE, it includes the SAID’s TEKs, encrypted with a KEK derived from the AK or the GSAID’s GTEK, encrypted with a. The Key Reply message is authenticated with a keyed message digest; the MMP_KEY used to do this is key is derived from the AK.

Key Reject: Response from the BS to the CPE to indicate this SAID is no longer valid and no key will be sent. The Key Reject message is authenticated with a keyed message digest; the authentication key is derived from the AK.

TEK Invalid: The BS sends an CPE this message if it determines that the CPE encrypted an uplink PDU with an invalid TEK, i.e., an SAID’s TEK key sequence number, contained within the received PDU’s MAC Header, is out of the BS’s range of known, valid sequence numbers for that SAID.

7.2.4.2.3 Events

Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate TEK FSM and remove the corresponding SAID’s keying material from the CPE key table. See Figure 7.2-1.

Authorized: Sent by the Authorization FSM to a non-active (START state) TEK FSM to notify TEK FSM of successful authorization. See Figure 7.2-1.

Authorization Pending (Auth Pend): Sent by the Authorization FSM to TEK FSM to place TEK FSM in a wait state while Authorization FSM completes reauthorization. See Figure 7.2-1.

Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational

Reauthorize Wait or Rekey Reauthorize Wait states to clear the wait state begun by the prior Authorization Pending event. See Figure 7.2-1.

TEK Invalid: This event is triggered by either an CPEs data packet decryption logic or by the receipt of a TEK Invalid message from the BS.

A CPE’s data packet decryption logic triggers a TEK Invalid event if it recognizes a loss of TEK key synchronization between itself and the encrypting BS. For example, an SAID’s TEK key sequence number, contained within the received downlink MAC PDU header, is out of the CPECPEs range of known sequence numbers for that SAID.

A BS sends an CPEs a TEK Invalid message, triggering a TEK Invalid event within the CPE, if the BS’s decryption logic recognizes a loss of TEK key synchronization between itself and the CPE.

Timeout: A retry timer timeout. Generally, the particular request is retransmitted.

GTEK/TEK Refresh Timeout: The TEK refresh timer timed out. This timer event signals the TEK state machine to issue a new Key Request in order to refresh its keying material. The refresh timer is set to fire a configurable duration of time (GTEK/TEK Grace Time) before the expiration of the newer TEK the CPE currently holds. This is configured via the BS to occur after the scheduled expiration of the older of the two TEKs.

GKEK Updated: This event is triggered when the CPE receives the new GKEK through the SCM GSA Add.

TEK/GTEK Updated: This event is triggered when the CPE receives the new TEK or GTEK and traffic keying material through the Key Reply message.

7.2.4.2.4 Parameters

All configuration parameter values take the default values from Table 318 or may be specified in Auth Reply message. Theses value shall be 4 octets long.

Operational Wait Timeout: Timeout period between sending of Key Request messages from the Op Wait state (see Table 318).

Rekey Wait Timeout: Timeout period between sending of Key Request messages from the Rekey Wait state (see Table 318).

GTEK/TEK Grace Time: Time interval, in seconds, before the estimated expiration of a GTEK/TEK that the CPE starts rekeying for a new GTEK/TEK. GTEK/TEK Grace Time takes the default value from Table 318 or may be specified in a configuration setting within the Auth Reply message and is the same across all SAIDs (see Table 318).

7.2.4.2.5 Actions

Actions taken in association with state transitions are listed by <event> (<rcvd message>) --> <state>:

1-B Op Wait (Stop) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

1-C Op Reauth Wait (Stop) Start

a) Terminate TEK FSM

1-D Operational (Stop) Start

a) Clear TEK refresh timer, which is timer set to go off “GTEK/TEK Grace Time” seconds prior to the TEK’s scheduled expiration time

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-E Rekey Wait (Stop) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-F Rekey Reauth Wait (Stop) Start

a) Terminate TEK FSM

b) Remove SAID keying material from key table

2-A Start (Authorized) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

3-B Op Wait (Auth Pend) Op Reauth Wait

a) Clear Key Request retry timer

3-E Rekey Wait (Auth Pend) Rekey Reauth Wait

a) Clear Key Request retry timer

4-C Op Reauth Wait (Auth Comp) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

4-F Rekey Reauth Wait (Auth Comp) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

5-D Operational (TEK Invalid) Op Wait

a) Clear TEK refresh timer

b) Send Key Request message to BS

c) Set Key Request retry timer to Operational Wait Timeout

d) Remove SAID keying material from key table

5-E Rekey Wait (TEK Invalid) Op Wait

a) Clear TEK refresh timer

b) Send Key Request message to BS

c) Set Key Request retry timer to Operational Wait Timeout

d) Remove SAID keying material from key table

5-F Rekey Reauth Wait (TEK Invalid) Op Reauth Wait

a) Remove SAID keying material from key table

6-B Op Wait (Timeout) Op Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Operational Wait Timeout

6-E Rekey Wait (Timeout) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

7-D Operational (TEK Refresh Timeout) Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

7-G M&B Rekey Interim Wait (TEK Refresh Timeout) -> Rekey Wait

a) Send Key Request message to BS

b) Set Key Request retry timer to Rekey Wait Timeout

8-B Op Wait (Key Reply) Operational

a) Clear Key Request retry timer

b) Process contents of Key Reply message and incorporate new keying material into key database

c) Set the TEK refresh timer to go off “GTEK/TEK Grace Time” seconds prior to the newer key’s scheduled expiration

8-E Rekey Wait (Key Reply) Operational

a) Clear Key Request retry timer

b) Process contents of Key Reply message and incorporate new keying material into key database

c) Set the TEK refresh timer to go off “GTEK/TEK Grace Time” seconds prior to the newer key’s scheduled expiration

9-B Op Wait (Key Reject) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

9-E Rekey Wait (Key Reject) Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

c) Remove SAID keying material from key table

10-D Operational (GKEK Updated) -> M&B Rekey Interim Wait

a) Process contents of Key Update Command message for the GKEK update mode and incorporate new GKEK into key database

11-G M&B Rekey Interim Wait (GTEK Updated) -> Operational

a) Clear Key Request retry timer

b) Process contents of Key Update Command message for the GTEK update mode and incorporate new traffic keying material into key database

c) Set the TEK refresh timer to go off “GTEK/TEK Grace Time” seconds prior to the key’s scheduled expiration

7.2.5 Key derivation

The SCM key hierarchy defines what keys are present in the system and how the keys are generated. IEEE 802.22 systems shall use either one of two, either ECC-based or RSA based, authentication schemes. The keys used to protect management message integrity and transport the TEKs are derived from source key material generated by the authentication and authorization processes. The ECC/RSA-based authorization process yields the pre-Primary AK (pre-PAK. All SCM key derivations are based on the Dot22KDF algorithm as defined in 7.2.5.7.

7.2.5.1 AK derivation

The AK will be derived by the BS and the CPE from the PAK (from ECC/RSA-based authorization procedure). Note that PAK can be used according to the value of Authorization Policy Support field included in the CBC-REQ/RSP messages.

After the authorization procedure has been performed, the CPE and BS will both posses the PAK. The derivation of the AK varies based on which keys are possessed.

The AK shall be generated as follows:

AK ⇐ Dot22KDF (PAK, CPE MAC Address | BSID | PAK | “AK”, 160)

7.2.5.2 KEK derivation

The KEK is derived directly from the AK. It is used to encrypt the TEKs, GKEK and all other keys sent by the BS to CPE in unicast message.

7.2.5.3 GKEK derivation

There are two methods for GKEK generation. If 0x05 is among the crytptographic suites (See Table 7.4-1) that is selected for a particular GSA, then GKEK is randomly generated at the BS or a network entity (for example, an ASA server) and transmitted to the CPE encrypted with the KEK. If 0x06 is among the cryptographic suites (See Table 7.4-1), then a pre-arranged an operator specific method can be used to drive GKEK derivation. When using the operator specific method, the KEK is used to transport the keying material or any other data the operator specific method requires for GKEK generation. There is one GKEK per Group Security Association. GKEK is used to encrypt the GTEKs sent by the BS to the CPEs in the same multicast/broadcast group.

7.2.5.4 Traffic encryption key (TEK)

The TEK is generated as a random number in the BS and is encrypted using the corresponding TEK encryption algorithm (e.g., AES key wrap for SAs with TEK encryption algorithm identifier in the cryptographic suite is equal to 0x01-0x04), keyed with the KEK and transferred between BS and CPE in the TEK exchange.

7.2.5.5 Group traffic encryption key (GTEK)

The GTEK is used to encrypt data packets of the multicast/broadcast service and it is shared among all CPEs that belong to the multicast/broadcast group. There are two GTEKs per GSA. Just as with TEKs, GTEKs will have overlapping lifetimes. Regardless of the cryptographic suite selected for GTEK generation (e.g. 0x05 or 0x06), the GTEK is encrypted using same algorithms applied to encryption for TEK and transmitted to the CPE in broadcast or unicast messages. The GTEK in a SCM Key-Reply message shall be encrypted by the GKEK or the KEK. Also, the GTEK in a SCM Group Key Update Command message will be encrypted by the GKEK.

7.2.5.6 Management Message Protection key and KEK derivation

7.2.5.6.1 MMP_PN management

The CPE shall maintain a MMP_PN counter for each AK. The BS is assumed to maintain a MMP_PNcounter for each AK context as well. This is done to keep the MMP_PN value synchronized with the corresponding counter at the CPE. The value of this counter maintained by the CPE is denoted as MMP_PNC and the value maintained by the BS is denoted as MMP_PNB.

7.2.5.6.1.1 Maintenance of MMP_PNC by the CPE

Upon successful completion of the SCM Authorization or Re-authorization, and establishment of a new AK, the CPE shall instantiate a new MMP_PN counter and set its value to 1. The CPE shall initiate re-authorization before the MMP_PNC expires, e.g. reaches the half of the counter space (0x7FFFFF)., The AK Lifetime (See 7.3.1.1) shall not be set to a value less than the expected amount of time required to for the MMP_PN to be exhausted. The CPE shall manage a separate MMP_PNC counter for every active AK context. Specifically, during re-authorization, but before the activation of the new AK, the old MMP_PNC (corresponding to the old AK) shall be used signing and/or encryption of MAC control messages, while the new MMP_PNC shall be used for signing and/or encrypting of SCM Key Request messages. If CPE should lose its AK context, then CPE shall restart the authentication process.

7.2.5.6.1.2 Processing of MMP_PNB by the BS

The BS may possess one or more AK contexts associated with the CPE, each of which includes the value of MMP_PNB. This value shall be maintained as specified in subsequent paragraphs of this section.

Upon successful completion of the SCM Authorization and Re-authorization, and establishment of a new AK context, the BS shall set MMP_PNB of the corresponding newly instantiated AK context to 1. The BS shall manage a separate MMP_PN_U/DB for every AK context it is maintaining. Specifically, during re-authorization, but before the activation of the new AK, the old MMP_PNB (corresponding to the old AK context) shall be used for signing and/or encryptionof MAC control messages, while the new MMP_PNB shall be used for signing and/or encrypting of SCM Key Replymessages.

During periodic ranging, the CPE can optionally transmit the RNG-REQ containing the MMP_PN parameter, the BS shall compare the received MMP_PN value, which is MMP_PNC, with MMP_PNB (i.e., the value of MMP_PN counter maintained by the BS for the corresponding AK context). If MMP_PNC < MMP_PNB, the BS shall process the message as being invalid and send a RNG-RSP message requesting re-authorization. This can happen when either the BS or CPE loses the AK context.

7.2.5.6.2 Derivation of Management Message Protection (MMP) keys and KEKs

MMP keys are used to sign and/or encrypt management messages in order to validate the authenticity and protect the contents of these messages. The level of protection to be used for management messages is negotiated during the Authorization exchange.

There is a different key for US and DS messages. Also, a different message authentication key is generated for a broadcast message (this is DS direction only) and for a unicast message.

The MMP_KEY and KEK are derived are as follows:

MMP_PREKEY | KEK ⇐ Dot22KDF(AK, CPE MAC Address | BSID | “MMP_KEY+KEK”, 256)

MMP_KEY ⇐ AESMMP_PREKEY (MMP_PN)

For a fixed CPE, the MMP_PN shall be set to 0 in the derivation of the MMP_KEY at the BS and the CPE. Specifically, the preprocessed value of MMP_PREKEY is treated as the Cipher Key of the Advanced Encryption Standard (AES) algorithm AES128 [FIPS197]. The MMP_PN is treated as the Input Block Plain Text of this algorithm. The AES128 algorithm is executed once. The Output Block Cipher Text of this algorithm is treated as the resulting MMP_KEY. When MMP_PN is used as an input of AES128 algorithm, 104 zero bits are prepadded before the 24-bit MMP_PN where the MMP_PN is regarded as most-significant-bit first order.

7.2.5.7 Key Derivation Function

The Dot22KDF algorithm is a CTR mode construction that may be used to derive an arbitrary amount of keying material from source keying material.

Dot22KDF(key, astring, keylength)

{

result = null;

Kin = Truncate (key, 128);

for (i = 0; i < ceil((keylength-1)/128); i++) {

result = result | AESKin(i | astring | keylength | result);

}

return Truncate (result, keylength);

}

7.2.6 Key hierarchy

Figure 7.2-3 outlines the process to calculate the AK when the RSA/ECC-based authorization process has taken place.

Figure 7.2-3. AK from PAK Only (from RSA/ECCbased authorization)

Figure 7.2-4 outlines the unicast key hierarchy starting from the AK.

Figure 7.2-4. MMP / KEK derivation from AK

7.2.7 Maintenance of AK

The BS and CPE maintain cached AK as follows:

- Successful completion of the SCM ECC/RSA Authorizaiton and establishment of a PAK, causes the activation of all the AK associated with the new PAK.

- If the packet counter belonging to MMP key reaches its maximum value, the associated AK becomes permanently deactivated.

- The BS and CPE must maintain the AK context (i.e., replay counters etc.) as long as they retain the AK.

7.2.8 AK switching methods

Once the SCM Key Request/Key Reply handshake begins, the BS and CPE shall use the new AK for the 3-way handshake messages. Other messages shall continue to use the old AK until the 3-way handshake completes successfully. Upon successful completion of the 3-way handshake, all messages shall use the new AK. The old AK may be used for receiving packets until the expiration of the AK Grace Time (Section 7.3).

7.2.9 Security Associations

Keying material is held within associations. There are two types of association: The security associations (SA) that maintain keying material for unicast connections; and group security associations (GSA) that hold keying material for multicast groups. If CPEand BS decide “No authorization” as their authorization policy, they do not have any security association. In this case, Null SAID shall be used as the target SAID field in DSA-REQ/RSP messages.

7.2.9.1 Security associations

A security association contains keying material that is used to protect unicast connections. The contents of an SA are as follows:

- The SAID, a 16-bit identifier for the SA. The SAID shall be unique within a BS.

- The KEK, a 128-bit key encryption key, derived from the AK.

- MMP Keys for encryption of upstream and downstream management traffic, MMP_KEY

- MMP_PN, 24 bit packet numbers for use by link cipher to protect management messages

- RxMMP_PN, 24-bit receive sequence counter, for use by link cipher to protect management messages

- TEK0 and TEK1, 128-bit traffic encryption keys, generated within the BS and transferred from the BS to the CPE using a secure key exchange.

- The TEK Lifetimes TEK0 and TEK1, a key aging lifetime value.

- PN0 and PN1, 24-bit packet numbers for use by the link cipher.

- RxPN0 and RxPN1, 24-bit receive sequence counter, for use by the link cipher.

7.2.9.2 Group Security Association

The Group Security Association (GSA) contains keying material used to secure multicast or broadcast transmissions. These are defined separately from SAs since GSA offer a lower security bound than unicast security associations, since keying material is shared between all members of the group, allowing any member of the group to forge traffic as if it came from any other member of the group.

The contents of a GSA are as follows:

- The SAID (GSAID), a 16-bit identifier for the GSA. The SAID shall be unique within the BS

- The Group Key Encryption Key (GKEK). Serves the same function as an KEK but for a GSA.

- The Group Traffic Encryption Key (GTEK). Served the same function as an SA TEK but for a GSA.

  - GTEK0 and GTEK1, 128-bit traffic encryption keys

- The GTEK Lifetimes for GTEK0 and GTEK1, a key aging lifetime value

- GPN0 and GPN1, 24-bit packet numbers for use by the link cipher

- RxGPN0 and RxGPN1, 24 bit receive sequence counter, for use by the link cipher

7.2.10 Security context

The security context is a set of parameters linked to a key in each hierarchy that defines the scope while the key usage is considered to be secure. Examples of these parameters are key lifetime and counters ensuring the same encryption will not be used more than once. When the context of the key expires, a new key should be obtained to continue working.

The purpose of this subclause is to define the context that belongs to each key, how it is obtained and the

scope of its usage.

7.2.10.1 AK context

The AK key has two phases of lifetime: the first begins at AK creation and the second begins after validation by the 3-way handshake.

The phases ensure that when the AK is created it will be defined with the PAK pre-handshake lifetime and after successful 3-way handshake.

If the cached AK and associated context is lost by either BS or CPE, no new TEKs can be derived from this AK on handover. Cached AKs that were derived from the PMK can continue to be used in HO. Reauthentication is required to obtain a new AK so as to derive new TEKs.

The AK context is described in Table 7.2-3.

Table 7.2-3 – AK Context in SCM

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| AK | 160 | The authorization key, calculated as defined in 7.2.2.2.3. |
| AK Lifetime | 32 | This is the time this key is valid; it is calculated AK lifetime = PAK lifetime = Authorization Grace Time. When this expires, re-authorization is needed. |
| AK Sequence Number | 4 | The sequence number of the PAK from which this AK is derived. |
| KEK | 128 | Used to encrypt transport keys from the BS to the CPE. |
| MMP_KEY | 128 | The key which is used for signing and/or encrypting DS/US management messages |
| MMP_PN | 24 | Used to avoid DS/US replay attack on management connection. When this expires re-authentication is needed. |

7.2.10.2 GKEK context

The GKEK is the head of the group key hierarchy. There is a separate GKEK for each group (each GSA).

This key is randomly generated by the BS and transferred to the CPEencrypted with KEK. It is used to encrypt group TEKs (GTEK) when broadcasting them to all CPEs. The GKEK context is described in Table 7.2-4.

Table 7.2-4 – GKEK Context

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| GKEK | 128 | Used to encrypt transport keys from the BS to the CPE |
| GKEK Sequence Number | 4 | The sequence number of the GKEK. The new GKEK sequence number shall be one greater than the preceding GKEK sequence number. |
| GKEK Lifetime | 32 | This is the time this key is valid; prior to expiration a new GKEK should be obtained. |
| GTEK0 | 128 | Older of two keys used to sign and/or encypt multicast/broadcast management and data traffic messages |
| GTEK1 | 128 | Newer of two keys used to sign and/or encypt multicast/broadcast management and data traffic messages |
| GPN0 | 24 | Packet number counter associate with older of two keys used to sign and/or encrypt multicast/broadcast management and data traffic messages. Used to avoid DS/US replay attack on multicast/broadcast connection. When this expires re-keying is needed. |
| GPN1 | 24 | Packet number counter associate with older of two keys used to sign and/or encrypt multicast/broadcast management and data traffic messages. Used to avoid DS/US replay attack on multicast/broadcast connection. When this expires re-keying is needed. |

7.2.10.3 PAK context

The PAK context includes all parameters associated with the PAK. This context is created when RSA

Authentication completes.

The PAK context is described in Table 7.2-5.

Table 7.2-5 – PAK Context

| Parameter | Size (bits) | Usage |
| --- | --- | --- |
| PAK | 160 | A key yielded from ECC/RSA-based authentication |
| PAK Lifetime | 4 | PAK lifetime, from when the ECC/RSA-based authorization is achieved. The value of the PAK lifetime is initially set to a default value, Authorization Grace Time. The Authorization exchange may subsequently change this value. |
| PAK Sequence Number | 4 | PAK sequence number, when the ECC/RSA-based authorization is achieved and a key is generated. The MSB 2 bits are the sequence counter, and the least significant bits are set to 0. |

7.3 Key Usage

7.3.1 BS Key Usage

The BS is responsible for maintaining keying information for all SAs. The SCM protocol defined in this specification describes a mechanism for synchronizing this keying information between a BS and its client CPE.

7.3.1.1 AK Lifetime

At initial network entry, if the security is enabled during the basic capabilities negotiation, the authorization procedure shall be initiated. The authorization procedure activates a new AK. This AK shall remain active until it expires according to its predefined Authorization Grace Time, a BS system configuration parameter. In SCM, AK lifetime is determined the PAK lifetime or by the expiration of the MMP_PN.

A CPE will initiate re-authorization before the expiration of it’s current AK. This leads the the CPE to have two active sets of keying material. If an CPE fails to reauthorize before the expiration of its current AK, the BS shall hold no active AKs for the CPE and shall consider the CPE unauthorized. A BS shall remove from its keying tables all TEKs associated with an unauthorized CPEs SA.

7.3.1.2 AK Transition Period on BS

The BS shall always be prepared to start re-authentication upon request. The BS shall be able to support two simultaneously active AKs for each client CPE. The BS has two active AKs during an AK transition period; the two active keys have overlapping lifetimes.

In SCM, an AK transition period begins when the BS receives an Auth Request message from an CPE and the BS has a single active AK for that CPE. In response to this Auth Request, the BS activates a second AK [see point (a) and (d) in Figure 7.3-1], which shall have a key sequence number one greater (modulo 16) than that of the existing AK and shall be sent back to the requesting CPE in an Auth Reply message. The BS shall set the active lifetime of this second AK to be the remaining lifetime of the first AK [between points (a) and (c) in Figure 7.3-1], plus the predefined AK Lifetime; thus, the second, “newer” key shall remain active for one AK Lifetime beyond the expiration of the first, “older” key. The key transition period shall end with the expiration of the older key. This is depicted on the right-hand side of Figure 7.4-1.

As long as the BS is in the midst of an CPE AK transition period, and thus is holding two active AKs for that CPE, it shall respond to Auth Request messages with the newer of the two active keys. Once the older key expires, an Auth Request shall trigger the activation of a new AK, and the start of a new key transition period.

Figure 7.3-1 – AK management in BS and CPE

7.3.1.3 BS usage of AK

The BS shall make use of keys derived from the CPE’s AK for the following purposes:

- Verify MAC-Digests and/or decrypt (See 7.4) of SCM Key Request messages as well as all other non-SCM related management messages received from CPE

- Calculate MAC-Digests and/or encrypt (See 7.4) for SCM Key Reply messages as well as other non-SCM related management messages sent to CPE

- Encrypting TEK when it is sent to CPE in the Key Reply message

The MMP_KEY is used to verify MAC Digests and/or decrypt (see 7.4) of SCM Key Request messages as well as other non-SCM related management messages received from CPE. The PN used in this operation is MMP_PN (See 7.2.5.6.1.2). When a CPE issues an SCM Key Request message, it shall include the AK Sequence Number, to indicate that it is using the newer of the two AKs that it is assigned.

The MMP_KEY is used to verify MAC Digests and/or decrypt (see 7.4)of SCM Key Reply messages as well as other non-SCM related management messages sent to the CPE. The PN used in this operation is MMP_PN (See 7.2.5.6.1.1). If the BS has been informed by the CPE that the Key Request issued by the CPE was issued with the newer AK, then BS shall use the MMP_KEY derived from the new AK and initialize MMP_PN associated with the new AK to 0. If the BS has no knowledge that the CPE is using the new AK, then it will use the MMP_KEY derived from the old AK and use the current value of MMP_PN associated with the older AK. The CPE indicates which AK is being used by including the AK Sequence Number of that AK in the Key Request message.

Prior to expiration of the MMP_PN, when half the key space (2^23) has been used up, the CPE conduct re-authorization.

7.3.1.4 TEK/GTEK lifetime

The BS maintains two, active TEKs per SA or GTEKs per GSA. Both TEKs/GTEKs will have overlapping lifetimes. TEK/GTEK Lifetime is a system parameter that determines the length of time that the TEK/GTEK is valid for. TEK/GTEK Lifetime is only invalidated when the PN associated with that TEK is about to expire. The TEK/GTEK Lifetime is communicated to the CPE, along with both generations of TEKs, from the BS in the SCM Key Reply message.

In the GMH, the usage of the newer TEK/GTEK is indicated in the value of the EKS field. For the newer TEK/GTEK, the EKS field is 1 greater (modulo 4) than that of the older TEK/GTEK.

7.3.1.5 BS usage of TEK and GTEK

Two generations of TEKs shall be maintained per SA, and two generations of GTEKs shall be maintained per GSA. Transitioning between both generations of TEKs/GTEKs and how they are used are dependent on whether or not the TEK/GTEK is used for DS or US traffic.

The BS transitions from using the older generation to the newer generation (for each of its’ SAs) based on the following rules:

- At the expiration of the older TEK/GTEK, the BS shall immediately transition to using the newer TEK/GTEK for encryption.

- The transitional period for the encrypting US traffic begins when the BS issues the newer TEK/GTEK to the CPE in the SCM Key Reply message and concludes that the older TEK/GTEK has expired (i.e. it’s PN has expired)

The BS makes use of both generations of TEKs/GTEKs, based on the following rules:

- BS shall use the older TEK/GTEK for encrypting DS traffic.

- BS shall be able to decrypt US traffic using either of the two TEK

It is the responsibility of the CPE to update its keys in a timely fashion; the BS shall transition to a new DS encryption key regardless of whether a client CPE has retrieved a copy of that TEK/GTEK.

Note that the BS encrypts with a given TEK/GTEK for only the second half of that TEK’s total lifetime (See Figure 7.3-2). The BS is able, however, to decrypt with a TEK/GTEK for the TEKs/GTEKs entire lifetime.

Figure 7,3-2 - TEK Management in BS and CPE

7.3.2 CPE Key Usage

CPEs shall maintain an active AK and stay authorized with the BS. This requires the CPE to initiate re-aurhorization periodically. Upon completing authorization and re-authorization, the CPE shall also be responsible for maintaining active keying material (e.g. TEKs) for encrypting DS and US traffic.

7.3.2.1 CPE Reauthorization

AKs have a limited lifetime and shall be periodically refreshed. In SCM, a CPE refreshes its AK by reissuing an Authorization Request to the BS. The Authorization State Machine (See 7.2-1) manages the scheduling of Authorization Requests for refreshing AKs. In SCM RSA/ECC-based authorization, the CPE refreshes its AK by issuing a SCM Auth-Request message. In ECC-based authorization the CPE refreshes its AK by issuing a SCM ECC-Request message.

In SCM, an CPE’s Authorization state machine schedules the beginning of reauthorization a configurable duration of time, the Authorization Grace Time, [see points (x) and (y) in Figure 7.3-1], before the CPE’s latest AK is scheduled to expire. The Authorization Grace Time is configured to provide an CPE with an authorization retry period that is sufficiently long to allow for system delays and provide adequate time for the CPE to successfully complete an Authorization exchange before the expiration of its most current AK. The Authorization Grace Time should be set at a value that doesn’t expire for the MMP_PN expires.

Note that the BS does not require knowledge of the Authorization Grace Time. The BS, however, shall track the lifetimes of its AKs and shall deactivate a key once it has expired.

7.3.2.2 CPE usage of AK

A CPE shall use the MMP_KEY derived from the newer of the two AKs it has when calculating MAC Digests and/or encryption of SCM Key Reqeust, and other non-SCM related management messages that are transmitted to the BS.

The CPE shall be capable of using the MMP_KEY derived from either one of its AKs to verify and/or decrypt SCM Key Reply, SCM TEK Invalid, and other non-SCM related management messagests thare are received from the BS.

A CPE shall use the MMP_KEY derived from the newer of its two most recent AKs when calculating the MAC-Digests of management messages it transmits to the BS.

7.3.2.3 CPE usage of TEK and GTEK

Through operation of its TEK state machines, a CPE shall maintain two, successive sets of keying material for encrypting traffic per SA. The CPE schedules requests for a new set of traffic keying material, based a configurable amount of time, the TEK/GTEK Lifetime [see points (x) and (y) in Figure 7.3-2], before the CPE’s latest TEK is scheduled to expire.

For each of its authorized SAIDs, the CPE

- Shall use the newer of its two TEKs to encrypt US traffic

- Shall be able to decrypt DS traffic encrypted with either of the TEKs.

The left-hand side of Figure 7.3-2 illustrates the CPE’s maintenance and usage of an SA’s TEKs, where the shaded portion of a TEK’s lifetime indicates the time period during which that TEK shall be used to encrypt MAC PDU payloads.

GTEKs shall be treated in the same manner as TEKs, with regard to how they are managed (see Figure 7.3-2).

7.4 Cryptographic Methods

This subclause specifies the cryptographic algorithms and key sizes used by the SCM protocol. All CPE and BS implementations shall support the method of packet data encryption and authentication defined in 7.4.2, and encryption of the TEK as specified in 7.4.1

All inputs to key derivation and other supporting functions shall be byte aligned. Furthermore, each byte shall be in canonical form as defined in IEEE Std. 802-2001 where the leftmost bit in each byte is the most significant bit and the rightmost bit is the least significant bit.

### 7.4.1 Selection of Data Encryption and Authentication Methods

Only one data encryption and authentication algorithm is supported in 802.22, AES in CCM mode, therefore no specific configuration item is required to define the use of AES CCM. The parameters (see Table 318) associated with SCM Authorization Request/Reply define how AES CCM is to be applied in order to provide authentication and/or encryption for MAC PDU payloads. The cryptographic suite configuration is made up of selecting an Authentication Method, and Encryption Method, and a TEK Encryption method.

Possible values for Authentication method are:

- No Authentication

- Authentication for Unicast

- Authentication for Multicast/Broadcast

Possible values for Encryption method are:

- No Encryption

- Encryption for Unicast

- Encryption for Multicast/Broadcast

Possible values for TEK/GTEK Encryption Method are:

- AES-128 key wrap of TEK/GTEK using KEK/GKEK

The cryptographic suite list is defined in Table 7.4.-1 is a 1-byte construct defined in the following table:

Table 7.4-1

| Value | Cryptographic Suite |
| --- | --- |
| 0x00 | No Authentication, No Encryption |
| 0x01 | Authentication for Unicast, No Encryption, AES-128 key wrap of TEK using KEK |
| 0x02 | Authenticaion and Encryption for Unicast, AES-128 key wrap of TEK using KEK |
| 0x03 | Authentication for Multicast/Broadcast, No Encryption, AES-128 key wrap of GTEK with GKEK |
| 0x04 | Authentication and Encryption for Multicast/Broadcast, AES-128 key wrap of GTEK with GKEK |
| 0x05 | BS random generation of GKEK and GTEK |
| 0x06 | Operator-specific function for GKEK and GTEK generation |
| 0x07 | RSA-based authorization |
| 0x08 | ECC-based authorization |
| 0x09-0xFF | Reserved |

In Table 318, a configuration parameter for the list of Cryptopgrahic Suites supported will be transmitted to the BS by the CPE in the SCM RSA/ECC Authorization Information, Request and Authorization Reply messages.

7.4.2 Data Encryption and Authentication with AES CCM

If the cryptographic suite selected (see Table 7.4-1 for operation during the Authorization process is 0x01-0x04 than AES in CCM [NIST Special Publication 800-38B and FIPS 197] mode to provide authentication and/or encryption [known as CCM* in IEEE 802.15.4-2006] of MAC PDUs.

7.4.2.1 PDU Format

7.4.2.1.1 Packet Number

The MAC PDU payload shall be prepended with a 3-byte PN (Packet Number). The PN shall be encoded in the MAC PDU least significant byte first. The PN shall not be encrypted.

The PN associated with an SA shall be set to 1 when the SA is established and when a new TEK is installed. Upon completion of Authorization/Re-Authorization and after the MMP_KEY has been derived has been derived, the MMP_PN is set to 1. After each PDU transmission, the PN and MMP_PN shall be incremented by 1.

When admitting a CPE to an existing multicast/broadcast group, the BS will take the current value of the PN related to the newest generation of material for that GSA, and increment by 1 when establishing. The maximum number of CPEs that can be admitted to a multicast/broadcast group simultaneously is one half the PN_WINDOW_SIZE (see 7.4.2.3).

On DS connections, the PN shall be XORed with 0x800000 prior to encryption and transmission. This effectively splits the PN space into two ranges for DS (0x000000-0x7FFFFF) and DS (0x800001-0xFFFFFF), thereby avoiding collision of PN values when using a single PN for DS and DS. On DS connections, the PN shall be used without such modification.

Any tuple value of {PN, KEY} shall not be used more than once for the purposes of transmitting data. This measure is known a protection against replay attacks.

The CPE shall ensure that a new TEK is requested and transferred before the PN on either the CPE or BS reaches 0x7FFFFFFF. If the PN in either the CPE or BS reaches 0x7FFFFFFF without new keys being installed, transport communications on that SA shall be halted until new TEKs are installed. In the case of the MMP_KEY, if MMP_PN expires, then current AK is invalidated and must start Re-Authorization.

7.4.2.1.2 PDU Format – Authentication Only

The cyphersuites allow for authentication and/or encryption of MAC PDUs. If the suites 0x01 or 0x03 is assigned to the SA, then only authentication is provided for any MAC PDUs transmitted on service flows that are mapped to these SAs.

The AES in CCM protocol is applied in the following manner:

- The Plaintext Payload is processed, generating an Integrity Check Value (ICV) that is 8 bytes long.

- Only the ICV is encrypted using the active TEK/GTEK, generating the Ciphertext ICV

- The Authenticated PDU is formed by appending the Ciphertext ICV to the Plaintext Payload form the authenticated PDU

This requires the EC bit in the GMH to be set to 0. If EC bit is not set to 0, the the PDU shall be discarded, as this would indicate a conflict between the configured cryptographic suite and how it is being applied.

Figure 7.4-1 illustrates how MAC PDUs are processed and formatted when suite 0x01 or 0x03 is configured and the EC bit in GMH is set to 0. The Ciphertext ICV is transmitted so that byte index 0 [as enumerated in NIST Special Publication 800-38] is transmitted first and byte index 7 is transmitted last (i.e., LSB first).

Figure 7.4-1 – Authentication Only PDU Format

7.4.2.1.3 PDU Format – Authentication and Encryption

The cyphersuites allow for authentication and/or encryption of MAC PDUs. If the suites 0x02 or 0x04 is assigned to the SA, then authentication and encryption is provided for any MAC PDUs transmitted on service flows that are mapped to these SAs.

The AES in CCM protocol is applied in the following manner:

- The Plaintext Payload is processed, generating an Integrity Check Value (ICV) that is 8 bytes long.

- Then the ICV is encrypted using the active TEK/GTEK, generating the Ciphertext ICV

- Then the Plaintext Payload is then encrypted with AES using the active TEK/GTEK, generating a Ciphertext Payload

- The encrypted PDU is formed by appending the Ciphertext ICV to the Ciphertext Payload

This requires the EC bit in the GMH to be set to 1. If EC bit is not set to 1, the the PDU shall be discarded, as this would indicate a conflict between the configured cryptographic suite and how it is being applied.

Figure 7.4-2 illustrates how MAC PDUs are processed and formatted when suite 0x02 or 0x04 is configured and the EC bit in GMH is set to 1. The Ciphertext ICV is transmitted so that byte index 0 (as enumerated in NIST Special Publication 800-38) is transmitted first and byte index 7 is transmitted last (i.e., LSB first).

Table 7.4-2 – Encrypted PDU Format

7.4.2.2 CCM Algorithm Constraints

The CCM specification [NIST SP 800-38B] defines specific values for several parameters.

The Tlen parameters specifices then length of the ICV (otherise known as Message Authentication Code, MAC) in bits. This value, as stated in section 7.4.2.1, shall be 64 bits (8bytes).

Consistent with the CCM specification, the 3-bit binary encoding [(t–2)/2)]3 of bits 5, 4, and 3 of the Flags byte in B0 shall be 011.

The size q of the Length field Q shall be set to 2. Consistent with the CCM specification, the 3-bit binary encoding [q-1]3 of the q field in bits 2, 1, and 0 of the Flags byte in B0 shall be 001.

The length a of the associated data string A shall be set to 0.

The nonce shall be 13 bytes long as shown in Figure 7.4-3. Bytes 0 through 3 shall be set to the first 4 bytes of the generic MAC header (thus excluding the HCS). The HCS of the generic MAC header is not included in the nonce since it is redundant. Bytes 4 through 9 are reserved and shall be set to 0x000000000000. Bytes 10 through 12 shall be set to the value of the PN. The PN bytes shall be ordered so that byte 10 shall take the least significant byte and byte 12 shall take the most significant byte.

| Byte | 0 4 | 5 9 | 10 12 |
| --- | --- | --- | --- |
| Field | GMH | Reserved | PN |
| Contents | GMH (without HCS) | 0x0000000000 | PN field from Payload |

Figure 7.4-3 – Nonce Construction

Consistent with the CCM specification, the initial block B0 is formatted as shown in Figure 7.4-4.

| Byte | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte Significance |  |  | MSB LSB |
| Number of Bytes | 1 | 13 | 2 |
| Field | Flag | Nonce | L |
| Contents | 0x19 | As specificed in Figure 7.4.2-3 | Length of plaintext payload |

Figure 7.4-4 – Initial Block, B0, construction for CCM

Consistent with the NIST CCM specification, the counter blocks Ctrj are formatted as shown in Figure 7.4.2-5.

| Byte | 0 | 1 13 | 14 15 |
| --- | --- | --- | --- |
| Byte Significance |  |  | MSB LSB |
| Number of Bytes | 1 | 13 | 2 |
| Field | Flag | Nonce | Counter |
| Contents | 0x1 | As specificed in Figure 7.4.2-3 | i |

Figure 7.4-5 – Counter Block, Ctrj, construction for CCM

7.4.2.3 Receive Processing Rules

On receipt of a PDU the receiving CPE or BS shall decrypt and authenticate the PDU consistent with the NIST CCM specification configured as specified in 7.4.2.2.

Packets that are found to be not authentic shall be discarded.

Receiving BS or CPEs shall maintain a record of the highest value PN and MMP_PN received for each SA. The receiver shall maintain a PN window whose size is specified by the PN_WINDOW_SIZE parameter for SAs and management connections as defined in Table 318.

Any received PDU with a PN lower than the beginning of the PN window shall be discarded as a replay attempt. The receiver shall track PNs within the PN window. Any PN that is received more than once shall be discarded as a replay attempt. Upon reception of a PN, which is greater than the end of the PN window, the PN window shall be advanced to cover this PN.

7.4.3 Public Key Encryption of AK

AKs in Auth Reply messages shall be RSA or ECC public-key encrypted, using the CPE’s public key.

For RSA certificates, RSA public-key encryption uses 65537 (0x010001) as its public exponent and a modulus length of 1024 bits. The PKM protocol employs the RSAES-OAEP encryption scheme (PKCS #1). RSAES-OAEP requires the selection of a hash function, a mask-generation function, and an encoding parameter string. The default selections specified in PKCS #1 shall be used when encrypting the AK. These default selections are SHA-1 for the hash function, MGF1 with SHA-1 for the mask-generation function, and the empty string for the encoding parameter string.

For ECC certificates, ECC public key encryption is based upon the Elliptic Curve Integrated Encryption Scheme (ECIES) detailed in Section 5.1 of [SEC1 – insert proper reference]. Elliptic Curve domain parameters shall be selected based upon the recommendations in Section 7.5.1.5.2. The MAC and ENC schemes that are part of the ECEIS method will be handled by AES-CCM method as defined in Section 7.4.2.1.3. KDF to be used in ECIES will make use of Dot22KDF function defined in Section 7.2.5.7.

7.4.4 Digital Signatures

For RSA-based certificate, the RSA Signature Algorithm (PKCS #1) with SHA-1 [FIPS 186-2] shall be used. As with its RSA encryption keys, Privacy uses 65537 (0x010001) as the public exponent for its signing operation. Manufacturer CAs shall employ signature key modulus lengths of at least 1024 bits and no greater than 2048 bits.

For ECC-based certificate, the Elliptic Curve Digital Signature Algorithm (ECDSA) defined in Section 4 of [SEC1 – insert proper reference] shall be used. Elliptic Curve domain parameters shall be selected based upon the recommendations in Section 7.5.1.5.2. Domain parameters sets that are selected will produce keys of no less than 160 and no greater than 224 bits in length. SHA-1 shall be used as the hash function.

## 7.5 Certificate Profile

### 7.5.1 Certificate Format

This subclause describes the X.509 Version 3 certificate format and certificate extensions used in IEEE 802.22-compliant CPEs.  The X.509 Version 3 format is defined in IETF RFC 2459.  ASN.1 encoding of algorithm identifiers are also further described in IETF RFC 3279.  The basic X.509 Version 3 certificate format is retained from the reference system. Table 7.5.1-1 highlights the fields of a X.509v3 certificate.

Table 7.5.1-1- Fields of X.509 Version 3 Certificate

| X.509 Version 3 Field | Description |
| --- | --- |
| tbsCertificate.version | Indicates X.509 certificate version. Always set to 3 |
| tbsCertificate.serialNumber | Unique integer the issuing CA assigns to the certificate. |
| tbsCertificate.signature | Object identifier (OID) and optional parameters defining algorithm used to sign the certificate. This field shall contain the same algorithm identifier as the signatureAlgorithm field. |
| tbsCertificate.issuer | Distinguished Name of the CA that issued the certificate. |
| tbsCertificate.validity | Specifies when the certificate becomes active and when it expires. |
| tbsCertificate.subject | Distinguished Name identifying the entity whose public key is certified in the subjectpublic key information field. |
| tbsCertificate.subjectPublicKeyInfo | Field contains the public key material (public key and parameters) and the identifier of the algorithm with which the key is used |
| tbsCertificate.issuerUniqueID | Optional field to allow reuse of issuer names over time. |
| tbsCertificate.subjectUnique ID | Optional field to allow reuse of subject names over time. |
| tbsCertificate.extensions | The extension data. |
| signatureAlgorithm | OID and optional parameters defining algorithm used to sign the certificate. This field shall contain the same algorithm identifier as the signature field in tbsCertificate. |
| signatureValue | Digital signature computed upon the ASN.1 DER encoded tbsCertificate. |

All certificates described in this specification shall be based on RSA of ECC. With RSA, the RSA signature algorithm SHA-1 is used as the one-way hash function. The RSA signature algorithm is described in PKCS #1; SHA-1 is described in FIPS 180-1.

For ECC certificates elliptic curve domain parameters can be generated according to procedures defined in Section A.3 of ANSI X9.62-2005.  Example parameters sets of parameters can be found in FIPS 186-3 and ANSI X9.63-2001.  Domain parameters sets that are selected will produce keys of no less than 160 and no greater than 256 bits in length.

Restrictions posed on the certificate values are described in 7.5.1.x through 7.5.1.x.

#### 7.5.1.1 tbsCertificate.validity.notBefore and tbsCertificate.validity.notAfter

CPE certificates shall not be renewable and shall thus have a validity period greater than the operational lifetime of the CPE. A Manufacturer/ServiceProvider CA certificate’s validity period should exceed that of the CPE certificates it issues. The validity period of an CPE certificate shall begin with the date of generation of the device’s certificate; the validity period should extend out to at least 10 years after that manufacturing date. Validity periods shall be encoded as UTCTime. UTCTime values shall be expressed Greenwich Mean Time (Zulu) and shall include seconds (i.e., times are YYMMDDHHMMSSZ), even where the number of seconds is zero.

#### 7.5.1.2 tbsCertificate.serialNumber

Serial numbers for CPE certificates signed by a particular issuer shall be assigned by the manufacturer in increasing order. Thus, if the tbsCertificate.validity.notBefore field of one certificate is greater than the tbsCertificate.validity.notBefore field of another certificate, then the serial number of the first certificate shall be greater than the serial number of the second certificate.

#### 7.5.1.3 tbsCertificate.signature and signatureAlgorithm

Certificates can be signed with the RSA or ECDSA (ANSI X9.62-2005) algorithms.  ECDSA is the elliptic curve analog of the DSA signature algorithm.  The following two sub-sections define OIDs that represent values for tbsCertificate.signature and signatureAlgorithm fields of the X.509v3 certificate to describe each algorithm.

##### 7.5.1.3.1 RSA signature & signatureAlgorithm

The RSA signature algorithm (PKCS #1, IETF RFC 2313), which makes use of SHA-1 is described in FIPS 180-1) as the one-way hash algorithm.  The ASN.1 OID used to describe the RSA signature algorithm using SHA-1 is as follows:

sha-1WithRSAEncryption OBJECT IDENTIFIER ::=

{ iso(1) member-body(2) us(840) rsadsi(113549) pkcs(1) pkcs-1(1) 5}

The calculation and encoding of the signature value is described in PKCS #1 (IETF RFC 2313) specification.  The signature value is calculated over the rest of ASN.1 encoded certificate,  then inserted signatureValue field of the certificate.

##### 7.5.1.3.2 ECC signature & signatureAlgorithm

ECDSA itself is identified by OIDs arranged in the following manner:

ansi-X9-62  OBJECT IDENTIFIER ::= {

iso(1) member-body(2) us(840) 10045 }

id-ecSigType OBJECT IDENTIFIER  ::=  {

ansi-X9-62 signatures(4) }

ECDSA also uses SHA-1 as the one-way hash function.  The ASN.1 OID used to described the ECDSA signature algorithm using SHA-1 is as follows:

ecdsa-with-SHA1  OBJECT IDENTIFIER ::= {

id-ecSigType 1 }

The calculation and encoding of the signature value is described in ANSI X9.62-2005.  This process outputs two values (r and s).  The signature value is calculated over the rest of ASN.1 encoded certificate,  then inserted signatureValue field of the certificate using the following ASN.1 encoded data structure:

Ecdsa-Sig-Value  ::=  SEQUENCE  {

r     INTEGER,

s     INTEGER  }

#### 7.5.1.4 tbsCertificate.issuer and tbsCertificate.subject

X.509 Names are SEQUENCES of RelativeDistinguishedNames, which are in turn SETs of AttributeTypeAndValue. AttributeTypeAndValue is a SEQUENCE of an AttributeType (an OBJECT IDENTIFIER) and an AttributeValue. The value of the countryName attribute shall be a two-character PrintableString, chosen from ISO 3166; all other AttributeValues shall be encoded as either T.61/TeletexString or PrintableString character strings. The PrintableString encoding shall be used if the character string contains only characters from the PrintableString set. Specifically:

abcdefghijklmnopqrstuvwxyz

ABCDEFGHIJKLMNOPQRSTUVWXYZ

0123456789

’()+,–./:=? and space

The T.61/TeletexString shall be used if the character string contains other characters. The following OIDs are needed for defining issuer and subject Names in SCM certificates:

id-at OBJECT IDENTIFIER ::= {joint-iso-ccitt(2) ds(5) 4}

id-at-commonName OBJECT IDENTIFIER ::= {id-at 3}

id-at-countryName OBJECT IDENTIFIER ::= {id-at 6}

id-at-localityName OBJECT IDENTIFIER ::= {id-at 7}

id-at-stateOrProvinceName OBJECT IDENTIFIER ::= {id-at 8}

id-at-organizationName OBJECT IDENTIFIER ::= {id-at 10}

id-at-organizationalUnitName OBJECT IDENTIFIER ::= {id-at 11}

The following subclauses describe the attributes that comprise the subject Name forms for each type of SCM certificate. Note that the issuer name form is the same as the subject of the issuing certificate. Additional attribute values that are present but unspecified in the following forms should not cause a device to reject the certificate.

##### 7.5.1.4.1 Manufacturer/ServiceProvider certificate

countryName=<Country of Manufacturer/ServiceProvider>

[stateOrProvinceName=<state/province>]

[localityName=<City>]

organizationName=<Company Name>

organizationalUnitName=WirelessRAN

[organizationalUnitName=<Manufacturing Location>]

commonName=<Company Name> <Certification Authority>

The countryName, organizationName, and commonName attributes shall be included and shall have the values shown. The organizationalUnitName having the value “WirelessMAN” shall be included. The organizationalUnitName representing manufacturing location should be included. If included, it shall be preceded by the organizationalUnitName having value “WirelessMAN.” The stateOrProvinceName and localityName may be included. Other attributes are not allowed and shall not be included.

##### 7.5.1.4.2 CPE Certificate

countryName=<Country of Manufacturer/ServiceProvider>

organizationName=<Company Name>

organizationalUnitName=<manufacturing location>

commonName=<Serial Number>

commonName=<MAC Address>

The MAC address shall be the CPE’s MAC address. It is expressed as six pairs of hexadecimal digits separated by colons (:), e.g., “00:60:21:A5:0A:23.” The Alpha HEX characters (A–F) shall be expressed as uppercase letters.

The organizationalUnitName in an CPE certificate, which describes the modem’s manufacturing location, should be the same as the organizationalUnitName in the issuer Name describing a manufacturing location.

The countryName, organizationName, organizationalUnitName, and commonName attributes shall be included. Other attributes are not allowed and shall not be included.

##### 7.5.1.4.3 BS Certificate

countryName=<Country of Operation>

organizationName=< Name of Infrastructure Operator>

organizationalUnitName=<WirelessRAN>

commonName=<Serial Number>

commonName=<BS Id>

The BS Id field shall contain the operator-defined BSId.1 It is expressed as six pairs of hexadecimal digits separated by colons (:), e.g., “00:60:21:A5:0A:23.” The Alpha HEX characters (A-F) shall be expressed as uppercase letters.

#### 7.5.1.5 tbsCertificate.subjectPublicKeyInfo

The tbsCertificate.subjectPublicKeyInfo field contains the public key and the public key algorithm identifier. The following two subsections describe OIDs used to encode this information for RSA public keys (7.6.1.5.1) and ECDSA/ECDH public keys (7.6.1.5.2).

##### 7.5.1.5.1 RSA Public Keys

The OID that identify RSA encryption for a certificate are defined as follows:

pkcs-1 OBJECT IDENTIFIER ::= { iso(1) member-body(2) us(840)

rsadsi(113549) pkcs(1) 1}

rsaEncryption OBJECT IDENTIFIER ::= { pkcs-1 1}

The OID that identifies RSA public keys in a certificate is defined as follows:

RSAPublicKey ::= SEQUENCE {

modulus            INTEGER,    -- n

publicExponent     INTEGER  }  -- e

##### 7.5.1.5.2 ECDSA/ECDH Public Keys

ECDH (ANSI X9.63-2001) is the elliptic curve analog of Diffie-Hellman agreement.  The OIDs and parameters used to encode information are similar for ECDSA signatures and ECDH encryption, and arranged in the following manner:

ansi-X9-62 OBJECT IDENTIFIER ::=

{ iso(1) member-body(2) us(840) 10045 }

id-public-key-type OBJECT IDENTIFIER  ::= { ansi-X9.62 2 }

id-ecPublicKey OBJECT IDENTIFIER ::= { id-publicKeyType 1 }

Each elliptic curve public key is associated with a set of elliptic curve parameters.  The OIDs that define the elliptic curve parameters are arranged as follows:

EcpkParameters ::= CHOICE {

ecParameters  ECParameters,

namedCurve    OBJECT IDENTIFIER,

implicitlyCA  NULL }

If ecParameters are inherited from the certificate authority, then the implicitlyCA value is included in EcpkParameters and is set to NULL.  ecParameters and its components are defined as follows:

ECParameters ::= SEQUENCE {

version   ECPVer,          -- version is always 1

fieldID   FieldID,         -- identifies the finite field over

-- which the curve is defined

curve     Curve,           -- coefficients a and b of the

-- elliptic curve

base      ECPoint,         -- specifies the base point P

-- on the elliptic curve

order     INTEGER,         -- the order n of the base point

cofactor  INTEGER OPTIONAL -- The integer h = #E(Fq)/n

}

ECPVer ::= INTEGER {ecpVer1(1)}

Curve ::= SEQUENCE {

a         FieldElement,

b         FieldElement,

seed      BIT STRING OPTIONAL }

FieldElement ::= OCTET STRING

ECPoint ::= OCTET STRING

ECPoint represents the base point of an elliptic curve and can take on two forms, compressed and uncompressed [defined in ANSI X9.62-2005].  For certificates the encoding of ECPoint shall be supported by the uncompressed form.  The compressed form may (optionally) be used instead.

The elliptic curve domain parameters can be generated according to procedures defined in Section A.3 of ANSI X9.62-2005.  Example parameters sets of parameters can be found in FIPS 186-3 and ANSI X9.63-2001.

#### 7.5.1.6 tbsCertificate.issuerUniqueID and tbsCertificate.subjectUniqueID

The issuerUniqueID and subjectUniqueID fields shall be omitted for all of the SCM’s certificate types.

#### 7.5.1.7 tbsCertificate.extensions

##### 7.5.1.7.1 CPE Certificates

CPE certificates may contain noncritical extensions; they shall not contain critical extensions. If the KeyUsage extension is present, the keyAgreement and keyEncipherment bits shall be turned on, keyCertSign and cRLSign bits shall be turned off, and all other bits should be turned off.

##### 7.5.1.7.2 BS Certificates

Manufacturer/ServiceProvider certificates may contain the Basic Constraints extension. If included, the Basic Constraints extension may appear as a critical extension or as a noncritical extension. Manufacturer/ServiceProvider certificates may contain noncritical extensions; they shall not contain critical extensions other than, possibly, the Basic Constraints extension. If the KeyUsage extension is present in a Manufacturer/ServiceProvider certificate, the keyCertSign bit shall be turned on and all other bits should be turned off.

#### 7.5.1.8 signatureValue

In all three SCM certificate types, the signatureValue can contain either the RSA (with SHA-1) or ECDSA signature computed over the ASN.1 DER encoded tbsCertificate. The ASN.1 DER encoded tbsCertificate is used as input to the RSA signature function. The resulting signature value is ASN.1 encoded as a bit string and included in the Certificate’s signatureValue field.

### 7.5.2 Certificate Storage and Management

#### 7.5.2.1 Certificate Storage and Management in CPE

Manufacturer/ServiceProvider-issued CPE certificates shall be stored in CPE permanent, write-once memory. SSs that have factory-installed RSA private/public key pairs shall also have factory-installed CPE certificates. SSs that rely on internal algorithms to generate an RSA key pair shall support a mechanism for installing a manufacturer-issued CPE certificate following key generation. The CA certificate of the Manufacturer/ServiceProvider CA that signed the CPE certificate shall be embedded into the CPE software. If a manufacturer issues CPE certificates with multiple Manufacturer/ServiceProvider CA certificates, the CPE software shall include ALL of that manufacturer’s CA certificates. The specific Manufacturer/ServiceProvider CA certificate installed by the CPE [i.e., advertised in Authentication Information messages and returned by the management information base (MIB) object] shall be that identifying the issuer of that modem’s CPE certificate.

#### 7.5.2.2 Certificate Storage and Management in the BS

SCM employs digital certificates to allow BSs to verify the binding between an CPE’s identity (encoded in an X.509 digital certificate’s subject names) and its public key. The BS does this by validating the CPE certificate’s certification path or chain. Validating the chain means verifying the Manufacturer/ServiceProvider CA Certificate through some means.

7.6 Security Mechanisms for the Cognitive Functions

Unlike IEEE 802.11 and IEEE 802.16 standards based waveforms which usually operate in the licensed spectrum, cognitive radio based WRANs need to operate in the unlicensed spectrum or White spaces as secondary occupiers of the spectrum.  Mechanisms have been defined in various sections of the 802.22 standard to protect the incumbents who are the primary occupiers of the spectrum (e.g. spectrum sensing, geolocation, spectrum management, spectrum etiquette etc.). This section provides further details on how to protect the incumbents as well as the 802.22 sytems against various types of Denial of Service (DoS) attacks targeted at the cognitive functions of the 802.22 systems. As a result, the Security Sub-layer 2 has been introduced in the cognitive plane of the 802.22 entity. The cognitive plane consists of a Spectrum Sensing Function (SSF), a Geolocation Function and a Spectrum Manager (SM) at the BS or the Spectrum Automaton at the CPE.

Figure XCP1 (a) shows the 802.22 Protocol Reference Model (PRM) with its Cognitive Plane functions containing Security Sublayer 2 at the BS. Figure XCP1 (b) shows the 802.22 PRM with its Cognitive Plane functions containing Security Sublayer 2 at the CPE. Details on the PRM itself can be found in Section 6.2.

Organization - Section 7.6 defines detailed attributes and mechanisms that 802.22 systems are required to provide to ensure security and protection of the cognitive functions. Some cognitive plane security related mechanisms are an integral part of other cognitive functions required for the 802.22 system implementation such as Spectrum Sensing Function, Geolocation, Spectrum Manager, Spectrum Automaton, Management Plane procedures and functions etc. Since these functions are described in sections other than Section 7, Section 7.6 briefly describes the required security features and provides references as to where in IEEE 802.22 standard the mechanisms or the messaging for the same can be found. These security mechanisms are related to availability, authentication, authorization, identification, integrity, confidentiality and privacy. Section 7.6.7 describes collaborative sensing, and decision making to enhance security for WRAN systems and the incumbents. The information fusion and decision making mechanism are explained in greater detail in Section 9 on Cognitive Radio Capability. The theoretical background for collaborative sensing and decision making is provide in Annex E related to Collaborative Sensing and Decision Making to Provide Potection against the Spurious Signals. Some other security mechanisms related to the cognitive capability are also defined in greater detail in this section.  Section 7.6.8 is related to the CBP Authentication Mechanisms and Section 7.6.9 related to the 802.22.1 Wireless Microphone Beacon Authentication.

(a)

(b)

Figure XCP1 (a) PRM of the 802.22 BS showing Security Sublayer 2 which provides protection for the cognitive functions at the BS (b) PRM of the 802.22 CPE with Security Sublayer 2 which provides protection for the cognitive functions at the CPE

Some of basic security functions of this layer and the remediation measures required to protect these functions include:

7.6.1 Availability

This is the primary function of any network.  If there is a perception, real or not, that a particular type of network is unreliable due to service or security issues then that network type will not be utilized by WRAN operators or end-users.  In the case of cognitive networks availability refers to ability of the BSs to properly sense the available spectrum and make it available to the CPEs.   This means that the BSs must have built-in security mechanisms that shall:

  - Ensure the availability of the spectrum for the primary (incumbents) and the secondary (WRAN) users

  - Mitigate any DoS-type attacks against BS, CPE, and other supporting devices such as the ones used to generate 802.22.1 beacons.

The details of various types of sensing and classification mechanisms have been provided in Section 9, and the details for 802.22.1 beacon authentication has been described in Section 7.6.0

7.6.2 Authentication

This functionality provides assurance that the communicating parties, sender and receiver, are who they purport to be.  In cognitive networks there is the added problem of distinguishing between the valid incumbents of the spectrum and the secondary users.  WRAN operators must be able to:

  - Validate incumbent TV signals and the wireless microphone beacons

  - Detect and counter man-in-the-middle and similar type attacks that attempt to steal available spectrum space.

  - Detect and counter any spoofing and similar type attacks

  - Authenticate geolocation information

  - Authenticate co-existence information of neighboring WRAN systems

  - Detection and reporting of spurious transmissions from other CPEs.

The details on spectrum sensing and signal classification have been provided in Section 9, Section 7.6.8 describes mechanisms to authenticate the Co-existence Beaconing Protocol (CBP) used to exchange the co-existence information and Section 7.6.9 describes mechanisms to authenticate the 802.22.1 wireless microphone beacons. Authentication of the geolocation information will be described in the Management Plane Procedures Addendum at a future date.

7.6.3 Authorization

Different network entities will have different privilege levels.  For example, a BS may be authorized to forcibly remove an interfering CPE from the network.  In cognitive networks the ability of the BS spectrum manager to sense the available spectrum, make decisions regarding its use and enforce those decisions at the CPE level is an important authorization example.  For a cognitive network to properly function:

  - Only the authorized parties shall be allowed to configure the spectrum manager at the BS and the spectrum automaton at the CPE

  - Configuration information shall be identified and protected

  - The BS shall be authorized to remove a CPE from the network if it was found to cause interference to the incumbents

The authorization process is briefly described in Section 7.2. The BS has the ability at any time to de-authorize a CPE from its network. The mechanism on how this decision is made is described in Section 9, related to the Cognitive Radio Capabilities through various policy sets. The mechanisms for protecting configuration information will be described in the Management Plane Procedures Addendum at a future date.

7.6.4 Identification

Identification works hand-in-hand with authentication in assuring both incumbent and secondary spectrum users that the communicating entities are known.  To that end it is necessary that cognitive networks provide mechanisms that shall:

  - Positively identify transmitting/receiving BS and CPE equipment

  - Ensure that the identification methods employed cannot be compromised through spoofing or similar type attacks.

  - Protect against replay-type attacks which employ previously transmitted valid identifiers.

The MAC information elements to identify the transmitting receiving BS and CPE equipment are defined in Section 6. The security mechanisms to ensure identification and integrity have been described in Section 7.4 and 7.5.

7.6.5 Integrity

Integrity is the assurance that the information transmitted over the medium arrives at its destination unaltered. Integrity provides write protection for the content. This is especially difficult in wireless networks because of the uncontrolled nature of the medium.  Also the fact that certain portions of the data must be altered to ensure proper transmission and delivery (timestamps, source, destination etc.) compounds the problem.  Cognitive networks must:

  - Protect against Co-Existence Beaconing (“CBP”) falsification

  - Protect against replay-type attacks using previously transmitted valid data

The security mechanisms to authenticate and perform integrity check on the CBP packets is described in Section 7.6.8. The security mechanisms to provide some degree of protection against replay-type attacks has been provided in Section 7.4.

7.6.6 Confidentiality/Privacy

Confidentiality works closely with integrity to ensure read as well as write protection for data. This is usually carried out using encryption and ciphers that can operate at the link and higher layers.  One must account for the fact that the wireless medium is more sensitive to transmission errors due to propagation effects such as shadowing, fading as well as un-intentional interference.  This sensitivity can wreak havoc with complex ciphers and cause numerous re-transmissions resulting in wasted bandwidth.  With cognitive networks this sensitivity is especially troublesome because of the opportunistic nature of spectrum use by the secondary users and the fact that use of the spectrum is not guaranteed.  Cognitive networks, therefore, must provide:

  - Support for robust ciphers and encryption methods

  - Mechanisms to safeguard WRAN operator’s spectrum availability information from eavesdropping by competitors or would-be hackers.

The security mechanisms to ensure confidentiality and privacy are provided in Sections 6 and Section 7.4.

7.6.7 Collaborative Sensing and Decision Making to Enhance Security

7.6.7.1 Introduction

The essential functions of the Security Sublayer 2 at the cognitive plane shall be to provide protection for the incumbents as well as protection to the 802.22 systems against DoS attacks of various types. Threat model for the 802.22 systems has been discussed in [5 – PRM and Security Enhancements in 802.22]. Figure [Security_Sublayer_Cognitive_Plane] shows the security sublayer architecture for the Cogitive Plane. One of the most important mechanisms employed by this security sublayer is that of collaborative sensing and correlation with geolocation information from various sensors (CPEs) located in the network. This information from various sensors shall be combined in a certain way which is referred to as information fusion followed by decision making. Figure [Security_Sublayer_Cognitive_Plane] represents these steps which enable the 802.22 systems to provide protection against some types of DoS attacks. Since this functionality is essentially a part of the Spectrum Manager, the information fusion, decision making and policy mechanisms are defined in detail in Section 9, on Cogntive Radio Capability,[16] however a brief description is provided in the following section.

Figure Security_Sublayer_Cognitive_Plane Security Sublayer 2 at the Cognitive Plane

7.6.7.2 Collaborative Sensing Mechanisms

Co-operative or Collaborative sensing is a scheme wherein more than one Radio Frequency (RF) sensors are used to sense the spectrum. The local sensing information shall be combined with information from the incumbent database to see if any of the subscribers (CPEs) lie in the protected contour of an incumbent. This local sensing information is then passed on to a central node such as the BS which makes a decision on whether to make opportunistic usage of the spectrum.

Appendix E shows the benefits of collaborative sensing and information fusion to improve security in a network consisting of cognitive radios such as the one being considered for 802.22 systems. In Appendix E, various rules are considered for information fusion where it has been shown that rather than using simple OR or AND type rules, Voting based rules provide flexibility and accuracy needed to provide security protection in cognitive radio systems.

Collaborative sensing takes all it strength when the information fusion described above takes into account the knowledge on the geographic location of the specific sensing devices providing sensing reports.  The correlation of the reports of the sensing devices that may have sent an “Urgent Coexistence Situation (UCS)” message to the BS and their respective geographic location will provide powerful means to identify the characteristics of the sensed signal. This will, in turn improve the decision making to determine the presence of an incumbent and its type versus presence of other devices based on their expected extent of coverage.

In the case of DTV detection, the area where correlation will exist between sensing devices will be large and will be in the direction of the DTV transmitter.  In the case of a wireless microphone, the correlarion will exist over a more limited area.  This limited area correlation will also be used in the case of the detection of an 802.22.1 beacon and will help increasing the level of confidence of the decision that it is indeed an 802.22.1 beacon based on its expected area of coverage, and possibly reducing the need to rely on the full PPDU decoding.

Since all the information will be available at the BS, it will be up to the manufacturers to implement various levels of complexity in fusing the sensing reports and the geographic location of the sensing devices to come up with a reliable assessment of the presence of incumbents, that is true positives, while minimizing the probability of false positives. Since incumbents will need to be protected in all cases and in case of doubt, action will need to be taken to avoid interference to incumbents, more advanced data fusion and decision processes will have the advantage of avoiding false positives and false alarms and therefore reducing cases where WRAN systems would have to change frequency while it is not necessary.

Colloborative sensing and information fusion will provide protection to the primary users of the spectrum since the probability of missed detection reduces with the number of sensors. Collaborative sensing provides protection to the secondary users of the spectrum against DoS attacks since it is easy to fool one sensor into believing the state of the spectrum, however, it is difficult to fool many sensors located in proximal, but disparate geographical locations.

Hence, collaborative sensing, correlation with incumbent database, information fusion and decision making can provide protection to not just the primary users, but also to the secondary users of the spectrum.

The detailed mechanism on the information fusion and decision making is provided in Section 9, related to the Cognitive Radio Capability [16].

7.6.8 CBP Authentication Mechanisms

This section discusses a method to provide protection over CBP.  This method provides authentication and integrity protection for CBPs by using ECC-based implicit certificates to calculate a signature over the CBPs.  This signature is then added to the CBPs as an IE.  This method was derived from the method used to protect beacon messages transmitted by low powered, licensed devices operating in television broadcast bands as defined in the current IEEE P802.22 TG1 draft document (IEEE P802.22.1/D1).  The receiving station (CPE or BS) uses the certificate information to verify the signature.  CBP Protection can be provided using one of four options.  Figure 7.11.2-1 describes the CBP Protection scheme options.  Sections 7.6.8.1-7.6.8.3 describe the options in detail.

In Figure 7.7.2-1 the following is depicted:

- We are depicting tx of CBPs from Bss to another BS (over-the-air or through backhaul) or received by a CPE at cell-edge or w/in over-lapping area.

- If Rx station is CPE:

  - It can validate cert/signature or received CBP, then strip out that information before relaying CBP to its current serving BS, or

  - Just blindly relay the CBP to its serving BS

- Process by which this certificates are generated is outside the scope of the standard.  Definition of this process is left for the duty of an industry consortium formed to develop profiles for products based on this standard.

Figure 7.7.2-1 CBP Authentication Mechanisms

##### 7.6.8.1 CBP Protection Mode 1

Mode 1 of CBP Protection entails BS ECC implicit certificates and signatures (calculated using CMAC) to be transmitted in each CBP.  The format of the of the BS certificate data that is pre-distributed shall follow Table 7.7.2-1. The format of the signature is defined in Table 7.7.2-2. Option 1 is executed in the following manner:

1) Certificate authority provides certificate (w/ public key) and private key to each BS individually.

- BS adds certificate to CBP as an IE.  This may happen only via a periodic broadcast or during a certificate request/response (CERT-REQ/RSP). In these cases, it calculates the signature over the CBP IE's (CERT-REQ/RSP) and adds signature to CBP into the Signature IE of the CBP MAC PDU.

- BS certificate public key is not used directly to sign the CBP.  Instead it is fed into a Key Derivation Function (KDF), defined in Section 7.x.x.x (TBD).  The output, or CBP Signature Key is then used  by the signature function.  The KDF is called by the following manner; 		CBP_SIGNATURE_KEY_i = KDF(BS Implicit Certificate Public Key, BS ID | i | “CBP Signature Key”, 128). “i” in the formula refers to the current time stamp that is part of the signature IE (Table 7.11.2-4).

4) Signature will be calculated via CMAC (e.g. IETF RFC 4493).   CMAC output is 8 bytes.

4) Receiving station validates certificate & signature.  The receiving station then validates the BS implicit certificate.  If this validation fails, the receiving station drops the CBP.  If successful, the receiving stations uses the BS implicit certificate public key and time stamp from the signature IE to derive the CBP Signature Key, then recomputes the signature, and compares it to signature value in signature IE.  If there is a mismatch, then validation of CBP signature has failed, and receiving station will drop the CBP.

##### 7.6.8.2 CBP Protection Mode 2

Mode 2 of CBP Protection entails pre-distribution of BS ECC implicit certificates to receiving station.  This can be done via a SIM card, when CPE is activated, or via NCMS (for periodic certificate updates).  BS calculates signature (using CMAC) of CBP using its implicit certificate and adds the signature as an IE to each CBP. This option is applicable when sharing BS implicit certificates between BSs that belong to the same operator. The format of the of the BS certificate data that is pre-distributed shall follow Table 7.7.2-1. The format of the signature is defined in Table 7.7.2-2. Mode 2 is executed in the following manner:

1) Certificate Authority generates certificate (w/ public key) and private key for BSs.

2) Certificate Authority then distributes keys to:

a)  a service provider, who then would distribute certificates to all BSs within its network, possibly through NCMS

b) central database that it or service provider consortium provides, that houses certificates for all BSs in a given geo-graphical area, & individual providers use NCMS to gain certs for their own and competitor's  BSs

3) BS certificate public key is not used directly to sign the CBP.  Instead it is fed into a Key Derivation Function (KDF), defined in Section 7.x.x.x (TBD).  The output, or CBP Signature Key is then used  by the signature function.  The KDF is called by the following manner; 		CBP_SIGNATURE_KEY_i = KDF(BS Implicit Certificate Public Key, BS ID | i | “CBP Signature Key”, 128). “i” in the formula refers to the current time stamp that is part of the signature IE (Table 7.7.2-4).

4) BS calculates signature over each CBPs, adds signature to each CBPs via an IE

5) Signature will be calculated via CMAC.  CMAC output is 8 bytes.

6) Receiving station validates certificate & signature.  The receiving station then validates the BS implicit certificate.  If this validation fails, the receiving station drops the CBP.  If successful, the receiving stations uses the BS implicit certificate public key and time stamp from the signature IE to derive the CBP Signature Key, then recomputes the signature, and compares it to signature value in signature IE.  If there is a mismatch, then validation of CBP signature has failed, and receiving station will drop the CBP.

##### 7.6.8.3 CBP Protection Certificate and Signature Structures

The following subsections describe the structures of the certificate and signature information.  For Mode 2, the BS ECC implicit certificate is not transmitted over the air.

Table 7.7.2-1: Mode 1 and Mode 2 BS Implicit Certificate (BSIC)

| Item | Size | Description |
| --- | --- | --- |
| CA ID | 8 bits | Id of CA that issued implicit certificate to BS |
| Key Validity Date (Not Before) | 41 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Date (Not After) | 3 bits | # of years after Key Validity Date (Not Before) date that certificate expires. Encoded as follows: - 000 – 1 year - 001 – 2 years - 010 – 3 years - 011 – 4 years - 100 – 5 years - 101 – 10 years - 110 – 15 years - 111 – 20 years |
| Public Key Reconstruction Data | 33 byte | Key data used to reconstruct public key: - 33 bytes for 256 bit ECC keys |

Table 7.7.2-2: Mode 1 and Mode 2 Signature (added to 2nd Symbol of Self Coexistence Window)

| Item | Size | Description |
| --- | --- | --- |
| Key ID/Serial # | 10 bits | Serial # of key associated with BS implicit certificate. This is generated by the CA. |
| Time Stamp | 54 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit,encoded by different # of bits): - YYYY: 4 digit year, e.g. 2008; each Y is from 0-9 & is encoded by 4 bits, total is 16 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - ss: seconds, 00-59, 6 bits - .ss: 10 ms boundary, .000-.99, 7 bits - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Signature | 8 bytes | CMAC output is 128 bits (16bytes) signature is a truncation of this value to no less than 8 bytes |

##### 7.6.8.4 Certificate Generation, Processing, and Validation Requirements

###### 7.6.8.4.1 Certificate Generation Requirements

- Infrastructure as described in Figure 7.7.2-1 for certificate generation and distribution

- Recommended EC domain parameters to be used can be read from ANSI X9.63-2001 and FIPS 186-3

  - Domain parameters that provide 256 bit primes for ECs are to be used

- BS shall be identified by its 48bit MAC Address

- ‘to-be-signed certificate data’, IU construction differs between options 1/2 and options 3/4 (IU data used by CA in requirement 4a/4b below)

  - For option 3/4 IU = KeyID/Serial # || BS MAC Address || CA ID || Key Validity Date (Not Before) || Key Validity Date (Not After) ; these are fields from the Option 3/4 Implicit Certificate IE

  - For option 1/2 IU = KeyID/Serial # || BS MAC Address || CA ID || Key Validity Date (Not After) ; these are fields from the Option 1/2 Implicit Certificate IE

- If each operator is allowed to maintain it’s its’ own BS implicit certificates (i.e. act as its’ own Certificate Authority)

  - Operator will register its’ Certificate Authority ID when registering its’ Operator ID. Mechanism by registration is executed is outside the scope of the standard.

  - Operator’s shall share their own CA Root certificate with other operators that have BSs that border or overlap with BSs in their own network. The mechanism for CA Root Certificate sharing is outside the scope of the standard.

###### 7.6.8.4.2 Certificate Generation Process

- In order to start certificate generation process (see Figure 7.7.2-1) a request from the BS (or on behalf of BS) must be made to CA.

  - This process requires that BS generate its own ‘ephemeral’ key pair using established ECC domain parameters.  ECC domain parameters used for 'ephemeral' key pair generation shall be the same parameters used by CA for its own certificate and BS implicit certificate generation

  - Certificate generation initiation follows process as defined in Section 4.1 “Initiator Transformation” in SEC 4

- CA executes process as defined in Section 4.2 of SEC 4 (“Standards for Efficient Cryptography 4: Elliptic Curve Cryptography), with some exceptions.

###### 7.6.8.4.3 Certificate Validation Process

- Whether certificate information for a BS is received in Certificate IE or is distributed through the NCMS, a BS received that certificate shall validate it.

  - If certificate is received in Certificate IE & validation of certificate fails, then packet is dropped

  - BS or NCMS operator may check a CA’s CRL during installation and validation of other BSs implicit certificates

  - Verification of a BS Implicit Certificate is defined in Section 5 “ECQV Implicit Certificate Processing Transformation” of SEC 4

- The Certificate/License authority shall have its own certificate with it’s own EC key pair, that is generated from the EC domain parameters that are selected

  - ECC domain parameters that CA uses for generation of its' own certificate must be same as parameters used for BS implict certs and 'ephemeral' key pair generation

- The CA certificate and its associated KeyId shall be delivered to BS through NCMS, and stored in TBD MIB entry, this happens offline and not part of WRAN operation

- Certificates for all CA's and a CA's associated ECC domain parameters shall be shared between operator’s and distributed to all BSs within an operator’s network.. The mechanism by which is done is outside the scope of the standard.

##### 7.6.8.5 Signature Generation, Processing, and Validation Requirements

###### 7.6.8.5.1 Signature Generation Requirements

- Key Derivation Function (KDF) is used to derive the key used to sign each CBP transmission. KDF is defined in Section 7.2.5.7.

- For CMAC-based signature calculation use RFC 4493 (AES-CMAC) Output of AES-CMAC algorithm shall be truncated to 64 bits.

###### 7.6.8.5.2 Signature Generation Process

- BS uses system clock (synchronized to GPS), to get time (to millisecond) for "Time" field of Signature data in 2nd symbol of Self-coexistence window

- Message input parameter into CMAC calculation shall be all of the non-signature data in 2nd symbol of self-coexistence window and all IE data in 3rd symbol of self-coexistence window (if 3rd symbol is transmitted)

- Key input parameter is the public key associated with the certificate of the BS that transmitted the CBP

- Signature algorithm outputs:

  - For CMAC a digest that is 16bytes or 128bits long.  The MAC value shall be truncated to no less than 64bits, as recommended by NIST Special Publication 800-38B.  NIST 800-38B suggests taking 64 MSB.

##### 7.6.8.6 BS Implicit Certificate Exchange

CBP Protection Mode 1 allows for pre-distribution of CA Root and BS Implicit certificates between all BSs within an operator’s network. This would be accomplished using the NCMS. This may not be possible when BSs from different operators border/overlap with each other. This section defines a certificate exchange process to handle this case.

If there is a single CA, then the CA Root certificate and single set of EC domain parameters are available to all operators. If each operator is allow to sign/create their own certificates, then operators shall make available their CA Root Certificate and EC domain parameters available to other operators.

If a BS receives a CBP from a BS, for whom it does not have the BS implicit certificate, it cannot verify the signature in the 2nd symbol (1st data symbol) of CBP and must drop the CBP packet. Upon doing this the BS (Certificate Requestor) will initiate a Certificate Request directed towards the source BS (Certificate Responder) of the unverifiable CBP. The Certificate Requestor BS does this by adding a CERT-REQ IE (Section 6.7.1.2.x), with its’ own BS implicit certificate, as an IE in the CBP MAC PDU.

When the Certificate Responder receives a CBP with a CERT-REQ IE from a Certificate Requestor it verifies the certificate and Signature data in the Signature IE of the CBP MAC PDU. If both BSs are under the same CA then the Certificate Responder already has the CA Root certificate and EC domain parameters it needs to verify the BS implicit certificate from Certificate Requestor. If they are not, e.g. each operator is it’s own CA or CAs exist on a regional basis, than the Certificate Responder shall use a mechanism to request the CA Root certificate and EC domain parameters of another CA. This mechanism shall be provided, definition of this mechanism is outside the scope of the system.

After verifying the Certificate Requestor’s certificate and signature over CBP in which CERT-REQ IE was received, the Certificate Responder initiates a Certificate Response directed towards the Certificate Requestor. The Certificate Responder does this by adding a CERT-RSP IE (Section 6.7.1.2.x), with its’ own BS implicit certificate to the CBP MAC PDU.

When Certificate Requestor receives the CBP with the Certificate Response, it verifies it in the same manner that Certificate Responder verifies the Certificate Reqeust. After this, any pending coexistence signalling (e.g. Channel Contention) can proceed. Figure XXXX illustrates the BS Implicit Certificate Exchange.

Figure XXXX – BS Implicit Certificate Exchange

[Editor’s Note: Timers and/or some random backoff should be used to control re-transmission of certificate requests and/or responses. If working group deems this necessary then timers and the logic for controlling the retransmissions will be introduced at a later date.]

7.6.9 IEEE 802.22.1 Beacon Authentication

The Beacon, as defined in IEEE 802.22.1 standard, is used to afford protection to devices that fall under FCC Part 74 from harmful interference from license-exempt devices in the television bands. Authenication information has been added to the beacon to allow 802.22 CPEs and BSs to verify the authenticity of the transmission of the beacon. Without this verification, it may be possible for a rogue device/operator to send out an illegitimate beacon in an effort to cause the receiver of the beacon to vacate that particular channel.

[Editor’s Note: need to add proper reference to TG1 annex that is to be included into the main standard, text here reflects 07/491r5 or latest revision]

Section 3.2, in Annex 1 defines 10 modes for sensing and decoding the TG1 beacon. Sensing Types 1-8 (Section 3.2.1-3.2.8, Annex 1) are not adequate for sensing and decoding enough of the TG1 Beacon Frame to provide the receiving device with the information to authenticate the TG1 beacon. To authenticate the TG1 beacon, Sensing Types 9 and 10 shall be used.

If Sensing Type 9 (Section 3.2.9, Annex 1) is used, then MSF1 and MSF2 of the TG1 beacon frame is captured and decoded. The signature calculated over the TG1 beacon frame is contained with the Signature field of MSF2. To verify the signature, the public key of the certificate used to generate the signature is required. In this case, the certificate and public key shall be obtained via some off-line method (Sectio 5.6.1 and 7.5.5, IEEE 802.22.1) and installed at the 802.22 BS/CPE via a MIB. [Editor’s Note: This implies that there is some Part74 database that has info/locations for licensed wireless microphones. This database could hold info about the licensee, including the certificate. This method also implies use of a MIB, which will be defined later. Use of this method requires further discussion.]

If Sensing Type 10 (Section 3.2.10, Annex 1) is used, then MSF1, MSF2, and MSF3 of the TG1 beacon frame is captured and decoded. With this method the receiving device has all of the information it needs to verify the signature.

The process for verifying the signature in the TG1 beacon frame is detailed in Section 7.5.4 of IEEE 802.22.1. If the signature is processed, and verified as being valid, the receiving device has knowledge that the originator of the beacon is a valid PPD as defined by IEEE 802.22.1 and must vacate the channel. If the signature is not verified as being valid, then ...[Editor’s Note: Left open for now. Currently not sure what to do if signature is processed and found to be invalid. Does the BS/CPE still have to vacate the channel? Has this case been considered properly in Section 9?[

[--------------------------------------------End of Text Modification------------------------------------------------]

# III. Modifcations to Clause 11

[--------------------------------------------Start of Text Modification------------------------------------------------]

[add the following entries to Table 318 in Section 11]

| CPE | Auth Wait Timeout | Timeout period between sending Auth Request message from the Auth Wait State (Section 7.2.3.4) | 2 s | 10 s | 30 s |
| --- | --- | --- | --- | --- | --- |
| CPE | Auth Grace Timeout | Amount of time before authorization is scheduled to expire that the CPE starts reauthorization (Section 7.2.3.4) | 5 min (300 s) | 10 min (600 s) | 35 days (3,024,000 s) |
| CPE | Auth Reject Wait Timeout | Amount of time a CPE’s Authorization FSM remains in the Auth Reject Wait state before transitioning to the Start state (Section 7.2.3.4) | 10 s | 60 s | 10 min (600 s) |
| CPE | Operational Wait Timeout | Timeout period between sending of Key Request messages from the Op Wait state (Section 7.2.4.2) | 1 s | 1 s | 10 s |
| CPE | Rekey Wait Timeout | Timeout period between sending of Key Request messages from the Rekey Wait state (Section 7.2.4.2) | 1 s | 1 s | 10 s |
| CPE | GTEK/TEK Grace Time | Time interval, in seconds before the estimated expiration of a GTEK/TEK (Section 7.2.4.2) | 5 min (300 s) | 1 h (3600 s) | 3.5 days (302,400 s) |
| BS | AK Lifetime | Lifetime BS assigns to new AK | 1 day (86,400 s) | 7 days (604,800 s) | 70 days (6,048,000 s) |
| BS | PAK Lifetime | Lifetime BS assigns to new PAK | 1 day (86,400 s) | 7 days (604,800 s) | 70 days (6,048,000 s) |
| BS | TEK Lifetime | Lifetime BS assigns to new TEK | 30 min (1,800 s) | 12 h (43,200 s) | 7 days (604,800 s) |
| CPE | CPE_Random | Random number generated in CPE, SCM Auth-Request/Auth-Reply | -- | 8 bytes | -- |
| BS | BS Random | Random number generated in BS, SCM Auth-Reply | -- | 8 bytes | -- |
| CPE | CPE Certificate | Contains CPE’s X.509 certificate | -- | Size should not cause SCM Auth-Request to exceed the maximum MAC management message size | -- |
| BS, CPE | SAID | Security Association (SA) identifier | -- | 2 bytes | -- |
| CPE | Cryptographic Suite List | List of crytopgraphic suites, the CPE supports, SCM Auth-Request | 1 byte | -- | 9 bytes |
| CPE | SigCPE | An RSA or ECC signature over all the attributes in the SCM Auth-Request message | 40 bytes (160 bit ECC Key) | 56 bytes (224 bit ECC Key) | 128 bytes |
| BS | PAK Sequence Number | Sequence number of PAK | -- | 4 bits | -- |
| BS | CPE SA List | List of SAs that CPE is authorized for each SA, the SAID and Cryptographic suite configured for that SA. | -- | N*(2 bytes for SAID + 1 byte for Cryptographic Suite) | -- |
| BS, CPE | SCM Configuration Settings Indicator | Indicates existence of new settings for SCM Parameters, 1 byte long. (Section 6.9.28.2) | -- | 1 byte | -- |
| BS, CPE | SCM Configuration Settings Field | New values for SCM parameters (Section 6.9.28.2) that BS may change from defaults in Table 318 | 4 bytes | -- | 24 bytes |
| BS | SigBS | An RSA or ECC signature over all the attributes in the SCM Auth-Reply/Reject message | 40 bytes (160 bit ECC Key) | 56 bytes (224 bit ECC Key) | 128 bytes |
| BS, CPE | Message Response Code | Error code identifying reason for rejection of Auth Request, Section XXXX | -- | 1 byte | -- |
| CPE | Key Sequence Number | AK Sequence Number, SCM Key-Request/Reply | -- | 4 bits | -- |
| CPE | Group Key Indicator | Flag to inidicate if Key Request message is for a GKEK or GTEK. Only applicable if SAID in Key Request message refers to a GSA | -- | 1 bit | -- |
| BS | Older TEK/GTEK Parameters | Older generation of TEK Parameters relevant to SAID, or GTEK parameters relevant to GSAID in Key Reply message | -- | 128 bits (16 bytes) | -- |
| BS | Newer TEK/GTEK Parameters | Older generation of TEK Parameters relevant to SAID, or GTEK parameters relevant to GSAID in Key Reply message | -- | 128 bits (16 bytes) | -- |
| BS | GKEK Parameters | GKEK related parameters for multicast/broadcast service | -- | 128 bits (16 bytes) | -- |
| BS, CPE | SCM Flow Control | The maximum number of concurrent SCM transactions | 0 (Default: unlimited # of transactions) | -- | 255 |
| BS, CPE | Number of Supported Security Associations | The maximum # of supported security associations | 1 | -- | 255 |
| BS, CPE | PN_WINDOW_SIZE | Window that defines the acceptable PNs for received PDUs that are to be processed by encryption/decryption process |  | 2 bytes |  |

[--------------------------------------------End of Text Modification------------------------------------------------]

# IV.Annex E

[--------------------------------------------Start of Text Modification------------------------------------------------]

[insert new Annex, E, into document[

Annex E.

Collaborative Spectrum Sensing and Decision Making to Provide Protection Against Spurious Signals

Let

H0: Hypothesis such that the incumbent is not occupying the channel being sensed

H1:  Hypothesis such that the incumbent is occupying the channel being sensed

Let x(n) be the received signal at the input of the SSF, where n is the running index of the samples

x(n)=w(n)→H0s(n)+w(n)→H1        (C.1),

where s(n) be the authentic incumbent signal, and w(n) be the noise.  Hence the goal of the SSF is to decide between hypotheses H0 and H1.

Let y(n) be the decision statistic, and  be some threshold used to choose between the Hypotheses H0 and H1 respectively. Then the detection probability (Pd) and the false alarm probability (Pf) can be denoted by

Pd=Py>λ|H1Pf=Py>λ|H0                  (C.2)

Consider a group of N localized sensors, monitoring a specific area, collaboratively trying to distinguish between Hypotheses H0 and H1. If the group collectively decides that the incumbent signal is detected when the detection statistic at, at least one of the sensors exceeds the threshold, then the sensors are said to follow the OR rule of the collaborative sensing. Under these circumstances, the network detection and false alarm probabilities are denoted by Qd and Qf respectively, and given by

QdOR=1−1−PdNQfOR=1−1−PfN                  (C.3)

Under the OR rule of collaborative sensing, the net detection as well as false alarm probabilities increase.

However, consider another signal c(n), which statistically looks similar to the authentic signal s(n), such that, when transmitted, it may result in false detection of the signal s(n). Assume that c(n) is being transmitted at a lower power than s(n) and hence, will be detected by L sensors where (L<N). Let PC be the probability that c(n) is transmitted (ON) at a particular instant of time. If the OR rule of the collaborative sensing is followed, then the probability of detection of an authentic incumbent signal is give by

QdC,OR=PC+1−PC1−1−PdN=1−1−PC1−PdN           (C.4)

and the false alarm probability = probability that Hypothesis H1 is selected when H0 is true is given similarly by

QfC,OR=1−1−PC1−PfN                     (C.5)

This means that if the other signal is present along with the true signal, it helps detection. For instance, if c(n) is ON all the time (that is PC = 1), then, QdC,OR=QfC,OR=1, which means that a particular channel always appears to be occupied, whether or not an authentic incumbent signal is present. If c(n) is OFF all the times then QdC,OR=1−1−PdN and probability of authentic incumbent detection is the same as in C.3. Note, that the false alarm probability increases with an increasing value of PC.

Now consider a case where instead of the OR rule of collaborative sensing, the AND rule is followed. Under these circumstances, the incumbent signal is said to be detected if and ONLY if all the N localized sensors detect the signal. In absence of c(n), the detection and false alarm probabilities are given by

QdAND=PdNQfAND=PfN            (C.6),

which means that both, the detection as well as the false alarm probabilities decrease with increasing N.

Let PC be the probability that c(n) is transmitted at a particular instant of time and some L<N sensors are able to detect it. If the AND rule of the collaborative sensing is followed, then the probability that an authentic incumbent signal is detected is given by,

QdC,AND=PdN−L              (C.7).

which means that even in presence of a false signal c(n), the probability that an authentic incumbent signal is detected increases, but is not automatic. Similarly, the false alarm probability = probability that Hypothesis H1 is selected when H0 is true, and is given by

QfC,AND=PfN−L             (C.8).

As an example, let Pd = 0.9, let Pf = 0.1, let N = 2, L = 1, and PC = 0.5, then

| Pd= 0.9 | QdOR = 0.99 | QdC,OR = 0.995 | QdAND = 0.81 | QdC,AND = 0.9 |
| --- | --- | --- | --- | --- |
| Pf= 0.1 | QfOR = 0.19 | QfC,OR = 0.595 | QfAND = 0.01 | QfC,AND = 0.1 |

Rather than using simple OR or AND based decision making, it makes sense to use some form of a VOTING based rule for decision making which provides a finer granularity, flexibility and protection against low powered signals c(n) which may have statistical properties that are similar to s(n) – the signal of interest.

Given N collaborative detectors, let k be the number of these radios that detect a transmitter present in a band. To decide when we declare a transmitter present, we define the collaborative sensing voting rule. The rule has one parameter, the voting threshold T, a number between 0 and 1. It can be expressed as a percentage, e.g.  T = 0.5 is a threshold of 50%.

The rule is that we declare the signal s(n) to be present if  d  TN nodes detect its presence.

To evaluate the rule, suppose that all detectors can detect the signal of interest s(n) with probability Pd, independent of each other. Then the probability of k successes from N transmitters is given by the binomial distribution,

B(k,N;Pd)=N!k!(N−k)!Pdk1−PdN−k.      (C.9)

The probability the voting rule declares that the signal of interest s(n) is present if

QDcollaborative=k≥TNNB(k,N;Pd).         (C.10)

The probability of the rule declaring no transmitter present is 1 − QDcollaborative.

Under this formulation, the AND rule that everyone must detect to declare a transmitter present is T = 1. The OR rule that if anyone detects we declare a transmitter present is 0 < T ≤ 1/N.

If a transmitter is actually present, then QD represents the true detection rate and 1 − QD represents the missed detection rate. If a transmitter is not present then QD represents the false positive rate and 1 − QD represents the true negative rate.

Figure C.1 OR Rule of Collaborative Sensing and Signal Detection

Figure C.1 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the OR rule of collaborative sensing with Pd = 0.9 and Pf = 0.1. Simulations assume the sensors are independent. It can be seen that with the OR rule, false positives increase with the number of detectors. Hence the OR rule of collaborative sensing fails to provide robust detection of the signal of interest.

Figure C.2 AND Rule of Collaborative Sensing and Signal Detection

Figure C.2 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the AND rule of collaborative sensing. It can be seen that the AND rule performs much better than the OR rule for false positives, however, the true positive performance suffers as the number of independent observations from the detectors increases. Hence once again the AND rule of collaborative sensing fails to provide robust detection of the signal of interest s(n).

Figure C.3 Voring Based Rule of Collaborative Sensing and Signal Detection where the signal is said to be detected if half the number of sensors detect the signal.

Finally, Figure C.3 shows the true positive and false positive detection probability curves as a function of the number of independent signal detectors for the VOTING based rule of collaborative sensing with voting threshold set to 0.5: that is half the number of independent detectors (N/2) must confirm the presence of the signal. It can be seen that the VOTING based rule performs much better than the OR as well as the AND rules for true positives as well as the false positives. Probability of detection for true and false positives reaches an optimal value with observations from around 6 sensors.

The voting rule can also perform well in the presence of a false signal, c(n), that affects a limited number, L, of detectors. As the number of independent detectors increases, these L detectors will be overruled by the detection from other sensors.

Hence collaborative sensing with information fusion and decision making based on a VOTING rule can help in enhancing the network detection performance of the true signal s(n) in presence of a false signal c(n). Based on the simulations results, information fusion and decision making from approximately 6 to 10 independent sensors is likely to be sufficient to obtain satisfactory performance results.

Correlation of the voting results with the geographical location of the sensing CPEs

Collaborative sensing takes all it strength when the information fusion described above takes into account the knowledge on the geographic location of the specific sensing devices providing sensing reports.  The correlation of the reports of the sensing devices that will have sent an “Urgent Coexistence Situation (UCS)” message to the base station and their respective geographic location which is, by definition, known at the base station from the time of their registration on the network will provide powerful means to identify the characteristics of the sensed signal and help improve the decision making as to the presence of an incumbent and its type versus presence of other devices based on their expected extent of coverage.

In the case of DTV detection, the area where correlation will exist between sensing devices will be large and will be in the direction of the DTV transmitter.  In the case of a wireless microphone, the correlarion will exist over a more limited area.  This limited area correlation will also be used in the case of the detection of an 802.22.1 beacon and will help increasing the level of conficence of the decision that it is indeed a TG1 beacon based on its expected area of coverage, and possibly reducing the need to rely on the full PPDU decoding.

Since all the information will be available at the base station, it will be up to the manufacturers to implement various levels of complexity in fusing the sensing reports and the geographic location of the sensing devices to come up with a reliable assessment of the presence of incumbents, that is true positives, while minimizing the probability of false positives. Artificial intelligence algorithms could be taken advantage of to improve this data fusion and decision process.  Since incumbents will need to be protected in all cases and in case of doubt, action will need to be taken to avoid interference to incumbents, more advanced data fusion and decision processes will have the advantage of avoiding false positives and false alarms and therefore reducing cases where WRAN systems would have to change frequency while it is not necessary.

[--------------------------------------------End of Text Modification------------------------------------------------]

References:

- A. Mody, T. Kiernan, “Meeting Minutes of the Security Ad-Hoc Group in 802.22” https://mentor.ieee.org/802.22/file/08/22-08-0163-07-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Recommended text for Section 7 on Security in 802.22” - https://mentor.ieee.org/802.22/file/08/22-08-0174-07-0000-recommended-text-for-section-7-on-security-in-802-22.doc was presented and discussed

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Table of Contents for Section 7”. https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, “Scope and the Workplan for the Security Ad-Hoc Group” –  https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- A. Mody, R. Reddy, T. Kiernan, M. Sherman, DJ Shyy, “Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis” https://mentor.ieee.org/802.22/file/08/22-08-0083-07-0000-security-and-prm-enhancements-in-80222-v3.ppt

- A. Mody, R. Reddy, M. Sherman, T. Kiernan, DJ Shyy, “Security and Protocol Reference Model Enhancements in 802.22,” https://mentor.ieee.org/802.22/file/08/22-08-0083-04-0000-security-and-prm-enhancements-in-80222-v3.ppt

- IEEE 802.16 Draft Standard for Broadband Wireless Access – P802.16Rev2/D5 (June 2008) – Section 7 – Security Sublayer

- Security Enhancement for 802.16e - A SDD Proposal for 802.16m -www.wirelessman.org/tgm/contrib/C80216m-08_046.doc

- http://www.navigadget.com/index.php/2007/01/29/gps-and-gsm-jammer/

- M. Barbeau, “WiMAX Threat Analysis”, Proceedings of the ACM, Q2SWinet’05, October 13, 2005, Montreal, Quebec, Canada.

- S. Xu,  M. Matthews, “Security Issues in Privacy and Key Management Protocols of 802.16,” Proceedings of the ACM SE’06, March 10-12, 2006, Melbourne, Florida, USA

- D. Johnston and J. Walker, “Overview of IEEE 802.16 Security,” IEEE Security and Privacy, Magazine Published by the IEEE Computer Society, 2004

- Y. Xiao, X. Shen and D. Du, Wireless Network Security, Springer Series on Signals and Communications Technology, 2006

- Qusay H. Mahmoud, Cognitive Networks, Towards Self Aware Networks, Wiley, Sept. 2007 – Chapter 11, Security Issues in Cognitive Radio Networks

- Amita Sethi, Potential Denial of Service Threat Assessment for Cognitive Radios, MS Thesis, University of Colorado at Boulder, 2008.

- A. Mody, “Section 9 – Cognitive Radio Capability – Missing Sections and Details,” Document 22-09-057 Rev 6 - https://mentor.ieee.org/802.22/dcn/09/22-09-0057-06-0000-section-9-cognitive-radio-capabilities-missing-sections-and-details.doc