IEEE P802.22
Wireless RANs

| ATM Convergence Sublayer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-06-24 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Edward Au | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68745666 | ks.au@ieee.org / ksau@i2r.a-star.edu.sg |
| Zander Lei | Institute for Infocomm Research | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-68745686 | leizd@i2r.a-star.edu.sg |

Abstract

This contribution provides the resolution to the technical comment #68.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This contribution provides the resolution to the technical comment #68. The additional texts are marked in blue.

5.6 ATM Convergence Sublayer

The Asynchronous Transfer Mode Convergence Sublayer (ATM CS) is a logical interface that associates different ATM services with the MAC Common Packet Sublayer (CPS) Service Access Point (SAP). The ATM CS accepts ATM cells from the ATM layer performs classification, and delivers CS PDUs to the appropriate MAC SAP.

5.6.1 CS service definition

The ATM CS is specifically defined to support the convergence of PDUs generated by the ATM layer protocol of an ATM network. Since ATM cell streams are generated according to the ATM standards, no ATM CS service primitive is required.

5.6.2 Data/Control plane

5.6.2.1 PDU formats

The ATM CS PDU payload shall be equal to the ATM cell payload. The ATM CS PDU is illustrated in Figure xx.

Figure xx – ATM CS PDU format.

5.6.2.2 Classification

An ATM connection, which is uniquely identified by a pair of values of virtual path identifier (VPI) and virtual channel identifier (VCI), is either virtual path (VP) switched or virtual channel (VC) switched. In VP-switched mode, all VCIs within one single incoming VPI are automatically mapped to that of an outgoing VPI. In VC-switched mode, input VPI/VCI values are individually mapped to output VPI/VCI values.

A classification rule is a set of matching criteria applied to each ATM cell entering the ATM CS. It consists of some ATM cell matching criteria, such as VPI and VCI, and a reference to a CID. If an ATM cell matches the specified matching criteria, it is delivered to the MAC SAP for delivery on the connection identified by the CID.

6.9.8.10.20 Convergence Sublayer Specification

This parameter specifies the CS that the connection being set up shall use.

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| [145/146].28 | 1 | 0: No CS 1: IPv4 2: IPv6 3: IEEE 802.3/Ethernet 4: IEEE 802.1Q VLAN 5: IPv4 over IEEE 802.3/Ethernet 6: IPv6 over IEEE 802.3/Ethernet 7: IPv4 over IEEE 802.1Q VLAN 8: IPv6 over IEEE 802.1Q VLAN 9: ATM 10 – 255: Reserved | DSA-REQ |

6.9.8.10.21  Convergence Sublayer Parameter Encoding Rules

| Element ID | Value |
| --- | --- |
| [145/146].cst | cst = 99: IPv4 cst = 100: IPv6 cst = 101: IEEE 802.3/Ethernet cst = 102: IEEE 802.1Q VLAN cst = 103: IPv4 over IEEE 802.3/Ethernet cst = 104: IPv6 over IEEE 802.3/Ethernet cst = 105: IPv4 over IEEE 802.1Q VLAN cst = 106: IPv6 over IEEE 802.1Q VLAN cst = 107: ATM |

6.9.8.10.23 ATM CS encodings for configuration and MAC messaging

The ATM CS encodings listed in 6.9.8.10.23.1 through 6.9.8.10.23.3 shall be used in the configuration file, in CPE’s registration requests (when applicable), and in Dynamic Service messages (when applicable). All ATM specific encodings are prefixed to begin with a Type value of [145/146].107.

6.9.8.10.23.1 ATM Switching Encoding field

This field defines the switching methodology for the service. If the field = 0, at least one VPI/VCI Classifier pair shall be defined for classifying the service. If the field = 1, exactly one VPI Classifier and zero or one VCI Classifier shall be specified for classifying the service. If the field = 2, exactly one VPI Classifier and one VCI Classifier shall be defined for classifying the service. If the field = 0, no switching methodology is applied.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.1 | 1 | 0: No switching methodology applied 1: VP switching 2: VC switching |

6.9.8.10.23.2 ATM Classifier Encoding field

This field defines an ATM classifier. It is a compound encoding used to describe the VPI and associated VCIs for ATM classification.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.2 | variable | Compound |

It shall have the following form.

| Field | Note |
| --- | --- |
| ATM Classifier ID | Always present |
| VPI Classifier | Always present except for DSC change action deleting classifier |
| VCI Classifier | Zero or more instances when the VPI classifier is present |

6.9.8.10.23.2.1 VPI Classifier field

This field defines the VPI on which to classify ATM cells for the service flow.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.2.1 | 2 | 8-bit or 12-bit VPI field value |

6.9.8.10.23.2.2 VCI Classification field

This field defines the VCI on which to classify ATM cells for the service flow. This encoding shall immediately follow the VPI encoding with which it is associated.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.2.2 | 2 | 16-bit VCI field value |

6.9.8.10.23.2.3 ATM Classifier ID field

This field is used to identify an ATM classifier.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.2.3 | 2 | 16-bit classifier ID |

6.9.8.10.23.3 ATM Classifier DSC Action field

When received in a DSC-REQ message, this field indicates the action to be taken on a classifier. If the action encoding is “Add” or “Replace”, the action is followed by a complete ATM Classifier compound encoding. If the action is “Delete”, the action encoding is followed by the ATM Classifier compound encoding composed only of the ATM Classifier ID encoding.

| Element ID | Length (bytes) | Value |
| --- | --- | --- |
| [145/146].107.3 | 1 | 0: DSC Add Classifier 1: DSC Replace Classifier 2: DSC Delete Classifier |

References: