IEEE P802.22
Wireless RANs

| Functional block diagram applicable to the PHY layer |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | (613) 998-2500 | gerald.chouinard@crc.ca |

Abstract

This document is a suggested text for the functional block diagram applicable to the WRAN PHY layer. As a resolution for comment ID #647, we will insert new subclause describing the block diagram that integrates all the functional blocks of the PHY layer.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution and action item for comment ID #647, I suggest the functional block diagram of the transmitter and receiver for OFDMA PHY.

With a general and single block diagram, we can describe all process of WRAN OFDMA signals, including preamble, headers (SCH, FCH, DS/US MAP, DCD/UCD etc.), data bursts, pilots, and CBP payload. For the preamble and pilot generation, the process starts from the preamble & pilot inserter in the block diagram. The headers are a kind of data burst, thus the process is same as the data transmission. The only difference is that the SCH is spreaded by a factor of 4. However, we can consider the spreading process as a kind of subcarrier allocation process because each QPSK symbols is transmitted on different four spreaded subcarriers. Moreover, in the case of CBP packet, though the spreading factor of CBP payload is 3 and the preamble/pilot pattern are different from data transmission, the process is also same as data transmission. Since the details of each block are already described in the related subclause, in this subclause it is reasonable to describe the general process of WRAN PHY layer as following suggested text.

2. Suggested text

8.3 Functional block diagram applicable to the PHY layer

The functional block diagram of the transmitter and receiver for the PHY layer is shown in Figure 1. This subclause specifies the general processing of the WRAN baseband signal. The binary data for transmission is supplied to the PHY layer from the MAC layer. This is input to a channel coding processor which includes data scrambler, encoder, puncturer, and bit interleaver. It is specified in subclause 8.6 in detail. The interleaved data is mapped to data constellations according to modulation schemes specified in subclause 8.7. The subcarrier allocator assigns the data constellations to the corresponding subchannels according to the subcarrier allocation method described in subclause 8.5.2. In order to support the synchronization, channel estimation, and tracking process, the preamble is inserted in the first 1 or 2 OFDM symbols of each frame specified in subclause 8.3.1, and the pilot subcarriers are transmitted at fixed positions in the frequency domain within each OFDM symbol as specified in subclause 8.5.1. In the frequency-domain, an OFDM symbol contains the data, pilot, and null subcarriers, as defined in Table 247. The resultant stream of constellations is subsequently input to an inverse Discrete Fourier Transform after a serial-to-parallel convertion. The inverse Fast Fourier Transform (IFFT) is the expected means of performing the inverse Discrete Fourier Transform. In order to prevent inter-symbol interference (ISI) and inter-carrier interference (ICI) due to delay spread, the OFDM symbol is cyclically extended by a cyclic prefix. Finally, the OFDM signal is transferred to the front-end modules via an analog-to-digital converter.

The OFDM receiver basically performs the operations of the transmitter in reverse order. In addition to the data processing, the synchronization and channel estimation must be performed at the receiver.

The CBP packet can also be generated through the same process as the data transmission. The only difference is in the subcarrier allocator and the preamble and pilot inserters. The subcarrier allocation of CBP data is a kind of spreading process because each QPSK symbol is transmitted on three different spreaded subcarriers. The subcarrier allocation and preamble and pilot patterns are described in subclause 8.4.

Figure 1. Transmiter and receiver block diagram for the OFDMA PHY