IEEE P802.22
Wireless RANs

| PHY Discussion Topics in July Meeting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-7-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Zander Zhongding Lei | Institute for Infocomm Research, Singapore | 21 Heng Mui Keng Terrace, Singapore 119613 | 65-6874-5686 | leizd@i2r.a-star.edu.sg |

Abstract

This document contains the PHY topics proposed to be discussed in the working group and/or PHY specific sessions in July plenary meeting. A brief summary / updates for each of the topic have been given as well. Please refer to the comments database and PHY conference call minutes for more details.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

WG Discussion

- Power Control for CDMA Transmission (ID 717, DCN 145, Jung Sun)

  - The power adjustment equation for transmission in the contention window needs to be restricted explicitly (4W EIRP limit). If a CPE is only transmitting in the contention window at a time slot, the power cap would be simply 4W. However, if the same CPE is transmitting data burst at the same time, then its power cap at the time would be 4W power_of_data_burst.

  - The contention window is used for 3 types of transmission: UCS notification, BW request, and Ranging. The opportunistic transmission of UCS will not happen together with the data transmission.  Instead, a UCS bit within MAC header (Table 3, pg 23 in draft 1.0) is transmitted when data transmission is on. Similarly, a MAC subheader for bandwidth request (Table 28, pg 35 in draft 1.0) is transmitted when data transmission is on. There are two types of ranging which could use the opportunistic window: initial ranging and periodic ranging. Initial ranging is used for establishing links and synchronization, which is precedent to the data transmission. However, it was not clear whether the only possibility in question is the periodic ranging

  - The question posted to the meeting is whether there is any scenario requiring simultaneous transmission of data burst and periodic ranging signal from the same CPE

  - The consensus reached in WG (Straw Poll) was there would not be parallel transmission of data and periodic ranging from the same CPE.

  - Modified equation is approved

  - A few points not allowing the parallel transmission if possible:

    - CPE complexity.  The processing would be more complex if a CDMA burst is to be transmitted along with a normal OFDMA data burst by the same CPE

    - Since more sub-channels would be used in a parallel transmission, the ranging burst could not use the full 4 W EIRP. The power of the CDMA ranging burst would need to be scaled back so that the total EIRP meets the 4W limit.

    - Ranging request received from a CPE does not have to occupy the entire upstream sub-frame. The condition is that CPE A only transmits its CDMA ranging request and no other data in parallel during the 3 symbols allocated for it but that those subchannels could be used by other CPEs for the rest of the sub-frame.

    - The ranging request is not to occur that often and not transmitting in parallel should not affect the system QoS

- Inter WRAN Coexistence (ID 664, DCN 137r2, Gerald)

  - Rev2 presented and to be further discussed in the working group. In a summary, it was proposed to

    - Allow for two modes of operation: normal mode and coexistence mode

    - Use SFN operation for the superframe header in self-coexistence situation

  - Big impact to the PHY and MAC and PHY/MAC needs to design/re-design

    - “Normal operation” superframe header and “self-coexistence operation” superframe header to be transmitted simultaneously by all overlapping BSs

    - How to switch between “normal operation” and “self-coexistence operation”

    - Modulation and coding of the superframe header (preamble/SCH) in SFN mode

    - Distributed frame scheduling among BSs with fast operation

    - A new superframe structure (preambles/SCH) to operate in a coexistence environment

    - CBP burst with the right information

  - open

- Antenna Pattern/Gain Storage (ID 709, Ivan)

  - It is suggest adding a requirement that the antennas have their actual antenna pattern and gain stored in a non-volatile memory and the mandatory means for the BS/CPE unit to interface and read this memory out of the antenna. A BS/CPE shall not transmit without delivering the above data. It is supposed to help in alignment and in making sure that the operator or customer does not substitute the antenna for another "higher gain" one and thereby possibly violate regulatory requirements.

  - The proposal may be a simple (?) way to make sure the transmitter of any 802.22 device knows the antenna gain and automatically limits the conducted power delivered to the antenna so that EIRP limits are not violated. The consideration is that

  - Furthermore, it is suggested that the cable/connector between the transmitter and the antenna, if any, should also be standardized, such as cable impedance/type (coax or is a twin lead)/shield performance, connector type (F, TNC, SMA, BNC?), max acceptable VSWR?

  - It is said that there is precedence in the connector definition of 802.3 and 802.5, the cable media impedance of 802.3 and 802.5, the voltage levels and waveforms applied on the media, etc.

  - However, it is arguable that the way to control the CPE EIRP is considered as implementation/legal issue and beyond the scope of the standard. Examples of possible implementations of the way the EIRP would be controlled at the CPE could be contained in an annex.

  - PHY group needs the direction from WG to decide whether it should be standardized in .22. If positive, the most appropriate/tamper-proof solution should be discussed in the WG level involving antenna experts.

- Antenna Sidelobes Spec (ID 725, DCN 188, S. Kuffner)

  - The purpose of the discussion is to determine the percentage of sidelobes and backlobes allowed to exceed the mask. There does exist regulatory precedent for allowing a small percentage of pattern sidelobes to exceed a specified envelope, provided those exceptions stay within a specified small number of dB. However, similar allowance if enacted for IEEE 802.22 could turn out to be extremely difficult to characterize and enforce.  IEEE 802.22 CPE antennas may not be professionally installed and could be in proximity to roofs or other structures that could influence the pattern of the antenna.  Further empirical study is recommended to determine the extent of pattern distortions for typical mounting scenarios.

- RF Mask (IDs 726~734, DCN 141, Gerald)

  - The FRD text was used as a place-holder because no resolution could be reached on an RF mask by the time of the first Draft.  Discussions are to take place on technical bases to define an RF mask that will protect the broadcast incumbents while being technically feasible before issuing the second version of the Draft.

PHY Discussion

- Interleaver (IDs 680, 698, John)

  - The interleaver description clarifying the functionality/implementation is to be updated (ID 680, Apurva).

  - There was also a concern on the latency for the interleaving scheme as in comment ID 698 (Geroge). Table 274 has 32 rows that an algorithmic approach will be needed to build the interleaver, interleaving a bit at a time per clock cycle.  This has an impact on latency. It is requested to have a calculation of the worst-case row for the latency assuming only one bit can be interleaved per clock.

  - Approved for clarification text

  - John will work on the latency issue

- PHY Functional Block Diagram/Description (ID 647, DCN 186, Sung Hyun)

  - Further amendment to the functional block description is needed as suggested by Gerald, including listing of randomizer, encoder, puncturer, and interleaver in the FEC block, separating preamble inserter and pilot inserter, using "Cyclic Prefix Inserter” instead of “Guard Inserter", and some editorial changes

  - Approved with modification

- Normalized C/N Values for Power Control (IDs 718~722, DCN 181, Sung Hyun)

  - The simulation results determining the normalized C/N values for TPC has been presented. The simulations incorporated the new subcarrier and bit interleaving scheme for all FEC rates under both AWGN and channel B.

  - For frequency domain CDMA transmission (messages/ranging), C/N values are determined based on reference performance at 90% detection probability. For non CDMA type message transmission, C/N values are determined for reference BER at 2*10-4 after Viterbi decoder

  - AWGN results

    - C/N values are consistent to published DVB-T simulation results

    - C/N value differences between different FEC rates are close to that for 802.16 system

  - Channel B results

    - Results are deviated from available results of DVB-T and 802.16 due to the different channel models

  - 90% detection probability for CDMA transmission is conservative compared to BER 2*10-4 for non-CDMA transmission, taking into consideration packet collision for non-CDMA transmission? (CDMA-type transmission may not have the collision issue due to code orthogonality)

  - One column for AWGN only or 2 columns incorporating Channel B results?

  - 2nd column is default and 3rd column is informative

- CBP Redesign? (ID 672, DCN 190, Cheng Shan)

  - Proposed design presented. It was claimed to be more efficient in terms of overall throughput, latency and collision probability with comparable BER performance but slightly increased complexity.

  - Concerns to be addressed

    - The number of pilot carriers is much fewer (1/2 or ¼ for different modes) and would degrade the channel estimation performance. It becomes worse in the channel with more fluctuation in frequency domain due to multipaths.

    - The payload size does not match to the current CBP payload size from MAC

    - Synchronization is of big concern with a single CBP preamble. More preamble sequences are needed and this brings up the new issue of the preamble sequence design

    - Robustness is more important vs the overhead and latency issue as the CBP transmission does not happen frequently

    - How to working with inter-WRAN coexistence issue.

  - Postponed for further contribution

- CBP Encoding Text (ID 674, DCN 191, Jung Sun)

  - Diagram and text was discussed. It was further suggested adding in sentences and stating clearly that the encoding steps are meant for CBP data only, not forSCH, to avoid causing confusion.

  - Suggest to change the figure 106 in MAC to align with figure 140 in PHY. Dave is assigned to provide the figure and corresponding text.

- Interleaver (IDs 680, 698, John)

  - The interleaver description clarifying the functionality/implementation is to be updated (ID 680, Apurva).

  - There was also a concern on the latency for the interleaving scheme as in comment ID 698 (Geroge). Table 274 has 32 rows that an algorithmic approach will be needed to build the interleaver, interleaving a bit at a time per clock cycle.  This has an impact on latency. It is requested to have a calculation of the worst-case row for the latency assuming only one bit can be interleaved per clock.

- Frequency Range (ID 629, Charles)

  - It was identified (by Charles) that the frequency range of 54 to 862 MHz does not comply with the ITU-R Radio Regulations, the recommended text/reference is needed.

  - Gerald to make editorial change and make sure it is the spectrum allocated to the Television Broadcasting Service in the frequency range 54 to 862 MHz

  - Make remarks in Table 245

  - Closed

- Geo-ranging Tone Location (ID 682, Ivan)

  - It was suggested (by Apurva) to ”Either re-write the text about Table 254 (Geo-ranging text) or provide and example in the informative annex on downstream / upstream allocations.” Action is required for Ivan.