IEEE P802.22
Wireless RANs

| Resolutions MAC CIDs 111, 142, 147 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-08-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |

Abstract

This document provides the proposed resolutions for comments: 111, 142, and 147.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 111

Replace Figure 13 with the following:

Comment 142

Replace the text In Section 6.7.1.1 and Table 3 – Generic MAC header format with the following table. Text modifications are indicated in RED collor.

Note: the EKS field does not appear anywhere else in the draft. If it is needed, additional text would be required to explain its usage, and in this case, the design of Table 3 would need to be revisited to add the 2 extra bits.

      - General

The format of the general MAC header is shown in . This header begins each MAC PDU containing either higher layer traffic or MAC management data.

Since the MAC is a connection-oriented MAC, an important component of the general MAC header is the CID which serves to identify an existing service flow between the BS and CPE. Two other critical fields included in the header for the purpose of coexistence are the UCS and the CN subheader indication (Type = 6). These fields are used by CPEs to quickly signal the BS of a newly detected urgent coexistence situation with incumbents. For example, these can be utilized to cope with the case where the BS is engaged in communication with a CPE while incumbents are detected by this CPE in a TV channel currently in use. Here, this urgent situation can be immediately conveyed to the BS by having the CPE set the UCS bit to 1 and including the CN subheader to indicate the corresponding TV channel number now occupied by an incumbent service. The general MAC header also includes other fields (e.g., security related), and these can be found in .

- Table 3— Generic MAC header format

| Syntax | Size | Notes |
| --- | --- | --- |
| General_MAC_Header_Format() { |  |  |
| Length | 11 bits | The length in bytes of the MAC PDU including the MAC header and the CRC |
| UCS | 1 bit | Urgent Coexistence Situation Used by the CPE to alert the BS about an urgent coexistence situation with incumbents in the TV channel(s) currently being used by the BS. 0 = no incumbent (default) 1 = incumbent detected |
| EC | 1 bits | Encryption control 0 = payload is not encrypted 1 = payload is encrypted |
| EKS | 2 bits | Encryption key sequence The index of the traffic encryption key (TEK) and initialization vector used to encrypt the payload. This field is only meaningful if the EC field is set to 1 |
| Type | 7 bits | Indicates the subheaders and special payload types present in the message payload See Table 1 |
| CID | 12 bits | Connection identifier |
| HCS | 8 bits | Header check sequence The transmitter shall calculate the HCS value for the first six bytes of the header, and insert the result into the HCS field (the last byte of the MAC header). It shall be the remainder of the division (Modulo 2) by the generator polynomial g(D = D8 + D2 + D + 1) of the polynomial D8 multiplied by the content of the header excluding the HCS field. (Example: [EC Type]=0x80, BR=0xAAAA, CID=0x0F0F; HCS should then be set to 0xD5). |
| } |  |  |

- — Encoding of the Type field

| Type bit | Value |
| --- | --- |
| #6 most significant bit (MSB) | CN (Channel Number) subheader Indicates whether a special CN subheader follows the generic MAC header. The CN subheader shall be used only if the UCS bit is set to 1 (see 6.7.1.2.1 and Table 28). 1 = CN subheader present 0 = CN subheader absent |
| #5 | Bandwidth Request subheader Indicates whether this is a bandwidth request frame, and hence contains a special payload related to bandwidth allocation (see Table 29) 1 = present; 0 = absent |
| #4 | ARQ feedback payload 1 = present; 0 = absent |
| #3 | Extended type Indicates whether the present Packing or Fragmentation Subheader is Extended 1 = Extended 0 = not Extended. Applicable to connections where ARQ is not enabled |
| #2 | Fragmentation subheader 1 = present; 0 = absent |
| #1 | Packing subheader 1 = present; 0 = absent |
| #0 | Downstream: FAST-FEEDBACK Allocation subheader Upstream: Grant Management subheader 1 = present; 0 = absent |

In Section 6.7.1.3, modify the text as indicated below. Text changes are indicated in RED.

- 6.7.1.3 MAC Subheaders and Special Payloads

Seven types of subheaders may be present. The per-PDU subheaders (i.e., CN, Bandwidth Request, Fragmentation, FAST-FEEDBACK_Allocation, and Grant Management) may be inserted in MAC PDUs immediately following the Generic MAC header. If indicated, the CN subheader shall always follow the Generic MAC header. If both the Fragmentation subheader and Grant Management subheader are indicated, the Grant Management subheader shall come first. If both the Grant Management subheader and Bandwidth Request subheader are indicated, the Grant Management subheader shall come first. The FAST-FEEDBACK Allocation subheader shall always appear as the last per-PDU subheader.

The only per-SDU subheader is the Packing subheader. It may be inserted before each MAC SDU if so indicated by the Type field. The Packing and Fragmentation subheaders are mutually exclusive and shall not both be present within the same MAC PDU.

When present, per-PDU subheaders shall always precede the first per-SDU subheader.

Insert new subclause 6.7.1.3.1 CN Subheader and adjust the references and table numbers accordingly.

- 6.7.1.3.1 CN Subheader

The CN subheader shall be used only in case of an UCS event (when UCS bit in the generic MAC header shall be set to one) to indicate the channel number in which an incumbent has been detected. If the UCS bit in the generic MAC header is set to one, but no CN subheader is indicated, the BS shall assume the incumbent has been detected on the operating channel (N). The CN subheader shall be indicated when an incumbent is detected on a channel different from the operating channel (e.g. adjacent channels N+1 or N-1).

- Table 28 — CN subheader format

| Syntax | Size | Notes |
| --- | --- | --- |
| CN_Subheader_Format() { |  |  |
| Channel Number | 8 bits | TV channel Number It indicates which TV channel is under an urgent coexistence situation with incumbents. |
| } |  |  |

Comment 147

In Section 6.7.1.2 remove the following fields from Table 5.

| Number of TV Channels for Backup | 4 bits | See Error: Reference source not found |
| --- | --- | --- |
| Reserved | 4 bits | All bit shall be set to 0. |
| For (i=0; i < Number of TV Channels for Backup; i++) { |  | List of backup TV channels in order of priority to be used by CPEs in case of loss of communication with the BS. The backup TV channels shall be a disjoint set including the current operating TV channel. |
| TV channel Number for Backup [i] | 8 bits | See Error: Reference source not found |
| } |  |  |

In Section 6.7.1.2.1, replace the first sentence in line 4 with the following:

CBP packets shall carry at least one Backup Channel information element (IE) in their payload. This is to allow WRANs to execute the spectrum etiquette mechanism before deciding to execute the other spectrum sharing mechanisms described in 6.21.2.3.

In Section 6.7.1.2.1, insert a new row 0 into Table 6 with the following text and re-number the other element Ids accordingly:

| 0 | Backup Channels IE |
| --- | --- |

Insert new sub-clause 6.7.1.2.1 Backup Channels IE as described below and re-number remaining sub-clasuses accordingly:

- 6.7.1.2.1.1 Backup Channels IE

The Backup Channel IE carries a list of the current backup channels defined by the BS. The Backup Channel IE shall be included in the CBP MAC PDU to allow for network discovery and implementation of the spectrum etiquette coexistence mechanism.

- Backup Channels IE

| Syntax | Size | Notes |
| --- | --- | --- |
| Backup_channels_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Length | 8 bits |  |
| Number_Backup_Channels | 8 bits | Number of backup channels in the list |
| For (i=0; i < Number_Backup_Channel; i++) { |  | List of backup channels in order of priority to be used by CPEs in case of loss of communication with the BS due to incumbents. |
| Backup Channel Number [i] | 8 bits |  |
| } |  |  |

References: