IEEE P802.22
Wireless RANs

| Resolution for MAC comment 198: Boosting bits allocation in DS-MAP IE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document provides the proposed resolutions for comment  198.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 198

Comment 198 suggests changing of boosting bits allocation for Table 50 in Draft v.1.0. For convenience suggestion of comment 198 and Table 50 are summarized in Table 1.

Table 1 Summary of suggestion of comment 198

| Cordwords | Draft v.1.0 | Comment 198 (Suggestion) |
| --- | --- | --- |
| 000 | normal(not boosted) | +9dB |
| 001 | +6dB | +6dB |
| 010 | -6dB | +3dB |
| 011 | +9dB | normal(not boosted) |
| 100 | +3dB | -3dB |
| 101 | -3dB | -6dB |
| 110 | -9dB | -9dB |
| 111 | -12dB | -12dB |

Suggestion of comment 198 is re-odering of original one, but it is easy to read by readers. Note that boosting bits allocation of current Draft v.1.0 is the same as that of IEEE 802.16d standard. Eventhough DS-MAP management frame is protected by CRC which is attached after a MAC management frame as shown in Figure 15 on Draft v.1.0. Our purpose for allocation of boosting bits is to support reliability of WRAN system.

After determining average its Tx power, BS must compensate for certain CPEs which need higher or lower power with respect to average BS Tx power. This is called power boosting. The basic tatic of bits allocation is to allocate a minimum hamming distance (HD: bits difference between two codewords) between adjacent codewords, because minimum HD allocation guarantees small change of actual boosting value. For example, 1 bit error, which is most likely occurred, makes only 1 step difference boosting value (3dB). Moreover, in case of round trip (i.e., change of -12dB to 9 or vice versa), we should allocate maxmum HD to protect critical 21dB variation. Table 2 summrized suggested bits allocation as HD point of view

Table 2 Comparison of Hamming distance between cordwords

| Boosting value(dB) | -12 | -9 | -6 | -3 | 0 | 3 | 6 | 9 | -12 | Avg. HD(**) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft v.1.0 | 111 | 110 | 010 | 101 | 000 | 100 | 001 | 011 | 111 |  |
| HD | 1(*) | 1 | 3 | 2 | 1 | 2 | 1 | 1 |  | 1.571428571 |
| Comment 198 | 111 | 110 | 101 | 100 | 011 | 010 | 001 | 000 | 111 |  |
| HD | 1 | 2 | 1 | 3 | 1 | 2 | 1 | 3 |  | 1.571428571 |
| Suggestion | 111 | 110 | 010 | 011 | 001 | 101 | 100 | 000 | 111 |  |
| HD | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 3 |  | 1 |

(*)Hamming distance is calculated by summation of bits difference between adjacent cordwords. Here two codewords such as 111 and 110 only different the last bit. Thus HD(111,110)=1.

(**) The average hamming distance is calculated over all cordwords except round trip cordwords.

Finally, we suggests a new boosting cordwords which guarantee reliability of system as follows

| Cordwords | Boosting value |
| --- | --- |
| 000 | +9dB |
| 001 | 0dB |
| 010 | -6dB |
| 011 | -3dB |
| 100 | +6dB |
| 101 | +3dB |
| 110 | -9dB |
| 111 | -12dB |

Or

| Boosting value | Cordwords |
| --- | --- |
| +9dB | 000 |
| +6dB | 100 |
| +3dB | 101 |
| Normal(not boosting) | 001 |
| -3dB | 011 |
| -6dB | 010 |
| -9dB | 110 |
| -12dB | 111 |

F