IEEE P802.22
Wireless RANs

| Additional text for Figure 14 as a resolution for comment ID #116 - The burst selection method which considers the characteristics of the MAC data in the upstream |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document is an additional text for the Figure 14-Example of a time/frequency structure of a MAC frame. As a resolution for comment ID #116, we need to add the technical ground for dividing the full burst and column burst. This document clarifies when the full bursts and column bursts are used in the upstream subframe.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution and action item for comment ID #116, I submit the additional text for Figure 14. The additional text describes the ground for dividing the full burst and column burst in order to adaptively allocate the appropriate bursts in consideration of the CPE position, channel environment, and service type.

2. Additional text in Red

6.5 General Frame Structure

.

.

.

- Example of a time/frequency structure of a MAC frame

Figure 1 shows an example of the two-dimensional (time/frequency) structure of the MAC frame which shall consist of an integer number of fixed size OFDM slots, each of which shall consist of one OFDM symbol by one sub-channel (i.e., 1 OFDM slot = 1 symbol x 1 sub-channel). To help understand Figure 1, the MAC packets are assumed to be structured in a linear TDM manner (see Error: Reference source not found) while the PHY packets are arranged in a two-dimensional time/frequency domain (symbol in the horizontal direction, logical sub-channels in the vertical direction). For the FCH, the DS/US-MAP, the DCD and UCD, and for the downstream payload the MAC information is first layed vertically by sub-channels, then stepped horizontally in the time direction.

The MAC data elements from Error: Reference source not found, starting from the FCH and including the first broadcast burst, are entered into the second OFDM symbol, as shown in Figure 1, in the increasing order of logical subchannels until all logical subchannels are occupied. Then, the subsequent data elements, if they have not all been mapped, are placed in the same order on the following OFDM symbols. The balance of the last OFDM symbols is then padded with zeros.

The MAC data elements that are contained in upstream bursts are mapped to the US sub-frame in a different order as shown in Figure 1. They are first mapped OFDM symbol by OFDM symbol, in the same logical subchannel. Once a logical subchannel has been filled to the maximum capacity, the balance of the MAC data elements shall be mapped to the next logical sub-channel, in an increasing subchannel order. This process continues until all of the subchannels and symbols alloted to the burst are filled. If the quantity of MAC data elements is insufficient to fill in the burst, padding shall be inserted at the end.

Alternatively, the horizontal laying of the MAC data elements may fill one burst of 7 OFDM symbols at a time.  The MAC data elements are then entered in sequence, on an OFDM symbol-by-symbol order. However, when all logical subchannels have been filled, the next MAC data elements shall be placed in the first available logical subchannel in the following burst. The width of the last vertical burst will be between 7 and 13 symbols depending on the total number of symbols in the upstream sub-frame. When all MAC data elements have been placed in the US subframe, the balance of the last OFDM symbols shall then be padded with zeros. (Note that the US-MAP indicates the length of the US MAC element, not their absolute position in the US map.)

Since bursts of an entire time zone of an upstream subframe are allocated to a power-limited CPE that is located at a cell boundary and does not permit line of sight (LOS) propagation in order to maximize transmitted power per subchannel of the power-limited CPE and the power-limited CPE maps data to the allocated bursts, a maximum signal to noise ratio (SNR) can be obtained. Furthermore, since bursts of a previous time zone in the upstream subframe are allocated to a CPE requiring a quick response from a downstream subframe of a next frame, decoding latency till delivering to media access control (MAC) can be minimized. Moreover, since bursts of a later time zone in the upstream subframe are allocated to a CPE that is located in a cell boundary and permits line-of-sight propagation, the CPE that does not have sufficient time to shift from a reception mode to a transmission mode with only a Tx to Rx transition gap (TTG) can have enough time.

The downstream’s PHY PDU preamble is followed by a FCH burst.  This FCH MAC burst, which is described in Error: Reference source not found, specifies the burst profile and the length of the DS-MAP, if transmitted or the US-MAPs. The DS-MAP message, if transmitted, shall be the first MAC PDU in the burst following the FCH.  An US-MAP message shall immediately follow either the DS-MAP message, if transmitted, or the FCH.  If UCD and DCD messages are transmitted in the frame, they shall immediately follow the DS-MAP and US-MAP messages.  The symbols containing these broadcast MAC control messages shall be modulated using QPSK, rate: ½ (see section Error: Reference source not found) with the mandatory convolutional code mode (see section Error: Reference source not found).

In the upstream direction, if a CPE does not have any data to be transmitted in its US allocation, it shall transmit an US PHY burst containing a general MAC header (see Error: Reference source not found) with its basic CID, together with a bandwidth request subheader (see Error: Reference source not found) with BR = 0. This would allow the BS to reclaim this CPE’s allocation and use the resource for some other purpose.

The BS may schedule up to four types of contention windows (see Error: Reference source not found): the Ranging window is used for initializing the association, the BW request window is for CPEs to request upstream bandwidth allocation from the BS, the UCS Notification window is used by CPEs to report an urgent coexistence situation with incumbents, while the Self-coexistence window is employed by CBP packet for signaling key information to adjacent and overlapping WRAN cells for the purpose of self-coexistence and for carrying out geolocation between CPEs of the same WRAN cell.

The Self-coexistence window is scheduled at the end of the upstream subframe as depicted in Figure 1. The CBPpackets are transmitted by selected CPEs or the BS, and carry information, among other things, about the transmitter’s ongoing service flows with the BS and also about the 802.22 cell as a whole, as well as information to support the self-coexistence mechanism. (see Error: Reference source not found).

A CBP packet shall be transmitted by each CPE associated to a base station at least once every [15] minutes for periodic identification of its MAC address and the associated base station ID.

Whenever a CPE is neither receiving nor sending data to its BS (idle state), it shall be capable of decoding CBP packets transmitted by nearby CPEs belonging to other BSs operating on the same TV channel (N), on adjacent channels (N+/-1), and/or on alternate channels (N+/-2 and beyond).  This mechanism shall also be available at CPE initialization.  In addition, a frame synchronization mechanism is defined so that multiple collocated 802.22 cells can efficiently communicate with each other and align their quiet periods for sensing incumbents.

In addition, the BS has the possibility of using the Self-coexistence window and define what should be done during this time window. The Self-coexistence window may have two purposes as indicated by the BS: listening for beacons (from nearby CPEs associated with the same BS or associated with other BSs) or transmitting beacons.