IEEE P802.22
Wireless RANs

| Proposed Resolution for Comment #151 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Cheng Shan | Samsung Electronics | S. Korea | +82 31 279 7557 | cheng.shan@samsung.com |
| Sungbum Kim | Samsung Electronics | S. Korea | +82 31 279 7092 | sb07.kim@samsung.com |
| Euntaek Lim | Samsung Electronics | S. Korea | +82 31 279 5917 | et.lim@samsung.com |
| Junhong Hui | Samsung Electronics | China | +86 10 5925 3333 3108 | jh.hui@samsung.com |
| Baowei Ji | Samsung Telecom. America | USA | +1 972 761 7167 | bji@sta.samsung.com |

Abstract

This document proposes related text changes on reducing the redundant fields in CBP IEs in section 6.7.1.2 as the resolution to comment #151.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

I. Related Comment & Resolution

CID 151

Comment:

Some of the CPE IEs defined in 6.7.1.2.1 are unnecessarily too long. Considering the maximum number of CBP IE bits a single CBP can carry is only 418, a 3-symbol CBP can hardly carry multiple long IEs in one CBP packet.

Suggested Remedy:

Review section 6.7.1.2.1, and reduce the size of IEs by deleting unnecessary/redundant fields.

Resolution:

"Accept in Principle. Assigned to Wendong Hu , Dave and Cheng Shan to review it.\

Action: Cheng Shan to explain the calculation on the maximum length of the payload.

Action: Cheng Shan to submit contribution suggesting how the size of IE's can be reduced."

II. Proposed Text Changes

(Replace the text from section 6.7.1.2 to section 6.7.1.3 but not including 6.7.1.3 with the following texts.)

      - CBP Beacon frame format

The coexistence beacon protocol (CBP) beacons (discussed in Error: Reference source not found) are inter-cell and intra-cell packets that are transmitted with the goal of improving self-coexistence amongst overlapping 802.22 cells and of enhancing geolocation ranging resolution between CPEs within a cell (section Error: Reference source not found). These beacons are transmitted under the control of the BS during a self-coexistence window.

As discussed in Error: Reference source not found, the beacon MAC header described in Table 5 is utilized by the CBP packets. Overall, the CBP packets provide information about the CPE’s current cell with which it is associated and usually a description of the CPE’s traffic flows with its BS.

To describe its traffic flows with the BS, the beacons transmitted by CPEs shall carry beacon IEs (see 7) in their payload that provide necessary and sufficient information about the CPEs traffic reservations with the BS unless the CPEs have no traffic reservation. Stations (either CPEs or BSs) belonging to other 802.22 cellsthat receive a CBP beacon, can then improve coexistence amongst BSs through a variety of mechanisms such as interference-free scheduling.

- — Beacon MAC header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Beacon_MAC_Header_Format() { |  |  |
| Frame Number | 8 bits | The frame number in which this message is transmitted. See definition in Error: Reference source not found |
| Transmission Offset | 8 bits | Indicates the offset (in symbols) relative to the start of the first symbol of the PHY PDU (including preamble) where the current frame is transmitted. The time instants indicated by the Transmission Offset values are the transmission times of the first symbol of the beacon, that is the preamble. |
| Tx Type | 1 bit | Indicate if the present CBP is sent by a BS or CPE 0: CBP sent by BS 1: CBP sent by CPE |
| If Tx Type = 1 |  |  |
| { |  |  |
| BS ID | 48 bits | Address that uniquely identifies the BS to which this CPE is associated (BS MAC address) |
| } |  |  |
| Reserved | 3 bits | All bit shall be set to 0. |
| HCS | 8 bits | Header check sequence. See definition in Error: Reference source not found |
| } |  |  |

- CBP Information Elements

CBP packets shall carry at least one information element (IE) in their payload among the set described in Table 6. The CBP packets transmitted by CPEs shall carry at least one beacon IE (see Table 7) in their payload, since it provides the basic information required to enable self-coexistence. CBP packets transmitted by the CPEs may also carry a single DS/US Boundary IE. CBP packets transmitted by the BS shall carry at least one DS/US Boundary IE, and may also carry one or more Beacon IE.

- — CBP IEs

| Element ID | Name |
| --- | --- |
| 0 | Beacon IE |
| 1 | DS/US Boundary IE |
| 2 | BS IP Address IE |
| 3 | Inter-BS Capabilities IE |
| 4 | CC_REQ IE |
| 5 | CC_REP IE |
| 6 | CC_ACK IE |
| 7 | RR-REQ IE |
| 8 | RA-RSP IE |
| 9 | RA-ACK IE |
| 10 | RC-REQ IE |
| 11 | RC-RSP IE |
| 12 | RC-ACK IE |
| 13 | RE-REQ IE |
| 14 | RE-RSP IE |
| 15 | RE-ACK IE |
| 16 | RS-SEM IE |
| 17 | RS-ADV IE |
| 18 | BS Channel Parameter IE |
| 19 | CBP Location IE |
| 20 | CBP Geolocation IE |

- Beacon IE

The format of the Beacon IE is described in Table 7.

- — Beacon IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CPE_Beacon_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Direction | 1 bit | Indicates whether this reservation is for Upstream direction (set to 0) or Downstream direction (set to 1) |
| CoS | 3 bits | Indicates the priority of the reservation |
| Reserved | 4 bits | Reserved All bits set to zero. |
| Burst_type | 1 bit | Valid only if Direction = 0 (Upstream direction) This value specifies the Upstream burst type (see Table 63). 0 = Bursts are mapped in the time axis over the full width of the upstream sub-frame before incrementing in the frequency axis; 1 = Bursts are mapped in the time axis over segments of 7 symbols before incrementing in the frequency axis and then re-tracing to the lowest unused subchannel in the next 7 symbol segment. The width of the last segment is to be between 7 and 13 symbols depending on the width of the upstream sub-frame |
| Frame Offset | 8 bits | Indicates the offset (in units of symbols) of this CPE’s reservation with the BS (whether DS or US) relative to the start of the first symbol of the PHY PDU (including preamble) where the frame is transmitted. The time instants indicated by the Frame Offset values are the transmission times of the first symbol of the CPE reservation including preamble (if present). |
| Sub-channel Number | 8 bits | The initial logical sub-channel number of this reservation |
| Duration | 8 bits | Indicates the duration (or length) in units of OFDM slots of this CPE’s reservation with the BS (whether DS or US) |
| } |  |  |

- DS/US Boundary IE

The DS/US Boundary IE is described in Table 8. The DS/US Boundary IE provides information about contiguous blocks of DS or US allocations scheduled by the BS, but it does not provide detailed information about a particular CPE’s allocations. At least one DS/US Boundary IE shall be included in the coexistence beacons transmitted by the BS. Coexistence beacons transmitted by CPEs may also carry a single DS/US Boundary IE.

- — DS/US Boundary IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| DS/US_Boundary_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Starting DS Allocation Channel | 8 bits | Starting logical channel from the perspective of the overall DS allocation by the BS. |
| Ending DS Allocation Channel | 8 bits | Ending logical channel from the perspective of the overall DS allocation by the BS. |
| Ending DS Allocation Symbol | 7 bits | Ending symbol from the perspective of the overall DS allocation by the BS. |
| Starting US Hoziontal Allocation Channel | 8 bits | Starting logical channel from the perspective of the overall horizontal US allocation by the BS. |
| Ending US Horizontal Allocation Channel | 8 bits | Ending logical channel from the perspective of the overall horizontal US allocation by the BS. |
| Ending US Horizontal Allocation Symbol | 7 bits | Ending symbol from the perspective of the overall horizontal US allocation by the BS. |
| Starting US Columns Allocation Channel | 8 bits | Starting logical channel from the perspective of the overall columns US allocation by the BS. |
| Ending US Columns Allocation Channel | 8 bits | Ending logical channel from the perspective of the overall columns US allocation by the BS. |
| Ending US Columns Allocation Symbol | 7 bits | Ending symbol from the perspective of the overall US columns allocation by the BS. |
| Reserved | 3 bits |  |
| } |  |  |

- BS IP Address IE

The BS IP Address IE is sent in a CBP packet payload to indicate the IP address of the BS to which the CPE is associated. The BS that originates the CBP packet supplies its own IP address using this IE.

- — BS IP Address IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| IP Address_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Reserved | 7 bits | All bits set to zero. |
| IP Address Type | 1 bit | 0 = IPv4 1 = IPv6 |
| BS IP Address | 64 bits | IP address of the BS. In case of IPv4 addresses, the 32 most significant bits shall be ignored and the actual IP address shall be carried in the 32 least significant bits. |
| } |  |  |

- Inter-BS Capabilities IE

The Inter-BS Capabilities IE indicates other capabilities in terms of inter-BS communications that are supported by the BS, such as Adaptive on Demand Channel Contention and Dynamic Resource Renting and Offering discussed in 6.21.2.3.

- — Inter-BS Capabilities IE

| Syntax | Length (bytes) | Value |
| --- | --- | --- |
| - Element ID | - 1 |  |
| Inter-BS Capabilities | 1 | Bitmapping indicating other inter-BS communication capabilities supported through CBP. Setting the bit corresponding to a specific capability indicates that the capability is supported by the BS. Bit #0 – Adaptive on Demand Channel Contention (AODCC) Bit #1 – Dynamic Resource Renting and Offering Bit #2-7 – reserved |

- Channel Contention Request IE

The Channel Contention Request (CC_REQ) IE defined in Table 11 is used in the AODCC mechanism. The CC_REQ is sent in a CBP packet payload by the contention source in order to contend for the working TV channels with the contention destination. This is transmitted by the contention source once it has data transmission requirements and occupies no TV channel. If the current working TV channel can not satisfy the QoS requirement, the contention source is also send out this message.

- — CC_REQ IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CC_REQ_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Source Operator Id | 16 bits | Operator identity of the contention source (operator making the sharing request). |
| Destination Operator Id | 16 bits | Operator identity of the contention destination (operator receiving the sharing request). |
| Destination BS Id | 48bits | The MAC address of the contention destination |
| Sequence number | 8bits | Incremented by 1 by the source whenever any of the following three fields change. The contention destinations shall discard the repeated CC_REQ IEs. |
| TV Channel contention number (CCN) | 8bits | A random number to show the priority to contend for the TV channel. CCN is dedicated to the contention resolution in the intra system operator situation. |
| TV Channel Contention Number of Credit Tokens (CCNCT) | 8 bits | Number of credit token per BIN proposed for the contention resolution. CCNCT is dedicated to the contention resolution in the inter operators situation. |
| Start time | 16bits | Starting from the next frame, the number of frames after which the contention source requires to start operation on the requested TV channel. |
| } |  |  |

- Channel Contention Response IE

The Channel Contention Response (CC_RSP) IE defined in Table 12 is used in the AODCC mechanism. The CC_RSP IE is sent in a CBP packet payload by the contention destinations in order to inform the contention source whether the request was accepted. Typically this is transmitted by the contention destination once it has received a CC_REQ IE and after running the contention resolution algorithm.

- — CC_RSP IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CC_RSP_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Source BS Id | 48 bits | Copy from the CC_REQ IE received |
| Sequence number | 8 bits | Copy from the CC_REQ. |
| TV Channel number | 8 bits | The TV channel requested by the source BS |
| Result indication | 2 bits | 00 - SUCCESS (accept the request) 01 -REJECT (reject the request) 10 and 11 - reserved |
| Reason of Rejection | 6 bits | The reason to reject the CC_REQ IE. This field can only be used when result = 01 000000 - the current working period of the contention destination is too short (applicable in the intra operator and inter operators situation). 000001 - the contention destination has smaller channel contention number (CCN) than the contention source (applicable in the intra operator situation). 000010 - the contention source proposes a smaller CCNCT than the contention destination (applicable in the inter operators situation). 000011 - The remaining time to the quiet period is too short (applicable in the intra operator and inter operators situation). 000100—111111 – reserved |
| TV Channel Release Time | 16 bits | Starting from the next frame, the number of frames after which the TV channel can be released. |
| } |  |  |

- Channel Contention Acknowlegdment IE

The Channel Contention Acknowledgment (CC_ACK) defined in Table 13 is used in the AODCC mechanism. The CC_ACK is sent in a CBP packet payload by the contention source in order to notify the contention destinations that the contention source will occupy the destinations’ working TV channels or give up the request. Typically, the contention source shall notify the contention destinations that it will occupy the TV channel if it receives CC_RSP IE with a SUCCESS indication from all the contention destinations. Otherwise the contention source shall notify the contention destinations that it will give up the channel request if it receives CC_RSP IE with a REJECT indication from anyone of the contention destinations.

- — CC_ACK IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| CC_ACK_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Destination Id | 48bits | The MAC address of the contention destination |
| Sequence number | 8 bits | Same as the corresponding CC_REQ IE. The contention destinations shall discard the repeated CC_ACK IE being received |
| TV Channel number | 8 bits | The TV channel being requested by the contention source |
| Start time | 16 bits | Starting from the next frame, the number frames after which the contention source will start operation on the requested TV channel. |
| Occupation code | 2 bits | 00 - Occupy the TV channel 01- Give up the request 10 and 11 - Reserved |
| Reserved | 6 bits | The field shall be set to 0 |
| } |  |  |

- Rent Request IE

The Rent Request (RR-REQ) IE defined in Table 14 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RR-REQ IE

| Syntax | Size | Notes |
| --- | --- | --- |
| RR-REQ_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Offeror BS ID | 48 bit | Indicates which BS the present request is sent to. |
| Renter’s bid | 16 bits | Number of credit tokens per resource unit bid by the renter |
| Renting in Start Time | 16 bits | Start time from which the renter is interested to rent in, and for which the renter’s bid applies for. |
| Renting in End Time | 16 bits | End time from which the renter is interested to rent in, and for which the renter’s bid applies for. |
| } |  |  |

- Resource Allocation Response IE

The Resource Allocation Response (RA-RSP) IE defined in Table 15 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RA-RSP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| RA-RSP_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Renter BS ID | 48 bits | Indicates which BS this response is sent to. |
| Allocation bit flag | 1 bits | Indicates whether offeror BS supplies the TV channel requested by Renter or not. 0 =dissatisfaction 1=satisfaction |
| Reserved | 7 bits | All bits shall be set to 0. |
| } |  |  |

- Resource Allocation Acknowledge IE

The Resource Allocation Acknowledge (RA-ACK) IE defined in Table 16 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RA-ACK IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RA-ACK_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Offeror BS ID | 48 bit | Indicates which BS the present acknowledgement is sent to. |
| Confirmation Code | 8 bits | Error: Reference source not found |
| } |  |  |

- Resource Collection Request IE

The Resource Collection Request (RC-REQ) IE defined in Table 17 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RC-REQ IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-REQ_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Renter BS ID | 48 bit | Indicates which BS the present request is sent to. |
| } |  |  |

- Resource Collection Response IE

The Resource Collection Response (RC-RSP) IE defined in Table 18 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RC-RSP IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-RSP_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Offeror BS ID | 48 bit | Indicates which BS the present response is sent to. |
| RCR bit flag | 1 bits | Resource Collection Response bit flag Indicates Accept or Reject 0 = Reject 1= Accept |
| Reserved | 7 bits | All bits shall be set to 0. |
| } |  |  |

- Resource Collection Response Acknowledge IE

The Resource Collection Response Acknowledge (RC-ACK) IE defined in Table 19 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RC-ACK IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RC-ACK_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Renter ID | 48 bit | Indicates which BS the present acknowledgement is sent to. |
| Confirmation Code | 8 bits | Error: Reference source not found |
| } |  |  |

- Returning Request IE

The Returning Request (RE-REQ) IE defined in Table 20 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RE-REQ IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-REQ_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Offerer BS ID | 48 bits | Indicates which BS the present request is sent to. |
| } |  |  |

- Returning Response IE

The Returning Response (RE-RSP) IE defined in Table 21 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RE-RSP IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-RSP_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Renter BS ID | 48 bits | Indicates which BS the present response is sent to. |
| Confirmation Code | 8 bits | Error: Reference source not found |
| } |  |  |

- Returning Response Acknowledge IE

The Returning Response Acknowledge (RE-ACK) IE defined in Table 22 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RE-ACK IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RE-ACK_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Offeror BS ID | 48 bits | Indicates which BS the present acknowledgement is sent to. |
| Confirmation Code | 8 bits | Error: Reference source not found |
| } |  |  |

- RS-SEM IE

The Resource Spectrum Etiquette Mechanism (RS-SEM) IE defined in Table 23 is used in the Spectrum Etiquette Mechanism.

- — RS-SEM IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS-SEM_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| TV channel number | 8 bits | Active TV channel number |
| Number of Backup TV channels | 8 bits |  |
| for (i=0; i < Number of Backup TV channels; i++) { |  | In priority order. |
| TV channel Number[i] | 8 bits |  |
| } |  |  |
| } |  |  |

- RS-ADV IE

The Resource Advertisement (RS-ADV) IE defined in Table 24 is used in the Dynamic Resource Renting and Offering Mechanism.

- — RS-ADV IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| RS-ADV_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| Renting out Start Time | 16 bits | Starting time of the renting period. It is the time from which the offering channel’s available resource is opened for renting. |
| Renting out End Time | 16 bits | Ending time of the renting period. It is the time till which the offering channel’s available resource is opened for renting. |
| MNCT | 16 bits | Minimum number of credit tokens per resource unit required per renter’s bid. |
| RS | 1 bit | Resource Sharing field Indicates the resource advertisement resource advertisement =1 normal =0 |
| reserved | 7 bits |  |
| } |  |  |

- BS Channel Parameter IE

- — BS Channel Parameter IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| BS-Channel-Parameter_IE_Format() { |  |  |
| Element ID | 8 bits |  |
| TV Channel Number | 8 bits | The operating TV channel of the sender BS |
| CBP Preferred TV Channel Number | 8 bits | Valid only if greater than 0. The preferred TV channel by this BS to exchange CBP packets |
| } |  |  |

- CBP Location IE

The CBP location IE specifies the geo-location of the present CBP source, which is either a BS or a CPE.

- — CBP Location IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| Location_Information() { |  | An LCI with Latitude resolution, Latitude, Longitude, and Longitude resolution set to zero shall indicate that the location is not known. |
| Element ID | 8 bits |  |
| Latitude Resolution | 6 bits | Latitude resolution indicates the number of valid bits in the fixed-point value of Latitude |
| Latitude | 34 bits | Latitude is a fixed point value consisting of 9 bits of integer and 25 bits of fraction |
| Longitude Resolution | 6 bits | Longitude resolution indicates the number of valid bits in the fixed-point value of Longitude |
| Longitude | 34 bits | Longitude is a fixed point value consisting of 9 bits of integer and 25 bits of fraction |
| Datum | 8 bits | Datum is encodes the horizontal and vertical references used for the coordinates. The Datum octet has 256 possibilities, of which 3 have been registered with the Internet Assigned Numbers Authority (IANA): 1: WGS 84 (Geographical 3D) – World Geodesic System 1984, Coordinate Reference System (CRS) Code 4327, Prime Meridian Name: Greenwich; 2: NAD83 – North American Datum 1983, CRS Code 4269, Prime Meridian Name: Greenwich; The associated vertical datum is the North American Vertical Datum of 1988 (NAVD88); 3: NAD83 – North American Datum 1983, CRS Code 4269, Prime Meridian Name: Greenwich; The associated vertical datum is Mean Lower Low Water (MLLW). The WGS 84 datum shall be used when referencing locations anywhere. The GeoConf Option referred to in IETF RFC 3825 defines two fields for which the IANA maintains a registry: The Altitude type (AT) field and the Datum field. The initial values of the Altitude registry are as follows: AT = 1 meters of Altitude defined by the vertical datum specified; AT = 2 building Floors of Altitude. |
| } |  |  |

- — CBP Geolocation IE format