IEEE P802.22
Wireless RANs

| Resolution for MAC comment 561: Initialization and Updating Channel Sets |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document provides the proposed resolutions for comment  561.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 561

To resolve the comment 561, this contribution suggests the original text as following:

Section 6.21.4.1 merged with Section 6.21.4.3 as Section 6.21.4.1( Doc:08/267)

Section 6.21.4.2Moved to Section 9.2.2.1(Doc:08/268)

Section 6.21.4.3merged with Section 6.21.4.1

Replace Section 6.21.4.1(Line 36 on pp.260) ~6.21.4.3 with the following:

      - (6.21.4.1) Initialization and Updating Channel Sets

For efficient channel management, Section 9.2.2 adressed channel classification and selection rules. In this subsection, procedure of channe list initialization and updating are addressed.

In order to maintain the channel sets, each BS maintains available channel sets (i.e., Operating, Backup, Occupied, Candidate, and Uncrassified). Also, each CPE maintains all channel sets. These individual sets have different update duration. For example, in CPE side, Operating sets are updated after every receiving of SCH, Backup sets are updated after receiving DCD and Occupied and Candidate sets are updated after receiving CHO-UPD message (See 6.8.21.9). After synchronization, BS should send CHO-UPD message to CPE to update the Occupied set for newly connected CPE. These relations are summarized in Table 1.  Whereas, in BS case, channel sets are updated after each quiet period either at a periodic interval or aperiodic interval.

Table 1 Update channel set information in CPE

| Message | Field | Information | Note |
| --- | --- | --- | --- |
| SCH | TxID,CN,NC | Operating channel |  |
| DCD | Channel Number for Backup | Backup channels |  |
|  | Number of Channles for Backup |  |  |
| CHO-UPD | Channel number and channel state | Occupied, Candidate and Uncrassified channels |  |

When a CPE turn on its power, the CPE executes channel sets initialization procedure as shown in Figure 2. This procedure is closely related with obtaining downlink parameters procedure (See. 6.15.4). After a new CPE connected to BS, BS shall send the CHO-UPD message to indicate occupied channel information to the CPE. Therefore the CPE initializes the Occupied Set by receiving CHO-UPD message. Channel sets in the CPE are updated after receiving corresponding messages such as SCH, DCD, and CHO-UPD. In the case of BS, after each sensing period, BS received BLM-REP message. If channel sets are changed depending on BLM-REP messages. BS sends CHO-UPD to corresponding CPEs and updates new channel information in its channel sets. This procedure is depicted in Figure 3.

| - —CPE Channel Sets initialization | - —BS Channel Sets update |
| --- | --- |