IEEE P802.22
Wireless RANs

| Modification of section 6.7.1.2.1 (Comment #152) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-10-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution proposes a resolution for comment #152 on the apparent redundancy in the text of the first paragraph in section 6.7.1.2.1 as well as a re-consideration of the need for transmitting the Beacon IE and the DS/US boundary IE from one cell to another in the CBP burst.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Re-write of text in section 6.7.1.2.1 (resolution of comment #152)

This contribution proposes a resolution of comment #152 on the CBP burst IE’s, the apparent redundancy of the text of the first paragraph and a reconsideration of the need for transferring information on the CPE traffic MAP information and DS/US boundary between WRAN cells.

As indicated on slide 28 of contribution 22-08-0137-03-0000-wran-coexistence-considerations.ppt, related to the “bi-directional burst collision” that may occur between two nearby CPEs belonging to two different WRAN cells where a CPE trying to receive a DS packet from its BS would be impacted by a nearby CPE that starts to transmit an upstream burst towards the BS to which it is associated before the first CPE has completed its reception.  As was conceived in the preparation of the first Working Document, one way to do it would be to alert the surrounding CPEs of the location of the actual DS and US bursts for the specific CPE and te location of the DS/US boundary so that the transmitting CPE does not start its transmission while the receiving CPE is still expecting its downstream burst or before the end of the DS sub-frame in the other WRAN cell.

Since it was decided that the upstream burst would be mapped horizontally to minimize the instantaneous EIRP required to reach the BS since a minimum number of sub-carrier would be used, there will not be much flexibility in scheduling the US burst in time.  However, since the DS burst is mapped vertically (using all sub-carriers during a minimum number of OFDM symbols), there will be a lot more flexibility in the DS sub-frame for the BS to schedule the DS burst going to a CPE where such “bi-directional burst collision“ may occur for an early transmission in this DS burst.  The only thing needed is therefore for the BS to identify the CPEs where such “bi-directional burst collision“ may occur and then make sure that the bursts going to these CPEs are scheduled first in the DS sub-frame.  The identification of these potentially affected CPEs can easily be done by monitoring the CBP bursts transmitted by nearby CPEs.  There is no need to indicate where the actual DS or US bursts will happen or where the DS/US boundary is to occur because the other CPE cannot really control when its US burst is scheduled because of the horizontal mapping (few sub-carriers but all symbols in the US sub-frame).

In order for this to work without forcing all DS/US boundaries to be the same across coexisting WRAN cells (negating the flexibility in adjusting the DS/US capacity ratio) and indicating such DS/US boundary, a maximum size US sub-frame should be set to allow a buffer zone for the DS to schedule the bursts going to these affected CPEs where the US bursts would never occur.  Depending on the number of CPEs in this situation (i.e., in the overlap area) and their expected download capacity, leaving a minimum of 6 symbols in the DS sub-frame (minimum to allow for channel training) would normally be sufficient to absorb the DS capacity going to these affected CPEs.  The size of the US sub-frame could then vary up to a maximum defined by the number of data symbols in the frame minus the 6 symbols needed for proper training or somewhat less if there are a large number of CPEs in the overlap area.

Under the above conditions for the “bi-directional burst collision” and based on the fact that it has been decided that the unit of capacity sharing in coexistence situation will be the frame, there is no longer a need to exchange burst scheduling information between WRAN cellsnor DS/US Boundary information as  described in sections 6.7.1.2.1.1 and 6.7.1.2.1.2.  It is therefore proposed to delete these two sections and update Table 6 accordingly.

Other modifications to the text are also proposed as indicated below:

        - CBP Information Elements

CBP packets shall carry at least one information element (IE) in their payload among the set described in Table 6 since it provides the basic information required to enable self-coexistence. CBP packets shall carry at least one Backup Channel information element (IE) in their payload. This is to allow WRANs to execute the spectrum etiquette mechanism before deciding to execute the other spectrum sharing mechanisms described in 6.21.2.3.

- — CBP IEs

| Element ID | Name |
| --- | --- |
| 0 | Backup Channel IE |
| 1 | Inter-BS Capabilities IE |
| 2 | CC_REQ IE |
| 3 | CC_RSP IE |
| 4 | CC_ACK IE |
| 5 | RR-REQ IE |
| 6 | RA-RSP IE |
| 7 | RA-ACK IE |
| 8 | RC-REQ IE |
| 9 | RC-RSP IE |
| 10 | RC-ACK IE |
| 11 | RE-REQ IE |
| 12 | RE-RSP IE |
| 13 | RE-ACK IE |
| 14 | RS-SEM IE |
| 15 | RS-ADV IE |
| 16 | BS Channel Parameter IE |
| 17 | CBP Location IE |
| 18 | CBP Geolocation IE |

- — Beacon IE format

          - DS/US Boundary IE

The DS/US Boundary IE is described in

— DS/US Boundary IE format

_______________________________