IEEE P802.22
Wireless RANs

| Resolution for Comment ID 161 and 163 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Kuffner | Motorola | Schaumburg, IL | 874-538-4158 | stephen.kuffner@ motorola.com |

Abstract

This submission addresses IEEE 802.22 Letter Ballot #1 Comment #161 and 163 pertaining to channel contention methods of coexistence.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### Introduction

The comments pertain to Table 11 in Clause 6.7.1.2.1.5 and the information elements used in channel contention resolution.

### Discussion

| COMMENT 161 |  |
| --- | --- |
| Commented Text | Table 11, Clause 6.7.1.2.1.5 |
| Comment Text | Is there text within the document explaining how the Source Operator Id is chosen or assigned? I'm not suggesting adding an explanation here, but I searched for "Source Operator Id" and couldn't find an explanation anywhere. |
| Resolution Statement | Term coming from Motorola coexistence proposal. The operator may own more than one BS. Scheme may depend on whether the BS's belongs to the same operator or not. Action: Steve Kuffner to clarify. |

The “Source Operator ID” term actually comes from the joint coexistence proposal [1], though it is mentioned only in the portion of that contribution (due to Wendong Hu and David Grandblaise) that differentiates contention techniques based on whether that contention is intra-operator or inter-operator.

For some background, the operator is of course understood to be the entity managing the communications infrastructure equipment and services.  If contention is between base stations under the control of a single operator, that contention is intra-operator and based on the Adaptive On-Demand Channel Contention (AODCC) method, where a random number is chosen by each base station participating in the contention, and the present holder of the spectrum compares the random numbers to determine the next occupant.  If the contention is inter-operator, a Credit-Tokens bid is offered by the requestor.  In the lexicon of channel contention, the “source” operator is the operator making the spectrum sharing request (the requestor) or bid.   See “Notes” column in Table 11 and also clause 6.21.2.3.4.1.  The ID isn’t specifically a source operator ID; it is an operator ID that is attributed to the source BS in the contention procedure.  So, an operator ID would function as a source operator ID when a BS is requesting spectrum and as a destination operator ID when receiving a request.

However, it is presumed that an operator would know the BS IDs (MAC addresses) of its stations in the vicinity and that these addresses could be shared via a higher layer.  Since MAC addresses are already contained in the CC_REQ and CC_RSP IEs, this information ought to be sufficient for a BS to determine whether the source BS is managed by the same operator or not.  It would seem that a specific operator ID isn’t necessary – only some “yes/no” means of determining whether the contending BS is inter- or intra- operator is required, and this can in principle be handled by inspecting a list of BS IDs.

Proposed Resolution

The source and destination operator IDs are superfluous and can be removed from the CC_REQ (Table 11), CC_RSP (Table 12), and CC_ACK (Table 13) IEs.  Elimination of these fields in the IEs reduces the size of the IEs by 4 bytes (16 bits each).  To enable decision making, the BSs will require access to a list of MAC addresses of other BSs in the vicinity that are managed by the same operator.  Then there is no need to explain how an operator ID is chosen or assigned and the comment is remedied.  Access to the list is beyond the scope of the draft and is probably more appropriately addressed in the recommended practice.  If the MAC address list was not accessible, the BS would have to assume that the contention was inter-operator.

| COMMENT 163 |  |
| --- | --- |
| Commented Text | Table 11, Clause 6.7.1.2.1.5 |
| Comment Text | The text says, "Number of credit token per BIN..." Why is "BIN" in capital letters? Also, it might be clearer if it were called "frequency bin" like it is called in the rest of the draft. "Token" should be plural. |
| Resolution Statement | The basic unit of spectrum sharing is a frame. Action: Steve Kuffner to modify the text accordingly. |

As originally conceived, the spectrum sharing quanta for the credit tokens approach was a time-frequency allocation (or “BIN”) within an OFDMA block.  However, the basic unit of spectrum sharing is now quantized in integer frames according to a motion made at the May 2008 Interim [2].  Thus the “BIN” is no longer relevant and should be stricken from the draft.  “BIN” is only used in Table 11.

Proposed Resolution

Text for the CC_REQ IE in the Notes column of Table 11 for the “TV Channel Contention Number of Credit Tokens (CCNCT)” row:

“Number of credit tokens per BINframe proposed for the contention resolution. CCNCT is dedicated to the contention resolution in the inter operators situation.”

References

[1]	IEEE 802.22-07/0121r3, “Proposed text changes and Comment Resolution to Section 6.21.2 Self-coexistence in IEEE 802.22/D0.2 Draft Standard,” D. Cavalcanti et al, 14 March 2007.

[2]	IEEE 802.22-08/0199r0, “Draft Minutes of the Jacksonville Interim Session of 802.22 May 2008,” E. Au et al, 22 May 2008.