IEEE P802.22
Wireless RANs

| Resolution for MAC comment 217: The length of timing adjustment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |

Abstract

This document provides the resolutions for comment 217. This document investigates the effect of timing offset to decide whether timing adjustment in the unit of fractional TU is required in the WRAN system.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 217

<<Related Section 6, table72>>

To change my vote from no to approve, change table 72 element 1 length to 32 bits (16 lower bits to be used as fractional TU to specify to a much more precise resolution).

Investigation of Comment 217

Table 72 element 1 is “time adjust” field of the RNG-RSP. It is to adjust the time at which CPEs transmit the signal in the upstream. In other words, it is for the timing synchronization of the signals which are received at the BS. The unit of the time adjustment is the sample duration of the OFDMA symbol, which is defined by Time Unit (TU) in the current draft v1.1.

This document investigates the effect of timing offset to decide whether timing adjustment in the unit of fractional TU is required.

In the OFDMA system, the signals transmitted by CPEs can be arrived at the BS among three possible positions.

- Case 1: On time of FFT starting point

- Case 2: In the guard interval duration

- Case 3: After the guard interval

Effect by timing offset

Case 1: There is no timing offset.

Case 2: Only phase rotation will be appeared in the frequency domain. It can be compensated by the channel equalization. There is no any additional noise and degradation of orginal signal by timing offset. So, it is meaningless to adjust the timing offset in the unit of fractional TU.

Case 3: Signal degration and additional noise will be appreaed by ICI and ISI.

Below equation represents the received signal with timing offset as case 3 [1].

(ICI)

(ISI)

(AWGN)               		                                                                                                                            (1)

Parameters:

l,k : Transmitted signal at the kth subcarrier in the lth OFDMA symbol

Y   : Received signal in the frequency domain

N   : Length of FFT

: Timing offset in the unit of sample time (TU)

Wl,k   : White Gaussian Noise

The first row in the equation shows the signal distortion and phase rotation. Like case 2, phase rotation can be easily compensated by channel equalizer.

The second and third rows in the equation shows the ICI(Inter Carrier Interference) and ISI(Inter Symbol Interferecne), respectively. The additional noise of two effects is approximated by Gaussian noise and its variance is  [1, 2].

(2)

If timing offset with one, 1/2 and 1/4 sample of case 3 are occurred, the signal degradation and additive noise based on equation (1) and (2) is below table.

| Degradation of Signal power |  |  |
| --- | --- | --- |
| Timing offset | Linear | dB Scale |
| 1/4 sample | 0.000244 | -36.12386456 |
| 1/2 sample | 0.000488 | -33.1138297 |
| One sample | 0.000976 | -30.10405999 |

| Additive Noise Power |  |  |
| --- | --- | --- |
| Timing offset | Linear | dB Scale |
| 1/4 sample | 0.000244 | -36.12386456 |
| 1/2 sample | 0.000488 | -33.1138297 |
| One sample | 0.000976 | -30.10405999 |

Both degradion of signal power and additive noise power occurred by one sample offset are almost -30 dB. It can be ignored on the synchronization performance. The effect of fraction timing offset is even less than that of one sample.

Furthermore, receivers of the OFDM system can intend to shift the position of FFT in the guard interval duration (case 2) in order that ISI doesn’t be happened. So, the effect of timing offset with case 3 can be minimized or eliminated. It means that BS try to make the signals transmitted from CPEs be located in the guard interval, more precisely in the duration from -25% of the CP to the last sample of the CP.

From the above all investigations, we don’t need the fractional TU for specifing to a much more precise resolution in the synchronization point of view.  Therefore, WRAN doesn't require the 32bits to indicate the franction TU on the time adjust field.

Reference

[1] Michael Speth, Stefan A. Fechtel, Gunnar Fock, Hsinrich Meyr, “Optimum Receiver Design for Wireless Broad-Band Systems Using OFDM-Part I,” IEEE Transactions on communications, Vol. 47, NO. 11, November 1999

[2] Hlaing Minn, Vijay K. Bharagava, “A Robust Timing and Frequency Synchronization for OFDM Systems,” IEEE Transactions on Wireless Communications, VOL. 2, NO. 4 July 2003