IEEE P802.22
Wireless RANs

| Resolution for MAC comment 372: OFDMA-based periodic ranging |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |

Abstract

This document provides the proposed resolutions for comment 372.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 372

<<Related Section 6.17.10.2>>

In addition to CDMA-based periodic ranging, is OFDMA-based periodic ranging supported as well? Referring to Table 57 (Additional UCD channel information elements), there is only CDMA-based UCD IE available for periodic ranging.

Please clarify and update the text accordingly.

Investigation of Comment 372

Assumption:

- Clarifying the OFDMA-based periodic ranging

 There is no any place using the term of “OFDMA-based” in the draft v1.0. So, we can’t exactly know the meaning of “OFDMA-based”. Therefore, we assume that “OFDMA-based periodic ranging” is contention-based periodic ragnging using the MAC message.

- The necessity of periodic ranging for WRAN system

Currently, the necessity of periodic ranging is being discussed in IEE802.22 WG. In this document, we don’t take care of necessity but the method to perform the periodic ranging for WRAN system.

Periodic ranging:

The objective of periodic ranging is to adjust the timing and power level of data transmitted from CPEs. It is performed through random access by CPEs. BS may estimate the timing offset and received SNR (or signal power) with the known signal transmitted by CPEs such as preamble or pilots. There can be two kinds of periodic ranging, CDMA-based periodic ranging and contention-based periodic raging. Two types of periodic ranging have both advantages and disadvantages which are described in the following table.

|  | CDMA-based | Contention-based |
| --- | --- | --- |
| Transmitted Data for periodic ranging | CDMA code | MAC Message (RNG-REQ) |
| User Classification | Impossible (CDMA code IE) | Possible (CPE MAC Address) |
| Required OFDMA symbols | One OFDMA symbol (a kind of Preamble) | At least Seven OFDMA symbols ( 7x7 pilot pattern) |
| Latency | At least two frames | One frame |
| Contention among CPEs | Possible (Different codes) | Possible (Random Backoff, More frames are required.) |
| Capacity | Be able to share the capacity with CDMA-based initial ranging | Required new capacity for contention-based periodic ranging |

Periodic Ranging in the IEEE 802.16:

In the IEEE802.16 WirelessMAN, there are three types of PHY specific, Single-Carrier(SC), OFDM and OFDMA. All of them support the periodic ranging. SC and OFDM specific use the contention-based periodic ranging since they can have a preamble in the time domain. OFDMA specific, however, uses only CDMA-based periodic ranging.

UIUC in the current IEEE802.22 draft v1.0:

UIUC (Table 62 in the draft v1.0) is related to the Periodic ranging since it is transmitted on the upstream.  However, there is no value of UIUC for contention-based periodic ranging in the current draft. (Contention-based BW request and contention-based UCS notification is defined. CDMA Ranging which includes the initial and periodic ranging is also defined.)

Conclusion

Periodic ranging can be performed by two types CDMA-based or contention-based. However, periodic ranging in the IEEE802.16 OFDMA PHY specific is performed by CDMA-based periodic ranging for contention free among CPEs and sharing the resource with initial ranging. Also, there is no any hook for contention-based periodic ranging in the UIUC of current draft v1.1. Therefore, it is correct that there is only CDMA-based UCD IE available for periodic ranging.