IEEE P802.22
Wireless RANs

| Proposed resolution to Comment 283 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Charles Einolf | CBS | 3007 Argentina Place Bowie, MD 20716-3824 USA | +1-301-806-1544 | c.einolf@ieee.org |

Abstract

The following proposed text addresses comment #283 relative to separating normative and informative text for inclusion in the IEEE P802.22 draft standard and the IEEE P802.22 recommended practice, respectively.  The proposed text is based upon documents 802.22_DRAFTv1.0.doc and 22-09-0022-00-0002 with all changes accepted in order to illustrate the proposed further changes made herein.  Text applicable to the recommended practice will be provided separately.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed text for Section 6.14.5 of the 802.22 standard:

    - Transmit Control at CPEs and BS for the Protection of TV band Incumbents

WRAN CPEs and BSs shall not operate on the same channel or on the first adjacent channels of a TV operation within the TV protected contour.  However they may operate on co-channels or adjacent channels outside this protected contour as long as they are located at sufficient separation distances beyond this protected contour.

Consequently, if there is a TV operation on channel N, a WRAN CPE located within the TV protected contour of that TV station:

- Shall not transmit on channel N-1

- Shall not transmit on channel N

- Shall not transmit on channel N+1

- May be constrained from transmitting on alternate channels (N±2 and beyond),

as defined by up-to-date information from the incumbent database.

Furthermore, if there is a TV operation on channel N, a WRAN CPE outside of the TV protected contour of that TV station:

- Shall not transmit on channel N within a separation distance from the TV protected contour.

- Shall not transmit within a separation distance from the TV protected contour on channel N-1

- Shall not transmit within a separation distance from the TV protected contour on channel N+1.

The geolocation data shall be entered at the base station.  The constraints, may be established by the local regulatory body or, alternatively, if no database exists and permitted by local regulatory body may be developed using the methodology provided in the 802.22.2 WRAN Recommended Practice.

References:

802.22_DRAFTv1.0.doc

Document 22-09-0022-00-0002-proposed-resolution-to-comment-283.doc