IEEE P802.22
Wireless RANs

| TV Whitespace Petition for Reconsideration Input from 802.22 to 802.18 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carl R. Stevenson | WK3C Wireless LLC | 4991 Shimerville Rd. Emmaus, PA 18049-4955 | 610-841-6180 | wk3c@wk3c.com |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario Canada K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |
| Victor Tawil | MSTV |  |  |  |
| Ivan Reede | Amerisys |  |  |  |

Abstract

This document contains text from 802.22 as input to 802.18 for a Petition for Reconsideration of the FCC’s Report and Order in the “TV Whitespace” docket.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

#### Before the

FEDERAL COMMUNICATIONS COMMISSION

Washington, D.C. 20554

| In the Matter of Unlicensed Operation in the TV Broadcast Bands |  | ) ) ET Docket No. 04-186 ) |
| --- | --- | --- |

IEEE 802.18 Petition For  Reconsideration

IEEE 802.18, the Radio Regulatory Technical Advisory Group (“the RR-TAG”) within IEEE 802 hereby respectfully submits its Petition for Reconsideration of the Commission’s November 4, 2008, decision in the above-captioned rulemaking relating to Part 15 devices operating on vacant TV channels (“white spaces”).  This document was prepared and approved by the RR-TAG, and also was reviewed by the IEEE 802 Executive Committee.

The members of the RR-TAG that participate in the IEEE 802 standards process are interested parties in this proceeding.  We appreciate the opportunity to express our concerns, seek clarifications and/or modifications to some of the rules adopted by the Commission for this instant proceeding.

introduction

On November 4, 2008, the Commission adopted a Second Report and Order (Second R&O) that establishes rules to allow new, sophisticated wireless devices to operate in broadcast television spectrum on a secondary basis at locations where that spectrum is available. (This unused TV spectrum is now commonly referred to as television “white spaces”).  In IEEE 802.18’s view, this represents a significant opportunity for industry to participate in developing new technology and applications for wireless services.

Etc......

the commission’s  adopted rules for fixed operation in the Tv Bands  will prohibit and/or limit the operation of IEEE 802.22 compliant devices

In its previous filing to the Commission  IEEE 802.18 supported the Commission’s first Report and Order findings that the protection of incumbent operations in the TV bands is a much more tractable problem when devices are limited to fixed operation.  Fixed point to multipoint systems with a master/slave relationship between base stations and user terminals, coupled with sensing across the network, geolocation/database techniques, and transmitter power control, can provide a viable means of bringing broadband fixed access services to less densely populated rural areas and other unserved/underserved areas where spectrum is available.

In this instant proceeding, however, the Commission adopted the final rules to authorize fixed system operation on unused spectrum in the TV bands containing some provisions that will limit or prohibit certain operation or functionality of IEEE-802.22 compliant devices and the proposed draft of the 802.22 Standard. Specifically the Commission need to clarify or reconsider modifying their rules on issues and/or topics listed below so as to allow for the most efficient and economical use for fixed operation in this band.

A- Fixed devices (base stations) should be allowed to have multiple fixed client/slave devicesSection 15.711f states that: "A fixed device may not operate as a client to another fixed device."  This rule would prohibit fixed devices from using a "master/slave” protocol as proposed for 802.22 systems.  The IEEE 802.22 architecture is designed around a protocol where a client/slave fixed device is required to operate under the control of and communicate with a master fixed device (a base station). As mentioned in our previous filing, IEEE 802.22 established a class of fixed client/slave devices known as Customer Premise Equipment (CPEs) with a power output of up to 4 W EIRP which can determine their geolocation parameters and send them to the master/base station as a condition for association.  The master/base station is responsible for querying the database on behalf a CPE requesting association to get a list of available channels which may be used by the CPE at its location.   In order to associate with a master/base station and begin normal operation, a CPE must receive a list of available channels and permission to operate from the master/base station.  This should have been provided for in the R&O.  The Commission should define a master/slave relationship as described above and should modify its rules to allow those classes of stations.

B- Sensing for television signals should be optional rather than mandated by the Commission in the case a reliable database exists

In Section 15.711a.1 and a.2, the Commission mandated the use of a geolocation and database access mechanism as well as sensing for both fixed and personal/portable operation.

If the database is used, there is no longer a need for a 10 meter minimum receive antenna height requirement and the fixed CPE antenna heights indicated in the proposed new Table below would be feasible for both transmit and receive antennas.  This change would significantly reduce the cost and complexity of CPE equipments and installation, benefiting consumers and making rural broadband access more affordable and ubiquitous.

- C- Fixed base station height should be based on HAAT rather than AGL and not limited to 30m AGL

In para. 181 of the R&O the Commission limited the antenna height for fixed unlicensed TVBDs to 30 meters above ground level (AGL) and used that limit for its calculation of the separation distance to the TV protected contour.  In the case of a fixed base station located on a mountain, using 30 meters AGL would considerably underestimate the required separation distance and compromise the protection of the TV service in some situations and would unnecessarily limit fixed base station coverage area in other situations, requiring the deployment of more base stations than necessary, thereby increasing cost to consumers.

IEEE 802.18 recommends that the antenna height for such fixed base stations should be expressed in terms of HAAT (Height Above Average Terrain) rather than height above ground level (AGL) to allow the determination of the appropriate separation distances necessary to protect the incumbents while allowing to achieve extended coverage.  We recommend that HAAT be used to determine the required separation distance from TV protected contours as described below in the Table below.  The method for calculating HAAT should be the same as was employed in Part 90 to protect the TV service from PLMRS.  In addition, we recommend no limits on the antenna height above ground for fixed base stations.  We further recommend that no changes in the assumption of antenna heights of 10m AGL for fixed user terminals (CPEs) be made for the purpose of calculating the separation distance to the TV protected contour.

| Antenna height of Unlicensed Device (AGL for personal/portable and fixed CPE, HAAT for fixed base station) | Required Separation (km) From Digital or Analog TV (Full Service or Low Power) Protected Contour |  |
| --- | --- | --- |
|  | Co-channel | Adjacent Channel |
| Less than 3 meters | 6 km | 0.1 km |
| 3 – Less than 10 meters | 6.9 km | 0.256 km |
| 10 – Less than 30 meters | 10.8 km | 0.285 km |
| 30 – Less than 50 meters | 13.6 km | 0.309 km |
| 50 – Less than 75 meters | 16.1 km | 0.330 km |
| 75 – Less than 150 meters | 22.6 km | 0.372 km |
| 150 – Less than 300 meters | 32 km | 0.405 km |
| 300 – Less than 600 meters | 45.7 km | 0.419 km |
| 600 – Less than 1200 meters | 68 km | 0.426 km |

D- PSD limits and minimum occupied bandwidth should be specified

In the January 2007 comments to the FCC, 802.18 recommended that the minimum channel bandwidth to be used in the TV white space should be 6 MHz.  In the second R&O, the Commission conducted all its tests based on a wideband noise-like signal.  However, in specifying the maximum power, it failed to specify a minimum bandwidth.  IEEE 802.18 believe that, there is a need to allow narrower bandwidths than proposed earlier while maintaining the same protection for incumbents on the 6 MHz systems.  We therefore believe that the Commission should specify a minimum bandwidth and a maximum spectrum power density.

Specifically, we propose that, in addition of a maximum of 1 W conducted power in 6 MHz, the maximum PSD per 100 kHz should not exceed 100 mW.  We also propose that a minimum occupied bandwidth of 500 kHz be required in order to facilitate correctly identifying a Part 74 wireless microphone from narrowband unlicensed transmissions.

Another possibility to define the maximum PSD limit is to use the approach as defined in Part 15.247e where the maximum PSD is specified as 8 dBm (conducted) in 3 kHz (which corresponds to 210 mW in 100 kHz).  Again, in this case, a minimum occupied bandwidth of 500 kHz is required in order to differentialte Part 74 wireless microphones from narrowband unlicensed transmissions for sensing purpose.

E- RF mask

The RF mask should be defined relative to the total power in 6 MHz rather than relative to the PSD in the reference bandwidth [100 kHz] as stated in paragraph 10 to preclude the allowable out-of-band emission levels from increasing when the transmission is concentrated in a narrower bandwidth, i.e., the PSD in the 100 kHz is higher (re: 15.709 c1&2).  A transmission narrower than the channel bandwidth (6 MHz) would be allowed to increase its first adjacent PSD by the same amount as its in-band PSD increases since the current mask defines the -55 dB level relative to the maximum PSD in 100 kHz within the 6 MHz TV channel.

F- The sensing threshold for wireless microphones should be -107 dBm rather than -114 dBm

IEEE 802 believes that the proposed -114 dBm sensing threshold is too low and results in the sensing device very often wrongly detecting the presence of wireless microphones.  The original proposal from 802.22 was –107 dBm and we still believe that this threshold would adequately protect the incumbents.  This higher threshold would help to avoid the problem of false detection of non-existent wireless microphones due to the presence of spurious and noise in the channel, as well as an easy denial of service attack resulting from the presence of very simple RF oscillators in the channel.  Having a minimum bandwidth of 500 kHz as proposed above would also prevent other unlicensed devices being erroneously detected as wireless microphones.

There is no way for sensing to differentiate between a legal Part 74 device and any other narrowband signals sources such as spurious signals as allowed by Part 15.209a since the specified threshold is 33.5 dB below the permitted level from a source at a 10 m distance.  Additionally, the  –114 dBm sensing threshold is comparable to the level of man-made noise in the bandwidth authorized for Part 74 devices.

It may be appropriate to use different thresholds to allow protection for different types of services such as DTV and wireless microphones

[Rationale for sensing: Beyond the database use, there is a need for sensing both wireless microphones and the 802.22.1 beacon which allows for enhanced protection because one cannot predict where a news event will happen.  Although database access can be very fast, this will not allow updates in realtime by the news crew. Broadcasting news crews rely heavily on wireless microphones and not sensing it would be very disruptive to the way they operate.  IEEE 802 needs to stay consistent with its previous position that wireless microphone sensing is needed.]

G- Part 74 devices need to be sensed within 2 seconds, not 60 seconds

The timing parameters proposed for wireless microphone sensing are different than the ones that were determined in the 802.22 Functional Requirement Document (e.g., sensing every 60 sec. versus 2 sec., channel move time is 2 sec., see 15.711c4&5).   The IEEE 802.22 is still to work on the basis of 2 sec. sensing.  802.18 would need to support the 2 sec. rather than 60 sec. in order to protect broadcast operation, consistent with our previous statements.   In summary, IEEE 802 propose to stick to 2 sec. sensing repetition time at the proposed sensing level of –107 dBm.

H- Synchronized quiet periods are necessary for incumbent sensing

IEEE 802 would like to bring to the attention of the Commission that, in order for RF sensing to be successful in TV white space where different TVBDs will operate, these devices will have to observe synchronous quiet periods to allow sensing of Part 74 devices that otherwise would be masked by other nearby TVBDs.  Such masking would occur due to co-channel and adjacent channel transmissions by TVBDs or out-of-band emissions from closer-by TVBS, hence TVBDs operating on any TV channels should use the same synchronized quiet period pattern.Because of the low sensing threshold specified in the FCC R&O, there is no known sensing technology that can operate without synchronized quiet periods to protect the incumbents

Since such devices may not all comply to IEEE 802 standards, the Commission should consider specifying a common timing for such quiet periods for RF sensing purposes, which could be based on the rising edge of the GPS 1-pulse-per-second, since that is the most practical means of both providing geolocation and timing information and is ubiquitously available.

CONCLUSION

We recognize that permitting operation of unlicensed devices in the TV band is a complex issue requiring carefully thought out rules to avoid interference with licensed systems. Our intention is to provide the Commission with our best engineering and technical assessment of the required changes in the current rules to allow for the most efficient and economical operation of fixed, specifically point-to-multipoint, systems supporting unlicensed WRAN operations in the TV band, while protecting incumbents in the band.etc……

Respectfully submitted,

Michael Lynch

/s/

Michael Lynch

Chair, IEEE 802.18 Radio Regulatory TAG

2221 Lakeside Blvd.

Richardson, TX 75082

(972) 684-7518

freqmgr@ieee.org

Annex A

- Propagation model applied to the calculation of separation distances to TV protected contour

We also note in para. 181 that the FCC curves from Section 73.699 of the Rules are used for a transmit antenna height of 30 m and the Okumura propagation model is applied for lower antenna heights.  It was found in our discussions that the Okumura propagation model is not well suited for non-urban areas and for antenna heights below 30 m.

The ITU-R P.1546 propagation model which contains the original FCC propagation curves (Section 73.699 of the Rules) and which has been updated successively to, among other things, extend the model for lower antenna heights has been used to obtain the new separation distances shown in the new proposed Table below.

- Calculation of separation distances to the protected TV contours

Analysis of the separation distances shown in Table 3 allowed the IEEE 802 to conclude that the predictions were made based on the following assumptions:

- Interference probability: F(50,10)

- TVBD EIRP: 4 Watts

- DTV field strength to be protected: 41 dBμV/m at mid-band UHF (Ch. 38, 617 MHz)

- Co-channel D/U: 15 dB , adjacent channel D/U: -33 dB

- DTV receiving antenna discrimination: 0 dB

It is recognized that the co-channel interference from fixed TVBDs will specifically affect DTV reception at the edge of the coverage areas and that the two following aspects need to be considered in the calculation of the required separation distances to the TV protected contours:

- OET Bulletin 69 recommends an increase of the co-channel D/U for low DTV SNR conditions at edge of coverage from 15 dB to 23 dB.  This results in a reduction of the DTV receiver desensitization from 3 dB to 0.7 dB.

- It is understood that the DTV receiving antenna will be pointing inward towards the DTV transmit station while the interfering TVBD will be, by definition, outside the protected contour in the opposite direction, thus allowing the consideration of a DTV receiving antenna discrimination corresponding to the front-to-back ratio indicated in the OET Bulletin 69, that is 14 dB.

This would result in a tightening of the interference requirement of 8 dB from the co-channel D/U at the same time as a relaxation of 14 dB from the DTV receiving antenna, with a net 6 dB relaxation in the separation distance calculations.

It is proposed to replace the existing Table 3 in para. 181 and in section 15.712a2 with the new Table 3 below based on the following assumptions:

- Interference probability: F(50,10)

- TVBD EIRP: 4 Watts

- DTV field strength to be protected: 41 dBμV/m and mid-band UHF (Ch. 38, 617 MHz)

- Co-channel D/U = 23 dB, adjacent channel D/U= -33 dB

- DTV receiving antenna discrimination toward TVBD’s= 14 dB

- Propagation model: ITU-R P.1546-2 extrapolated up to free-space for distances less than about 1 km to be representative of rural environment.

| Antenna height of Unlicensed Device (AGL for personal/portable and fixed CPE, HAAT for fixed base station) |  | Required Separation (km) From Digital or Analog TV (Full Service or Low Power) Protected Contour |  |  |
| --- | --- | --- | --- | --- |
|  |  | Co-channel |  | Adjacent Channel |
| Less than 3 meters | 6 km |  | 0.1 km |  |
| 3 – Less than 10 meters | 6.9 km |  | 0.256 km |  |
| 10 – Less than 30 meters | 10.8 km |  | 0.285 km |  |
| 30 – Less than 50 meters | 13.6 km |  | 0.309 km |  |
| 50 – Less than 75 meters | 16.1 km |  | 0.330 km |  |
| 75 – Less than 150 meters | 22.6 km |  | 0.372 km |  |
| 150 – Less than 300 meters | 32 km |  | 0.405 km |  |
| 300 – Less than 600 meters | 45.7 km |  | 0.419 km |  |
| 600 – Less than 1200 meters | 68 km |  | 0.426 km |  |

The IEEE Local and Metropolitan Area Networks Standards Committee (“IEEE 802” or the “LMSC”)

2 This document represents the views of IEEE 802.18.  It does not necessarily represent the views of the IEEE as a whole or the IEEE Standards Association as a whole.

(reference the IEEE 802.18 January 2007 Comments)

The 10m antenna height was driven by TV sensing and is not required for wireless microphone sensing since in the case of wireless microphones the RF path is reciprocal.

An explanation of the assumptions, propagation models, and calculations used to derive the table is found in Annex A

Note that for Part 74 device protection, higher antenna heights for fixed base station TVBDs will improve both coverage and sensing in a similar way since the RF parts are reciprocal.