IEEE P802.22

Wireless RANs

| CBP Design Recommendations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-02-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131 | 1-408-467-8410 | wendong.hu@st.com |
| Dave Calvalcanti | Philips | 345 Scarborough Rd. Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |
| Cheng Shan | Samsung Electronics | Korea | +82 31 279 7557 | cheng.shan@samsung.com |

Abstract

This document describes some recommendations on the design of the CBP MAC PDU within the SCW. Design of CBP has been streamlined to compensate for the elimination of Interference-Free Scheduling and Resourcing Renting coexistence mechanism. References to CBP Protection have been removed from this document and will be treated in a separate document once the security text (08/174 or latest revision), has been adopted into the draft.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 1. Introduction

The new CBP design is described by discussing the contents of the self-coexistence window (SCW), namely the CBP Preamble and CBP MAC PDU. The CBP MAC PDU is made up of a Header and a Payload, that spans the 2nd and 3rd symbols of the SCW. What follows is a harmonized solution to comment 147 in the current comment database (doc # 08/146r26).

1. CBP Preamble:

This CBP preamble is mandatory for all CBP transmissions and is transmitted in the 1st symbol of the SCW. CBP Preamble is defined in Section 8.4.1.4.

2. CBP MAC PDU –Header:

[-------------------------------------------------Start of Text Proposal---------------------------------------------]

[replace Table 5 in Section 6.7.1.2 with the following table and modify the caption accordingly]

Table 5 – CBP MAC PDU Header

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CBP_MAC_PDU_Header{ |  |  |
| SCH Data | 184 | Data from SCH of current BS sourcing the CBP, minus the HCS from that data. This data includes the BS ID of the BS this SCH data is identified with |
| Station ID | 48 | Station ID, e.g. 48 bit MAC Address, of station emitting CBP |
| Coexistence Capability Indicator | 4 | - 0000, no coexistence capability supported - 0001, only Spectrum Etiquette - 0010, Spectrum Etiquette and Channel Contention - 0011-1111, Reserved |
| Frame # | 4 | Frame # in which CBP is transmitted |
| Transmission Offset | 4 | Indicates the offset (in symbols) relative end of the frame. The time instants indicated by the Transmission Offset values are the transmission times of the first symbol of the beacon (CBP Preamble, Section 8.4.1). |
| Length | 8 | Length of CBP MAC PDU (header + payload) in bytes |
| HCS (Optional) | 8 | Header Check Sequence. If CBP Protection method is enabled, HCS will not be transmitted in CBP MAC PDU Header. |
| Reserved | 4 | 0x0F |
| } |  |  |

[-------------------------------------------------End of Text Proposal---------------------------------------------]

3. CBP MAC PDU –Payload:

[-------------------------------------------------Start of Text Proposal---------------------------------------------]

[replace Table 6, of section 6.7.1.2.1 with the following]

Table 6 – CBP IEs

| Element ID | Name |
| --- | --- |
| 0x00 | Backup Channel IE |
| 0x01 | CC_REQ IE |
| 0x02 | CC_RSP IE |
| 0x03 | CC_ACK IE |
| 0x04 | CBP Location IE |

[replace title and text for section 6.7.1.2.1.1 with the following]

6.7.1.2.1.1 Backup Channel IE

The Backup Channel IE carries a list of the current backup channels defined by the BS. The Backup Channel IE shall be included in the CBP MAC PDU to allow for network discovery and implementation of the spectrum etiquette coexistence mechanism.

[replace Table 7 in section 6.7.1.2.1 with the following]

Table 7  Backup Channel IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Backup_Channel_IE() { |  |  |
| Element ID | 8 bits | 0x00 |
| Number of Backup TV channels | 4 bits |  |
| for (i=0; i < Number of Backup TV channels; i++) { |  | List of backup channels in order of priority to be used by CPEs in case of loss of communication with the BS due to incumbents |
| TV channel Number[i] | 8 bits |  |
| } |  |  |
| Reserved | 4 | 0x0F |
| } |  |  |

[remove section 6.7.1.2.1.2 DS/US Boundary IE]

[remove section 6.7.1.2.1.3 BS IP Address IE]

[remove section 6.7.1.2.1.4 Inter-BS Capabilities IE]

[rename section 6.7.1.2.1.5 Channel Contention Request IE to 6.7.1.2.1.2 Channel Contention Request IE]

[renumber Table 11 caption  to Table 8 and replace the table with the following]

Table 8 – CC_REQ IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CC_REQ_IE_Format() { |  |  |
| Element ID | 8 | 0x01 |
| Destination BS Id | 48 | The MAC address of the contention destination |
| Sequence number | 16 | Incremented by 1 by the source whenever any of the following three fields change. The contention destinations shall discard the repeated CC_REQ IEs. |
| TV Channel contention number (CCN) | 16 | A random number to show the priority to contend for the TV channel. CCN is dedicated to the contention resolution in the intra system operator situation. |
| Start time | 16 | Starting from the next frame, the number of frames after which the contention source requires to start operation on the requested TV channel. |
| } |  |  |

[rename section 6.7.1.2.1.6 Channel Contention Response IE to 6.7.1.2.1.3 Channel Contention Response IE]

[renumber Table 12 caption  to Table 9 and replace the table with the following]

Table 9 – CC_RSP IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CC_RSP_IE_Format() { |  |  |
| Element ID | 8 bits | 0x02 |
| Source BS Id | 48 bits | Copy from the CC_REQ IE received |
| Sequence number | 16 bits | Copy from the CC_REQ. |
| TV Channel number | 8 bits | The TV channel requested by the source BS |
| Result indication | 2 bits | 00 - SUCCESS (accept the request) 01 -REJECT (reject the request) 10 and 11 – reserved |
| Reason of Rejection | 6 bits | The reason to reject the CC_REQ IE. This field can only be used when result = 01 000000 - the current working period of the contention destination is too short (applicable in the intra operator and inter operators situation). 000001 - the contention destination has smaller channel contention number (CCN) than the contention source (applicable in the intra operator situation). 000010 - the contention source proposes a smaller CCNCT than the contention destination (applicable in the inter operators situation). 000011 - The remaining time to the quiet period is too short (applicable in the intra operator and inter operators situation). 000100—111111 – reserved |
| TV Channel Release Time | 16 bits | Starting from the next frame, the number of frames after which the TV channel can be released. |
| } |  |  |

[rename section 6.7.1.2.1.7 Channel Contention Acknowledgement IE to 6.7.1.2.1.4 Channel Contention Acknowledgement IE]

[renumber Table 13 caption  to Table 10 and replace the table with the following]

Table 10 – CC_ACK IE

| Syntax | Size | Notes |
| --- | --- | --- |
| CC_ACK_IE_Format() { |  |  |
| Element ID | 8 bits | 0x03 |
| Destination Id | 48bits | The MAC address of the contention destination |
| Sequence number | 16 bits | Same as the corresponding CC_REQ IE. The contention destinations shall discard the repeated CC_ACK IE being received |
| TV Channel number | 8 bits | The TV channel being requested by the contention source |
| Start time | 16 bits | Starting from the next frame, the number frames after which the contention source will start operation on the requested TV channel. |
| Occupation code | 2 bits | 00 - Occupy the TV channel 01- Give up the request 10 and 11 – Reserved |
| Reserved | 6 bits | set to 000000 |
| } |  |  |

[remove section 6.7.1.2.1.8 Rent Request IE]

[remove section 6.7.1.2.1.9 Resource Allocation Response IE]

[remove section 6.7.1.2.1.10 Resource Allocation Acknowledge IE]

[remove section 6.7.1.2.1.11 Resource Collection Request IE]

[remove section 6.7.1.2.1.12 Resource Collection Response IE]

[remove section 6.7.1.2.1.13 Resource Collection Response Acknowledge IE]

[remove section 6.7.1.2.1.14 Returning Request IE]

[remove section 6.7.1.2.1.15 Returning Response IE]

[remove section 6.7.1.2.1.16 Returning Response Acknowledge IE]

[remove section 6.7.1.2.1.17 RS-SEM IE]

[remove section 6.7.1.2.1.18 RS-ADV IE]

[remove section 6.7.1.2.1.19 BS Channel Parameter IE]

[renumber 6.7.1.2.1.20 to 6.7.1.2.1.5 CBP Location IE]

6.7.1.2.1.5 CBP Location IE

The coordinates represented in this IE are based on the WGS 84 Datum.

[renumber Table 26 to Table 11 – CBP Location IE]

Table 11 – CBP Location IE

| Syntax | Size | Description |
| --- | --- | --- |
| CBP_Location_IE { |  |  |
| Element ID | 8 bits | 0x04 |
| Latitude | 29 bits | - Bit# 16-0: 20 bits of decimal degree fraction represents degrees out to six decimal places (.999999). - Bit# 23-17: 8 bits of degree, sufficient to cover 0° < latitude < 90° - Bit# 24: 1 bit indicator of hemisphere, N = 0, S = 1 |
| Longitude | 29 bits | - Bit# 16-0: 20 bits of decimal degree fraction represents degrees out to six decimal places (.999999). - Bit# 24-17: 8 bits of degree, sufficient to cover 0° < longitude < 180° - Bit# 25: 1 bit indicator of hemisphere, E = 0, W = 1 |
| Altitude | 14 bits | Represents altitude of station above sea level in 5m increments |
| } |  |  |

[-------------------------------------------------End of Text Proposal---------------------------------------------]

4. Analysis of CBP IE combinations for CBP Payload

In this section of the document, we try to go through the various combination of the 7 remaining CBP IEs are possible. The CBP MAC PDU carried in the 2nd and 3rd (if need be) symbols of the self coexistence window. The data capacity per symbol is 418 bits, giving a maximum total capacity of 836 bits. The CBP MAC PDU is comprised of the CBP Header and CBP Payload. The CBP Header is 264 bits. This leaves 572 bits for the CBP Payload. The CBP Payload is made up of various IEs.

The following lettering is used to encode the various IE configurations used in this analysis and how large (in bits) they are:

- A = CBP Header w/ CBP Protection disabled = 264

- B = Backup Channel IE (4 backup channels) = 48

- C = Backup Channel IE (8 backup channels) = 80

- D = Backup Channel IE (16 backup channels) = 144

- E = CC-REQ IE = 104

- F = CC-RSP IE = 104

- G = CC-ACK IE = 104

- H = CBP Location IE = 80

The following is a listing of possible combinations (≤ 836 bits) of Header + Payload IEs for CBP MAC PDU (values are in bits):

- A+B = 312

- A+B+E = 416

- A+B+2*E = 520

- A+B+3*E = 624

- A+B+4*E = 728

- A+B+5*E = 832

- A+B+F = 416

- A+B+2*F = 520

- A+B+3*F = 624

- A+B+4*F = 728

- A+B+5*F = 832

- A+B+G = 416

- A+B+2*G = 520

- A+B+3*G = 624

- A+B+4*G = 728

- A+B+5*G = 832

- A+B+E+F = 520

- A+B+E+G = 520

- A+B+F+G = 520

- A+B+E+2*F = 624

- A+B+E+2*G = 624

- A+B+2*E+F = 624

- A+B+2*E+G = 624

- A+B+F+2*G = 624

- A+B+2*F+G = 624

- A+B+E+2*F+G = 728

- A+B+E+F+2*G = 728

- A+B+2*E+F+G = 728

- A+B+E+3*F = 728

- A+B+E+3*G = 728

- A+B+F+3*G = 728

- A+B+3*F+G = 728

- A+B+2*E+2*F = 728

- A+B+2*E+2*G = 728

- A+B+2*F+2*G = 728

- A+B+3*E+F = 728

- A+B+3*E+G = 728

- A+B+E+3*F+G = 832

- A+B+E+F+3*G = 832

- A+B+3*E+F+G = 832

- A+B+E+4*F = 832

- A+B+E+4*G = 832

- A+B+F+4*G = 832

- A+B+4*F+G = 832

- A+B+2*E+3*F = 832

- A+B+2*E+3*G = 832

- A+B+2*F+3*G = 832

- A+B+3*F+2*G = 832

- A+B+4*E+F = 832

- A+B+4*E+G = 832

- A+C = 344

- A+C+E = 448

- A+C+2*E = 552

- A+C+3*E = 656

- A+C+4*E = 760

- A+C+F = 448

- A+C+2*F = 552

- A+C+3*F = 656

- A+C+4*F = 760

- A+C+G = 448

- A+C+2*G = 552

- A+C+3*G = 656

- A+C+4*G = 760

- A+C+E+F = 552

- A+C+E+G = 552

- A+C+F+G = 552

- A+C+E+2*F = 656

- A+C+E+2*G = 656

- A+C+2*E+F = 656

- A+C+2*E+G = 656

- A+C+F+2*G = 656

- A+C+2*F+G = 656

- A+C+E+2*F+G = 760

- A+C+E+F+2*G = 760

- A+C+2*E+F+G = 760

- A+C+E+3*F = 760

- A+C+E+3*G = 760

- A+C+F+3*G = 760

- A+C+3*F+G = 760

- A+C+2*E+2*F = 760

- A+C+2*E+2*G = 760

- A+C+2*F+2*G = 760

- A+C+3*E+F = 760

- A+C+3*E+G = 760

- A+D = 408

- A+D+E = 512

- A+D+2*E = 616

- A+D+3*E = 720

- A+D+4*E = 824

- A+D+F = 512

- A+D+2*F = 616

- A+D+3*F = 720

- A+D+4*F = 824

- A+D+G = 512

- A+D+2*G = 616

- A+D+3*G = 720

- A+D+4*G = 824

- A+D+E+F = 616

- A+D+E+G = 616

- A+D+F+G = 616

- A+D+E+2*F = 720

- A+D+E+2*G = 720

- A+D+2*E+F = 720

- A+D+2*E+G = 720

- A+D+F+2*G = 720

- A+D+2*F+G = 720

- A+D+E+2*F+G = 824

- A+D+E+F+2*G = 824

- A+D+2*E+F+G = 824

- A+D+E+3*F = 824

- A+D+E+3*G = 824

- A+D+F+3*G = 824

- A+D+3*F+G = 824

- A+D+2*E+2*F = 824

- A+D+2*E+2*G = 824

- A+D+2*F+2*G = 824

- A+D+3*E+F = 824

- A+D+3*E+G = 824

- A+H+B = 392

- A+H+B+E = 496

- A+H+B+2*E = 600

- A+H+B+3*E = 704

- A+H+B+4*E = 808

- A+H+B+F = 496

- A+H+B+2*F = 600

- A+H+B+3*F = 704

- A+H+B+4*F = 808

- A+H+B+G = 496

- A+H+B+2*G = 600

- A+H+B+3*G = 704

- A+H+B+4*G = 808

- A+H+B+E+F = 600

- A+H+B+E+G = 600

- A+H+B+F+G = 600

- A+H+B+E+2*F = 704

- A+H+B+E+2*G = 704

- A+H+B+2*E+F = 704

- A+H+B+2*E+G = 704

- A+H+B+F+2*G = 704

- A+H+B+2*F+G = 704

- A+H+B+E+2*F+G = 808

- A+H+B+E+F+2*G = 808

- A+H+B+2*E+F+G = 808

- A+H+B+E+3*F = 808

- A+H+B+E+3*G = 808

- A+H+B+F+3*G = 808

- A+H+B+3*F+G = 808

- A+H+B+2*E+2*F = 808

- A+H+B+2*E+2*G = 808

- A+H+B+2*F+2*G = 808

- A+H+B+3*E+F = 808

- A+H+B+3*E+G = 808

- A+H+C = 424

- A+H+C+E = 528

- A+H+C+2*E = 632

- A+H+C+3*E = 736

- A+H+C+F = 528

- A+H+C+2*F = 632

- A+H+C+3*F = 736

- A+H+C+5*F = 944

- A+H+C+G = 528

- A+H+C+2*G = 632

- A+H+C+3*G = 736

- A+H+C+E+F = 632

- A+H+C+E+G = 632

- A+H+C+F+G = 632

- A+H+C+E+2*F = 736

- A+H+C+E+2*G = 736

- A+H+C+2*E+F = 736

- A+H+C+2*E+G = 736

- A+H+C+F+2*G = 736

- A+H+C+2*F+G = 736

- A+H+D = 488

- A+H+D+E = 592

- A+H+D+2*E = 696

- A+H+D+3*E = 800

- A+H+D+F = 592

- A+H+D+2*F = 696

- A+H+D+3*F = 800

- A+H+D+G = 592

- A+H+D+2*G = 696

- A+H+D+3*G = 800

- A+H+D+E+F = 696

- A+H+D+E+G = 696

- A+H+D+F+G = 696

- A+H+D+E+2*F = 800

- A+H+D+E+2*G = 800

- A+H+D+2*E+F = 800

- A+H+D+2*E+G = 800

- A+H+D+F+2*G = 800

- A+H+D+2*F+G = 800

The list of combinations presented does need verification, as it represents numerically possible combinations, not all of which are applicable (using the single channel contention paradigm). This list  is provided here for informative reasons only.

There are still some issues to be noted, there was some discussion on these issues during the MAC teleconference call on 2009-02-17:

- For example, will it be possible that a BS acts a contention source on one channel, but is a contention destination on another? Would this be possible if not all of a particular BS's CPEs are on the same channel, or BSs are using a more active CBP detection mechanism (i.e. fighting over channels designated by some as preferred for CBP transmission and by others for their own CPE traffic)?

  - There was no definitive answer to this question.

- Is it expected that a BS can start a contention process with more than one BS simultaneously?

  - Answer, no. If there is more than one potential contention destination, the contention source will randomly pick one to initiate contention with. Upon resolution of the contention, the CC-ACK message would be sent to both contention destinations. The other source, that contention wasn’t initiated with, has to vacate the channel upon receiving the ACK.

- From the current analysis, only a few combinations exist that will fit into one symbol of the SCW. This means that the use of the 3rd Symbol of SCW for transmitting CBP MAC PDU will almost be necessary.

  - This holds true, even under the assumption of the single channel contention paradigm. Recommend, that we consider removing text regarding optional use of 2nd data symbol of CBP MAC PDU (3rd symbol of SCW)