IEEE P802.22
Wireless RANs

| Proposed Resolution MAC comment 387 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Dave Cavalcanti | Philips | 345 Scarborough Rd, Briarcliff Manor, NY | 914-945-6083 | Dave.Cavalcanti@philips.com |
| Jianfeng Wang | Philips | 345 Scarborough Rd, Briarcliff Manor, NY |  | Jianfeng.Wang@philips.com |

Abstract

This document provides the proposed resolutions for comment 387

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 387

Comment: Page 225, line 26: It's not clear how the BS acknowledge the measurement reports sent by a CPE?

Proposed remedy: Need further specifications if the mechanism is missing.

Response:

The existing text in Page 224, line 5 to 11 describes the process of acknowledging the measurement reports sent by the CPE.

“Once the measurement report message is successfully received at the BS, the BS shall respond back to the CPE with a BLM-ACK message to acknowledge its reception. In case the CPE does not hear the BLM-ACK message from the BS after some pre-specified timeout T28 (see Error: Reference source not found), it shall assume that its BLM-REP message was lost and shall initiate retransmission of the BLM-REP message. The CPE shall attempt retransmission or measurement report messages up to BLM-REP retries (see Error: Reference source not found). Once the BLM-ACK message is successfully received at the CPE, it shall then clear its local statistics to prepare for future measurements. Error: Reference source not found illustrates the measurement message flow between the BS and a CPE.

The nature of the reports received by the BS can be essentially of two types: regular or urgent. Regular reports refer to the cases where the BS has explicitly requested CPEs to report back to it with a certain periodicity (and so the BS can allocate sufficient upstream resources beforehand), and also when CPEs are allowed to report autonomously, for example, whenever enough data has been collected (in this case, CPEs may have to request for upstream resource allocation). Urgent reports are those that take place whenever an incumbent is detected in a channel affected by the 802.22 transmission in a cell, and need to be reported back to the BS immediately (see Error: Reference source not found). In this case, the BS shall provide periodic upstream UCS contention periods where CPEs can notify the BS about potential interference and any critical measurement results, or else the CPE can use any of its own reservations for this purpose.”

For further clarification, add the following text in Page 224, line 22:

In case the CPE uses UCS notification, the BS do not sends an acknowledgment right away. Instead, the BS can allocate bandwidth for the CPE to transmit the complete measurement report frame (BLM-REP), and the corresponding acknowledgment (BLM-ACK) is sent on the next downstream opportunity following the reception of the measurement report.

References: