IEEE P802.22
Wireless RANs

| Frame-based On-demand Spectrum Contention Protocol – the Specifications |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-12-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Wendong Hu | STMicroelectronics | 1060 East Brokaw Road, San Jose, CA 95131, USA | 1-408-467-8410 | wendong.hu@st.com |

Abstract

The document provides the specifications for the Frame-based On-demand Spectrum Contention Protocol that enables dynamic spectrum sharing for coexistence operations among IEEE 802.22 WRAN cells. The document is to resolve the comment with the following comment IDs: 164, 168, 405, 524, and 525.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Control Message Formats of Frame-based Spectrum Contention Protocol

Instruction to the Editor: Replace sub-clause 6.7.1.2.1.5 Channel Contention Request IE as the following:

6.7.1.2.1.5 Spectrum Contention Request (SC_REQ) IE

The Spectrum Contention Request (SC_REQ) IE as defined in Table 11 is used by the Frame-based Spectrum Contention Protocol. The SC_REQ IE is transmitted in the payload of a CBP packet by a WRAN cell – the spectrum contention source, which intends to acquire spectrum resources (data frame transmission opportunities) of a TV channel currently occupied by another neighbouring WRAN – the contention destination, in order to satisfy the QoS requirements of the spectrum contention source’s data transmission.

- 1 — SC_REQ IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_REQ_IE_Format() { |  |  |
| Element ID | 8 bits | Indication of the Message Type |
| Length | 8 bits |  |
| BS ID of Contention Destination | 48bits | The MAC address of the spectrum contention destination’s base station. |
| Spectrum Contention Number (SCN) | 16 bits | A random number to show the priority to contend for spectrum resource of the target TV channel. |
| Contention Request Frame Index Vector | 16bits | A bit vector indicating the index of each data frame ofa superframe that the Contention Source WRAN requests to acquire (through the contention) for its data services starting from TBD superframes after the current superframe. For each of the 16 bits as shown below, the corresponding frame is requested for the contention when a bit’s value is set to 1. Otherwise, the bit value of the corresponding frame is set to 0. Bit 0: Frame 0; Bit 1: Frame 1; Bit 2: Frame 2; Bit 3: Frame 3; Bit 4: Frame 4; Bit 5: Frame 5; Bit 6: Frame 6; Bit 7: Frame 7; Bit 8: Frame 8; Bit 9: Frame 9; Bit 10: Frame 10; Bit 11: Frame 11; Bit 12: Frame 12; Bit 13: Frame 13; Bit 14: Frame 14; Bit 15: Frame 15. |
| } |  |  |

Instruction to the Editor: Replace sub-clause 6.7.1.2.1.6 Channel Contention Response IE as the following:

6.7.1.2.1.6 Spectrum Contention Response (SC_RSP) IE

The Spectrum Contention Response (SC_RSP) IE defined in  is used by the Frame-based Spectrum Contention Protocol. The SC_RSP IE is sent in a CBP packet payload by the contention destination WRAN cell in order to inform the contention source WRAN cell with regarding to the contention results. This IE is transmitted by the contention destination WRAN cell after it has received a SC_REQ IE from the contention source WRAN cell and run the contention resolution algorithm.

- Table 12 — SC_RSP IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_RSP_IE_Format() { |  |  |
| Element ID | 8 bits | Indication of the Message Type. |
| Length | 8 bits |  |
| BS ID of the Spectrum Contention Destination | 48 bits | MAC address of the Spectrum Contention Destination BS. |
| Contention Response Frame Index Vector | 16 bits | A bit vector indicating the contention results determined by the channel contention algorithm for the data frames within a super-frame that the contention source WRAN requests to acquire. These contention results will be effective starting from the next super-frame. For each of the 16 bits as shown below, the corresponding frame is granted to the contention source when a bit’s value is set to 1. Otherwise, the frame is not granted. For a data frame that is not requested by any contention source, the corresponding bit is set to 0. Bit 0: Frame 0; Bit 1: Frame 1; Bit 2: Frame 2; Bit 3: Frame 3; Bit 4: Frame 4; Bit 5: Frame 5; Bit 6: Frame 6; Bit 7: Frame 7; Bit 8: Frame 8; Bit 9: Frame 9; Bit 10: Frame 10; Bit 11: Frame 11; Bit 12: Frame 12; Bit 13: Frame 13; Bit 14: Frame 14; Bit 15: Frame 15. |
| } |  |  |

Instruction to the Editor: Replace sub-clause 6.7.1.2.1.7 Channel Contention Acknowledgment IE as the following:

6.7.1.2.1.7 Spectrum Contention Acknowledgment (SC_ACK) IE

The Spectrum Contention Acknowledgment (SC_ACK) IE as defined in  is used in Frame-based Spectrum Contention Protocol. SC_ACK is a broadcast acknowledgement message transmitted by the winner SC-SRC indicating the confirmation of spectrum acquisitions.

- Table 13 — SC_ACK IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_ACK_IE_Format() { |  |  |
| Element ID | 8 bits | Indication of the Message Type. |
| Length | 8 bits |  |
| Spectrum Contention Number (SCN) | 16 bits | The winning SCN used in SC_REQ message, showing the priority to contend for spectrum resource of the target TV channel. |
| BS ID of the granting SC-DST | 48 bits | The ID of the SC-DST WRAN cell granting the access to the data frame that are being acquired by the winning SC-SRC (this is used to enable “clear to send”). |
| Contention Acknowledgement Frame Index Vector | 16 bits | A bit vector indicating the contention results determined by the channel contention algorithm for the data frames within a super-frame that the contention source WRAN will acquire starting from the next super-frame. For each of the 16 bits as shown below, the corresponding frame will be occupied by the contention source when a bit’s value is set to 1. Otherwise, the frame will not be occupied. Bit 0: Frame 0; Bit 1: Frame 1; Bit 2: Frame 2; Bit 3: Frame 3; Bit 4: Frame 4; Bit 5: Frame 5; Bit 6: Frame 6; Bit 7: Frame 7; Bit 8: Frame 8; Bit 9: Frame 9; Bit 10: Frame 10; Bit 11 Frame 11; Bit 12: Frame 12; Bit 13: Frame 13; Bit 14: Frame 14; Bit 15: Frame 15. |
| } |  |  |

Instruction to the Editor: Insert sub-clause 6.7.1.2.1.8 Spectrum Contention Release (SC_REL) IE as the following:

6.7.1.2.1.8 Spectrum Contention Release (SC_REL) IE

Spectrum Contention Release (SC_REL) is a broadcast message IE transmitted by the granting spectrum contention destination (SC-DST) indicating the announcement of the spectrum releases.

- Table 13 — SC_REL IE format

| Syntax | Size | Notes |
| --- | --- | --- |
| SC_REL_IE_Format() { |  |  |
| Element ID | 8 bits | Indication of the Message Type. |
| Length | 8 bits |  |
| Spectrum Contention Number (SCN) | 16 bits | The winning SCN used in SC_REQ message, showing the priority to contend for spectrum resource of the target TV channel. |
| BS ID of the winning SC-SRC | 48 bits | The ID of the the SC-SRC WRAN cell granted the access to the data frame that are being released by the granting SC-DST (this is used to enable efficient spectrum reuse). |
| Contention Release Frame Index Vector | 16 bits | A bit vector indicating the contention results determined by the channel contention algorithm for the data frames within a superframe that the contention source WRAN will acquire starting from the next superframe. For each of the 16 bits as shown below, the corresponding frame will be occupied by the contention source when a bit’s value is set to 1. Otherwise, the frame will not be occupied. Bit 0: Frame 0; Bit 1: Frame 1; Bit 2: Frame 2; Bit 3: Frame 3; Bit 4: Frame 4; Bit 5: Frame 5; Bit 6: Frame 6; Bit 7: Frame 7; Bit 8: Frame 8; Bit 9: Frame 9; Bit 10: Frame 10; Bit 11 Frame 11; Bit 12: Frame 12; Bit 13: Frame 13; Bit 14: Frame 14; Bit 15: Frame 15. |
| } |  |  |

2. Top-level Procedure of Inter-WRAN Coexistence

Instruction to the Editor: Replace sub-clause 6.21.2.3 Mechanism for Inter-BS Coexistence as the following:

6.21.2.3 Inter-WRAN Self Coexistence

The self coexistence operations among 802.22 WRAN cells shall follow the top-level procedure illustrated in Figure 1 and described as below:

- The BS of an 802.22 WRAN cell is powered on.

- The BS performs network discovery, which includes discovering

  - TV channel occupancies of the neighbouring WRAN cells;

  - Self-coexistence Window (SCW) reservations of the neighbouring WRAN cells;

  - Frame reservation patterns of the neighbouring WRAN cells on specific TV channels (this information can be obtained from the received CBP packets);

- The BS performs channel acquisitions based on Spectrum Etiquette algorithms (as described in sub-clause 6.21.2.3.1).

- If the BS successfully acquires a TV channel, it goes to normal service operations on the acquired TV channel (Step 5 below).

Else if the BS fails to acquire any TV channel, it selects a TV channel occupied by other WRAN cells and performs the Inter-WRAN Frame-based Spectrum Contention operations on the selected TV channel (Step 6 below).

- The BS performs normal service operations (as described in sub-clause …).

During the normal services operations, the BS may receive internal demands (received from the inside of the BS’s own cell) for additional spectrum resources (data frame transmission opportunities). When this occurs, the BS re-initiates the spectrum acquisition process starting from Step 3 (etiquette-based channel acquisition).

During the normal services operations, the BS may also receive external demands (received from other WRAN cells) for sharing its occupied spectrum resources (data frame transmission opportunities) on the operating channel. When this occurs, the BS performs the Inter-WRAN Frame-based Spectrum Contention operations on its operating TV channel (Step 6).

- The BS performs the Inter-WRAN Frame-based Spectrum Contention operations on the selected TV channel (as described in sub-clause ..3), and then goes to the normal services operations (Step 5).

Figure 1 – Top-level Inter-WRAN Self Coexistence Procedure

3. Inter-WRAN Frame-based Spectrum Contention

Instruction to the Editor: Remove sub-clause 6.21.2.3.2 Interference-free Scheduling, and sub-clause 6.21.2.3.3 CBP-based Dynamic Resource Renting and Offering.

Instruction to the Editor: Replace sub-clause 6.21.2.3.4 CBP-based Adaptive on Demand Channel Contention (Spectrum Contention) and re-number the figures accordingly as the following:

6.21.2.3.2 Inter-WRAN Frame-based Spectrum Contention

6.21.2.3.2.1 Terminologies (Should be in Abbreviations)

Spectrum Contention Source (SC-SRC) – a WRAN cell that is demanding additional spectrum resources (i.e. data frames transmission opportunities on a TV channel) and is initiating an interactive spectrum contention process with the target Spectrum Contention Destination (SC-DST).

Spectrum Contention Destination (SC-DST) – a WRAN cell that is the target of the spectrum contention request initiated by Spectrum Contention Source (SC-SRC), and is the occupier of the spectrum resources being requested to be shared with the Spectrum Contention Source (SC-SRC).

Spectrum Contention Number (SCN) – the contention number randomly generated by SC-SRCs and SC-DSTs for determining the priority of spectrum access.

6.21.2.3.2.2 Message Flow of the Spectrum Contention Protocol

6.21.2.3.2.2.1 Control Messages

Each of the following control messages of the spectrum contention protocol is encapsulated by one CBP information element (IE).

Spectrum Contention Request (SC_REQ) – This is a unicast request message transmitted by the SC-SRC for initiating the spectrum contention process. It contains the following information:

- The SC-DST’s ID as the destination;

- The selected TV channel number;

- The fame index-vector of the selected data frames within a super-frame.

- The spectrum contention number (one SCN is used for the contention of all data frames as indicated in the frame index vector).

Spectrum Contention Response (SC_RSP) – This is a unicast response message transmitted by the SC-DST responding to the requesting SC-SRC with regard to the contention results. It contains the following information:

- The SC-SRC’s ID as the destination;

- The selected TV channel number;

- The fame index containing the contention results for each of the selected data frame within the super-frame;

Spectrum Contention Acknowledgement (SC_ACK) – This is a broadcast acknowledgement message transmitted by the winner SC-SRC indicating the confirmation of spectrum acquisitions. It contains the following information:

- The selected TV channel number;

- The fame index indicating a confirmation of the spectrum acquisition for each of the selected data frame within the super-frame;

- The winning SCN (used to resolving possible collisions of frame acquisition);

- The ID of the granting SC-DST, which is the SC-DST WRAN cell granting the access to the data frame that are being acquired by the winning SC-SRC (this is used to enable “clear to send”).

Spectrum Contention Release (SC_REL) is a broadcast message transmitted by the granting SC-DST indicating the announcement of the spectrum releases. It contains the following information:

- The selected TV channel number;

- The fame index indicating an announcement of the spectrum release for each of the selected data frame within the super-frame;

- The winning SCN of the winning SC-SRC (used to resolving possible collisions of frame acquisition);

- The ID of the winning SC-SRC, which is the SC-SRC WRAN cell granted the access to the data frame that are being released by the granting SC-DST (this is used to enable efficient spectrum reuse).

6.21.2.3.2.2.2 Message Flow

The message flow of the spectrum contention protocol in the time domain is shown in Figure 2.

Figure 2 – Message Flow of the Spectrum Contention Protocol

6.21.2.3.2.3 Procedure of the Spectrum Contention Protocol

6.21.2.3.2.3.1 Spectrum Contention Algorithm

The spectrum contention algorithm is used to resolve contentions of spectrum resource among the neighboring WRAN cells. The algorithm is specified in Figure 3 as the following:

Figure 3 – Spectrum Contention Algorithm

Where in the algorithm,

N: total number of contending WRAN cells;

WRAN: the array of IDs of the contending WRAN cells, WRAN[i] for i←1 to N;

SCN: the array of spectrum contention numbers of the contending WRAN cells, in which SCN[i] is the spectrum contention number of WRAN[i] for i←1 to N;

Frame: the data frame (spectrum resource) being contended for;

SCNwinner(Frame): the winner spectrum contention number for accessing theFrame;

WRANwinner(Frame):the ID of the winner WRAN cell to access the Frame;

6.21.2.3.2.3.2 Spectrum Contention Number Generation

The spectrum contention number,SCNi, for WRAN cell i, WRANi, is generated as the following:

SCNi = RANDOM (0, 216−1).

6.21.2.3.2.3.3 Top-level Procedure of Frame-based Spectrum Contention Protocol

Figure 4 shows the top-level procedure of the Frame-base Spectrum Contention Protocol.

Figure 4 – Top-level Procedure of Frame-based Spectrum Contention Protocol

6.21.2.3.2.3.4 Procedure of Available Frame Acquisitions

Figure 5 – Available Frame Acquisition Procedure

6.21.2.3.2.3.5 Procedure of Frame Acquisition

Figure 6 – Frame Acquisition Procedure

6.21.2.3.2.3.6 Spectrum Contention Procedure at Spectrum Contention Source

Figure 7 – Spectrum Contention Procedure at Contention Source

6.21.2.3.2.3.7 Spectrum Contention Procedure at Spectrum Contention Destination

Figure 8 – Spectrum Contention Procedure at Contention Destination

Figure 9 – Operations of Contention Success of a Data Frame at Contention Destination

Figure 10 – Operations of Contention Failure of a Data Frame at Contention Destination

Figure 11 – Release Pending Operations at Contention Destination

4. Motion

Move to accept the recommended changes to IEEE 802.22 v1.0 noted above and instruct the editor to make these changes, in order to resolve the comment with the following comment IDs: 164, 168, 405, 524, 525.

- Motion by:  Wendong Hu

- Second:

| Approve: 0 | Disapprove: 0 | Abstain: 0 |
| --- | --- | --- |