IEEE P802.22
Wireless RANs

| Text Amendment for Spectrum Etiquette (Resolution for CID 492~494) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Cheng Shan | Samsung Electronics | S. Korea | +82 31 279 7557 | cheng.shan@samsung.com |
| Junhong Hui | Samsung Electronics | China | +86 10 5925 3333 3108 | jh.hui@samsung.com |
| Sangbum Kim | Samsung Electronics | S. Korea | +82 31 279 7092 | sb07.kim@samsung.com |
| Baowei Ji | Samsung Electronics | USA | +1 972 761 7167 | baowei.ji@samsung.com |
| Euntaek Lim | Samsung Electronics | S. Korea | +82 31 279 5917 | et.lim@samsung.com |

Abstract

This document proposes text amendments to clarify the spectrum etiquette section.

This contribution can be regarded to address the following comments: 492, 493 and 494, but not limit to those comments.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

I. Related Comments

This contribution is to (but not limit to) address the following comments: 492, 493 and 494.

II. Proposed Text Changes

        - Spectrum Etiquette

The channel selection at a cell obeys the spectrum etiquette rule so that the chosen channel does not interfere, or interferes with a minimum number of channels to be used by its neighbor cells. Without cooperation, the frequency selection in a cell may lead to one or more BS not having enough channels. For example, the available channel sets at BS1,BS2 and BS3 are {1, 3}, {1, 2, 3} and {1, 2, 3}, respectively. If BS2 decides to use channel {1} and BS3 decides to use channel {3}, BS1 would have no channel to use.  The cooperation also minimize the channel collision probability when multiple cells switch their operational channels by avoiding using the same backup channel set.

The channel-selection decision of each cell follows the flowchart in Figure 13. The terms of central cell and neighbor cells are relative concepts. Beside the definition of channel sets as in section 9.2.2, the following additional channel sets are defined in spectrum etiquette operation:

- Incumbent occupied Channel Set: channels that are identified to be occupied by incumbent devices/services.

- WRAN occupied Channel Set: channels that are operating channels of the discovered neighboring WRAN cells. (Note for editor: Need further alignment with section 9.2 if definition of occupied channel set changes. When incumbent- and WRAN- occupied channel sets are defined in section 9.2, no need to present the definitions here, the above two bullets can be removed then.)

- Neighbor WRAN Backup Channel Set: channels that are within the backup channel sets of the discovered neighboring WRAN cells. This channel set is built by listening to SCH/CBPs of the discovered neighboring cells carrying the backup channel information.

- Local Priority Set 1: = (Backup Channel Set∪Candidate Channel Set) \ (WRAN-Occupied Channel Set∪Neighbor WRAN Backup Channel Set)

- Local Priority Set 2: = (Backup Channel Set∪Candidate Channel Set) \ (WRAN-Occupied Channel Set)

- Local Priority Set 3: = (WRAN-Occupied Channel Set)

Note that:

  - Symbols ∪, ∩, and \ are set operation of union, intersection, and exclude, respectively.

  - Note that Local Priority Sets 1~3 is local information, which is not shared with neighbor cells.

The spectrum etiquette is triggered by the following events:

- Incumbent discovery;

- Neighbor WRAN cells’ discovery/update;

- Operating channel switch demand; (e.g., due to interference);

- Contention request received from neighbor WRAN cells;

- Time-out for a predefined timer;

- Other predefined mechanism in implementation.

The procedure of WRAN spectrum etiquette is as follows.

- The central cell builds/updates channel sets as described in section 6.21.4.1 and section 9.2.2, either by receiving indication from the geo-location database, or spectrum awareness of incumbents and WRANs. The neighboring WRAN occupied channel set and neighboring WRAN backup channel set are built/updated accordingly.

- Update backup channel set by choosing one or several channels from Local Priority Set 1; if Local Priority Set 1 is empty, update backup channel set by choosing one or several channels from Local Priority Set 2 which are backup channels by the least number of neighbor cells; (if several channels are backup channels of the same number of neighbor cells, the present cell shall choose from those channels randomly.) if Local Priority Set 2 is also empty, choose one or several channel from Local Priority Set 3 which are operating channels by the least number of neighbor cells; (if several channels are operating channels of the same number of neighbor cells, the present cell shall choose from those channels randomly.)

- If switching operating channel is necessary or demanded by higher layer indication (e.g., due to incumbent discovery in the operating channel, significant detected interference in the operating channel , or preferable channel quality of a backup channel, etc.), promote the first backup channel to operating channel; go to step 2 to update backup channel set;

- If the new operating channel is also an operating channel of a neighbor cell, initialize the coexistence contention procedure as described in section xxx.yyy.

- Update the neighbor cells with new operating and backup channel sets via SCH/CBP.

- The flow chart of spectrum etiquette process

The spectrum etiquette procedure is further illustrated using the example in Figure 14. The central cell has 6 neighbor cells. The central cell has one operating channel #1, one backup channel #2 (underlined), and one candidate channel #3 (double underlined). The channel sets information for the neighbor cells are accordingly noted as in Figure 110 (a). In Figure 110 (b), two incumbents appears in the central cell on the operating and backup channel, and thus the central cell is forced to change both its operating and backup channel. The spectrum etiquette is triggered. The former candidate channel #7 is promoted to operating channel, and channel #5 is promoted to backup channel, which is only used by one neighbor cell as the operating channel. All the neighbor cells perform spectrum etiquette after receiving the channel set update information from the central cell accordingly, and the resultant channel sets for each cell are illustrated in Figure 110 (b). If another incumbent appears in the central cell on channel #7, the central cell will have to use channel #5 as the operating channel and start contention based coexistence with its neighbor cell.

| Central Cell Status | Before | Upon Incumbent Appearance | After Spectrum Etiquette |
| --- | --- | --- | --- |
| Operating Channel Set | 1 | 1 | 7 |
| Backup Channel Set | 2 | 2 | 5 |
| Candidate Channel Set | 7 | 3,4,5,6,7,8 | φ |
| WRAN-Occupied Channel Set | 3,4,5,6,8 | 3,4,5,6,8 | 3,4,5,6,8 |
| Neighbour Backup Channel Set | 4,5,6,7,8 | 3,4,5,6,7,8 | 4,5,6,7,8 |
| Local Priority Set 1: | 2 | 2 | φ |
| Local Priority Set 2: | 2,7 | 2,7 | φ |
| Local Priority Set 3: | 3,4,5,6,8 | 2,3,4,5,6,7,8 | 3,4,5,6,8 |

- Illustration of the procedure of spectrum etiquette