IEEE P802.22
Wireless RANs

| Meeting Minutes of the Cognitive Radio Capability Ad-Hoc Group in 802.22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |

Abstract

This document provides the minutes of the Cognitive Radio Capabilities Ad-Hoc conference calls which were held on April 3rd, April 9th and April 16th 2009.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. Minutes of the Telecon on Friday, April 3rd, 2009

1. Attendance

Apurva Mody

Gerald Chouinard

Ranga Reddy

Winston Caldwell

Dan Lubar

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy – No one seemed to be unfamiliar with the IEEE Patent Policy

  - http://standards.ieee.org/board/pat/pat-slideset.pdf

- Charter and Agenda of the Cognitive Radio Capability Ad-Hoc Group –  https://mentor.ieee.org/802.22/dcn/09/22-09-0056-02-0000-section-9-cognitive-radio-capability-ad-hoc-group-charter-and-agenda.doc

- Comment Database - https://mentor.ieee.org/802.22/dcn/08/22-08-0146-32-0000-wran-draft-1-0-draft-comments-database.xls

- Cognitive Radio Capabilities – Missing Sections and Details – Comment #735  can be found at - https://mentor.ieee.org/802.22/dcn/09/22-09-0057-05-0000-section-9-cognitive-radio-capabilities-missing-sections-and-details.doc

- Proposed Resoutions for the Comments Related to Section 9 can be found at

https://mentor.ieee.org/802.22/dcn/09/22-09-0066-00-0000-proposed-resolutions-to-comments-section9.doc

Minutes and Discussions

- Apurva Mody briefed the people on the telecom about the progress made in the Cognitive Radio Capabilities Ad-hocs at the March 2009 Plenary session in Vancouver.

- Apurva Mody went over the charter and the agenda of the group

- Document (https://mentor.ieee.org/802.22/dcn/09/22-09-0057-05-0000-section-9-cognitive-radio-capabilities-missing-sections-and-details.doc) was presented once again to the group

- Comments 744 and 747 of the Comment Database (https://mentor.ieee.org/802.22/dcn/08/22-08-0146-32-0000-wran-draft-1-0-draft-comments-database.xls) were addressed. The proposed resolutions can be found in later versions of the Comment Database posted to the mentor.

- Comment 744 was rejected but the section dealing with this comment needs to be enhanced. It was found that the term association was never defined anywhere in the 802.22 draft. (Association = Time/ Freq. Synchronization + Authorization Key Exchange + GPS/ Geolocation + Registration) –

- Comment 735 Missing Sections - Table 280 needs to be amended for a case when CPE in a protected contour tries to associate - Clarify the text. – Possible Options - Do not allow the CPE to register, De-register CPE, BS moves to another channel - Another scenario - Co-existence between two WRANs

- Comment 747 – Dan Lubar and Winston Caldwell will work together to provide the solution. Dan Lubar proposes to use method similar to the one in 802.11 Standard, to modify the sensing requirements based on regulatory classes.

2. Minutes of the Telecon on Friday, April 9th, 2009

1. Attendance

Apurva Mody

Gerald Chouinard

Winston Caldwell

Dan Lubar

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy – No one seemed to be unfamiliar with the IEEE Patent Policy

  - http://standards.ieee.org/board/pat/pat-slideset.pdf

- Charter and Agenda of the Cognitive Radio Capability Ad-Hoc Group –  https://mentor.ieee.org/802.22/dcn/09/22-09-0056-02-0000-section-9-cognitive-radio-capability-ad-hoc-group-charter-and-agenda.doc

- Comment Database - https://mentor.ieee.org/802.22/dcn/08/22-08-0146-32-0000-wran-draft-1-0-draft-comments-database.xls

- Cognitive Radio Capabilities – Missing Sections and Details – Comment #735  can be found at - https://mentor.ieee.org/802.22/dcn/09/22-09-0057-05-0000-section-9-cognitive-radio-capabilities-missing-sections-and-details.doc

- Proposed Resoutions for the Comments Related to Section 9 can be found at https://mentor.ieee.org/802.22/dcn/09/22-09-0066-00-0000-proposed-resolutions-to-comments-section9.doc

Minutes and Discussions

- Apurva Mody informed the group that Dan Lubar wished to involve others from P1900 to participate in the 802.22 Cognitive Radio Capability Ad-hoc.

- The general feeling was that this is an international standard and any one who wants to participate is welcome.

- Winston Caldwell pointed out that he would like to provide a complete end to end systems solution to the problem of Whitespace access using cognitive radio capability which included access to the geolocation database and the security mechanisms for the same.

- Apurva Mody said that if we can leverage the complementary synergies between 802.22 and P1900, then that would help a great deal. For example if P1900 can provide the interfaces to the geolocation / incumbent database, then 802.22 can design a compliant solution.

- Apurva Mody also suggested that certain basic security mechanisms on database access will be suggested in the Security subsection of the 802.22 text.

- Comments 805, 863, 904, 906, 907 and 908 were resolved. The proposed resolutions can be found in future versions of the Comment Database

- Comment 805 - Reject - since databse has priority over sensing. This information is superfluous. Broadcasters view is that the incumbent database, if it exists, should be prime and that sensing results could be neglected in this case.

- Comment 863 – Pending - Ask Victor for values that are present in Annex 1.

- Comment 904 – Superceded - Refer to the motion passed in Vancouver in March 2009 that allows terrestrial geolocation method

- Comment 906 – Counter - Read as "Operation by satellite based geolocation is mandatory where prescribed by the regulators" -Refer to the motion passed in Vancouver in March 2009 that allows terrestrial geolocation method

- Comment 907 – Reject - Refer to the motion failed in Vancouver in March 2009 that allows professional and secure manual entry

- Comment 908 – Reject - Motion PM 09-13 passed in Vancouver, March 2009 on terrestrial geolocation

3. Minutes of the Telecon on Friday, April 16th, 2009

1. Attendance

Apurva Mody

Gerald Chouinard

Winston Caldwell

Ranga Reddy

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy – No one seemed to be unfamiliar with the IEEE Patent Policy

  - http://standards.ieee.org/board/pat/pat-slideset.pdf

- Charter and Agenda of the Cognitive Radio Capability Ad-Hoc Group –  https://mentor.ieee.org/802.22/dcn/09/22-09-0056-02-0000-section-9-cognitive-radio-capability-ad-hoc-group-charter-and-agenda.doc

- Comment Database - https://mentor.ieee.org/802.22/dcn/08/22-08-0146-32-0000-wran-draft-1-0-draft-comments-database.xls

- Cognitive Radio Capabilities – Missing Sections and Details – Comment #735  can be found at - https://mentor.ieee.org/802.22/dcn/09/22-09-0057-05-0000-section-9-cognitive-radio-capabilities-missing-sections-and-details.doc

- Proposed Resoutions for the Comments Related to Section 9 can be found at https://mentor.ieee.org/802.22/dcn/09/22-09-0066-00-0000-proposed-resolutions-to-comments-section9.doc

Minutes and Discussions

- Comment 863 was re-visited – Ranga Reddy suggested that the values can be found in Doc 07-0491-Rev5. Ask Dave to see which MAC sections are impacted by that.

- Comment 768, 780, 782, 787, 788, 789 - Apurva needs to ask Dave to attend the following week’ s telecon

- Comment 815, 892 – Action Steve Shellhammer

- Comment 834 – Accept – Electronic Ballot – Confirmation in Montreal Session

- Comment 836 -  Ask Chris Clanton to clarify his comment, join the conference call the following week.

- Ranga – Table 280 needs to reflect different states of wireless 802.22.1 beacon sensing. MSF1, MSF2 etc.

- Following comments were resolved:  848, 862, 863

- Resolution to Comment 848 - This comment is superceded by Comment 847 which was approved in the electronic ballot (Clarify whether the table should contain three different indices to identify which parts of the PPDU need to be captured: MSF1, MSF2 and MSF3.

- Resolution to Comment 862 - "The 802.22.1 beacon was developed with the goal of allowing detection within a reasonable time window that would still allow 802.22 systems to provide good QoS.  It was possible to design the beacon so that the synch burst and index can be acquired within 5.1 ms, including the slippage due to the asynchronous capture of the burst.  When types of signals such as DTV and analog TV need to be sensed, proper sensing schemes should be used to allow  detection at the required sensing threshold within the same sensing window.  The 802.22.1 beacon can also provide additional information such as the location of the beacon (MSF1), the beacon signature (MSF2) and its authentication (MSF3) with correspondingly larger sensing periods.  See Table 2 of Annex "TG1"."

- Resolution to Comment 863 - See 07/0491/Rev5 - Annex 1 contains the values –