IEEE P802.22
Wireless RANs

| Regulatory Classes Example for Comment 747 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-04-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains exerpts from Annex I and J from the P802.11REVmb_D0.04 draft.  The regulatory class design that is used by 802.11 will be presented using this document to propose a possible solution to resolve comment 747 in IEEE 802.22-08/0146r33.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Annex I

(normative)

Regulatory classes

I.1 External regulatory references

The documents listed in Table I.1 specify current regulatory requirements for various frequency bands

and geographic areas at the time this standard was developed. They are provided for information only and

are subject to change or revision at any time.

Table I.1—Regulatory requirement list

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| Japan | Ministry of Internal Affairs and Communications (MIC) | MIC Equipment Ordinance (EO) for Regulating Radio Equipment Articles 7, 49.20, 49.21a | MIC |
| United States | Federal Communications Commission (FCC) | FCC CFR47 [B8], Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401–15.407, Section 90.210, Sections 90.1201–90.1217, Sections 90.1301–90.1337 | FCC |
| Europe | European Conference of Postal and Telecommunications (CEPT) Administrations and its Electronic Communications Committee (ECC). Also, European Radiocommunications Office, European Telecommunications Standards Institute (ETSI) | ECC DEC (04) 08, ETSI EN301 893, ETS 300-328 [B6] | CEPT |

aFrequency planning for licensed STAs in Japan is performed by the regulatory authority and the licensees, addressing

the coexistence among STAs operating with a variety of air propagation times and the coexistence between STAs

using 20 MHz channel spacing, STAs operating with 10 MHz channel spacing, and STAs operating with 5 MHz

channel spacing. Note also the CCA mechanism is preserved in licensed operation.

Table I.2—Emissions limits sets

| Emissions limits set | United States | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47 [B8], Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 interference-limited areasa | Reserved | Reserved | MIC EO Article 49.21 |
| 3 other interference areasa | Reserved | Reserved | MIC EO Article 49.21 |
| 4 Part 15 License Exempt bands | FCC CFR47 [B8], Section 15.247 | ETS 300-328 [B6] | MPHPT EO Articles 7, 49.20 |
| 5 public safety | FCC CFR47 [B8], Section 90.210 | Reserved | Reserved |
| 6 shared 3650 MHz band | FCC CFR47 [B8], Section 90.1323 | Reserved | Reserved |
| 7–255 | Reserved | Reserved | Reserved |

aThe deployment in Japan of licensed radios in dense urban areas (i.e., interference-limited areas) and other areas (i.e.,

other interference areas) are specified to allow more radios to operate in dense urban areas, while permitting what

can be less expensive radio designs to be used elsewhere.

Table I.3—Behavior limits sets

| Behavior limits set | United States | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47 [B8], Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 indoor only use | FCC CFR47 [B8], Section 15.407(e) | ETSI EN 301 389-1 | MIC EO Article 49.20 |
| 3 transmit power controla | Reserved | ETSI EN 301 389-1 | Reserved |
| 4 dynamic frequency selectiona | Reserved | ETSI EN 301 389-1 | Reserved |
| 5 IBSS restrictions | FCC CFR47 [B8], Section 90.1333 | Reserved | MIC EO Article 49.21 |
| 6 4 ms CSab | 4 ms, no exceptions | Reserved | MIC EO Articles 49.20, 49.21 |
| 7 licensed base STA | Reserved | Reserved | MIC EO Article 49.21 |
| 8 mobile STA | Reserved | Reserved | MIC EO Articles 49.20, 49.21 |
| 9 public safety | FCC CFR47 [B8], Section 90.1209 | Reserved | Reserved |
| 10 Part 15 license exempt bands | FCC CFR47 [B8], Section 15.247 | ETS 300-328 [B6] | MIC EO Article 49.20 |
| 11 registered STAc | FCC CFR47 [B8], Section 90.1331 | Reserved | Reserved |
| 12 dependent STAc | FCC CFR47 [B8], Section 90.1333 | Reserved | Reserved |
| 13 Reserved | Reserved | Reserved | Reserved |
| 14 Reserved | Reserved | Reserved | Reserved |
| 15 CCA-EDa | Reserved | Reserved | Reserved |
| 16–255 | Reserved | Reserved | Reserved |

aProcedures that may be used to improve sharing spectrum in addition to explicit regulatory requirements.

bThe Japanese 4 ms CS rule says no STA can transmit for more than 4 ms without carrier sensing, whether transmitting

fragments or frames, unless it is controlled by another STA

cThe deployment in US 3650 MHz band excludes operation near grandfathered satellite earth stations, federal government

radiolocation facilities, and Canadian and Mexican borders. If mutual agreement can be reached, the FCC will

permit operation within the declared exclusion zones. Operation near Canadian and Mexican borders is subject to

current and future agreements with Canada and Mexico. NOTE—The FCC regulation grandfathers the regulatory

status of 36 satellite earth stations and three radiolocation facilities in that they retain their primary status, and any

new ones will have secondary status.

I.2 Radio performance specifications

I.2.2 Transmit power levels

Transmit power levels described here are provided for information only and are subject to change or revision

at any time.

Table I.4—Transmit power level by regulatory domain

| Frequency band (GHz) | United States (Maximum output power with up to 6 dBi antenna gain) (mW) | United States (EIRP) | Europe (EIRP) |
| --- | --- | --- | --- |
| 2.400–2.4835 | 1 000 with antenna gain per FCC CFR47 [B8], Section 15.247(b)(4)(i)... | — | 100 mW |
| 3.650–3.700 registered STA | — | 1W/MHz | Reserved |
| 3.650–3.700 dependent mobile/ portable STA | — | 40 mW/MHz | Reserved |
| 5.15–5.25 | 40 (2.5 mW/MHz) | — | 200 mW |
| 5.25–5.35 | 200 (12.5 mW/MHz) | — | 200 mW |
| 5.470–5.725 | — | — | 1 W |
| 5.725–5.825 | 800(50 mW/MHz) | — | — |

I.2.3 Transmit spectrum mask

Transmit spectrum masks defined in regulation are described here for information only and are subject to

change or revision at any time.

I.2.4 CCA-ED threshold

For operation in the 3.65–3.7 GHz band, the CCA-ED thresholds shall be less than or equal to –72 dBm for

20 MHz channel widths, –75 dBm for 10 MHz channel widths, and –78 dBm for 5 MHz channel widths

(minimum sensitivity for BPSK, R=1/2 + 10 dB in Table 17-13).

Annex J

(normative)

Country information element and regulatory classes

J.1 Country information and regulatory classes

WLANs implemented in accordance with this standard and the specifications and definitions referenced in it

are subject to equipment certification and operating requirements established by regional and national

regulatory administrations. The specification establishes minimum technical requirements for

interoperability, based upon established regulations at the time this standard was issued. These regional and

national regulations are subject to revision or may be superseded. Regulatory requirements that do not affect

interoperability are not addressed in this standard. Implementers are referred to the regulatory sources in

Table I.1 for further information. Operation in countries within defined regulatory domains may be subject

to additional or alternative national regulations.

Table J.1—Regulatory classes in the United States

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Transmit power limit (EIRP) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 36, 40, 44, 48 | 40 | — | 1 | 1, 2 |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | — | 1 | 1 |
| 3 | 5 | 20 | 149, 153, 157, 161 | 800 | — | 1 | 1 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | — | 1 | 1 |
| 5 | 5 | 20 | 149, 153, 157, 161, 165 | 1000 | — | 4 | 10 |
| 6 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 25 | — | 5 | 9 |
| 7 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 500 | — | 5 | 9 |
| 8 | 4.89 | 10 | 11, 13, 15, 17, 19 | 50 | — | 5 | 9 |
| 9 | 4.89 | 10 | 11, 13, 15, 17, 19 | 1000 | — | 5 | 9 |
| 10 | 4.85 | 20 | 21, 25 | 100 | — | 5 | 9 |
| 11 | 4.85 | 20 | 21, 25 | 2000 | — | 5 | 9 |
| 12 | 2.407 | 25 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11 | 1000 | — | 4 | 10 |
| 13 | 3.000 | 20 | 133, 137 | — | 1 W/MHz | 6 | 3, 4, 6, 11, 15 |
| 13 | 3.000 | 20 | 133, 137 | — | 40 mW/ MHz | 6 | 3, 4, 5, 6, 12, 15 |
| 14 | 3.000 | 10 | 132, 134, 136, 138 | — | 1 W/MHz | 6 | 3, 4, 6, 11, 15 |
| 14 | 3.000 | 10 | 132, 134, 136, 138 | — | 40 mW/ MHz | 6 | 3, 4, 5, 6, 12, 15 |
| 15 | 3.0025 | 5 | 131, 132, 133, 134, 135, 136, 137, 138 | — | 1 W/MHz | 6 | 3, 4, 6, 11, 15 |
| 15 | 3.0025 | 5 | 131, 132, 133, 134, 135, 136, 137, 138 | — | 40 mW/ MHz | 6 | 3, 4, 5, 6, 12, 15 |
| 16–255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

J.2 Band-specific operating requirements

J.2.1 3650–3700 MHz in the United States

The registration authority is the FCC’s Universal Licensing System (ULS).

Regulations specify the following:

— Certified mobile and portable STAs do not need to be registered, but they “must” operate under the

control of an enabling STA (using DSE procedures).

— A registered STA “must” be a fixed STA.

— A registered STA “must” not operate as an enabling STA until the licensee has registered it as a

“base station” in ULS.

Enabling STAs and fixed STAs are registered STAs. Dependent non-AP STAs and dependent APs are

dependent STAs.

STAs shall use the following:

— CCA-ED

— CS/CCA

— TPC

— DFS

No STA shall use channel switch announcement.

STAs shall have the following elements set to TRUE:

— dot11LCIDSERequired

— dot11SpectrumManagementRequired

— dot11MultiDomainCapabilityEnabled

— dot11ExtendedChannelSwitchEnabled

STAs shall be capable of receiving all channels associated with regulatory classes 13–15.

STAs shall be capable of transmitting on all channels associated with regulatory class 15.

STAs shall set the value of dot11DSETransmitDivisor to 256 and the other dot11DSE timer values as shown

in Table J.4.

Table J.4—DSE timer limits

| Parameter | Seconds |
| --- | --- |
| dot11DSEEnablementTimeLimit | 32 |
| dot11DSEEnablementFailHoldTime | 512 |
| dot11DSERenewalTime | 60 |

References: