IEEE P802.22
Wireless RANs

| Remaining PHY Issues as in May 2009 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-4-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Zander Zhongding Lei | Institute for Infocomm Research, Singapore | 1 Fusionopolis Way #21-01 Connexis, Singapore 138632 | 65-6408-2436 | leizd@i2r.a-star.edu.sg |

Abstract

This document summarizes the remaining PHY related comments yet to be resolved up to May meeting, 09. A brief description of the status of each of the comments is included.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Remaining PHY Comments Summary

Remaining PHY Comments as in May 09

| Status | # of Comments | Comments ID | Zander’s Notes |
| --- | --- | --- | --- |
| Pending | 4 | 725, 682, 701, 731/734 |  |
| From MAC | 2 | 301, 236 |  |

- Updates on TR/T Comments

Pending

- ID 725 (TR) – Ivan/antenna pattern

  - It was decided in Sept. 08 that the reference transmit antenna pattern that is contained in the draft 1.2 figure 167 shall not be exceeded

  - The antenna pattern defined in section 8 corresponds to the antenna pattern of current DTV receiving antennas. The WISP operators and manufacturers commented in the Sep. 08 meetings that the antennas that will be used for WRAN transmission, not only for reception, would need to have better performance than the current DTV receive antennas. Otherwise, the HPA would not work properly because of the highly variable mismatch experienced as a function of frequency with low-cost DTV receive antennas (highly variable VSWR). Therefore, the antenna pattern needs to be re-visited.

In Vancouver Mar09, the antenna pattern as in slide 6 of #08-308r0 (removing the orange line) was agreed (motion PM09-05) and is pending for the approval in the electronic ballot.

- ID 682 (T) – Apurva/Table 254

Accept: Refer to Ivan Reede for action on clarifying the text before the table.
Find a way to show that this Table 254 is a one-dimensional array in its tabular representation.

(Editor added a darker line in the center.)

Action in Vancouver Mar09: Ivan to provide text for the May session.

- ID 701 (TR) – George/Ranging when channel switch

Sept. 08 meeting Pre-ranging before channel switch is not necessary and the normal periodic ranging should be sufficient in most cases.  Gwangzeen Ko will be proposing a proposal for channel switching procedure in MAC with an expedited way of doing ranging after channel switching.

Vancouver Mar09 Gwangzeen referred to DCN 08-310 addressing CID 372 which is still pending in the MAC group

- ID 731/734 (TR) – RF Mask

FCC R&O 08-260 stated 55dB rejection in the first adjacent channel as a guideline for RF mask (a special and more stringent mask for channels 36 and 38)

- Measured in PSD instead of the average power within 6 MHz channels

- 100 kHz resolution BW

Jan09 (09-023r1) Meeting the 55 dB level (PSD 55 dB w/ 100 kHz) is feasible since it requires a 256-tap FIR filter rather than an 8-tap filter and a 0.6 dB additional HPA back-off to meet the -55 dB for the first adjacent channel rather than only the -68.5 dB requirement for the second channel.

Vancouver Mar09 The FCC 55 dBr mask was included at the end of Doc. # 09-0023r2. It was found to be premature to put the FCC R&O 08-260 mask in the standard. Each administration may need a different mask.  The mask appearing in the standard should be a common minimum RF mask.

From MAC/TG2

- ID 301 Ivan/Lead cable radiation

The comment asks to consider the radiation from the lead cable between the CPE PA and the antenna that would result from a poor matching of the antenna with the cable attached to it. Such re-radiation can be higher than the back-lobe levels of the antenna and in such case, even if the antenna itself meets the reference pattern, the antenna installation will not meet the requirement, thus potentially creating more interference than expected.

- ID 236 Ivan/PHY modes

Vancouver Mar09 (TM09-16): PHY to identify no more than 16 mandatory (no optional) modulation/FEC modes.