IEEE P802.22
Wireless RANs

| Privacy Concerns |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061 | 603-885-2621, 404-819-0314 | apurva_mody@yahoo.com, apurva.mody@baesystems.com |
| Ranga Reddy | US Army |  |  | Ranga.Reddy@us.army.mil |

Abstract

This contribution seeks to amend the ranging/network entry procedure to allow for privacy of CPEs (Customer Premisis Equipment) in 802.22 Wireless Regional Area Networks (WRAN).

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Privacy Concerns

Apurva Mody (BAE Systems), Ranga Reddy (US Army)

CPE MAC address is sent in the clear during Ranging Request (RNG-REQ) and Ranging Response (RNG-RSP) process. This can allow malicious users to track an individual CPE in the network, which is a security concern. Also, some regulatory restrictions and laws in various countries may impose privacy requirements on the operation of TV whitespace devices.

This contribution proposes two approaches for solving this problem.

NOTE -- Concepts discussed here are some-what based upon contribution 09/112r0 or the latest revision. Acceptance of the solutions proposed is pending review and discussion of contribution 09/112r1 (or latest revision), but is not dependent on them.

Approach 1:

- Upon receiving CDMA Allocation IE, CPE randomly selects a “Temporary CPE MAC address”. This temporary MAC address is based upon the specification of a locally assigned/administered MAC address as defined in Section 9.2 of IEEE 802-201. In this case, the I/G and U/L bits of the MAC address are set to 0 and 1, respectively. The remaining 46 bits will be randomly generated by an appropriate PRNG. The CPE will then transmit the RNG-REQ with this temporary MAC address on the Initial Ranging CID.

- The BS receives this RNG-REQ, and transmits the RNG-RSP to the inteded CPE with the temporary MAC address received in the RNG-REQ. This RNG-RSP will contain the CPE’s Permanent Station ID (or Basic CID+Primary Mangement CID) if 09/112r0 or latest revision isn’t accepted).

- CPE and BS “hold” on to temporary MAC address until CPE completes authorization. Until this time, no other CPE attempting network entry can use the same temporary MAC address

- Rest of network entry process procedes as currently defined

Approach 2:

- Upon receiving CDMA Allocation IE, CPE transmits the RNG-REQ with its MAC address on the Initial Ranging CID.

- The BS receives the RNG-REQ, and transmits the RNG-RSP to the intended CPE using with the MAC address received in the RNG-REQ. The RNG-RSP will contain a “Temporary Station ID” (or a temporary Basic CID and temporary Management CID) selected from the pool of unused Multicast Station IDs (or Multicast Transport/Mangement CIDs)

- The Temporary Station ID (or Temporary Basic & Management CIDs) is then used by the BS and CPE to conduct the basic capabilities exchange

- The CPE transmits the SCM Authorization Request message using the Temporary Station ID (or Temporary Basic & Management CIDs)

- BS sends the SCM Authorization Reply to the CPE using the Temporary Station ID (or Temporary Basic & Management CIDs). Within the Authorization Reply is the CPE’s Permanent Station ID ( or Basic CID) encrypted by the CPE’s public key.

- Once Authorization is complete, the BS and CPE release used of the Temporary Station ID (or Temporary Basic & Management CIDs)

Issues:

The ultimate goal is to make sure that a malicious user can’t map the CPE MAC address to the Permanent Station ID or Basic CID. That is how privacy can be insured.

The first approach is much simpler than the second. However, during the SCM Authorization exchange the CPE’s MAC address may still be viewable. Transmission of the SCM Authorization request, requires that the CPE transmit its’ certificate to the BS. The certificate is bound to the CPE’s MAC address, and parsing the certificate may allow the user to view the MAC address (certificates are signed so the information within them can’t be easily forged). Approach 1 assigns the Permanent Station ID during the RNG-RSP, so by the time the CPE transmits the RNG-REQ, a malicious user could parse the Authorization Request and may be able to make the connection between the CPE’s MAC Address and Permanent Station ID. This approach is feasible if CPE MAC address doesn’t have to be included in the CPE certificate.

The second approach is slightly more complex than the first. The problem with this approach is that as the cell becomes more crowded, more and more Station IDs (or CIDs) get used up. This makes pool of available temporary Station IDs (or Basic+Management CIDs) shrink over time. However, this approach avoids the problem with the 1st approach, because the Permanent Station ID (or Basic CID+Management CID) is encrypted when the BS sends it to the CPE.

This contribution presents two approaches for CPE privacy during initial ranging and network entry only. The rest of the 802.22 draft must be reviewed to make sure no other messages/procedures reveal the some type of information.

Text Modification Proposal:

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Modify Section 6.17.2.6.3 and 6.18 to reflect this new procedure]

[If Approach 2 is selected, update section 6.17.3 “Basic Capability Negotiation” and 7.2.2 “CPE Authorization Process” to reflect the new procedure]

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

References:

- Hamiti, Shkumbin, “IEEE 802.16m System Description Document [Draft]”, IEEE 802.16m-08/003r9a, May 2009.

- “IEEE P802.22TM / DRAFTv2.0, Draft Standard for Wireless Regional Area Networks – Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Phyiscal Layer (PHY) specifications: Policies and Procedures for operation in the TV Bands”, IEEE P802.22/D2.08, May 2009

- Mody, Apurva and Reddy, Ranga, “New Connection Identifier Approach”, IEEE 802.22 working group contribution, 22-09-0112-01-0000-new-connection-identifier-approach.ppt, June 2009.