IEEE P802.22
Wireless RANs

| Privacy Concerns |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2018-01-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061 | 603-885-2621, 404-819-0314 | apurva_mody@yahoo.com, apurva.mody@baesystems.com |
| Ranga Reddy | N/A |  |  | Ranga.reddy@me.com |

Abstract

This contribution seeks to amend the ranging/network entry procedure to allow for privacy of CPEs (Customer Premisis Equipment) in 802.22 Wireless Regional Area Networks (WRAN).

R0: Original Version of this document

R1: Updates to this document to reflect comment resolution around Revision Project LB1

R2: Adding reference to IEEE P802c/D2

R3: Adding updates to text references to CPE Privacy in the rest of the Draft

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Privacy Concerns

Apurva Mody (BAE Systems), Ranga Reddy (N/A)

CPE MAC address is sent in the clear during Ranging Request (RNG-REQ) and Ranging Response (RNG-RSP) process. This can allow malicious users to track an individual CPE in the network, which is a security concern. Also, some regulatory restrictions and laws in various countries may impose privacy requirements on the operation of TV whitespace devices.

In R0 of this document two approaches for solving this problem were proposed. Approach 1 involved using a “Temporary MAC Address” selected by the CPE during the Ranging Procedure, and having the CPE sending  it’s actual MAC address during REG-REQ (after Authentication). Approach 2 involved using a “Temporary SID” selected by the BS and sent in the RNG-RSP, and then assigning a permanent SID during the REG-RSP.

During development/finalization of 802.22-2011, Approach 2 was selected. This approach is no longer the appropriate solution for ensuring CPE privacy. With the augmenting of the standard (i.e. IEEE 802.22b-2015), larger networks, with more devices will be deployed. This will lead to a dearth of unused SIDs to support CPE privacy. Therefore, it is recommended to go with approach 1.

In the intervening time, since development of 802.22-2011, the 802 WG has begun undertaking development of 802c draft, to address assignment of Local MAC Addresses. This contribution describes text changes to be applied to the Revision document to support CPE privacy using Local MAC Address primitives (as defined in 802c/D2.1).

NOTE -- Concepts discussed here are some-what based upon contribution 09/112r0 or the latest revision. Acceptance of the solutions proposed is pending review and discussion of contribution 09/112r1 (or latest revision), but is not dependent on them.

Approach 1:

- Upon receiving CDMA Allocation IE, CPE randomly selects a “Temporary CPE MAC address”. This temporary MAC address is based upon the specification of a locally assigned/administered MAC address as defined in Section 9.2 of IEEE 802-201. In this case, the I/G and U/L bits of the MAC address are set to 0 and 1, respectively. The remaining 46 bits will be randomly generated by an appropriate PRNG. The CPE will then transmit the RNG-REQ with this temporary MAC address on the Initial Ranging CID.

- The BS receives this RNG-REQ, and transmits the RNG-RSP to the inteded CPE with the temporary MAC address received in the RNG-REQ. This RNG-RSP will contain the CPE’s Permanent Station ID (or Basic CID+Primary Mangement CID) if 09/112r0 or latest revision isn’t accepted).

- CPE and BS “hold” on to temporary MAC address until CPE completes authorization. Until this time, no other CPE attempting network entry can use the same temporary MAC address

- Rest of network entry process procedes as currently defined

Approach 2:

- Upon receiving CDMA Allocation IE, CPE transmits the RNG-REQ with its MAC address on the Initial Ranging CID.

- The BS receives the RNG-REQ, and transmits the RNG-RSP to the intended CPE using with the MAC address received in the RNG-REQ. The RNG-RSP will contain a “Temporary Station ID” (or a temporary Basic CID and temporary Management CID) selected from the pool of unused Multicast Station IDs (or Multicast Transport/Mangement CIDs)

- The Temporary Station ID (or Temporary Basic & Management CIDs) is then used by the BS and CPE to conduct the basic capabilities exchange

- The CPE transmits the SCM Authorization Request message using the Temporary Station ID (or Temporary Basic & Management CIDs)

- BS sends the SCM Authorization Reply to the CPE using the Temporary Station ID (or Temporary Basic & Management CIDs). Within the Authorization Reply is the CPE’s Permanent Station ID ( or Basic CID) encrypted by the CPE’s public key.

- Once Authorization is complete, the BS and CPE release used of the Temporary Station ID (or Temporary Basic & Management CIDs)

Issues:

The ultimate goal is to make sure that a malicious user can’t map the CPE MAC address to the Permanent Station ID or Basic CID. That is how privacy can be insured.

The first approach is much simpler than the second. However, during the SCM Authorization exchange the CPE’s MAC address may still be viewable. Transmission of the SCM Authorization request, requires that the CPE transmit its’ certificate to the BS. The certificate is bound to the CPE’s MAC address, and parsing the certificate may allow the user to view the MAC address (certificates are signed so the information within them can’t be easily forged). Approach 1 assigns the Permanent Station ID during the RNG-RSP, so by the time the CPE transmits the RNG-REQ, a malicious user could parse the Authorization Request and may be able to make the connection between the CPE’s MAC Address and Permanent Station ID. This approach is feasible if CPE MAC address doesn’t have to be included in the CPE certificate.

The second approach is slightly more complex than the first. The problem with this approach is that as the cell becomes more crowded, more and more Station IDs (or CIDs) get used up. This makes pool of available temporary Station IDs (or Basic+Management CIDs) shrink over time. However, this approach avoids the problem with the 1st approach, because the Permanent Station ID (or Basic CID+Management CID) is encrypted when the BS sends it to the CPE.

This contribution presents two approaches for CPE privacy during initial ranging and network entry only. The rest of the 802.22 draft must be reviewed to make sure no other messages/procedures reveal the some type of information.

Text Modification Proposal:

Below are the text instructions required to implement the changes required to implement CPE Privacy using Local MAC Addresses. All changes are being made against the current revision draft

[Modify Section 7.7.7.3.4.12 as follows]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

7.7.7.3.4.12	Permanent Station IDMAC Address

This field specifies the permanent SID assigned to aMAC Address CPE. This IE is included if CPE Privacy (see Clause 8) during network entry is supported by the operator.

Table 61 – Permanent Station ID information element

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 15 | 6 | Permanent MAC AddressSID (Bit 0000 000b bbbb bbbb) | REG-REQ |

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[Replace text in Section 8.7 with the following]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

CPE SID and MAC Address are sent in the clear during the Ranging procedure. This can allow malicious users to track an individual CPE in the network, which is both a security concern and may (in some regulatory domains) violate laws regarding privacy of user information.

The following process details a procedure that can be used to ensure user privacy:

- Upon receiving the CDMA Allocation IE, CPE selects a temporary MAC Address is selected using the Administratively Assinged Identifier (AAI)-type Structured Local Address Plan (SLAP) as defined in Section 8.4.4.3 of IEEE Std. 802.

  - This entails setting the I/G, U/L, Y, Z bits of MAC Address to 0100

  - Remaing 44 bits can be assigned randomly

- The CPE then transmits this temporary MAC address in the RNG-REQ

- BS receives the RNG-REQ and selects a (permanent) SID for the CPE. It sends the permanent SID along with the temporary MAC address received in the RNG-REQ to the CPE in the RNG-CMD message.

- The CPE and BS proceed with the Authorization process to setup keying on the CPE.

- The CPE’s permanent (actual) MAC address is sent to the BS as an IE in the REG-REQ (see 7.7.7.3.4.12)

The CPE and BS “hold” onto the temporary MAC Address until the CPE completes the REG-REQ/RSP. Until then no other CPE can enter the network utilizing the same temporary MAC. Use of this procedure is optional and at the discretion of the operator.

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[Modify Table 44 (RNG-CMD) in the following manner]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

| Name | Element ID (1 byte) | Length (bits) | Description |
| --- | --- | --- | --- |
| Timing advance | 1 | 16 | Unsigned timing advance at the CPE, in number of TU, to compensate for the signal propagation delay on both, the downstream and the upstream RF paths, so that the upstream burst arrives at the BS within the tolerance specified in 403HError: Reference source not found. The timing advance shall be set to 0 when the CPE is co-located with the BS and shall increase as the CPE is located further away from the BS. |
| EIRP per subcarrier | 2 | 8 | Prange: EIRP per transmitted subcarrier (see 404HError: Reference source not found). Signed in units of 0.5 dB and ranging from –104 dBm (encoded 0x00) to +23.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme. |
| Offset frequency adjust | 3 | 16 | Signed number in Hertz |
| Ranging status | 4 | 3 | 000: Continue 001: Abort 010: Success 011: Re-range 100: Reauthenticate 101: Re-range and Re-register 110–111: Reserved |
| CPE MAC address | 6 | 48 | CPE MAC address that’s assigned universally by manufacturer., or locally assigned if CPE Privacy (see 8.7) is being used. |
| Station ID | 7 | 9 | Required parameter when RNG-CMD is sent in response to initial ranging. This may be temporary if the CPE Privacy is enabled (see 8.7). |
| Action Superframe Number | 8 | 8 | The superframe number (modulo 256) at which Channel Action shall be performed. |
| Action Frame Number | 9 | 4 | Integer value greater than or equal to zero that indicates the starting frame number, within the Action Superframe Number, at which the Channel Action shall be performed by all CPEs. |
| CDMA code | 10 | 1 | A unique code assigned to the CPE, to be used for dedicated ranging. Code is from the initial ranging codeset. |
| Transmission opportunity offset | 11 | 1 | A unique transmission opportunity assigned to the CPE, to be used for dedicated ranging in units of symbol duration. |

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[pg 246, lines 34-36, section 7.14.12.11]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

The BS shall respond with a REG-RSPCMD message. The REG-RSPCMD message shall include the Permanent Station IDMAC Address (see Table 61), if CPE Privacy (see 8.7) is enabled. Figure 42 shows the procedure that shall be followed by the CPE to initiate registration.

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[pg 732, lines 27-29, section 13.1.2.2.17.28, modify text as follows]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

13.1.2.2.17.28 wranIfRegCapabilityPermanentSidMacAddress

Permanent station IDMAC Address assigned to CPE, when CPE is entering the network under the CPE Privacy method (see 8.7). The format of this IE is defined in 7.7.7.3.4.12.

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[pg 894, lines 21-28, modify text as follows]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

wranIfRegCapabilityPermanentStationIdMacAdress	::= TEXTUAL-CONVENTION

STATUS	current

DESCRIPTION

"Permanent station IDMAC Address assigned to CPE, when CPE is

entering the network under the CPE Privacy method

(see 8.7). The format of this IE is defined in

7.7.7.3.4.12."

SYNTAX	MacAddressInteger32 (1..512)

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

[pg 1190, table 329, modify last three entries as follows]

[---------------------------------------------------------Start of Text Proposal---------------------------------------------------]

| If(CPE Privacy Enabled) { |  |  | CPE Privacy (see 8.7). |
| --- | --- | --- | --- |
| Permanent Station IDMAC Address | Integer | 48 bits | Permanent SIDMAC Address assigned to CPE if CPE Privacy (see 8.7) is enabled. |
| Padding | Bits | 7 bits | Set to 0000000. |

[---------------------------------------------------------End of Text Proposal---------------------------------------------------]

References:

- Hamiti, Shkumbin, “IEEE 802.16m System Description Document [Draft]”, IEEE 802.16m-08/003r9a, May 2009.

- “IEEE P802.22TM / DRAFTv2.0, Draft Standard for Wireless Regional Area Networks – Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Phyiscal Layer (PHY) specifications: Policies and Procedures for operation in the TV Bands”, IEEE P802.22/D2.08, May 2009

- Mody, Apurva and Reddy, Ranga, “New Connection Identifier Approach”, IEEE 802.22 working group contribution, 22-09-0112-01-0000-new-connection-identifier-approach.ppt, June 2009.

- “IEEE P802c/D2, Standar for Local and Metropolitan Area Networks: Overview and Architecture – Draft Amendment: Local Medium Access Control (MAC) Address Usage”, IEEE P802c/D2, January 2017