IEEE P802.22
Wireless RANs

| LB2 Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ahmed Sadek | Qualcomm | 5775 Morehouse Dr San Diego, CA 92121 | (858) 658-1362 | asadek@qualcomm.com |
| Steve Shellhammer | Qualcomm | 5775 Morehouse Drive San Diego, CA 92121 | (858) 658-1874 | Shellhammer@ieee.org |

Abstract

Comments on LB2 regarding Subclause 6.22.2.3.2.3.6.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This document provides recommended editing instructions to support a comment on LB2.

- Editing Instructions: At the beginning of the subclause Subclause 6.22.2.3.2.3.6  add the following text

If neighbouring WRANs on the same channel are sharing their SCWs, the random backoff mechanism described in section 6.16.2 is used in the contention process to send the CBP packet. The following algorithm reduces the number of contention frames required to decide the winner WRAN in the frame contention procedure.

Prioritized On-Demand Frame Contention (N, FC_SRC, FCN, SCWBackoffMax, SCWBackoffMax)

- FC_SRC[i] generates FCN[i]=Random(0,216-1) (as described in subclause 6.22.2.3.2.3.2) for 1≤i≤N

- Select SCWBackoffMax[i] as follow

SCWBackoffMax[i]=SCWBackoffMax216−1FCN[i]

- FC_SRC[i] generates a random backoff timer as follows

SCW_Backoff_Timer[i]=Random(i)×SCWSlotTime

Where Random(i) is an integer uniformly generated over the interval [0, SCWBackoffMax[i]]

- FC_SRC[k] that wins the access contention sends FC_REQ[k] including the FCN[k].

- All FC_SRC[i] 1≤i≤N, i≠k, try to decode FC_REQ[k].

- If FCN[k]>FCN[i], FC_SRC[i] marks this frame(s) as a loser frame(s).

- If FCN[k]<FCN[i], FC_SRC[i] continues contention for SCW to send FC_REQ as in step 3.

- FC_DST decodes all FC_Req messages received within a Frame Contention Window and generate its own FCN. The WRAN with minimum FCN is the winner WRAN.

Where in the above algorithm

N: the number of frame contention sources (FC_SRCs).

FC_SRC: array of the FC_SRCs IDs.

FCN: array of the frame contention number generated by the WRANs

SCWBackoffMax: array of the maximum backoff window for each WRAN

SCWBackoffMax: an integer denoting the maximum backoff window parameter as defined in subclause 11.1

- Editing Instructions: In Subclause 6.22.2.3.2.3.6  modify Figure 11 as follows,

- In the block in which the WRAN generates a frame contention number Nc, add

  - “Generate the SCW backoff window proportional to the contention number Nc”

- After the block in which the WRAN generates a frame contention number Nc, add a block with

  - “Receive FC_REQ from WRAN k with contention number Nk”

    - “If no proceed with sending FC_REQ message containing Nc”

    - “If yes, check if Nc<Nk”

      - “If yes, proceed with sending FC_REQ message containing Nc”

      - “If no, mark the corresponding frames as loser frames and stop contending for these frames in this contention round”